3GPP TSG-RAN WG5 Meeting #104                                                                  R5-244289

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Source:	MediaTek Inc.

Title:		Discussion paper on early implementable features and corrections

Agenda item:	6.4.7

# Document for: Discussion and Decision

- Introduction

In TS 36.331 Annex G and TS 38.331 Annex C, there are tables containing the feature and correction CR lists which may be implemented by a UE of an earlier release than which the CR was approved in. And RAN5 refers to these tables to define the release requirement of the relative PICS and test case applicability. According to the ASN.1 definition, early implementation is doable for UE as defined in TS 36.331 Annex F and TS 38.331 Annex D.

- Discussion

Currently there are some circumstances as below:

- Not every feature or correction CR in RAN2 has been clearly discussed whether it is early implementable in RAN2, e.g. contains "Implementation of this CR from Rel-N will not cause interoperability issues" on the CR coversheet and then adds them into the lists. Therefore, the list in TS 36.331 Annex G or TS 38.331 Annex C maybe incomplete. And it`s not feasible for RAN5 to request RAN2 to go back to clarify whether the features earlier than Rel-18 are early implementable.

- From UE perspective, some features are significant for security enhancement, e.g. UPIP (User plane integrity protection) is introduced in Rel-17 but UE may implement it since Rel-15 to enhance EN-DC security as EN-DC is based on Rel-15.

- Some features impact both NR and E-UTRA part, but UE may have different NR and E-UTRAN AS release. For example, RACS (UE Radio Capability Signaling Optimization) and RRC UL segmentation are Rel-16 features in both NR and E-UTRA core specification, and network may query nr, eutra-nr and/or eutra capabilities from gNB or eNB side. If UE reports NR accessStratumRelease as Rel-16 and E-UTRA accessStratumRelease as Rel-15, and the UE supports RACS or UL segmentation in NR, that means it also needs to support these features in E-UTRA to ensure network having the same UE capabilities.

Considering above situations, in this discussion paper, it is proposed to allow features and corrections CRs, which are not in TS 36.331 Annex G nor TS 38.331 Annex C, being early implementable in RAN5 based on meeting CRs [1][2][3][4].

3.	Proposal

Proposal:	RAN5 to accept the handling of early implementable features and corrections based on meeting CR process.

4.	References

[1]	R5-244297: Updates to the PICS implementation note, MediaTek Inc.

[2]	R5-244280: Updates to the applicability of UPIP TC, MediaTek Inc.

[3]	R5-244298: Updates to the applicability of RACS TC, MediaTek Inc.

[4]	R5-244299: Updates to the applicability of RACS TC, MediaTek Inc.