3GPP TSG-RAN5 Meeting #104	R5-244333

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Title:	Discussion on minimum test time simulation results for NB-IoT NTN

Source:	MediaTek Inc

Agenda item:	5.5.1.13

Document for:	Discussion and endorsement

1.	Introduction

As discussed in prerequisite discussion R5-242427 [1] and R5-243890 [2].

This paper is aiming to resubmit the simulation results and additionally provide SNR vs Tput curve for NB-IoT NTN Demod TC 8.3.1.1.1 and 8.3.1.1.2 subtest 1.

2. 	Discussion

The simulation assumption and updated simulation steps [3] are shown below,

1.	Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

2.	The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

3. 	If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold (for example, +/- 3.5%) of the target throughput value.

4.	To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

5.	That SNR value is the added SNR uncertainty due to finite test time.

30 random seeds are used in this simulation, Fig.1 and Fig.2 shows the results. Several pair of Tput corridor were added in these Fig. Red, green, purple and grey line represented 2%, 3.5%, 5% and 10% limit around 70% max Tput.

2.1	Simulation results of SNR vs Tput and Requested SNR

The time vs tput simulation should be run under the SNR of 70% max Tput. The SNR vs Tput curve was done to find the requested SNR.

Fig.1 SNR vs Tput curve for 8.3.1.1.1 test 1

Fig.2 SNR vs Tput curve for 8.3.1.1.1 test 2

Fig.3 SNR vs Tput curve for 8.3.1.1.2 test 1

The requested SNR of the test cases is -9.5 dB, -15.1 dB, -10 dB respectively.

2.2	Simulation results of Time vs Tput

The Time vs Tput curve is simulated under the requested SNR.

Fig.4 Time vs Tput simulation results for 8.3.1.1.1 test 1

Fig.5 Time vs Tput simulation results for 8.3.1.1.1 test 2

Fig.6 Time vs Tput simulation results for 8.3.1.1.2 test 2

Then the convergence time within different corridor is summarised in Table.1.

Table.1 Convergence time for test cases

| Convergence time(s) | Tput 70%: +/-2% | Tput 70%: +/-3.5% | Tput 70%: +/-5% | Tput 70%: +/-10% |
| --- | --- | --- | --- | --- |
| 8.3.1.1.1 Case1 TDLC5-200 NGSO | N/A | N/A | 107 | 31 |
| 8.3.1.1.1 Case2 TDLA100-10 NGSO | N/A | N/A | 388 | 200 |
| 8.3.1.1.2 Case1 EPA5 NGSO | N/A | 99 | 61 | 24 |

The simulation is run under 0.113ppm doppler shift (0.013 doppler shift for NGSO according to the ephemeris in 36.508 and 0.1 ppm max residual doppler according to RAN4 requirement)

The Number of NPDSCH sample is calculated based on the configuration of NPDSCH,

Table.2 Number of NPDSCH sample for test cases under NGSO condition

|  | Tput 70%: +/-2% | Tput 70%: +/-3.5% | Tput 70%: +/-5% | Tput 70%: +/-10% |
| --- | --- | --- | --- | --- |
| 8.3.1.1.1 Case1 TDLC5-200 | N/A | N/A | 669 (107000/32/4) | 243 |
| 8.3.1.1.1 Case2 TDLA100-10 | N/A | N/A | 758 (388000/128/4) | 391 |
| 8.3.1.1.2 Case1 EPA5 | N/A | 774 (99000/32/4) | 477 | 188 |

Considering the large test time in current NTN IODT, it’s better to use smaller Number of NPDSCH sample and add extra SNR uncertainty for these test cases due to finite test time.

Proposal 1: Use 669 as the NPDSCH sample for TC 8.3.1.1.1 Test 1 and 758 for Test 2 in 36.521-4.

SNR uncertainty shall be added since the Tput corridor is expanded. That means the performance can’t be convergent in a reasonable test time with original corridor.

Table.3 SNR uncertainty

| SNR uncertainty (dB) | Tput 70%: +/-3.5% | Tput 70%: +/-5% | Tput 70%: +/-10% |
| --- | --- | --- | --- |
| 8.3.1.1.1 Case1 TDLC5-200 | 0.05 | 0.15 | 0.35 |
| 8.3.1.1.1 Case2 TDLA100-10 | 0.1 | 0.2 | 0.4 |
| 8.3.1.1.2 Case1 EPA5 | 0.15 | 0.35 | 0.95 |

Proposal 2: Add SNR uncertainty due to finite test time.

3. 	Conclusion

This paper is aiming to provide the simulation results for TC 8.3.1.1.1 in 36.521-4.

The following observations and proposals are made:

Proposal 1: Use 669 as the NPDSCH sample for TC 8.3.1.1.1 Test 1 and 758 for Test 2 in 36.521-4.

Proposal 2: Add SNR uncertainty due to finite test time.

The associated CRs is R5-244335, R5-244336, R5-244337.

4. 	References

- R5-242427 Discussion on minimum test time simulation methods for NB-IoT NTN, MediaTek, RAN5 #103

- R5-243890 Discussion on minimum test time simulation results for 8.3.1.1.1, MediaTek, RAN5 #103

- R5-242588 Update to the simulation method of minimum test time, MediaTek, RAN5 #103