3GPP TSG-RAN5 Meeting #104	R5-244370

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4532 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TC 12.1.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The closing bracket is lost and the indentation is incorrect in information element of RRC message. There are some editorial errors in the TC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The errors have been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The errors will exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12.1.4.2	PC5-only operation / Sidelink Reconfiguration via PC5 RRC / SL DRB management / Peer UE side

12.1.4.2.1	Test Purpose (TP)

(1)

with { UE having established PC5 RRC connection with peer UE}

ensure that {

when { UE receives an RRCReconfigurationSidelink message that can comply to add a unicast SL-DRB}

then { UE applies the parameters in RRCReconfigurationSidelink and sends an RRCReconfigurationCompleteSidelink message to peer UE}

}

(2)

with { UE having established PC5 RRC connection with peer UE }

ensure that {

when { UE receives an RRCReconfigurationSidelink message that can comply to modify a unicast SL-DRB}

then {UE applies the parameters in RRCReconfigurationSidelink and sends an RRCReconfigurationCompleteSidelink message to peer UE }

}

(3)

with { UE having established PC5 RRC connection with peer UE }

ensure that {

when { UE receives an RRCReconfigurationSidelink message that can comply to release a unicast SL-DRB}

then { UE applies the parameters in RRCReconfigurationSidelink and sends an RRCReconfigurationCompleteSidelink message to peer UE}

}

12.1.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 , subclause 5.8.9.1a.1.2, 5.8.9.1a.2.1, 5.8.9.1a.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.8.9.1a.1.2]

For each sidelink DRB, whose sidelink DRB release conditions are met as in sub-clause 5.8.9.1a.1.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the sidelink DRB release was triggered after the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	release the PDCP entity for NR sidelink communication associated with the sidelink DRB;

2>	if SDAP entity for NR sidelink communication associated with this sidelink DRB is configured:

3>	indicate the release of the sidelink DRB to the SDAP entity associated with this sidelink DRB (TS 37.324 [24], clause 5.3.3);

2>	release SDAP entities for NR sidelink communication, if any, that have no associated sidelink DRB as specified in TS 37.324 [24] clause 5.1.2;

1>	for groupcast and broadcast; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB release was triggered due to the configuration received within the sl-ConfigDedicatedNR:

2>	for each sl-RLC-BearerConfigIndex included in the received sl-RLC-BearerToReleaseList that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel for NR sidelink communication, associated with the sl-RLC-BearerConfigIndex.

1>	for unicast, if the sidelink DRB release was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB release was triggered due to the configuration received within the SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	release the RLC entity and the corresponding logical channel for NR sidelink communication associated with the sidelink DRB;

2>	perform the sidelink UE information procedure in sub-clause 5.8.3 for unicast if needed.

1>	if the sidelink radio link failure is detected for a specific destination:

2>	release the PDCP entity, RLC entity and the logical channel of the sidelink DRB for the specific destination.

[TS 38.331, subclause 5.8.9.1a.2.1]

For NR sidelink communication, a sidelink DRB addition is initiated only in the following cases:

1>	if any sidelink QoS flow is (re)configured by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR and is to be mapped to one sidelink DRB, which is not established; or

1>	if any sidelink QoS flow is (re)configured by RRCReconfigurationSidelink and is to be mapped to a sidelink DRB, which is not established;

For NR sidelink communication, a sidelink DRB modification is initiated only in the following cases:

1>	if any of the sidelink DRB related parameters is changed by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or RRCReconfigurationSidelink for one sidelink DRB, which is established;

[TS 38.331, subclause 5.8.9.1a.2.2]

For the sidelink DRB, whose sidelink DRB addition conditions are met as in sub-clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the sidelink DRB addition was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB addition was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	if an SDAP entity for NR sidelink communication associated with the destination and the cast type of the sidelink DRB does not exist:

3>	establish an SDAP entity for NR sidelink communication as specified in TS 37.324 [24] clause 5.1.1;

2>	(re)configure the SDAP entity in accordance with the sl-SDAP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-SDAP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, associated with the sidelink DRB;

2>	establish a PDCP entity for NR sidelink communication and configure it in accordance with the sl-PDCP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-PDCP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, associated with the sidelink DRB;

2>	establish a RLC entity for NR sidelink communication and configure it in accordance with the sl-RLC-ConfigPC5 received in the RRCReconfigurationSidelink or sl-RLC-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, associated with sidelink DRB;

2>	if this procedure was due to the reception of a RRCReconfigurationSidelink message:

3>	configure the MAC entity with a logical channel in accordance with the sl-MAC-LogicalChannelConfigPC5 received in the RRCReconfigurationSidelink associated with the sidelink DRB, and perform the sidelink UE information procedure in sub-clause 5.8.3 for unicast if need;

2>	else:

3>	configure the MAC entity with a logical channel associated with the sidelink DRB, by assigning a new logical channel identity, in accordance with the sl-MAC-LogicalChannelConfig received in the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR.

NOTE 1:	When a sidelink DRB addition is due to the configuration by RRCReconfigurationSidelink, it is up to UE implementation to select the sidelink DRB configuration as necessary transmitting parameters for the sidelink DRB, from the received sl-ConfigDedicatedNR (if in RRC_CONNECTED), SIB12 (if in RRC_IDLE/INACTIVE), SidelinkPreconfigNR (if out of coverage) with the same RLC mode as the one configured in RRCReconfigurationSidelink.

For the sidelink DRB, whose sidelink DRB modification conditions are met as in sub-clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the sidelink DRB modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB modification was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	reconfigure the SDAP entity of the sidelink DRB, in accordance with the sl-SDAP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-SDAP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

2>	reconfigure the PDCP entity of the sidelink DRB, in accordance with the sl-PDCP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-PDCP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

2>	reconfigure the RLC entity of the sidelink DRB, in accordance with the sl-RLC-ConfigPC5 received in the RRCReconfigurationSidelink or sl-RLC-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

2>	reconfigure the logical channel of the sidelink DRB, in accordance with the sl-MAC-LogicalChannelConfigPC5 received in the RRCReconfigurationSidelink or sl-MAC-LogicalChannelConfig received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included.

12.1.4.2.3	Test description

12.1.4.2.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

-	NR-SS-UE 1 is synchronised on GNSS.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 0-A as defined in TS 38.508-1 [4].

12.1.4.2.3.2	Test procedure sequence

Table 12.1.4.2.3.2-1: Main behaviour

|  | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). | - | - | - | - |
| 3 | The NR-SS-UE1 sends a DIRECT LINK ESTABLISHMENT REQUEST message. | <-- | PC5-S: DIRECT LINK ESTABLISHMENT REQUEST |  |  |
| 4 | The UE sends a DIRECT LINK SECURITY MODE COMMAND message. | --> | PC5-S: DIRECT LINK SECURITY MODE COMMAND | - | - |
| 5 | The NR-SS-UE1 sends a DIRECT LINK SECURITY MODE COMPLETE message. | <-- | PC5-S: DIRECT LINK SECURITY MODE COMPLETE | - | - |
| 6 | The UE sends a DIRECT LINK ESTABLISHMENT ACCEPT message. | --> | PC5-S: DIRECT LINK ESTABLISHMENT ACCEPT | - | - |
| 7 | The NR-SS-UE1 sends an RRCReconfigurationSidelink message to establish a unicast mode SL-DRB. | <-- | PC5-RRC: RRCReconfigurationSidelink | - | - |
| 8 | Check: Does the UE send an RRCReconfigurationCompleteSidelink message? | --> | PC5-RRC: RRCReconfigurationCompleteSidelink | 1 | P |
| 9 | The NR-SS-UE1 sends an RRCReconfigurationSidelink message to indicate modification of unicast mode SL-DRB. | <-- | PC5-RRC: RRCReconfigurationSidelink | - | - |
| 10 | Check: Does the UE send an RRCReconfigurationCompleteSidelink message? | --> | PC5-RRC: RRCReconfigurationCompleteSidelink | 2 | P |
| 11 | The NR-SS-UE1 sends an RRCReconfigurationSidelink message to indicate release of unicast mode SL DRB. | <-- | PC5-RRC: RRCReconfigurationSidelink | - | - |
| 12 | Check: Does the UE send an RRCReconfigurationCompleteSidelink message? | --> | PC5-RRC: RRCReconfigurationCompleteSidelink | 3 | P |

12.1.4.2.3.3	Specific message contents

Table 12.1.4.2.3.3-1: RRCReconfigurationSidelink (step 7, Table 12.1.4.2.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 Conditions RX and SL_DRB

Table 12.1.4.2.3.3-2: RRCReconfigurationSidelink (step 9, Table 12.1.4.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 Conditions RX and SL_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { | 1 entry |  |  |  |
| SLRB-Config-r16[1] SEQUENCE { |  | entry 1 |  |  |
| sl-PDCP-ConfigPC5-r16 SEQUENCE { |  |  |  |  |
| sl-PDCP-SN-Size-r16 | len12bits |  |  |  |
| sl-OutOfOrderDelivery-r16 | true |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.1.4.2.3.3-3: RRCReconfigurationSidelink (step 11, Table 12.1.4.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 Condition RX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| slrb-ConfigToReleaseList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-PC5-ConfigIndex-r16 { | 1 entry |  |  |
| SLRB-PC5-ConfigIndex-r16 [1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |