3GPP TSG-RAN5 Meeting #104	R5-244404

# Maastricht, Netherlands, 19 – 23 Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2877 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PC1 operation for bands n25, n40, n66 and n77 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of PC1 operation for bands n25, n40, n66 and n77 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PC1 operation for bands n25, n40, n66 and n77 in clauses: - 6.2.1.3 Minimum conformance requirements - 6.2.1.5 Test requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 operation for bands n25, n40, n66 and n77 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n3 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n5 |  |  |  |  | 23 | ± 2 |
| n7 | 316 | +2/-3 |  |  | 23 | ± 23 |
| n8 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n13 |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  | 23 | ± 2 |
| n20 |  |  |  |  | 23 | ± 23 |
| n24 |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 23 | ± 23 |
| n26 |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  | 23 | ± 2 |
| n34 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n38 |  |  |  |  | 23 | ± 2 |
| n39 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n40 | 316 | +2/-3 | 26 | +2/-3 | 23 | ± 2 |
| n41 | 316 | +2/-3 | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  | 23 | ±2 |
| n66 | 316 | +2/-3 |  |  | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  | 23 | ± 2 |
| n77 | 316 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | + 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n85 |  |  |  |  | 23 | ±23 |
| n86 |  |  |  |  | 23 | ± 2 |
| n91 |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  | 23 | ±2 |
| n99 |  |  |  |  | 23 | +2/-33 |
| n100 | 316 | +2/-3 |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  | 23 | ±2 |
| n106 |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: FFS NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-2 for Power Class 2 as defined in TS 38.306.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n85 |  |  |  |  | 23 | ±(23+TT) |
| n91 |  |  |  |  | 23 | ±(23, 5+TT) |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n106 |  |  |  |  | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n8 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n75 | 31 | +2+TT/-3-TT |  |  |  |  |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n255 | 31 | +2+TT/-3-TT |  |  |  |  |
| n405 | 31 | +2+TT/-3-TT |  |  |  |  |
| n415 | 31 | +2+TT/-3-TT |  |  |  |  |
| n665 | 31 | +2+TT/-3-TT |  |  |  |  |
| n775 | 31 | +2+TT/-3-TT |  |  |  |  |
| n785 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3. NOTE 5: Generally, PC1 UEis not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

-------- End of changes --------