3GPP TSG-RAN5 Meeting #104	R5-244414

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4539 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.5.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With current Prose, if pc_noOf_PDUsSameConnection = 0 no PDU session will be established until Step 29. Basically, no DRB is established. This causes problems while establishing the PDU session as a result of Step 29. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the EXCEPTION after Step 17A, and void the Step17Aa1. This will be replaced with Steps 17BA – 17BCa2 which refer to Steps 18 – 19Ba2 of Table 4.5.2.2-2 of the generic procedure in TS38.508-1. Similar issue was fixed in RAN5#103 for testcase 6.3.1.10 (R5-243587). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s240433 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 9.1.5.1.4	Initial registration / 5GS services / MICO mode / TAI list handling

9.1.5.1.4.1	Test Purpose (TP)

(1)

with { The UE is in 5GMM-DEREGISTERED state and is switched off }

ensure that {

when { the UE supports MICO mode and requests the use of MICO mode }

then { the UE includes the MICO indication IE in the REGISTRATION REQUEST message }

}

(2)

with { The UE has received REGISTRATION ACCEPT message }

ensure that {

when { the REGISTRATION ACCEPT message included MICO indication IE indicating “all PLMN registration area allocated” }

then { the UE treats all TAIs in the current PLMN as a registration area and deletes its old TAI list }

}

(3)

with { The UE detecting a better NG cell in same PLMN }

ensure that {

when { the UE treats all TAIs in the current PLMN as a registration area and has deleted its old TAI list as a result of REGISTRATION ACCEPT message included MICO indication IE indicating “all PLMN registration area allocated” }

then { the UE does not perform the REGISTRATION procedure for mobility }

}

(4)

with { MICO mode activated in the UE }

ensure that {

when { the UE needs to transfer mobile originated signalling }

then { the UE deactivates MICO mode and transmits a SERVICE REQUEST message with IE Service type set to “signalling” }

}

9.1.5.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.304, clause 4.1 and TS 24.501, clause 5.5.1.2.2, clause 5.5.1.2.4 and clause 5.3.6. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.2]

…

If the UE supports MICO mode and requests the use of MICO mode, then the UE shall include the MICO indication IE in the REGISTRATION REQUEST message.

…

[TS 24.501, clause 5.5.1.2.4]

…

The AMF shall include the MICO indication IE in the REGISTRATION ACCEPT message only if the MICO indication IE was included in the REGISTRATION REQUEST message, the AMF supports and accepts the use of MICO mode. If the AMF supports and accepts the use of MICO mode, the AMF may indicate "all PLMN registration area allocated" in the MICO indication IE in the REGISTRATION ACCEPT message. If "all PLMN registration area allocated" is indicated in the MICO indication IE, the AMF shall not assign and include the TAI list in the REGISTRATION ACCEPT message. If the REGISTRATION ACCEPT message included an MICO indication IE indicating "all PLMN registration area allocated", the UE shall treat all TAIs in the current PLMN as a registration area and delete its old TAI list.

…

[TS 24.501, clause 5.3.6]

…

The UE may deactivate MICO mode and activate the AS layer at any time. Upon deactivating MICO mode, the UE may initiate 5GMM procedures (e.g. for the transfer of mobile originated signalling or user data).

…

[TS 38.304, clause 4.1]

…

When the UE is in RRC_IDLE state, upper layers may deactivate AS layer when MICO mode is activated as specified in TS 24.501 [14]. When MICO mode is activated, the AS configuration (e.g. priorities provided by dedicated signalling) is kept and all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while MICO mode is activated it is up to the UE implementation whether it performs the corresponding action immediately or the latest when MICO mode is deactivated. When MICO mode is deactivated, the UE shall perform all idle mode tasks.

…

9.1.5.1.4.3	Test description

9.1.5.1.4.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A and NGC Cell C are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [4].

UE:

None.

Preamble:

-	The UE is in state Switched OFF (State 0N-B) as per TS 38.508-1 [4] Table 4.4A.2-0.

9.1.5.1.4.3.2	Test procedure sequence

Table 9.1.5.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS configures: - NGC Cell A as the "Non-suitable “Off” cell. - NGC Cell C as the "Non-suitable "Off" cell". | - | - | - | - |
| 1 | The UE is switched ON. | - | - | - | - |
| 2 | The user requests enabling of MICO mode by MMI or AT command. | - | - | - | - |
| 3 | The SS configures: - NGC Cell A as the "Serving cell". - NGC Cell C as the "Non-suitable "Off" cell". | - | - | - | - |
| 4-6 | The UE establishes an RRC connection by executing steps 2–4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 7 | Check: Does the UE transmit a REGISTRATION REQUEST message including IE MICO indication? | --> | REGISTRATION REQUEST | 1 | P |
| 8-16 | Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 17 | The SS transmits a REGISTRATION ACCEPT message that includes IE MICO indication. | <-- | REGISTRATION ACCEPT | - | - |
| 17A | The UE transmits a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 17Aa1 | Void | - | - | - | - |
| 17B | Void | - | - | - | - |
| 17BA – 17BCa2 | Steps 18 – 19Ba2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 18 | The SS releases the RRC Connection. | - | - | - | - |
| 19 | The SS configures: - NGC Cell A as the "Non-suitable “Off” cell". - NGC Cell C as the "Serving cell". | - | - | - | - |
| 20 | Check: Does the UE transmit a RRCSetupRequest on NGC Cell C? This is checked for 60s? | --> | NR RRC: RRCSetupRequest | 2, 3 | F |
| 21 | Void |  |  |  |  |
| - | The following messages are to be observed on NGC Cell C unless explicitly stated otherwise. | - | - | - | - |
| 21A-28 | Void | - | - | - | - |
| 29 | Cause the UE to request connectivity to an PDU session (Note 1) | - | - | - | - |
| 30-31 | Steps 2-3 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. | - | - | - | - |
| 32 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message with service type IE set to “signalling”. | --> | SERVICE REQUEST | 4 | P |
| 33-36 | Steps 5-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. | - | - | - | - |
| 37 | The PDU session establishment procedure as specified in TS 38.508-1 [4] subclause 4.5A.2 take place. | - | - | - | - |
| Note 1: The request of connectivity to an additional PDU session may be performed by MMI or AT command +CGACT. |  |  |  |  |  |

9.1.5.1.4.3.3	Specific message contents

Table 9.1.5.1.4.3.3-1: REGISTRATION REQUEST (step 6, Table 9.1.5.1.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |
| 5GS registration type value | ‘001’B | Initial registration | INITIAL |
| MICO indication | ‘0000’B |  |  |

Table 9.1.5.1.4.3.3-2: REGISTRATION ACCEPT (step 17, Table 9.1.5.1.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MICO Indication | ‘0001’B | All PLMN registration area allocated |  |
| TAI list | Not present |  |  |

Table 9.1.5.1.4.3.3-3:  Void

Table 9.1.5.1.4.3.3-4: SERVICE REQUEST (step 32, Table 9.1.5.1.4.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Service type | ‘0000’B | signalling |  |