3GPP TSG-RAN5 Meeting # 104	R5-244422

# Maastricht, Netherlands,  19th August 2024 – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3501 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR NTN: Addition of NGSO Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current NTN test model is lacking a support for NGSO scenario simulation. This blocks implementation of NGSO environment for the relevant test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the NTN description with the missing information relevant for NGSO test model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR NTN tests cannot be executed in NGSO environment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1, 7.5.2.2, 7.5.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.5	Non-Terrestrial Network

### 7.5.1	NTN cell configuration

An NR cell to be used in a Non-Terrestrial Network (NTN) test environment is an NR cell configured with NTN-specific configuration parameters. It is called an NR NTN cell.

In particular an NR NTN cell is configured to operate on an NTN-specific NR band and is broadcasting a system information block combination, defined in TS 38.508-1 [5] clause 4.4.3.1.2, that contains SIB19.

Typically, an NTN test case can be executed under one of the following two sub-NTN test environments:

-	the Geosynchronous Orbit (GSO) scenario, or

-	the Non-Geosynchronous Orbit (NGSO) scenario.

7.5.2	NTN timing considerations

#### 7.5.2.1	General

DL and UL are frame aligned at the UL time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are defined using a Common Timing Advance (Common TA) and two offsets: Koffset and Kmac, with Koffset ≥ TTA.

The various NTN timing relationships are illustrated in Figure 7.5.2.1-1.

Figure 7.5.2.1-1: NTN timing relationship parameters

#### 7.5.2.2	NTN timing advance

The Timing Advance (TA) formula, specified in TS 38.211 [19] clause 4.3.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Tc: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common = NTA,adjcommon * Tc: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific = NTA,adjUE * Tc: the TA corresponding to the service link delay.

The NTN test environment in TS 38.508-1 [5] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore, the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7.5.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 38.213 [21] clause 4.2) |
| TTA,legacy |  | InitialValue integer (0..3846) | NTA = InitialValue * 16∙642μ |
| TTA,common | NTA,adjcommon | NtnCommonTA integer (0..66485757) | See below. |
| TTA,UE-specific | NTA,adjUE | NtnUeSpecificTA integer (0..66485757) | NTA,adjUE= 8* NtnUeSpecificTA See below. |
| NOTE: The time unit TC is defined in TS 38.211 [19] clause 4.1. |  |  |  |

From the TA-Info-r17 specified in TS 38.331 [16], only the value of ta-Common-r17 is defined in TS 38.508-1 [5] SIB19 for the GSO and NGSO scenarios. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of NTA,adjcommon according to the formula given in TS 38.213 [21] clause 4.2.

NOTE:	It is assumed that the SS uses the calculation: NTA,adjcommon = 8 * NtnCommonTA.

NTA,adjUE is calculated using the NTN serving satellite static position specified in TS 38.508-1 [5] SIB19's ephemeris data and the UE static position specified in TS 38.508-1 [5] clause 4.5C1. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the NTN serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

t=2*d/c

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geometry d in meters is derived by:

d=∆x+∆y+∆z

with

∆x=(xsat-xUE)2

∆y= (ysat-yUE)2

∆z=(zsat-zUE)2

and

xsat, ysat, zsat being serving satellite coordinates, in m

xUE, yUE, zUE being UE coordinates, in m

To use above formulas the positioning data provided in TS 38.508-1 [3] is first converted to the coordinates accordingly.

Example of the calculation of ∆x:

given UE latitude 25.08439333 and positionX-r17 of -16976014:

xsat= -16976014* 1.3 m = -22068818.2 m

xUE= -3025296.94 m

∆x=(-22068818.2 m-(-3025296.94 m))2=(-19043521.26 m)2= 362655701980071.94 m2

Following the above calculations, the final value t for a GSO scenario results in 0.24338972709427653 sec that yields NtnUeSpecificTA to have a value of 59771543.981895022, that is rounded to 59771544.

For an NGSO scenario, the calculated final value of t results in 0.004062662259880647 sec and a corresponding NtnUeSpecificTA has a value of 7987519.015906143. The last one is rounded to 7987519.

#### 7.5.2.3	NTN scheduling offsets

The NTN timing parameters listed in Table 7.5.2.3-1 come into play when scheduling data in an NTN cell.

Table 7.5.2.3-1: NTN timing parameters in an NTN cell

| NTN timing parameter | Definition | Reference |
| --- | --- | --- |
| UE-gNB RTT | TTA + Kmac | TS 38.321 [13] clause 3.1 |
| Kmac | k-Mac or 0 | TS 38.213 [21] clause 8.2 |
| Koffset | cellSpecificKoffset - Differential Koffset | TS 38.213 [21] clause 4.2 |

TTCN configures the NTN offsets Koffset and Kmac in the SS as part of the NTN cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL.

#### 7.5.2.4	RRC connection release sequence

The sequence specified in clause 7.3.5.2 applies, with the following differences: T is set by the value of UE-gNB RTT ms in advance of RRCRelease.

7.5.3	NTN frequency considerations

#### 7.5.3.1	General

According to TS 38.508-1 [5] clause 6.3.4.2 for the NGSO scenario, a constant frequency Doppler shift is applied to each configured NR NTN cell.

### Once an NR NTN cell has been configured in the SS by TTCN, the SS shall alter the operating carrier frequency (NR-ARFCN) of this cell by the value of this shift.