3GPP TSG-RAN5 Meeting #104	R5-244439

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	6.6.6.6

Source:	MCC TF160

Title:	Specification and usage of conditions in IMS messages

Document for:	Agreement

# 1	Introduction

In context of the withdrawal of R5-242620 at RAN5#103 an action point AP#103.02 has been created to analyse the conditions used by the specific message contents of test cases and test procedures in 34.229-5.

In general, there is no consistency regarding whether or not conditions like IMS security and GIBA are specified in the references to default message contents. In addition, conditions are missing where needed or even wrong.

This document summarises the application of conditions for all default message contents of 34.229-1 annex A and their references in specific message contents of 34.229-1 and 34.229-5 test cases.

Basically, unless specified, none of the prose changes proposed in this document has impact on the TTCN implementation.

# 2	Analysis

### 2.1	Common issues

### 2.1.1	RAN type

For several messages there are conditions for the RAN type (E-UTRAN, NR, UTRAN, WLAN):

- A.1.1 REGISTER: A31, A32, A33

- A.1.4 SUBSCRIBE: A6, A7

- A.2.1 INVITE: A27, A28

- A.2.3 183 Session Progress: A13, A14

- A.2.4 PRACK: A6, A7

- A.2.5 UPDATE: A5, A6

- A.2.6 180 Ringing: A13, A14

- A.2.8 BYE: A7, A8

- A.2.10 MO REFER: A4, A5

- A.3.1 200 OK: A21, A22

- A.4.3 PUBLISH: A4, A5

- A.5.1 SUBSCRIBE: A6, A7

- A.6.1 SUBSCRIBE: A5, A6

- A.7.3 MESSAGE for MO SMS: A4, A5

- A.8.2 MESSAGE UE providing information for CS to PS SRVCC: A3, A4

But these conditions are not used consistently in specific message contents and on the other hand in general it is clear which IPCAN is used so that there is no need to specify these conditions in a specific message content.
(only in case of fall-back scenarios it may be useful to specify the RAN type for clarification).

Proposal 2.1.1:	In general conditions specifying a RAN type shall be removed from specific message contents except for fall-back scenarios where the RAN type may be kept for clarification.

### 2.1.2	PICS

There are many cases where a message content depends on UE capabilities (PICS). For these cases it does not make sense to specify the respective condition in the specific message content of a test case or test procedure (34.229-1 annex C, 34.229-5 annex A) as it is done e.g. in session timer test cases where condition A26 (UE supports Session Timer) is specified for the INVITE.

Proposal 2.1.2:	Conditions corresponding to PICS shall be removed from specific message contents.

NOTE 1:	In TTCN PICS related conditions are considered statically as a UE e.g. supports a feature (like Session Timer) independent from the test case.

NOTE 2:	There are cases where the condition in a default message content is a mixture of PICS and a further condition  These conditions need to be considered case-by-case.
(e.g. condition A8 in A.2.9: INVITE for creating a video call and UE supports video media feature tag)

### 2.1.3	Direction of a message

Several default message contents are specified for both direction and there are conditions to distinguish whether the message is sent from the SS or from the UE. As in general it is clear from the test case's or test procedure's message sequence in which direction the message is sent, there is no need to specify the respective condition. In addition, for some of these messages the direction is combined with the security scheme (GIBA, IMS security; see clause 2.1.4).

Proposal 2.1.3:	Conditions specifying the direction in which a message is sent shall be removed from specific message contents.

### 2.1.4	IMS security vs. GIBA

Several default message contents have conditions to distinguish GIBA from IMS security, which are not specified in a consistent way in 34.229-1 and 34.229-5:

| Specific message content | Clause | Conditions (IMS security / GIBA) | 34.229-1 | 34.229-5 |
| --- | --- | --- | --- | --- |
| 401 Unauthorized | A.1.2 | A1/A2 | not specified | A1 is specified in all test cases and procedures except test cases 10.12 and 10.13 |
| SUBSCRIBE for reg-event package | A.1.4 | A1/A2 | not specified | A1 is specified in all test cases and procedures except A.11 |
| 200 OK for SUBSCRIBE | A.1.5 | A1/A2 | not specified | A1 is specified in all test cases and procedures except A.11 |
| NOTIFY for reg-event package | A.1.6 | A1/A2 | not specified | A1 is specified in all test cases and procedures |
| INVITE for MO Call Setup | A.2.1 | A1/A2 | not specified except test case 22.1 | A1 is specified in all test cases and procedures except test case 10.10 and procedures A.6 and A.23 |
| 183 Session Progress for INVITE | A.2.3 | A1/A3 by the SS A2/A4 by the UE | not specified except test cases 22.1 and 22.3 | A1/A2 is specified in all test cases and procedures except A.15.2 where A3 is specified instead of A1 (what is wrong) |
| PRACK | A.2.4 | A1/A2 by the UE A3/A4 by the SS | not specified | A1/A3 is specified in all test cases and procedures |
| UPDATE | A.2.5 | A1/A2 by the UE A3/A4 by the SS | not specified except test cases 22.1, 22.2, 22.3, 22.5, 22.6, 22.8 | A1/A3 is specified in all test cases and procedures |
| BYE | A.2.8 | A1/A2 by the UE A3/A4 by the SS | not specified except test cases 12.13b, 12.13c | A1/A3 is specified in all test cases and procedures except test case 8.40 |
| INVITE for MT Call | A.2.9 | A1/A2 | not specified except test cases 22.5, 22.6, 22.7, 22.8 | A1 is specified in all test cases and procedures |
| MO REFER | A.2.10 | A1/A2 | not specified | A1 is specified in all test cases and procedures except test case 8.36 |
| MT NOTIFY for refer package | A.2.11 | A1/A2 | not specified | A1 is specified for steps 3/5 in A.20 and for step 5 in A.26 but not for step 7 in A.26 and not in test case 8.36 |
| MT REFER | A.2.12 | A1/A2 | not specified in C.43 | Not used |
| MO NOTIFY | A.2.13 | A1/A2 | Not used | Not used |
| 200 OK for other requests than REGISTER or SUBSCRIBE | A.3.1 | A1/A3 by the SS A2/A4 by the UE (for INVITE/UPDATE) | not specified except test cases 22.x | A1/A2 is specified in many but not all test cases and procedures |
| PUBLISH | A.4.3 | A1/A2 | No references to A.4.3 | A1 is specified in all test cases and procedures (test case 6.1 and A.10) |
| SUBSCRIBE for conference event package | A.5.1 | A1/A2 | not specified | A1 is specified for step 10 of A.19 and A.25 but not for step 0A of A.11 |
| 200 OK for SUBSCRIBE | A.5.2 | A1/A2 | not specified | A1 is specified for step 11 of A.19 and A.25 but not for step 0B of A.11 |
| NOTIFY for conference event package | A.5.3 | A1/A2 | not specified | A1 is specified in test cases 8.34 and 8.35 and in A.20 but not in A.19, A.25, A.26, A.27 |
| SUBSCRIBE for message-summary event package | A.6.1 | A1/A2 | not specified | A1 is specified in test case 8.30 but not in A.11 |
| NOTIFY for message-summary event package | A.6.2 | A1/A2 | not specified | A1 is specified in test case 8.30 |
| 200 OK for SUBSCRIBE for message-summary event package | A.6.3 | A1/A2 | not specified | A1 is specified in test case 8.30 but not in A.11 |
| MESSAGE for MO SMS | A.7.3 | A2/A3 | not specified | not used |

Proposal 2.1.4:	Clarification shall be added to 34.229-1 that unless specified otherwise by a test case IMS security is applied when the UE supports it (34.229-2 A.6a/2: pc_IMS_Sec).
 Only if the UE does not support IMS security GIBA may be used instead (34.229-2 A.6a/1: pc_IMS_GIBA_Sec). This is what is implemented in TTCN (f_IMS_PTC_Init).
 In general, conditions specifying the security scheme shall be removed from specific message content except for GIBA specific test cases.

### 2.2	Conditions of particular default message contents

### 2.2.1	General

In this clause the conditions of default message contents are considered which may/shall be specified in a specific message content referencing a default message content.

### 2.2.2	REGISTER (34.229-1 A.1.1)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A1/A2 | initial or subsequent REGISTER (IMS Security) (NOTE 1) |
| A3 | GIBA (NOTE 1) |
| A7 | emergency registration |
| A17 | re/de-registration |

NOTE 1:	In contrast to the other messages the security scheme needs to be specified for REGISTER as in case of IMS security initial and subsequent REGISTER need to be distinguished.

### 2.2.3	200 OK for REGISTER (34.229-1 A.1.3)

In principle test cases and test procedures shall specify either A2 or A3:

| Condition | Explanation |
| --- | --- |
| A2 | non-emergency registration |
| A3 | emergency registration |

A2 is specified for some but not all cases in 34.229-1 and 34.229-5.

Proposal 2.2.3:	A2 (Response for an non-emergency registration) shall be declared as default in A.1.3, so that there is no need any more to specify it in test cases and procedures.

### 2.2.4	NOTIFY for reg-event package (34.229-1 A.1.6)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A6 | NOTIFY sent by SS for de-registering the UE |

### 2.2.5	420 Bad Extension for REGISTER (34.229-1 A.1.8)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A1 | IMS emergency call for an anonymous emergency call |

### 2.2.6	INVITE for MO Call Setup (34.229-1 A.2.1)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A4 | INVITE creating a dialog (NOTE 1) |
| A5 | re-INVITE within a dialog for session modification (NOTE 1, 2) |
| A6 | INVITE for creating an emergency session in case of no registration |
| A7 | INVITE for creating an emergency session within an emergency registration using IMS security |
| A8 | UE is capable of obtaining location information, has obtained its location and is setting up an emergency session (NOTE 3) |
| A11 | INVITE for creating a video call (NOTE 3) |
| A19 | INVITE for creating an emergency session within an emergency registration using GIBA |
| A20/A21 | INVITE for creating an eCall over IMS session manually/automatically |
| A25 | INVITE for creating a test eCall over IMS session |
| A32 | re-INVITE within a dialog for purposes other than session modification (NOTE 1, 2) |

NOTE 1:	Test case and procedures should specify either A4 or A5/A32. Nevertheless, in general 34.229-1 does not specify A4 and 34.229-5 does not specify A4 in case of emergency call establishment (A6, A7) even though without A6 e.g. From- and To-tag are not specified.
 A4 shall be declared as default in A.2.1, so that there is no need any more to specify it in test cases and procedures (i.e. A4 shall be applied implicitly unless A5/A32 is specified explicitly by a specific message content of a test case or test procedure).

NOTE 2:	34.229-1 A.2.1 only uses "A5 OR A32". Furthermore, it is not fully clear when exactly to use A32 instead of A5. The only cases where A32 is used are in the session timer test case 22.7 (LTE) at steps 14 and 17 and 7.33 (NR) at steps 21 and 24.
 A5 shall be changed to "re-INVITE within a dialog" so that A32 can be removed.

NOTE 3:	It is not fully clear from the wording whether "INVITE for creating a video call" shall be applied too when adding video to an ongoing speech call. 
Nevertheless IR.94 (clause 2.2.2) and NG.114 (clause 2.2.4.1) say in this context "The UE … must be able to establish a video call directly during session establishment and by adding video to a voice session …" and TTCN checks the Accept-Contact header field according to A11.
 A11 shall be added for these cases too; the wording of A11 shall be changed (e.g. "INVITE for creating a video stream").

Proposal 2.2.6-1:	Clarification to be added to condition A4 (INVITE creating a dialog) of A.2.1 that A4 shall be applied unless specified otherwise by the test case or test procedure.

Proposal 2.2.6-2:	Wording of A5 in A.2.1 to be changed to "re-INVITE within a dialog"; A32 to be removed from A.2.1 and replaced by A5 in test cases and test procedures.

Proposal 2.2.6-3:	Wording of A11 in A.2.1 to be changed to "INVITE for creating a video stream"

### 2.2.7	183 Session Progress for INVITE (34.229-1 A.2.3)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A5 | 183 sent by the SS for INVITE for a non-UE detectable emergency call |
| A15 | 183 sent for re-INVITE within an established dialog |

### 2.2.8	180 Ringing for INVITE (34.229-1 A.2.6)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A3 | Response sent reliably |
| A4 | 180 sent by the SS when setting up an emergency call or a non-UE detectable emergency Call |
| A7 | Response sent by SS for emergency call without emergency registration |
| A8 | Response sent by SS for emergency call with emergency registration or a non-UE detectable emergency call |

A12 (180 Ringing is first provisional response sent reliably in this dialog) is not used in any test case or test procedure but needs to be considered implicitly in the TTCN implementation.

### 2.2.9	ACK (34.229-1 A.2.7)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A3/A4 | ACK for 2xx/non-2xx response (NOTE 1) |
| A5 | ACK for re-INVITE |

NOTE 1:	In principle test case and procedures should specify either A3 or A4, but A3 could be considered as normal case.

Proposal 2.2.9:	A3 (ACK for 2xx response) shall be declared as default in A.2.7, so that there is no need any more to specify it in test cases and procedures.

### 2.2.10	BYE (34.229-1 A.2.8)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A6 | BYE for emergency call with no registration |

### 2.2.11	INVITE for MT Call (34.229-1 A.2.9)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A4 | INVITE creating a dialog (NOTE 1) |
| A5 | re-INVITE within a dialog (NOTE 1) |
| A8 | INVITE for creating a video call and UE supports video media feature tag (NOTE 2) |

NOTE 1:	Test case and procedures should specify either A4 or A5. Nevertheless, in general 34.229-1 does not specify A4.

Proposal 2.2.11-1:	A4 (INVITE creating a dialog) shall be declared as default in A.2.9, so that there is no need any more to specify it in test cases and procedures: A4 shall be applied implicitly unless A5 is specified explicitly by a specific message content of a test case or test procedure.

NOTE 2:	Similar as in A.2.1 (A10, A11) A8 should be split into two conditions: "UE supports video media feature tag" (PICS: pc_IMS_Video_FeatureTag) and "INVITE for creating a video session". Nevertheless, it is not clear whether/why a simulated remote UE has to provide the “video” media feature tag in the Accept-Contact header field depending on a UE capability and up-to now it is not considered in TTCN.

Proposal 2.2.11-2:	A8 shall be changed to "INVITE for creating a video session" in A.2.9.

### 2.2.12	MO INFO for eCall over IMS (34.229-1 A.2.19)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A1/A2 | eCall over IMS was started manually/automatically |
| A3/A4 | UE able/not able to provide an updated MSD |

Test cases 11.4/5/6 in 34.229-5 specify conditions A1/A2 and A3/4 for step 6 of A.23. But in 34.229-1 none of the test cases using C.47 specify the conditions for the SIP INFO at step 6 of C.47.

### 2.2.13	486 Busy Here (34.229-1 A.2.21)

A.2.21 has only condition A1 (Response sent by SS for INVITE for eCall over IMS) which shall be specified but is not specified in 34.229-1 test cases 21.13 and 21.14.

### 2.2.14	600 Busy Everywhere (34.229-1 A.2.22)

A.2.22 has only condition A1 (Response sent by SS for INVITE for eCall over IMS) which shall be specified but is not specified in 34.229-1 test cases 21.15 and 21.16.

### 2.2.15	603 Decline (34.229-1 A.2.23)

A.2.23 has only condition A1 (Response sent by SS for INVITE for eCall over IMS) which shall be specified but is not specified in 34.229-1 test cases 21.17 and 21.18.

### 2.2.16	200 OK for other requests than REGISTER or SUBSCRIBE (34.229-1 A.3.1)

As in conditions A1/A2/A3/A4 the security scheme is mixed up with the message type of the request to which the 200 OK belongs (INVITE/UPDATE), in principle conditions A1/A2/A3/A4 should be specified for specific message contents referring to A.3.1 in case of a 200 OK for INVITE or UPDATE. Nevertheless, in general it is clear from the message sequence and/or the table title of the specific message content whether the 200 OK is for an INVITE or UPDATE or some other request.

Proposal 2.2.16-1:	Conditions A1/A2/A3/A4 (Response sent by SS/UE for INVITE/UPDATE, IMS security/GIBA) for A.3.1 shall be applied implicitly, i.e. there is no need to specify these conditions in references to A.3.1.

The same applies for conditions A19/A20 (Response sent by SS/UE for INVITE).

Proposal 2.2.16-2:	Conditions A19/A20 (Response sent by SS/UE for INVITE) for A.3.1 shall be applied implicitly, i.e. there is no need to specify these conditions in references to A.3.1.

In contrast to the message type of the request for which the 200 OK is sent, it is not always clear from the message sequence whether the 200 OK is sent within a dialog and whether it is sent for a re-INVITE.

Proposal 2.2.16-3:	A clarification shall be added to condition A8 (Any response sent by the UE within a dialog, except for CANCEL requests), that in context of A8 the only case where a 200 OK is not sent within a dialog is a 200 OK for a SIP MESSAGE request (which does not initiate a dialog according to RFC 3428).
 With this clarification it is clear that the A8 is applicable for all 200 OK responses of A.3.1 except for 200 OK for CANCEL and 200 OK for SIP MESSAGE and therefore there is no need to specify this condition in references to A.3.1.

Proposal 2.2.16-4:	Condition A23 (Response sent for re-INVITE within an established dialog) shall be specified explicitly in references to A.3.1.

Condition A5 (Any response sent by the UE within a dialog) is applicable for all 200 OK responses except a 200 OK response for a SIP MESSAGE and in case of eCall establishment where the 200 OK for INVITE creates the dialog. In all other cases according to RFC 3261 the dialog is created already e.g. by a 1XX response. On the other hand, there seems to be no need for condition A5 at all as the conditions for the Record-Route and Contact header fields can be expressed (and corrected) in a different way (see clause 3.3).

Proposal 2.2.16-5:	Conditions A5 (Any response sent by the UE within a dialog) shall be removed from A.3.1 and from all references to A.3.1.

Taking into account the above proposals the following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A6 | Response sent by SS for INVITE for emergency call or non-UE detectable emergency call |
| A7 | Response sent by SS for INVITE for emergency call without emergency registration |
| A12 | Response sent by SS for INVITE for eCall over IMS session with either ACK or NACK |
| A13 | Response sent by SS for INVITE for eCall over IMS session with ACK element = TRUE |
| A14 | Response sent by SS for INVITE for eCall over IMS session with ACK element = FALSE |
| A23 | Response sent for re-INVITE within an established dialog |

### 2.2.17	403 FORBIDDEN (34.229-1 A.3.2)

A.3.2 has only condition A1 (IMS emergency call for an anonymous emergency call) which is specified wherever A.3.2 is referred to.

### 2.2.18	NOTIFY for conference event package (34.229-1 A.5.3)

The following conditions shall be specified in test cases and test procedures (where applicable):

| Condition | Explanation |
| --- | --- |
| A3 | SS sends NOTIFY to indicate that UE is now subscribed to the conference event package |
| A4 | SS sends NOTIFY to indicate that UE’s subscription to conference event is terminated now |
| A5 | SS sends NOTIFY to indicate that UE is now subscribed to the video conference event package |

# 3	Further issues

### 3.1	34.229-5 Test cases 10.6, 10.12, 10.13 and 10.14

In test case 10.6 at steps 12 and step 23 the test procedure sequence refers to step 1 of A.6 (INVITE for emergency call) but there are no specific message contents specifying which of conditions A6 (INVITE for creating an emergency session in case of no registration) and A7 (INVITE for creating an emergency session within an emergency registration) shall be applied. The same issue is in test cases 10.12 and 10.13 at step 6 and in test case 10.14 at steps 2-14.

Proposal 3.1:	Clarification to be added to 34.229-5 A.6 that at step 1 condition A7 (INVITE for creating an emergency session within an emergency registration using IMS security) shall be applied per default for the INVITE unless specified otherwise in the test case. Alternatively condition A7 can be specified in specific message content to be added to the test cases.

### 3.2	34.229-5 A.24 (UE putting a Video Call on hold or resume the call)

34.229-5 A.24 specifies explicitly condition A11 for the INVITE at step 1 resulting in the requirement that the UE has to include the a “video” media feature tag in the Accept-Contact header of the INVITE. Nevertheless, according to IR.94 and NG.114 this requirement seems to be mainly for establishment of the video session and in TTCN the INVITE is not checked for the “video” media feature in case of call hold/resume.
 Either A11 needs to be removed from the INVITE in A.24 or a TTCN CR is needed.

Proposal 3.2:	Decision to be made whether or not condition A11 (INVITE for creating a video call) is needed for the INVITE at step 1 of A.24.

### 3.3	Issues in 34.229-1 A.3.1 (200 OK for other requests than REGISTER or SUBSCRIBE)

### 3.3.1	Record-Route header field

For conditions A2, A4, A5 the Record-Route header field is specified to be the "same value as received in the request (if present in the request)", with
		A2, A4: Response sent by UE for INVITE/UPDATE, IMS security or GIBA
		A5: Any response sent by the UE within a dialog.
This would mean that for all 200 OK responses sent by the UE except 200 OK for MESSAGE (which is not in a dialog) the Record-Route header field should be copied from the request.

In contrast to this, the note in the same cell of the table says "for requests other than INVITE it is not regulated if and what the UE writes into this header in a response", what in principle conflicts with the above interpretation. On the other hand, when looking at the message contents of the SIP requests sent by the SS only the INVITE contains a Record-Route header field so that in practice there is no conflict (but just confusion).

Furthermore R5-223454 has added "any value if present" for "A2 AND A23", i.e. for IMS security and any re-INVITE the Record-Route header field can be any value if present. This creates an ambiguity as it is not clear which of "A2, A4, A5" and "A2 AND A23" has precedence in case of IMS security (in case of GIBA "A2, A4, A5" would need to be applied as nothing else is specified).

A similar issue exists for the 200 OK sent by the SS where A1/A3 (Response sent by SS for INVITE/UPDATE) conflicts with "A1 AND A23" (Response sent by SS for re-INVITE with IMS security).

Conclusions:
According to the note for A2, A4, A5 and the reason for changes in related CRs it seems to be intended that in UL the 200 OK for the initial INVITE shall contain a Record-Route header field with the same value as in the INVITE and in all other cases the Record-Route header field is not specified (i.e. any value if present). The required expression for this is "A20 AND NOT A23" (which also covers the case of GIBA). This is what actually is implemented in TTCN.
In DL the conflict can be resolved by adding "AND NOT A23" to A1 and A3, i.e. the specific values apply for initial INVITE and UPDATE only (again this is what is actually implemented in TTCN).

Proposal 3.3.1:

| Record-Route |  | order of the parameters in this header must be like in the respective rows |
| --- | --- | --- |
| rec-route | A1 AND NOT A23 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |
|  | A3 AND NOT A23 | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |
|  | A6 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |
|  | A7 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |
|  | A20 AND NOT A23 | same value as received in the request (initial INVITE from the SS) |

NOTE 1: 	According to common rules in 34.229-1 for the UL "any value if present" and for the DL "not present" are the defaults which apply for a header field when there is no match with any expression in the 'condition' column. 
 There is no need to specify this explicitly.

### 3.3.2	Contact header field

A.3.1 specifies a Contact header field only for a 200 OK for INVITE or UPDATE (conditions A1, A2, A3, A4). In the 200 OK for other cases the Contact header field is not present when sent by the SS and any value if present when sent by the UE.

### 3.3.2.1	Contact address

In general, when a 200 OK for INVITE or UPDATE is sent, a dialog is created already, e.g. by the 183 Session progress (There is only the corner case of eCall establishment where the 200 OK for INVITE creates the dialog).

"Any response sent within a dialog" is not a proper criterion to distinguish call establishment from mid call scenarios and in general the Contact address for the 200 OK shall be the same as been provided in the INVITE or the 1xx response creating the dialog (NOTE: According to RFC 3261 the Contact address is part of the dialog state information).

Furthermore, the expression "A1 AND A5" does not make sense at all as A1 (Response sent by SS for INVITE/UPDATE) and A5 (Any response sent by the UE within a dialog) are mutually contradictory; in addition, GIBA is not considered.

Conclusions:
With A5 as it is the 200 OK's Contact address cannot be specified correctly. On the other hand, there seems to be even no need to use A5 for specification of the Contact address.

Proposal 3.3.2.1:

| Contact | A1,A2 A3,A4 |  |
| --- | --- | --- |
| addr-spec | A1,A3 | MO call: px_IMS_CalleeContactUri MT call: Contact address provided by SS in dialog creating INVITE |
|  | A2,A4 | MO call: Contact address provided by UE in dialog creating INVITE MT call: Contact address provided by UE in 1xx response establishing the dialog according to RFC 3261 [15] clause 12. (NOTE: The 'transport' url parameter may change, url parameters are not checked) |

NOTE 1:	With the above changes condition A9 gets obsolete and can be removed.

NOTE 2:	Together with the changes proposed in 3.3.1 A5 gets obsolete and can be removed.

### 3.3.2.2	feature-param audio

Expression "A11 AND A14 AND (A15 OR A16)" does not make sense at all as A11 (Response sent by UE) and A14 (Response sent by SS for INVITE for eCall …) are mutually contradictory.

Assumption: should be "A11 AND A15 AND (A16 OR A17)".

Proposal 3.3.2.2:	Condition for audio feature-param of Contact header field in A.3.1 to be corrected to 
"A11 AND A15 AND (A16 OR A17)".

### 3.3.3	Content-Length header field

When the 200 OK is sent by the SS (condition A10) Content-Length is specified to be 0 in case of NOT A12 and as "length of message body" in case of A12. This is in contrast to other messages where the Content-Length is simply specified as "length of message body" which results in 0 when there is no Message-Body.

Proposal 3.3.3:	Specification of Content-Length in A.3.1 shall be simplified to "length of message body" for A10 without further conditions.

### 3.4	Conditions A10 and A11 in 34.229-1 A.2.3 (183 Session Progress)

In 34.229-1 A.2.3 conditions A10 and A11 are void but still used in the default message content:

| Contact |  |  |
| --- | --- | --- |
| … |  |  |
| feature-param |  | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel"(see NOTE 2, 3) |
|  | A6 | video |
|  | A10 | audio |
|  | A11 AND A12 AND (A13 OR A14) | audio |

With

| Condition | Explanation |
| --- | --- |
| A10/A11 | Void (haven been "183 sent by SS/UE") |
| A12 | UE supports audio media feature tag |
| A13 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message |
| A14 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message |

A10 and A11 have been voided but left for the audio feature tag by R5-176371. A12, A13 and A14 have been introduced by R5-177127 at the same meeting.

Looking at the history it seems to be intended that the audio feature tag is always included in DL (in contrast to the video feature tag which, for what reason ever, is include only when the UE supports it). In UL it seems that the UE is expected to include the audio feature tag when it supports it and, in case of EUTRA and UTRAN/GERAN, the network has indicated support of IMS voice over PS Session.

Nevertheless, the case of NR5GC is not considered i.e. according to A.2.3, in contrast to TTCN (f_NR_IMS_IpcanInfo), for NR5GC the UE does not need to provide the audio feature tag at all.

Proposal 3.4:	A.2.3 to aligned with TTCN: 
In DL (A1, A3) the SS shall provide the audio feature tag without further condition.
In UL the condition can be "(A2 OR A4) AND A12 AND (A13 OR A14 OR A15) 
with the new A15: "UE uses NR access (A.18/5 3GPP TS 34.229-2 [5])"
NOTE:	A similar expression is e.g. in A.1.1 already.

# 4	Summary of Proposals

The following list summarises the proposals for prose changes of clauses 2 and 3. Basically the proposed changes don't affect the TTCN implementation but align the prose to TTCN, except proposal 3.2 which depending on the RAN5 decision may cause a TTCN change.

List of proposals:

Proposal 2.1.1:	In general conditions specifying a RAN type shall be removed from specific message contents except for fall-back scenarios where the RAN type may be kept for clarification.

Proposal 2.1.2:	Conditions corresponding to PICS shall be removed from specific message contents.

Proposal 2.1.3:	Conditions specifying the direction in which a message is sent shall be removed from specific message contents.

Proposal 2.1.4:	Clarification shall be added to 34.229-1 that unless specified otherwise by a test case IMS security is applied when the UE supports it (34.229-2 A.6a/2: pc_IMS_Sec).
 Only if the UE does not support IMS security GIBA may be used instead (34.229-2 A.6a/1: pc_IMS_GIBA_Sec). This is what is implemented in TTCN (f_IMS_PTC_Init).
 In general, conditions specifying the security scheme shall be removed from specific message content except for GIBA specific test cases.

Proposal 2.2.3:	A2 (Response for a non-emergency registration) shall be declared as default in A.1.3, so that there is no need any more to specify it in test cases and procedures.

Proposal 2.2.6-1:	Clarification to be added to condition A4 (INVITE creating a dialog) of A.2.1 that A4 shall be applied unless specified otherwise by the test case or test procedure.

Proposal 2.2.6-2:	Wording of A5 in A.2.1 to be changed to "re-INVITE within a dialog"; A32 to be removed from A.2.1 and replaced by A5 in test cases and test procedures.

Proposal 2.2.6-3:	Wording of A11 in A.2.1 to be changed to "INVITE for creating a video stream"

Proposal 2.2.9:	A3 (ACK for 2xx response) shall be declared as default in A.2.7, so that there is no need any more to specify it in test cases and procedures.

Proposal 2.2.11-1:	A4 (INVITE creating a dialog) shall be declared as default in A.2.9, so that there is no need any more to specify it in test cases and procedures: A4 shall be applied implicitly unless A5 is specified explicitly by a specific message content of a test case or test procedure.

Proposal 2.2.11-2:	A8 shall be changed to "INVITE for creating a video session" in A.2.9.

Proposal 2.2.16-1:	Conditions A1/A2/A3/A4 (Response sent by SS/UE for INVITE/UPDATE, IMS security/GIBA) for A.3.1 shall be applied implicitly, i.e. there is no need to specify these conditions in references to A.3.1.

Proposal 2.2.16-2:	Conditions A19/A20 (Response sent by SS/UE for INVITE) for A.3.1 shall be applied implicitly, i.e. there is no need to specify these conditions in references to A.3.1.

Proposal 2.2.16-3:	A clarification shall be added to condition A8 (Any response sent by the UE within a dialog, except for CANCEL requests), that in context of A8 the only case where a 200 OK is not sent within a dialog is a 200 OK for a SIP MESSAGE request (which does not initiate a dialog according to RFC 3428).

Proposal 2.2.16-4:	Condition A23 (Response sent for re-INVITE within an established dialog) shall be specified explicitly in references to A.3.1.

Proposal 2.2.16-5:	Conditions A5 (Any response sent by the UE within a dialog) shall be removed from A.3.1 and from all references to A.3.1.

Proposal 3.1:	Clarification to be added to 34.229-5 A.6 that at step 1 condition A7 (INVITE for creating an emergency session within an emergency registration using IMS security) shall be applied per default for the INVITE unless specified otherwise in the test case. Alternatively condition A7 can be specified in specific message content to be added to the test cases.

Proposal 3.2:	Decision to be made whether or not condition A11 (INVITE for creating a video call) is needed for the INVITE at step 1 of A.24.

Proposal 3.3.1:	Record-Route header field in A.3.1 to be corrected as proposed in clause 3.3.1.

Proposal 3.3.2.1:	Contact header field in A.3.1 to be corrected as proposed in clause 3.3.2.1.

Proposal 3.3.2.2:	Condition for audio feature-param of Contact header field in A.3.1 to be corrected to 
A11 AND A15 AND (A16 OR A17)".

Proposal 3.3.3:	Specification of Content-Length in A.3.1 shall be simplified to "length of message body" for A10 without further conditions.

Proposal 3.4:	A.2.3 to aligned with TTCN: 
In DL (A1, A3) the SS shall provide the audio feature tag without further condition.
In UL the condition can be "(A2 OR A4) AND A12 AND (A13 OR A14 OR A15) 
with the new A15: "UE uses NR access (A.18/5 3GPP TS 34.229-2 [5])"
NOTE:	A similar expression is e.g. in A.1.1 already.