3GPP TSG-RAN5 Meeting #104	R5-244521

# Maastricht, NL, Aug 19th - Aug 23th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.918 | CR | 0002 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of A.3.1.2.4.2 in TR 38.918 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, UNISOC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update the typo of slice service performance testing in A.3.1.2.4.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the typo of slcie service performance testing procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test procedure is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

A.3.1.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the MANAGE UE POLICY COMMAND message is defined in Table A.3.1.1.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the simulated APP A, begin uplink data transfer from UE to the Application Server Simulator.

6.	Wait for 15 seconds and then start to measure the throughput result in application layer (This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

8.	Calculate and record the average application layer data throughput across three iterations.

9.	Using the Application Client Simulator to simulate APP B, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

10.	Using the simulated APP B, begin uplink data transfer from UE to the Application Server Simulator.

11.	Repeat steps 6-8 to get the average application layer data throughput of the data traffic of simulated APP A.

12.	Observe that the throughput in step 11 is not lower than the throughput benchmark in step 8.

Table A.3.1.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000002 |  |  |

Table A.3.1.2.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | 1 |  |  |
| SD | 0x000001 |  |  |  |

Table A.3.1.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | 1 |  |
| SD |  | 0x000001 |  |  |

<End of modified section>