3GPP TSG-RAN5 Meeting #104	R5-244537

# Maastricht, Netherlands, 19 – 23 Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0668 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of bands n25, n40, n66, n71, n77, n85 PC1 for physical layer baseline implementation capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing PC1 for bands n25, n40, n66, n71, n77, n85 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding bands n25, n40, n66, n71, n77, n85 into Table A.4.3.1-4b for NR FR1 PC1 RF Baseline Implementation Capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No PC1 RF baseline implementation capabilities defined for bands n25, n40, n66, n71, n77, n85 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### A.4.3.1	RF Baseline Implementation Capabilities

NOTE:	The values indicated in column "Release" for bands are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 38.307 [19].

NOTE:	See Annex B for status of completed NR bands and power classes in this version of 3GPP UE conformance test specifications.

-------- Skipped unchanged clauses --------

Table A.4.3.1-4b: NR FR1 PC1 RF Baseline Implementation Capabilities

| Item | NR FR1 PC1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 6.2.1 | Rel-16 | pc_nrBand14_PC1_Supp | NR FR1 PC1 Band n14 |
| 2 | NR Frequency band: 874.4-880 MHz (UL), 919.4-925 MHz (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand100_PC1_Supp | NR FR1 PC1 Band n100 |
| 3 | NR Frequency band: 1900–1910 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand101_PC1_Supp | NR FR1 PC1 Band n101 |
| 4 | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand7_PC1_Supp | NR FR1 PC1 Band n7 |
| 5 | FFS |  |  |  |  |
| 6 | NR Frequency band: 3300–3800 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand78_PC1_Supp | NR FR1 PC1 Band n78 |
| 7 | NR Frequency band: 2496–2690 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand41_PC1_Supp | NR FR1 PC1 Band n41 |
| 8 | NR Frequency band: 1850–1915 MHz (UL), 1930–1995 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand25_PC1_Supp | NR FR1 PC1 Band n25 |
| 9 | NR Frequency band: 2300–2400 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand40_PC1_Supp | NR FR1 PC1 Band n40 |
| 10 | NR Frequency band: 1710–1780 MHz (UL), 2110–2200 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand66_PC1_Supp | NR FR1 PC1 Band n66 |
| 11 | NR Frequency band: 663–698 MHz (UL), 617–652 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand71_PC1_Supp | NR FR1 PC1 Band n71 |
| 12 | NR Frequency band: 3300–4200 MHz (UL / DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand77_PC1_Supp | NR FR1 PC1 Band n77 |
| 13 | NR Frequency band: 698–716 MHz (UL), 728–746 (DL) | 38.101-1, 6.2.1 | Rel-18 | pc_nrBand85_PC1_Supp | NR FR1 PC1 Band n85 |

-------- End of changes --------