3GPP TSG-RAN5 Meeting #104	R5-244591

# Maastricht, The Netherlands, 19th August 2024 – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2900 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CA_n30A-n77A PC2 to reference sensitivity case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification Oy, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CA_n30A-n77A PC2 reference sensitivity test is missing from 38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | CA_n30A-n77A PC2 reference sensitivity test is added to 38.521-1 Ch 6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_n30A-n77A PC2 reference sensitivity test case is not defined in 38.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.4, 7.3A.0.5, 7.3A.1_1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.3A.0.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Editor’s Note:

- 38.101-1 (V17.9.0) Table 7.3A.4-4 UL n77 agressor band impacts on DL band n5 indicates a case of UL/DL bandwidth of 5 MHz/5 MHz, but there is no definition of reference sensitivity for 5MHz bandwidth of n77. Therefore in Table 7.3A.0.4-4b corresponding case, UL n77 minimum bandwidth of 10 MHz is used instead.

Table 7.3A.0.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n1 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 10 | 15 | 50 (RBstart=0) | 1007 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 5 | 15 | 25 (RBstart=0) | 10 | 1.9 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n5 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n5 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n20 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n20 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n24 | n7712 | N/A | N/A | N/A | N/A | N/A | NOTE 2 | UL2/DL1 direct-hit |
| n24 | n7712 | N/A | N/A | N/A | N/A | N/A | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n25 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n28 | n1 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | n1 | 5 | 15 | 25 (RBstart=0) | 50 | 1.1 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 40 | 15 | 200 (RBstart=0) | 1007 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 1.9 | NOTE 6 | UL2/DL1 near-miss |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n71 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 8 (RBstart=0) | 5 | 9.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 20 (RBstart=0) | 25 | 4.1 | NOTE 3 | UL3/DL1 |
| n71 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: Void. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. NOTE 12: In the USA, n77 band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. There is no UL harmonic due to n24 UL in downlink for n77 operating in 3450 – 3550 MHz and 3700 – 3980 MHz. NOTE 13: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.2 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

Table 7.3A.0.4-1a: Void

Table 7.3A.0.4-2: Void

Table 7.3A.0.4-3: Void

Table 7.3A.0.4-3a: Void

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same CA configuration. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1 is specified in Table 7.3A.0.4-4b. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1 is specified in Table 7.3A.0.4-4c.

Table 7.3A.0.4-4: Void

Table 7.3A.0.4-4a: Void

Table 7.3A.0.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.3 | NOTE 11 | UL4/DL3 |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 30.3 | NOTE 4 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 4.3 | NOTE 12 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 0.8 | NOTE 12 | UL3/DL4 |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 3,7 | NOTE 7 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 | NOTE 8 | UL1/DL4 |
| n77 | n14 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 (RBstart=0) | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n25 | 20 | 15 | 100 (RBstart=0) | 40 | 1.1 | NOTE 7 | UL1/DL2 |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 24 (RBstart=0) | 10 | 8.0 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 6.3 | NOTE 2 | UL2/DL3 |
| n78 | n28 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| NOTE 1: FFS. NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: FFS. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 6: FFS. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 9: FFS. NOTE 10: FFS. NOTE 11: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 7.3A.0.4-4c: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 9.1 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 6.7 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| n77 | n14 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 25 (RBstart=0) | 10 | 10.6 | NOTE 3 | UL2/DL3 |
| n78 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 1 | NOTE 4 | UL1/DL2 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: FFS NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

#### 7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a, the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a. For these test points the reference sensitivity requirement specified in Table 7.3.2.3-1 and Table 7.3.2.3-2 are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a.

Table 7.3A.0.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) |  | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 |  | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 |  | N/A | FDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 |  | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 |  | N/A | FDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 |  | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 |  | N/A | TDD | N/A |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 |  | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 |  | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 |  | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 |  | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 |  | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 |  | 4 | FDD | IMD5 |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 |  | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 |  | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987.5 |  | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 |  | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 1 RBSTART=0 | 3945 |  |  |  |  |
| CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 |  | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 |  | N/A | FDD | N/A |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 |  | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 |  | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 |  | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 |  | N/A | FDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 |  | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 |  | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1721 | 10 | 50 | 1816 |  | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 |  | 24 | FDD | IMD23 |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 |  | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 |  | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 |  | 8.0 | FDD | IMD44 |
|  | n77 | 3435 | 10 | 50 | 3435 |  | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 |  | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 |  | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 |  | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 |  | N/A | TDD | N/A |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 |  | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 |  | N/A | FDD | N/A |
| CA_n5-n776 | n5 | 844 | 5 | 25 | 889 |  | 8.3 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 |  | N/A | TDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 |  | 5.5 | FDD | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 |  | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 |  | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 |  | N/A | TDD | N/A |
| CA_n14A-n77A | n14 | 793 | 5 | 20 | 763 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 |  | N/A | TDD |  |
| CA_n20A-n78A | n20 | 850 | 5 | 25 | 809 |  | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 |  | N/A | TDD | N/A |
| CA_n24-n7710 | n24 | N/A | N/A | N/A | N/A |  | N/A | FDD | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A |  | N/A | TDD | N/A |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 |  | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 |  | 20 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 |  | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 |  | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 |  | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 |  | N/A | FDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 |  | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 |  | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 |  | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 |  | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 |  | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 |  | N/A | FDD | N/A |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 |  | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 |  | N/A | TDD | N/A |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBSTART=0) | 2545 | N/A |  | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART=272) | 2625 |  |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 |  | FDD | IMD9 |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 |  | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 |  | 5.0 | FDD | IMD5 |
| CA_n48-n70 | n70 | 1697.5 | 25/15 | 25 | 1997.5 |  | 26 | FDD | IMD24 |
|  | n48 | 3695 | 10 | 50 | 3695 |  | N/A | TDD | N/A |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 |  | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 |  | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 |  | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 |  | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 |  | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 |  | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBSTART=0) | 3305 |  | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBSTART=8) | 3855 |  |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 |  | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 |  | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBSTART=0) | 3875 |  |  |  |  |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 |  | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 |  | N/A | TDD | N/A |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 |  | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 |  | N/A | FDD | N/A |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3309 | 10 | 50 | 3309 |  | N/A | TDD | N/A |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 |  | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 |  | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in subclause 6.2A.4 NOTE 2: RBSTART = 0, 15kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: TBD NOTE 7: TBD NOTE 8: TBD NOTE 9: TBD NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: TBD NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3A.0.5-1a: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 19.10 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 18.4 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
| CA_n5-n77 | 5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 34.33 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). |  |  |  |  |  |  |  |  |

Editor’s Note:

The n78 UL RB size of 10MHz BW in CA_n1-n3-n78 is different from the Table 7.3A.5-2 in TS 38.101-1 (V17.9.0), since the values in the latter cannot be applied to DFT-s-OFDM. The closest smaller valid value has been applied in Table 7.3A.0.5-2.

#### ------------MANY UNCHANGED SECTIONS SKIPPED HERE-----------

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier indicating power class 2 in TS 38.508-2 table A.4.3.2A.4.1-3. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA with PC3 single UL carrier or PC3 2UL CA

| CA configuration | Test ID |  | NR band | CBW | REFSENS test requirement of NR band (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (MHz) | 2Rx | 4Rx |
| CA_n1A-n3A | 1 |  | n1 | 5 | -100+23+TT | -100 +23+TT |
|  |  |  | n3 | 5 | -97+TT | --97-2.7+TT |
|  | 2 |  | n1 | 5 | -100+TT | -100-2.7+TT |
|  |  |  | n3 | 5 | -97+3+TT | --97+3+TT |
|  | 3 |  | n1 | 5 | -89.6+TT | -89.6-2.7+TT |
|  |  |  | n3 | 5 | -97+19.7+TT | -97+19.7+TT |
| CA_n1A-n8A | 1 |  | n1 | 5 | -100+6+TT | -100+6+TT |
|  |  |  | n8 | 5 | -97+TT | -97-2.76+TT |
| CA_n1A-n28A | 1 |  | n1 | 5 | -100+10.2+TT | -100+10.2+TT |
|  |  |  | n28 | 5 | -98.5+TT | -98.5-2.7+TT |
|  | 2 |  | n1 | 50 | -89.6+1.1+TT | -89.6+1.1+TT |
|  |  |  | n28 | 5 | -98.5+TT | -98.5-2.7+TT |
| CA_n1A-n41A | 1 |  | n1 | 50 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n41 | 10 | -94.8+6.1+TT | -94.8+6.1+TT |
|  | 2 |  | n1 | 20 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n41 | 100 | -94.8+0.7+TT | -94.8+0.7+TT |
|  | 3 |  | n1 | 5 | -93.8+18.1+TT | -93.8+18.1+TT |
|  |  |  | n41 | 100 | -94.8+TT | -94.8-2.7+TT |
| CA_n1A-n77A | 1 |  | n1 | 5 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 10 | -95.34+27.1+TT | -95.34+27.1+TT |
|  | 2 |  | n1 | 20 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 100 | -85.14+13.8+TT | -85.14+13.8+TT |
|  | 3 |  | n1 | 5 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 10 | -95.34+1.9+TT | -95.34+1.9+TT |
| CA_n1A-n78A | 1 |  | n1 | 5 | -100 +8+TT | -100 +8+TT |
|  |  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n2A-n48A | 1 |  | n2 | 5 | -98.0 +TT | -100.7+TT |
|  |  |  | n48 | 10 | -95.8+1.9+TT | -95.8+1.9+TT |
|  | 2 |  | n2 | 5 | -98.0 +12+TT | -98.0 +12+TT |
|  |  |  | n48 | 20 | -92.7 +TT | -94.9 +TT |
| CA_n2A-n66A | 1 |  | n2 | 5 | -98.0 +20+TT | -98.0+20+TT |
|  |  |  | n66 | 5 | -99.5+TT | -102.2+TT |
|  | 2 |  | n2 | 5 | -98.0 +TT | -100.7 +TT |
|  |  |  | n66 | 5 | -99.5 +4+TT | -99.5+4+TT |
| CA_n2A-n77A | 1 |  | n2 | 5 | -98+TT | -98-2.7+TT |
|  |  |  | n774 | 10 | -95.3+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n2 | 10 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n774 | 100 | -89.4+13.8+TT | -89.4+13.8+TT |
|  | 3 |  | n2 | 5 | -98+TT | -98-2.7+TT |
|  |  |  | n774 | 10 | -95.3+1.1+TT | -95.3+1.1+TT |
|  | 4 |  | n2 | 5 | -98+6.7+TT | -98+6.7+TT |
|  |  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 5 |  | n2 | 20 | -91.8+3,7+TT | -91.8+3,7+TT |
|  |  |  | n774 | 20 | -92.2+TT | -92.2-2.2+TT |
|  | 6 |  | n2 | 5 | -98.0 +26+TT | -98.0+28.7+TT |
|  |  |  | n774 | 10 | -95.3+TT | -97.5+TT |
|  | 7 |  | n2 | 5 | -98.0 +8+TT | -98.0 +8+TT |
|  |  |  | n774 | 10 | -95.3+TT | -97.5+TT |
|  | 8 |  | n2 | 5 | -98.0 +5+TT | -98.0 +5+TT |
|  |  |  | n774 | 10 | -95.3+TT | -97.5+TT |
| CA_n3A-n8A | 1 |  | n3 | 10 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n8 | 5 | -97.0+8+TT | -97.0-+8+TT |
|  | 2 |  | n3 | 10 | -93.8+6.4+TT | -93.8+6.4+TT |
|  |  |  | n8 | 5 | -97.0+TT | -97.0-2.76+TT |
| CA_n3A-n41A |  | 1 | n3 | 5 | -97.0+8.2+TT | -97.0+8.2+TT |
|  |  |  | n41 | 10 | -94.9 +TT | -94.9 -2.7 +TT |
|  |  | 2 | n3 | 5 | -97.0+0.6+TT | -97.0+0.6+TT |
|  |  |  | n41 | 100 | -84.6+TT | -84.6 -2.7+TT |
| CA_n3A-n77A | 1 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n77 | 10 | -95.3+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n3 | 10 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n77 | 100 | -85+13.8+TT | -85+13.8+TT |
|  | 3 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n77 | 10 | -95.3+1.1+TT | -95.3+1.1+TT |
|  | 4 |  | n3 | 5 | -97.0 +26.0 +TT | -97.0 +26 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2+TT |
|  | 5 |  | n3 | 5 | -97.0 +8.0 +TT | -97.0+8 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2+TT |
| CA_n3A-n78A | 1 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n78 | 10 | -95.8+23.9+TT | -95.8+23.9+TT |
|  | 2 |  | n3 | 10 | -93.8+TT | -93.8-2.7+TT |
|  |  |  | n78 | 100 | -85.5+13.8+TT | -85.5+13.8+TT |
|  | 3 |  | n3 | 5 | -97+TT | -97-2.7+TT |
|  |  |  | n78 | 10 | -95.8+1.1+TT | -95.8+1.1+TT |
|  | 4 |  | n3 | 5 | -97+26+TT | -97+26+TT |
|  |  |  | n78 | 10 | -95.8+TT | -95.8-2.2+TT |
|  | 5 |  | n3 | 5 | -97+8+TT | -97+8+TT |
|  |  |  | n78 | 10 | -95.8+TT | -95.8-2.2+TT |
| CA_n5A-n66A | 1 |  | n5 | 5 | -98.0 +30+TT | - |
|  |  |  | n66 | 5 | -99.5+TT | -102.2+TT |
| CA_n5A-n77A | 1 |  | n5 | 5 | -98+TT | - |
|  |  |  | n77 | 10 | -95.3+10.5+TT | -95.3+10.5+TT |
|  | 2 |  | n5 | 5 | -98+TT | - |
|  |  |  | n77 | 100 | -85+1.4+TT | -85+1.4+TT |
|  | 3 |  | n5 | 5 | -98+TT | - |
|  |  |  | n77 | 10 | -95.3+10.4+TT | -95.3+10.4+TT |
|  | 4 |  | n5 | 5 | -98+TT | - |
|  |  |  | n77 | 100 | -85+0.7+TT | -85+0.7+TT |
|  | 5 |  | n5 | 20 | -90.8 +2.7+TT | - |
|  |  |  | n77 | 20 | -92.2+TT | -94.4+TT |
|  | 6 |  | n5 | 5 | -98.0 +5.7+TT | - |
|  |  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 7 |  | n5 | 5 | -98.0 +8.3+TT | - |
|  |  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 8 |  | n5 | 5 | -98.0 +5.5+TT | - |
|  |  |  | n77 | 10 | -95.3+TT | -97.5+TT |
| CA_n5A-n78A | 1 |  | n5 | 5 | -98+TT | - |
|  |  |  | n78 | 10 | -95.8+10.5+TT | -95.8+10.5+TT |
|  | 2 |  | n5 | 5 | -98+TT | - |
|  |  |  | n78 | 100 | -85.5+1.4+TT | -85.5+1.4+TT |
| CA_n7A-n78A | 1 |  | n7 | 5 | -98+4.5+TT | -98+4.5+TT |
|  |  |  | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
| CA_n8A-n78A | 1 |  | n8 | 5 | -97+TT | -97-2.76+TT |
|  |  |  | n78 | 10 | -95.8+10.8+TT | -95.8+10.8+TT |
|  | 2 |  | n8 | 5 | -97+TT | -97-2.76+TT |
|  |  |  | n78 | 100 | -85.5+1.4+TT | -85.5+1.4+TT |
|  | 3 |  | n8 | 5 | -97+8.3+TT | -97+8.3+TT |
|  |  |  | n78 | 10 | -95.8+TT | -95.8+TT |
| CA_n14A-n77A | 1 |  | n14 | 5 | -97.0 +TT | - |
|  |  |  | n77 | 10 | -95.3 +10.4 +TT | -95.3+10.4 +TT |
|  | 2 |  | n14 | 5 | -97.0 +TT | - |
|  |  |  | n77 | 100 | -85.1 +0.7+TT | -85.1 +0.7+TT |
|  | 3 |  | n14 | 5 | -97.0 + 5.5 +TT | - |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
|  | 4 |  | n14 | 5 | -97.0 + 31 +TT | - |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
|  | 5 |  | n14 | 10 | -93.8 + 28 +TT | - |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
| CA_n20A-n78A | 1 |  | n20 | 5 | -97.0+TT | - |
|  |  |  | n78 | 10 | -95.8+10.8+TT | -95.8+10.8+TT |
|  | 2 |  | n20 | 5 | -97.0+TT | - |
|  |  |  | n78 | 100 | -85.5+1.4+TT | -85.5+1.4+TT |
|  | 3 |  | n20 | 5 | -97.0 + 11 +TT | - |
|  |  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n25A-n66A | 1 |  | n66 | 5 | -99.5+TT | -99.5-2.7+TT |
|  |  |  | n25 | 5 | -96.5+20+TT | - |
|  | 2 |  | n66 | 5 | -99.5+23+TT | -99.5+23+TT |
|  |  |  | n25 | 5 | -96.5+TT | - |
|  | 3 |  | n66 | 5 | -99.5+4+TT | -99.5+4+TT |
|  |  |  | n25 | 5 | -96.5+TT | - |
| CA_n25A-n77A | 1 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n77 | 10 | -95.3+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n25 | 10 | -93.3+TT | - |
|  |  |  | n77 | 100 | -85.1+13.8+TT | -85.1+13.8+TT |
|  | 3 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n77 | 10 | -95.3+1.1+TT | -95.3+1.1+TT |
| CA_n25A-n78A | 1 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n78 | 10 | -95.8+23.9+TT | -95.3+23.9+TT |
|  | 2 |  | n25 | 10 | -93.3+TT | - |
|  |  |  | n78 | 100 | -85.5+13.8+TT | -85.5+13.8+TT |
|  | 3 |  | n25 | 5 | -96.5+TT | - |
|  |  |  | n78 | 10 | -95.8+1.1+TT | -95.3+1.1+TT |
| CA_n26A-n66A | 1 |  | n26 | 5 | -97.55 + 30 +TT | - |
|  |  |  | n66 | 5 | -99.5 +TT | -99.5-2.7 +TT |
| CA_n26A-n70A |  | 1 | n26 | 5 | -97.55 + 30 +TT | - |
|  |  |  | n70 | 5 | -92.7 + TT | -92.7 - 2.7 +TT |
| CA_n28A-n40A | 1 |  | n28 | 5 | -98.5+37.8+TT | -98.5+37.8+TT |
|  |  |  | n40 | 5 | -100.0+TT | -100.0-2.7+TT |
|  | 2 |  | n28 | 20 | -90.8+30.3+TT | -90.8+30.3+TT |
|  |  |  | n40 | 20 | -100.0+TT | -100.0-2.7+TT |
| CA_n28A-n77A | 1 |  | n28 | 5 | -98.5 +TT | -98.5-2.76+TT |
|  |  |  | n77 | 10 | -95.3 +10.4 +TT | -95.3+10.4 +TT |
|  | 2 |  | n28 | 5 | -98.5 +TT | -98.5 -2.76+TT |
|  |  |  | n77 | 100 | -85.4 +0.7 +TT | -85.4+0.7 +TT |
|  | 3 |  | n28 | 5 | -85.4 +0.7 +TT | -85.4+0.7 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3-2.2 +TT |
| CA_n28A-n78A | 1 |  | n28 | 5 | -98.5 +TT | -98.5 -2.76 +TT |
|  |  |  | n78 | 10 | -95.8 +10.4 +TT | -95.8+10.4 +TT |
|  | 2 |  | n28 | 5 | -98.5 +TT | -98.5-2.76 +TT |
|  |  |  | n78 | 100 | -85.5 +0.7 +TT | -85.5 +0.7 +TT |
|  | 3 |  | n28 | 5 | -98.5 + 31 +TT | -98.5 + 31 +TT |
|  |  |  | n78 | 10 | -95.8 +TT | -92.7 -2.2 +TT |
| CA_n29A-n71A | 1 |  | n29 | 5 | -97.0 + 17.5 + TT | - |
|  |  |  | n71 | 20 | -86.0 + TT | -86.0-2.76 + TT |
| CA_n30A-n66A | 1 |  | n30 | 10 | -95.8 + TT | -95.8 -2.7 + TT |
|  |  |  | n66 | 15 | -94.5 + 8.3 + TT | -94.5 + 8.3 + TT |
| CA_n30A-n77A | 1 |  | n30 | 5 | -99 + 10.4 + TT | -99 + 10.4 + TT |
|  |  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 2 |  | n30 | 10 | -95.8 + 8 + TT | -95.8 + 8 + TT |
|  |  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 3 |  | n30 | 5 | -99 + 8 + TT | -99 + 8 + TT |
|  |  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
| CA_n39A-n41A | 1 |  | n39 | 5 | -100 +TT | -100 -2.7 +TT |
|  |  |  | n41 | 5 | -94.8 +8.1 +TT | -94.8 +8.1 +TT |
|  | 2 |  | n39 | 5 | -100 +TT | -100 -2.7 +TT |
|  |  |  | n41 | 100 | -94.8 +3.3 +TT | -94.8 +3.3 +TT |
|  | 3 |  | n39 | 10 | -100 +4.3 +TT | -100 +4.3 +TT |
|  |  |  | n41 | 5 | -94.8+TT | -94.8-2.7+TT |
|  | 4 |  | n39 | 10 | -100 +0.8 +TT | -100 +0.8 +TT |
|  |  |  | n41 | 40 | -94.8+TT | -94.8-2.7+TT |
|  | 5 |  | n39 | 40 | -90.6 +TT | -90.6 -2.7 +TT |
|  |  |  | n41 | 10 | -94.8 +3.3 +TT | -94.8 +3.3 +TT |
|  | 6 |  | n39 | 100 | -100 +1.6 +TT | -100 +1.6 +TT |
|  |  |  | n41 | 10 | -84.5 +TT | -84.5 -2.7+TT |
| CA_n41A-n66A | 1 |  | n41 | 100 | -84.6+TT | -84.6-2.7+TT |
|  |  |  | n66 | 5 | -99.5+10.5+TT | -99.5+10.5+TT |
| CA_n41A-n71A | 1 |  | n41 | 10 | -94,8 + 10.8 + TT | -94,8+ 10.8 + TT |
|  |  |  | n71 | 5 | -97.2 + TT | -97.2-2.76 + TT |
|  | 2 |  | n41 | 100 | -84.6 + 1.4 + TT | -84.6+ 1.4 + TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2 -2.76+ TT |
| CA_n41A-n77A | 1 |  | n41 | 10 | -94.8+10.4+TT | -94.8+10.4+TT |
|  |  |  | n77 | 20 | -95.3+TT | -95.3-2.2+TT |
|  | 2 |  | n41 | 100 | -94.8+6.3+TT | -94.8+6.3+TT |
|  |  |  | n77 | 20 | -95.3+TT | -95.3-2.2+TT |
|  | 3 |  | n41 | 60 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n77 | 10 | -95.3+2.7+TT | -95.3+2.7+TT |
|  | 4 |  | n41 | 100 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n77 | 10 | -95.3+2.7+TT | -95.3+2.7+TT |
|  | 5 |  | n41 | 100 | -94.8+TT | -94.8-2.7+TT |
|  |  |  | n77 | 10 | -95.3+8.3+TT | -95.3+8.3+TT |
|  | 6 |  | n41 | 10 | -94.8+4.5+TT | -94.8+4.5+TT |
|  |  |  | n77 | 100 | -95.3+TT | -95.3-2.2+TT |
|  | 7 |  | n41 | 100 | -94.8+4.5+TT | -94.8+4.5+TT |
|  |  |  | n77 | 100 | -95.3+TT | -95.3-2.2+TT |
| CA_n48A-n66A | 1 |  | n48 | 5 | -99.0 + TT | -99.0 -2.2+ TT |
|  |  |  | n66 | 5 | -99.5 + 5.0 + TT | -99.5 +5.0+TT |
|  | 2 |  | n48 | 5 | -99.0+27.1 + TT | -99.0+ 27.1+TT |
|  |  |  | n66 | 5 | -99.5 +TT | -99.5 -2.7 +TT |
|  | 3 |  | n48 | 100 | -85.5 + 13.8 +TT | -85.5-2.2 + 13.8 +TT |
|  |  |  | n66 | 40 | -90.1 +TT | -90.1 -2.7 +TT |
|  | 4 |  | n48 | 10 | -95.8 + 1.9+ TT | -95.8+ 1.9 + TT |
|  |  |  | n66 | 5 | -99.5 +TT | -99.5 -2.7 +TT |
| CA_n48A-n70A | 1 |  | n48 | 10 | -95.8 + TT | -95.8-2.2 + TT |
|  |  |  | n70 | 15 MHz UL /25 MHz DL | -92.7 +26 +TT | -92.7 +26 +TT |
| CA_n66A-n71A | 1 |  | n66 | 5 | -99.5 +5 +TT | -99.5+5 +TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2-2.76 +TT |
| CA_n66A-n77A | 1 |  | n66 | 5 | -99.5+TT | -102.2+TT |
|  |  |  | n77 | 10 | -95.3 +23.9+TT | -95.3 +23.9+TT |
|  | 2 |  | n66 | 20 | -93.3+TT | -96.0+TT |
|  |  |  | n77 | 100 | -85.1 +13.8+TT | -85.1+13.8+TT |
|  | 3 |  | n66 | 5 | -99.5+TT | -102.2+TT |
|  |  |  | n77 | 10 | -95.3 +1.1+TT | -95.3+1.1+TT |
|  | 4 |  | n66 | 5 | -99.5 +31 +TT | -99.5+31 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
|  | 5 |  | n66 | 5 | -99.5 +5 +TT | -99.5+5 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
| CA_n66A-n78A | 1 |  | n66 | 5 | -99.5+TT | -99.5-2.7+TT |
|  |  |  | n78 | 10 | -95.8+23.9+TT | -95.8+23.9+TT |
|  | 2 |  | n66 | 20 | -93.3+TT | -93.3-2.7+TT |
|  |  |  | n78 | 100 | -85.5+13.8+TT | -85.5+13.8+TT |
|  | 3 |  | n66 | 5 | -99.5+TT | -99.5-2.7+TT |
|  |  |  | n78 | 10 | -95.8+1.1+TT | -95.8+1.1+TT |
| CA_n70A-n71A | 1 |  | n70 | 25 | -92.7 +4.1 +TT | -92.7+4.1 +TT |
|  |  |  | n71 | 5 | -94.0 +TT | -94.0-2.76 +TT |
|  | 2 |  | n70 | 5 | -100.0 +9.9 +TT | -100.0+9.9 +TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2-2.76 +TT |
|  | 3 |  | n70 | 5 | -100.0 +5 +TT | -100.0+5 +TT |
|  |  |  | n71 | 5 | -97.2 +TT | -97.2-2.76 +TT |
| CA_n71A-n77A | 1 |  | n71 | 5 | -97.2 +5.5 +TT | -97.2+5.5 +TT |
|  |  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
| CA_n71A-n78A | 1 |  | n71 | 5 | -97.2 +TT | -99.9 +TT |
|  |  |  | n78 | 10 | -95.8 +10.4 +TT | -95.8 +10.4 +TT |
|  | 2 |  | n71 | 5 | -97.2 +5.5 +TT | -97.2 +5.5 +TT |
|  |  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n78A-n79A | 1 |  | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  |  | n79 | 10 | -96.1 +2+TT | -96.1 +2+TT |
|  | 2 |  | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  |  | n79 | 40 | -89.6 +2+TT | -89.6 +2+TT |
|  | 3 |  | n78 | 10 | -96.1 +2.6+TT | -96.1+2.6+TT |
|  |  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 4 |  | n78 | 100 | -85.5+2.6+TT | -85.5+2.6+TT |
|  |  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. Note 6: 4 Rx operation is targeted for FWA form factor |  |  |  |  |  |  |

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter band CA with PC2 single UL carrier or PC2 2UL CA

| CA configuration | Test ID | NR band | CBW | REFSENS test requirement of NR band (dBm) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | 2Rx | 4Rx |
| CA_n1A-n78A | 1 | n1 | 5 | -100 +17.8+TT | -100 +17.8+TT |
|  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n2A-n77A | 4 | n2 | 5 | -98+9.1+TT | -98+9.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 5 | n2 | 5 | -91.8+6.7+TT | -91.8+6.7+TT |
|  |  | n774 | 10 | -92.2+TT | -92.2-2.2+TT |
|  | 6 | n2 | 5 | -98.0+32.1+TT | -98.0 +32.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3+TT |
|  | 7 | n2 | 5 | -98.0 +19.1+TT | -98.0 +19.1+TT |
|  |  | n774 | 10 | -95.3+TT | -97.5+TT |
|  | 9 | n2 | 5 | -98.0 +1.0 +TT | -98.0+1.0 +TT |
|  |  | n774 | 100 | -85.1 +TT | -87.3 +TT |
| CA_n3A-n41A | 1 | n3 | 5 | -97.0+18.4+TT | -97.0 +18.4+TT |
|  |  | n41 | 10 | -94.9 +TT | -94.9 -2.7 +TT |
|  | 2 | n3 | 5 | -97.0+2.3+TT | -97.0+2.3+TT |
|  |  | n41 | 100 | -84.6+TT | -84.6 -2.7+TT |
| CA_n3A-n78A | 6 | n3 | 5 | -97.0 + 8.1 +TT | -97.0 +8.1 +TT |
|  |  | n78 | 5 | -97.0 +TT | -97.0 -2.7 +TT |
|  | 7 | n3 | 40 | -89.4 + 1.0 +TT | -89.4 + 1.0+TT |
|  |  | n78 | 40 | -89.4 +TT | -89.4 -2.7 +TT |
| CA_n5A-n77A | 5 | n5 | 20 | -90.8 +4.3+TT | - |
|  |  | n77 | 20 | -92.2+TT | -94.4+TT |
|  | 6 | n5 | 10 | -98.0 +8.1+TT | - |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 7 | n5 | 5 | -98.0 +18.6+TT | - |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
| CA_n14A_n77A | 4 | n14 | 5 | -97.0 + 34 +TT | - |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 5 | n14 | 10 | -93.8 + 31 +TT | - |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
| CA_n30A-n77A | 1 | n30 | 5 | -99 + 13.2 + TT | -99 + 13.2 + TT |
|  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 2 | n30 | 10 | -95.8 + 10.6 + TT | -95.8 + 10.6 + TT |
|  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 3 | n30 | 5 | -99 + 17.6 + TT | -99 + 17.6 + TT |
|  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
| CA_n41A-n79A | 1 | n41 | 100 | -84.6+TT | -84.6 -2.7 +TT |
|  |  | n79 | 10 | -98.9+3.1+TT | -98.9+3.1+TT |
|  | 2 | n41 | 10 | -94.8+3.5+TT | -94.8+3.5+TT |
|  |  | n79 | 100 | -85.6+TT | -85.6 -2.2+TT |
| CA_n66A-n77A | 4 | n66 | 5 | -99.5 +34.33 +TT | -99.5 +34.33 +TT |
|  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
|  | 5 | n66 | 5 | -99.5 +11.27 +TT | -99.5+11.27 +TT |
|  |  | n77 | 10 | -95.3 +TT | -97.5+TT |
| CA_n78A-n79A | 1 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 10 | -96.1+5+TT | -96.1+5+TT |
|  | 2 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 40 | -89.6 +5+TT | -89.6+5+TT |
|  | 3 | n78 | 10 | -96.1+5.6+TT | -96.1+5.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 4 | n78 | 100 | -85.5+5.6+TT | -85.5+5.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 5 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 100 | -85.5+5+TT | -85.5+5+TT |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. Note 6: 4 Rx operation is targeted for FWA form factor |  |  |  |  |  |

Table 7.3A.1_1.5-2: Reference sensitivity requirement for intraband non-contiguous CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| CA_n71(2A) | 1 | n71 | 15 | -97.2 +TT for PCC -97.2 + 4.0 +TT for SCC |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n71 | 15 |  | -94.0 +22.2 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n71 | 15 |  | -94.0 +5.2 +TT | -91.6 +TT |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |