3GPP TSG-RAN5 Meeting #104	R5-244694

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3285 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Physical layer parameters for DCI format 1_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Default physical layer parameters for DCI format 1_3 need to be added for the singleDCI test case 7.1.1.2.8.x,7.1.1.2.9.x,7.1.1.7.2.x and 7.1.1.7.4.x. DCI format 1_3 is used for the scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell. It is primarily used in CA scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Physical layer parameters for DCI format 1_3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MC_enh-UEConTest WI is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6.1.2.4	Physical layer parameters for DCI format 1_3

DCI format 1_3 is used for the scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell.

Default physical layer parameters for DCI format 1_3 are specified in table 4.3.6.1.2.4-1.

Table 4.3.6.1.2.4-1: Physical layer parameters for DCI format 1_3

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Identifier for DCI formats | Indicating a DL DCI format | “1” |  |
| Scheduled cell set indicator | Not present (0 bits, one entry is configured in MC-DCI-SetofCellsToAddModList as per Table 4.6.3-167) | - |  |
| Scheduled cells indicator | Not present (0 bits, scheduledCellComboListDCI-1-3 is not configured) | - | FDRA |
|  | The size is dependent on the number of entries configured in scheduledCellComboListDCI-1-3 The value is dependent on test parameters. | - | CellInd |
| Bandwidth part indicator | Not present | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | The first entry configured in tdra-FieldIndexListDCI-1-3 | “00” |  |
| VRB-to-PRB mapping | Not present (0 bits, vrb-ToPRB-Interleaver = Not present as per Table 4.6.3-100) | - |  |
| PRB bundling size indicator | Not present (semi-static PRB bundling) | - |  |
| Rate matching indicator | Not present(0 bits, rateMatchListDCI-1-3 is not configured) | - |  |
| ZP CSI-RS trigger | Not present(0 bits, zp-CSI-RSListDCI-1-3 is not configured) | - |  |
| Modulation and coding scheme (TB1) | Dependent on test parameters | - |  |
| New data indicator (TB1) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB1) | Dependent on test parameters | - |  |
| Modulation and coding scheme (TB2) | Dependent on test parameters | - |  |
| New data indicator (TB2) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB2) | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| Downlink assignment index | Dependent on test parameters | - |  |
| TPC command for scheduled PUCCH | 0 dB (accumulated TPC) as per Table 7.2.1-1 in TS 38.213 [22] | “01” |  |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “110” |  |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 |  |  |
|  | µ=0 (SCS=15kHz): K1=7 | “101” |  |
|  | µ=1 (SCS=30kHz): K1=5 | “011” |  |
|  | µ=3 (SCS=120kHz): K1=8 | “110” |  |
| One-shot HARQ-ACK request | Not present (0bit, pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3 and pdsch-HARQ-ACK-enhType3DCI-1-3 are not configured) | - |  |
| Enhanced Type 3 codebook indicator | Not present(0bit , pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3 is not configured) | - |  |
| HARQ-ACK retransmission indicator | Not present(0bit , pdsch-HARQ-ACK-retxDCI-1-3 is not configured) | - |  |
| Antenna port(s) | DMRS port 0 as per Table 7.3.1.2.2-1 in TS 38.212 [27] (dmrs-Type = DMRS type 1 and maxLength = len1 as per Table 4.6.3-50) | “0000” |  |
|  |  | “0000 0000” | DL_CELL2 |
| Transmission configuration indication | Not present (0bit, tci-PresentInDCI is not enabled) | - |  |
| SRS request | Not present (0bit, there is one entry in the higher layer parameter srs-RequestListDCI-1-3-r18) | - |  |
| SRS offset indicator | Not present (0bit, srs-OffsetListDCI-1-3 is not configured) | - |  |
| DMRS sequence initialization | fix length of 1 bit; '0'B for DMRS-DownlinkConfig.scramblingID0 (or physCellId if scramblingID0 is not present); see Table 4.6.3-50 | “0” |  |
| Priority indicator | Not present (0bit, priorityIndicatorDCI-1-3 is not configured) | - |  |
| ChannelAccess-CPext | Not present (0bit, channelAccessMode-r16 = "semiStatic" is not provided) | - |  |
| Minimum applicable scheduling offset indicator | Not present (0bit, minimumSchedulingOffsetK0DCI-1-3 is not configured) | - |  |
| SCell dormancy indication | Not present (0bit, dormancyDCI-1-3 is not configured) | - |  |
| PDCCH monitoring adaptation indication | Not present (0bit, pdcchMonAdaptDCI-1-3 is not enabled) | - |  |
| PUCCH Cell indicator | Not present (0bit, pucch-sSCellDynDCI-1-3 is not configured) | - |  |
| NOTE 1: NcellDL is defined in 38.212 clause 7.3.1.2.4. |  |  |  |

| Condition | Explanation |
| --- | --- |
| CellInd | The co-scheduled cell indication scheme based on Scheduled cells indicator filed of DCI format 1_3. |
| FDRA | The co-scheduled cell indication scheme based on FDRA filed of DCI format 1_3. |
| DL_CELL2 | NcellDL=2. NcellDL is defined in 38.212 [27] clause 7.3.1.2.4. |

<End of modified section 1>