3GPP TSG-RAN5 Meeting #104	R5-244702

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4570 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new single DCI test case 7.1.1.7.2 - Reference cell is scheduling cell - DCI format 1_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test configuration: DL CA test case, DCI 1-3 is configured(i.e DCI 0-3 is not configured), and multi–Cell PDSCH are scheduled based on FDRA field or Scheduled Cell indicator field. In addition, reference cell = scheduling cell = SpCell = NR Cell 1, while SpCell (NR Cell 1) and SCell (NR Cell 3) are co-scheduled Cell. The DCI size for USS DCI 0_1/DCI 1_1 may update in NR Cell 1 to satisfy the requirement in 38.212 “the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell” : - Before single DCI is configured: UE have to monitor CSS DCI 0_0/DCI 1_0 and USS DCI 0_1/DCI 1_1. The total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for NR Cell 1 DCI size 1: CSS DCI 0_0/DCI 1_0 DCI size 2: USS DCI 0_1 DCI size 3: USS DCI 1_1 - After single DCI(DCI 1-3) is configured: UE have to monitor CSS DCI 0_0/DCI 1_0 , USS DCI 0_1/DCI 1_1 and USS DCI 1_3. The total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for NR Cell 1 DCI size 1: CSS DCI 0_0/DCI 1_0 DCI size 2: USS DCI 0_1/DCI 1_1 with padding added according to 38.212 clause 7.3.1.0 step 4C DCI size 3: USS DCI 1_3 - Test case is added to test: - When SCell is deactivated, UE does not stop monitoring DCI format 1_3 on the SpCell (NR Cell 1). - When SCell is activated, UE receive DCI format 1_3 for SCell (NR Cell3) and start the sCellDeactivationTimer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new SingleDCI test case 7.1.1.7.2 Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MC_enh-UEConTest WP is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0497 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.7.2	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer

7.1.1.7.2.1	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.2.1.1	Test Purpose (TP)

(1)

with( UE in RRC_CONNECTED state with SCell activated, and the search space for single DCI is only configured in scheduling cell (i.e the Reference cell is scheduling cell) )

ensure that {

when{ UE receives a DCI format 1-3 on NR SpCell PDCCH to schedule SCell }

then {  UE restarts the sCellDeactivationTimer }

}

(2)

with ( UE in RRC_CONNECTED state with SCell activated, and the search space for single DCI is only configured in scheduling cell (i.e the Reference cell is scheduling cell))

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE deactivates the SCell and does not stop monitoring PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell (i.e does not stop monitoring DCI format 1_3 on the scheduling cell for the set of cells) }

}

7.1.1.7.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.9 and TS 38.212, clauses 7.3.1.0. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

…

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

Step 4:

-	Otherwise

Step 4A:

…

Step 4B:

…

Step 4C:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps

-	If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1.

-	If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1.

7.1.1.7.2.1.3	Test description

7.1.1.7.2.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.7.2.1.3.1-1 and in additionNR Cell 3 is configured as NR Active SCell.

Table 7.1.1.7.2.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.2.1.3.2	Test procedure sequence

Table 7.1.1.7.2.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.2.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR SpCell (NR Cell 1) to configure NR SCell (NR Cell 3) and DCI format 1_3. (Note 1) (Note 3) | <-- | (RRCReconfiguration) | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. (Note 2) (Note 4) | --> | (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 4 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 5 | 200 ms after step 4, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | DCI format 1_3 (only SCell is scheduled) | - | - |
| 6 | The SS transmits a MAC PDU (containing just padding or RLC status PDU, but no RLC data PDU) on NR SCell. | <-- | MAC PDU | - | - |
| 7 | 400 ms after step 4 the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | DCI format 1_3 (only SCell is scheduled) | - | - |
| 8 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SCell. | <-- | MAC PDU | - | - |
| 9 | Check: Does the UE transmit a Scheduling Request on PUCCH on NR SpCell? | --> | (SR) | 1 | P |
| 10 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 11 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 8 on NR SpCell. | --> | MAC PDU | - | - |
| 12 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 13 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 11 on NR SpCell. | <-- | MAC PDU | - | - |
| 14 | 400 ms (sCellDeactivationTimer = 320 ms) after step 7, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 15 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 2 | P |
| 17 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 18 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 15 on NR SpCell. | --> | MAC PDU | - | - |
| 19 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 20 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 18 on NR SpCell. | <-- | MAC PDU | - | - |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is also NR SpCell (NR Cell 1). Therefore, the single DCI (DCI format 1_3) is counted on the NR Cell 1, and DCI size alignment procedure specified in 38.212 clause 7.3.1.0 for NR Cell 1 is complete in step4C. Note 4: For SA the NR RRCReconfiguration message is scheduled by DCI format 0_0 to avoid DCI size disalignment for DCI format 0_1 between UE and SS temporarily. |  |  |  |  |  |

7.1.1.7.2.1.3.3	Specific message contents

Table 7.1.1.7.2.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.2.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-2: CellGroupConfig (Table 7.1.1.7.2.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 7.1.1.7.2.1.3.3-5 |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.2.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL and SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.2.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-3: ServingCellConfig (Table 7.1.1.7.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition DCI 1_3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.2.1.3.3-4 |  |
| mc-DCI-SetOfCellsToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18 { | 1 entry |  |  |
| MC-DCI-SetOfCells-r18[1] SEQUENCE { |  |  |  |
| scheduledCellComboListDCI-1-3 | Not present |  | px_coScheduledCellIndicationScheme_DCI_1_3 = FDRA |
| scheduledCellComboListDCI-1-3 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entris |  | px_coScheduledCellIndicationScheme_DCI_1_3 = CellInd |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.7.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-5: PhysicalCellGroupConfig (Table 7.1.1.7.2.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  | px_harqFeedbackType_DCI_1_3 =Type2 |
|  | semiStatic |  | px_harqFeedbackType_DCI_1_3 =Type1 |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-6: ServingCellConfig (Table 7.1.1.7.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition No_UL and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.2.1.3.3-7 |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-7: BWP-DownlinkDedicated (Table 7.1.1.7.2.1.3.3-6)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-8: Physical layer parameters for DCI format 1_3 (steps 5, 7, 12, 14 and 19 for px_coScheduledCellIndicationScheme_DCI_1_3 =CellInd, Table 7.1.1.7.2.1.3.2-2)

| Derivation path: 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The frist entry of scheduledCellComboListDCI-1-3 | “00” | Step 12,14,19 |
|  | The second entry of scheduledCellComboListDCI-1-3 | “01” | Step 5,7 |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 Note: NcellDL=1 | - | Step 12,14,19 |
|  | block number 1(SCell) : dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 Note: NcellDL=1 | - | Step 5,7 |

Table 7.1.1.7.2.1.3.3-9: Physical layer parameters for DCI format 1_3 (steps 5, 7, 12, 14 and 19 for px_coScheduledCellIndicationScheme_DCI_1_3 =FDRA, Table 7.1.1.7.2.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - | Step 12,14,19 |
|  | block number 1(SpCell): all bits set to 1 block number 2(SCell) : dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 Note: NcellDL=2 | - | Step 5,7 |

7.1.1.7.2.2	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Inter-Band CA

The scope and description of the present TC is the same as test case 7.1.1.7.2.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

- 	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

7.1.1.7.2.3	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.7.2.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

<End of modified section 1>