33GPP TSG-RAN5 Meeting #104	R5-244708

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

Source:	Huawei, HiSilicon, MCC TF160

Title:	Discussion to extend the scope for MBS Broadcast test case

Agenda item:	6.4.7

Document for:	Endorsement

Discussion

Observation 1(From MCC TF160): SearchspaceMCCH/ searchspaceMTCH are set to 1 (same as searchspace for CSS) in all MBS broadcast test case , so the scenario for SearchspaceMCCH/ searchspaceMTCH set to zero is not tested.

[38.331 5.9.1.2]

PDCCH monitoring occasion(s) for MCCH transmission are determined according to the common search space indicated by searchspaceMCCH. If searchspaceMCCH is set to zero, PDCCH monitoring occasions for MCCH message reception in the MCCH transmission window are the same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchspaceMCCH is not set to zero, PDCCH monitoring occasions for MCCH message are determined based on search space indicated by searchspaceMCCH.

Observation 2(From HiSilicon): It is possible that PCell and SCell have different PLMN list (for example, PCell PLMN list is PLMN1, while SCell PLMN list is PLMN1 and PLMN2, and the MBS is broadcasted in PLMN2 ), so RAN2 add plmn-IdentityInfoList-r17 in SCellConfig to indicate the PLMN list for SCell. Plmn-IdentityInfoList-r17 absent scenario has been tested in the 14.1.1.4.x, but plmn-IdentityInfoList-r17 present scenario has not been tested in any MBS broadcast test case.

[38.331]

SCellConfig ::=                     SEQUENCE {

sCellIndex                      SCellIndex,

sCellConfigCommon               ServingCellConfigCommon      OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated            ServingCellConfig      OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                  OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}       OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}            OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17)) OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17 OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }      OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17    SetupRelease {PLMN-IdentityInfoList}   OPTIONAL, -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17     SetupRelease {NPN-IdentityInfoList-r16} OPTIONAL -- Cond SCellSIB20-Opt

]]

}

npn-IdentityInfoList This field is used to transfer npn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to SNPN Identity. If this field and plmn-IdentityInfoList are both absent, the UE uses the npn-IdentityInfoList in SIB1 of the PCell.

plmn-IdentityInfoList This field is used to transfer plmn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to PLMN Identity. If this field and npn-IdentityInfoList are both absent, the UE uses the plmn-IdentityInfoList in SIB1 of the PCell.

| Conditional Presence | Explanation |
| --- | --- |
| SCellSIB20-Opt | This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise. |

[38.508-1 4.6]

Table 4.6.7-9: TMGI

| Derivation Path: TS 38.331 [6], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |
| plmn-Index-r17 | 1 |  |  |
| } |  |  |  |
| serviceId-r17 | ‘000001’H | OCTET STRING (SIZE (3)) | MBS_Broadcast |
|  | ‘000101’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MBS_Broadcast | MBS Broadcast service |

Proposal

Based on the above discussion, the followings are proposed:

Proposal 1: Update the title/TP and message content for one MBS Broadcast test case (i.e 38.523-1 14.1.1.2) to cover the searchspaceMCCH and searchspaceMTCH set to zero scenario.

The updated title and TP are highlight below:

14.1.1.2	MBS Broadcast/ MCCH Information Acquisition/ becoming interested to receive MBS broadcast services / searchspaceMCCH and searchspaceMTCH set to zero

14.1.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and camped on a cell providing SIB20 and not interested to receive MBS broadcast services and searchspaceMCCH/ searchspaceMTCH set to zero }

ensure that {

when { UE is becoming interested to receive one MBS broadcast service }

then { UE acquires the MBSBroadcastConfiguration message and starts MBS reception for this MBS broadcast service }

}

Proposal 2: Update theTP/test procedure and message content for MBS Broadcast CA test case (i.e 14.1.1.4) to cover the plmn-IdentityInfoList-r17 present scenario.

The updated TP and new TP are highlight below:

14.1.1.4	MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling

14.1.1.4.1	MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling / Intra-band Contiguous CA

14.1.1.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC CONNECTED state with SCell configured and interested to receive MBS broadcast services }

ensure that {

when { UE is receiving SIB20 for the Scell via dedicated signalling and plmn-IdentityInfoList-r17 is not present in the SCellConfig }

then { UE starts acquiring the MBSBroadcastConfiguration message on MCCH in the Scell at the next repetition period and starts MBS reception }

}

(2)

with { UE in NR RRC CONNECTED state with SCell configured and interested to receive MBS broadcast services }

ensure that {

when { UE is receiving SIB20 for the Scell via dedicated signalling and plmn-IdentityInfoList-r17 is present in the SCellConfig }

then { UE starts acquiring the MBSBroadcastConfiguration message on MCCH in the Scell at the next repetition period and starts MBS reception }

}