3GPP TSG-RAN5 Meeting #104	R5-244722

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3293 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of BWP-UplinkDedicated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - UE does not support PUCCH on SCell when the pc_twoPUCCH_group set to false. The default setting for UL CA does not need to configure PUCCH on Scell. If this feature need to be tested on a specific signalling test case, it can be configured in the test case. twoPUCCH-Group Indicates whether two PUCCH group in CA with a same numerology across CCs for data and control channel [at a given time] is supported by the UE. For NR CA, two PUCCH group is supported with the same numerology across NR carriers for data and control channel at a given time. For (NG)EN-DC/NE-DC, two PUCCH group is supported with the same numerology across NR carriers for data and control channel at a given time, wherein an NR PUCCH group is configured in FR1 and another NR PUCCH group is configured in FR2. The UE supports two PUCCH groups with PUCCH on a band X and a band Y if it sets this capability parameter for both band X and band Y. 2.The comma between NoPucchOnNulRF is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the PUCCH on SCell default configuration in 38.508-1 for signalling test case. - Correct the typo, update “NoPucchOnNulRF” to NoPucchOnNul,RF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE does not support PUCCH on SCell may fail in UL CA test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	BWP-UplinkDedicated

Table 4.6.3-15: BWP-UplinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config |  |  |
| release | NULL |  | SUL_NUL AND PucchOnNul |
| } |  |  |  |
| pucch-Config | Not present |  | SUL_NUL AND NoPucchOnNul,(RF OR SIG) AND SCell_add |
| pusch-Config | Not present |  | RESUME, SUL_NUL AND NoPuschOnNul AND pc_dynamicSwitch_SUL=False |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| release | NULL |  | SUL_NUL AND PuschOnNul AND pc_dynamicSwitch_SUL=False |
| } |  |  |  |
| configuredGrantConfig | Not present |  |  |
|  | ConfiguredGrantConfig |  | CG_Config |
| srs-Config | Not present |  | Short_DCI |
|  | SRS-Config |  |  |
| beamFailureRecoveryConfig | Not present |  |  |
| sl-PUCCH-Config-r16 | Not present |  |  |
| sl-PUCCH-Config-r16 CHOICE { |  |  | SIDELINK |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| cp-ExtensionC2-r16 | Not present |  |  |
| cp-ExtensionC3-r16 | Not present |  |  |
| useInterlacePUCCH-PUSCH-r16 | Not present |  |  |
| pucch-ConfigurationList-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 CHOICE { |  |  | SharedSpectrum |
| setup | LBT-FailureRecoveryConfig |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r16 | Not present |  |  |
| configuredGrantConfigToReleaseList-r16 | Not present |  |  |
| configuredGrantConfigType2DeactivationStateList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |
| RESUME | Used in RRCResume Message |
| SIDELINK | Used for sidelink communication |
| SCell_add | Add SCell |
| SharedSpectrum | Operation with shared spectrum channel access |
| CG_Config | Configured Grant Configuration |
| PucchOnNul | PUCCH-Config has been configured previously on the NUL carrier before supplementary carrier is configured |
| NoPucchOnNul | PUCCH-Config has not been configured previously on the NUL carrier before supplementary carrier is configured |
| PuschOnNul | PUSCH-Config has been configured previously on the NUL carrier before supplementary carrier is configured |
| NoPuschOnNul | PUSCH-Config has not been configured previously on the NUL carrier before supplementary carrier is configured |

<End of modified section 1>