3GPP TSG-RAN5 Meeting #104	R5-244793

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4597 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RRC test case 8.1.6.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per step 5 and 7, IP packets are looked back only on one DRB. Hence, there is no packet on other DRB in step 5A and 8 respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated step 5A and 8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this case unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

###### 8.1.6.1.1.2	Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB

8.1.6.1.1.2.1	Test Purpose (TP)

(1)

with {UE supporting UL PDCP Packet Delay per DRB measurement and in RRC_CONNECTED state}

ensure that {

when {UE receives RRCReconfiguration containing ReportConfigNR IE with ul-DelayValueConfig IE set to “setup”}

then {UE sends the RRCReconfigurationComplete message}

}

(2)

with {UE in RRC_CONNECTED and measurement configured for UL PDCP Packet Delay per DRB and having DRBs established}

ensure that {
  when {UE determines that UL PDCP SDUs per DRB}

then {UE sends the MeasurementReport message with ul-PDCP-DelayResultList for corresponding DRB}

}

(3)

with { UE in RRC_CONNECTED and measurement configured for UL PDCP Packet Delay per DRB}

ensure that {

when {UE receives RRCReconfiguration containing ReportConfigNR IE with ul-DelayConfig IE set to “release”)

then {UE stops monitoring the UL PDCP Packet Delay per DRB}

}

8.1.6.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.5.3.1, 5.5.4.1, 5.5.5.1 & TS 38.314, clause 4.3.1.1

[TS 38.331, clause 5.5.3.1]

…

The UE shall:

…

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the ul-DelayValueConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding average UL PDCP packet delay measurement per DRB;

…

[TS 38.331, clause 5.5.4.1]

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the corresponding reportConfig includes the ul-DelayValueConfig:

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided by all lower layers of the associated DRB identity;

…

[TS 38.331, clause 5.5.5.1]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

…

1>	if average uplink PDCP delay values are available:

2>	set the ul-PDCP-DelayValueResultList to include the corresponding average uplink PDCP delay values;

…

[TS 38.314, clause 4.3.1.1]

The objective of this measurement performed by UE is to measure Packet Delay in Layer PDCP for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501.

Protocol Layer: PDCP

Table 4.3.1.1-1: Definition for UL PDCP Packet Average Delay per DRB per UE

| Definition | PDCP Packet Delay in the UL per DRB. This measurement refers to PDCP queuing delay for DRBs in the UE, which captures the delay from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant). The measurement is done separately per DRB. Detailed Definition: MT,drbid=∀itDelivi, drbid-tArrivali, drbidIT, where explanations can be found in the table 4.3.1.1-2 below. |
| --- | --- |

NOTE:	UE measures UL PDCP queueing delay at DRB level. It is up to gNB to convert DRB level delay to QoS level delay with the assumption that all QoS flows mapped to the same DRB get the same QoS treatment, and it is up to gNB to calculate QoS level delay if multiple DRBs mapped with the same QoS.

Table 4.3.1.1-2: Parameter description for UL PDCP Packet Average Delay per DRB per UE

| M(T,drbid) | PDCP average delay in the UL per DRB, averaged during time period 𝑇. Unit: 0.1 ms. PDCP average delay in the UL per DRB is 1s if the actual value is larger than 1s. |
| --- | --- |
| tArrival(i) | The point in time when the UL PDCP SDU i arrivals at PDCP upper SAP. |
| tDeliv(i) | The point in time when the UL MAC PDU k including the first part of UL PDCP SDU i is scheduled for transmission. |
| i | A UL PDCP SDU that is received by the PDCP during time period 𝑇. |
| I(T) | Total number of UL PDCP SDUs received during time period 𝑇. |
| T | Time Period during which the measurement is performed |
| drbid | The identity of the measured DRB. |

8.1.6.1.1.2.3	Test description

8.1.6.1.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None

Preamble:

-	The UE is in 5GS state 3N-A with one PDU session active according to TS 38.508-1 [4], clause 4.4A.3 Table 4.4A.3-1.and using the message condition UE TEST LOOP MODE B active. 3 DRBs are configured where DRB#j is defined as default DRB.

8.1.6.1.1.2.3.2	Test procedure sequence

Table 8.1.6.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup ul-DelayValueConfig | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
|  | EXCEPTION: In parallel with steps 3-9, parallel behaviour defined in table 8.1.6.1.1.2.3.2-3 is executed to receive periodic measurement report. |  |  |  |  |
| 3 | The SS transmits UL grant to the UE at every 5ms in PDCCH occasion |  |  |  |  |
| 4 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #2 |  |  |  |  |
| 5 | The UE loop backs the IP packet received in step 4 on DRB#j+1 |  |  |  |  |
| 5A | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayValueResult with averageDelay set to non zero value for DRB#j+1 and zero for DRB# j+2? | --> | NR RRC: MeasurementReport | 2 | P |
| 6 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #3 |  |  |  |  |
| 7 | The UE loop backs the IP packet received in step 6 on DRB# j+2 |  |  |  |  |
| 8 | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayValueResult for DRB# j+1 and with with averageDelay set to non zero value DRB# j+2 and zero for DRB#j+1? | --> | MeasurementReport | 2 | P |
| 9 | The SS transmits an RRCReconfiguration message including MeasConfig to release the ul-DelayValueConfig measurement | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete |  |  |
| - | EXCEPTION: In parallel with steps 11-14, parallel behaviour defined in table 8.1.6.1.1.2.3.2-2 is executed. | - | - | - | - |
| 11 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #2 |  |  |  |  |
| 12 | The UE loop backs the IP packet received in step 11 on DRB# j+1 |  |  |  |  |
| 13 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #3 |  |  |  |  |
| 14 | The UE loop backs the IP packet received in step 13 on DRB#j+2 |  |  |  |  |
| 15 | Void | - | - | - | - |

Table 8.1.6.1.1.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayValueResult? | --> | MeasurementReport | 3 | F |

Table 8.1.6.1.1.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmit a MeasurementReport message | --> | MeasurementReport | - | - |

8.1.6.1.1.2.3.3	Specific message contents

Table 8.1.6.1.1.2.3.3-1: RRCReconfiguration (step 1 and 9, Table 8.1.6.1.1.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS

Table 8.1.6.1.1.2.3.3-2: MeasConfig (Table 8.1.6.1.1.2.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1.. maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR(57) | Thres=57(-100dBm ≤ SS-RSRP<-99dBm) |  |
|  | MeasObjectNR(56 + Delta(NRfs)) | SS RSRP corresponding to -100dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig | ReportConfigNR- PERIODICAL |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-3: ReportConfigNR- PERIODICAL (Table 8.1.6.1.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-142 with condition PERIODICAL, MDT_DELAY |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ul-DelayValueConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  | Step1 |
| delay-DRBlist SEQUENCE (SIZE(1..maxDRB)) OF DRB-Identity { | 2 entries |  |  |
| DRB-Identity[1] | j+1 | entry 1 |  |
| DRB-Identity[2] | j+2 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| Release | NULL |  | Step9 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-4: MeasurementReport (steps 5A and 8, Table 8.1.6.1.1.2.3.2-1 and step 1, Table 8.1.6.1.1.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE { | 1 entry |  |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | Physical CellID of the NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-PDCP-DelayValueResultList-r16 SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16{ | 2 entries | Step 5A, Table 8.1.6.1.1.2.3.2-1 |  |
| drb-Id-r16[1] | j+1 | entry 1 |  |
| averageDelay-r16[1] | Any value other than 0 |  |  |
| drb-Id-r16[2] | j+2 | entry 2 |  |
| averageDelay-r16[2] | 0 |  |  |
| } |  |  |  |
| ul-PDCP-DelayValueResultList-r16 SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16{ | 2 entries | Step8, Table 8.1.6.1.1.2.3.2-1 |  |
| drb-Id-r16[1] | j+1 | entry 1 |  |
| averageDelay-r16[1] | 0 |  |  |
| drb-Id-r16[2] | j+2 | entry 2 |  |
| averageDelay-r16[2] | Any value other than 0 |  |  |
| } |  |  |  |
| ul-PDCP-DelayValueResultList-r16 | Present | Step 1, Table 8.1.6.1.1.2.3.2-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-5: RRCReconfiguration (Preamble, Table 8.1.6.1.1.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-3DRBs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-3DRBs | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-6: RadioBearerConfig-3DRBs (Table 8.1.6.1.1.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition SRB2 and DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 3 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config::= SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | true |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBj | j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBj+1 |  |  |
| } |  |  |  |
| } |  |  |  |
| DRB-ToAddMod[3] SEQUENCE |  | entry 3 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBj+2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-7: CellGroupConfig-3DRBs (Table 8.1.6.1.1.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 and condition SRB2_DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 4 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRBj | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBj+1 | entry 3 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBj+2 | entry 4 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-8: PDU SESSION ESTABLISHMENT ACCEPT (Preamble) (Table 8.1.6.1.1.2.3.3-1)

| Derivation Path: TS 38.508-1, table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| Authorized QoS rules | 3 entries |  |  |
| QoS rule [1] | Reference QoS rule #1 as defined in Table 4.8.2.1-1. | QFI=1 |  |
| QoS rule [2] | Reference QoS rule #4 as defined in Table 4.8.2.1-4 except DQR bit set to ‘0’B. | QFI=2 |  |
| QoS rule [3] | Reference QoS rule #5 as defined in Table 4.8.2.1-5. | QFI=5 |  |
| Mapped EPS Bearer contexts | Not Present |  |  |
| Authorized QoS flow descriptions | 3 entries |  |  |
| QoS flow [1] | Reference QoS flow #1 as defined in Table8.1.5.4.1.3.3-7. | QFI=1 |  |
| QoS flow [2] | Reference QoS flow #2 as defined in Table8.1.5.4.1.3.3-8. | QFI=2 |  |
| QoS flow [3] | Reference QoS flow #3 as defined in Table 4.8.2.3-3. | QFI=5 |  |

<End of Change>