3GPP TSG-RAN5 Meeting #104	R5-244853

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0685 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SIG PICS for Further NR mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Further NR mobility enhancements test cases, new SIG PICS are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PICS were added for Further NR mobility enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of PICS for Further NR mobility enhancements TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.3.8	Mobility Capabilities

Table A.4.3.8-1: UE Mobility Capabilities

| Item | UE Mobility Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support inter-RAT Handover to EUTRA connected to EPC | 38.306, 4.2.9 | Rel-15 | pc_interRAT_EUTRA_Handover | Yes |  |  |
| 2 | Support inter-frequency Handover from the corresponding duplex mode or from the corresponding frequency range. | 38.306, 4.2.9 | Rel-15 | pc_handoverInterF | Yes |  |  |
| … |  |  |  |  |  |  |  |
| 36 | Supports Event A4 based conditional handover in NTN bands, i.e., CondEvent A4 | 38.306, 4.2.7.2 | Rel-17 | pc_eventA4BasedCondHandover_r17 | No |  | A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. |
| XX1 | Support conditional handover with candidate SCG, where conditional NR PSCell change is supported for FDD-FR1 bands, TDD-FR1 bands, TDD-FR2-1 bands and TDD-FR2-2 bands | 38.306, 4.2.7.2 | Rel-18 | pc_condHandoverWithCandSCG_change_r18 | No |  | The UE indicating support of this feature shall also indicate the support of condHandover-r16 and support of at least one NR-DC band combination. |
| XX2 | Support conditional handover with candidate NR PSCell addition | 38.306, 4.2.9a | Rel-18 | pc_condHandoverWithCandSCG_Addition_r18 | No |  | The UE indicating support of this feature shall also indicate the support of condHandover-r16 and support of at least one NR-DC band combination. |
| XX3 | Support conditional handover with candidate SCG, where conditional NR PSCell change is supported between FDD and TDD | 38.306, 4.2.9a | Rel-18 | pc_condHandoverWithCandSCG_FDD_TDD_change_r18 | No |  | The parameter can only be set if condHandoverWithCandSCG-change-r18 is set for both FDD and TDD. |
| XX4 | Support conditional handover with candidate SCG, where conditional NR PSCell change is supported between FR1 and FR2 | 38.306, 4.2.9a | Rel-18 | pc_condHandoverWithCandSCG_FR1_FR2_change_r18 | No |  | The parameter can only be set if condHandoverWithCandSCG-change-r18 is set for both FR1 and FR2. |
| XX5 | Support Subsequent CPAC for MN initiated subsequent conditional PSCell change or addition in NR-DC, which is configured by NR conditionalReconfiguration using MN configured measurement as the initial triggering condition and using candidate SN configured measurement as the following triggering condition | 38.306, 4.2.9a | Rel-18 | pc_mn_ConfiguredMN_TriggerSCPAC_r18 | No |  | The parameter can only be set if sn-InitiatedCondPSCellChangeNRDC-r17, mn-InitiatedCondPSCellChangeNRDC-r17 and condPSCellAdditionNRDC-r17 are supported. |
| XX6 | Support Subsequent CPAC for MN initiated subsequent conditional PSCell change or addition in NR-DC, which is configured by NR conditionalReconfiguration using MN configured measurement as the initial triggering condition and using candidate SN configured measurement as the following triggering condition, after the SCG from a previous SCPAC configuration is released | 38.306, 4.2.9a | Rel-18 | pc_mn_ConfiguredMN_TriggerSCPAC_afterSCG_release_r18 | No |  | A UE indicating support for this feature shall indicate support of mn-ConfiguredMN-TriggerSCPAC-r18. |
| XX7 | Support reference configuration for mn-ConfiguredMN-TriggerSCPAC-r18 and mn-ConfiguredSN-TriggerSCPAC-r18 | 38.306, 4.2.9a | Rel-18 | pc_mn_ConfiguredReferenceConfigSCPAC_r18 | No |  |  |
| XX8 | Support Subsequent CPAC as defined in TS 38.331 [9] for initial MN configured subsequent conditional PSCell change in NR-DC, which is configured by NR conditionalReconfiguration using SN configured measurement as the initial triggering condition | 38.306, 4.2.9a | Rel-18 | pc_mn_ConfiguredSN_TriggerSCPAC_r18 | No |  | The parameter can only be set if sn-InitiatedCondPSCellChangeNRDC-r17 is supported. |
| XX9 | Support reference configuration for sn-Configured-SCPAC-r18 | 38.306, 4.2.9a | Rel-18 | pc_sn_ConfiguredReferenceConfigSCPAC_r18 | No |  |  |
| XXA | Support Subsequent CPAC for SN configured subsequent conditional PSCell change (intra-SN) in NR-DC | 38.306, 4.2.9a | Rel-18 | pc_sn_ConfiguredSCPAC_r18 | No |  | The parameter can only be set if condPSCellChange-r16 is supported. |

[End of change]