3GPP TSG-RAN5 Meeting #104	R5-244869

# Maastricht, Netherlands, Aug 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2927 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test case 7.6C.3_1.2 Out-of-band Blocking for SUL with 3 DL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There’s no applicable in-band blocking test case for SUL operation with 3 DL CA configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 7.6C.3_1.2 Out-of-band Blocking for SUL with 3 DL CA, and adding test requirements for CA_n41C_n79A-n83A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The out-of-band Blocking blocking for CA_n41C_n79A-n83A can’t be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6C.3_1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Changes>

7.6C.3_1.1.5	Test requirement

Same test requirement specified in 7.6A.3.1.5.1 shall be met for downlink bands for SUL configuration with intra-band contiguous DL CA with following exception:

Table 7.6C.3_1.1.5-1: SUL operating band combination with exceptions allowed

NR Band combination for SUL

SUL_n78-n81

SUL_n78-n82

SUL_n78-n83

SUL_n79-n81

SUL_n79-n83

Table 7.6C.3_1.1.5-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when ¦fInterferer± fSUL- fDL¦≤(BWSUL+ BWDL)/2, where BWSUL and BWDL are the channel bandwidths configured for SUL and DL (victim) bands in MHz, respectively. |  |  |

Same test requirement specified in clause 7.6C.3.5 or 7.6.3.5 for each band or band combinations listed in Table 7.6C.3_1.1.4-1 shall be met for out-of-band blocking testing for SUL configuration with inter-band 2DL CA.

#### 7.6C.3_1.2	Out-of-band Blocking for SUL with 3 DL CA

Editor’s Note: No test points defined for Out-of-band Blocking for SULwith inter-band 3 DL CA testing. The testing is covered by 7.6A.3 and 7.6C.3

7.6C.3_1.2.1	Test purpose

Same test purpose as in clause 7.6A.3.2.

7.6C.3_1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands and 3DL CA.

7.6C.3_1.2.3	Minimum conformance requirements

Same minimum requirements as in subclause 7.6C.3_1.1.3.

7.6C.3_1.2.4	Test description

For SUL configuration with inter-band DL CA: No testing need to be performed since the testing has been covered in test case 7.6A.3 and 7.6C.3. For band combination CA_nX_nY-nZ with one NUL band configured with 2DL CA, test the out-of-band blocking of SUL configuration or NR bands as listed in Table 7.6C.3_1.1.4-1.

Table 7.6C.3_1.2.4-1: Test band combinations and configuration

| Band configuration | Verifying out-of-band blocking of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n41C_n79A-n83A | n41C | 7.6A.3.1 | Table 7.6A.3.1.4.1-1 |
|  | SUL_n79A-n83A | 7.6C.3 | Table 7.6C.3.4-1 |

7.6C.3_1.2.5	Test requirement

Same test requirement specified in subclause 7.6C.3.5 or 7.6A.3.1.5.1 for each band or band combinations listed in Table 7.6C.3_1.2.4-1 shall be met for out-of-band blocking testing for SUL configuration with inter-band 3DL CA.

### <End of Changes>