3GPP TSG-RAN5 Meeting #104	R5-244871

# Maastricht, Netherlands, Aug 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2929 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Rx requirements for new SUL with inter-band CA configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Updating Rx requirements for new SUL configuration CA_n41A_n79A-n83A and. And the table formats in clause 7.3C.0.3 are updated to align with the core requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating ΔRIB,c value for CA_n41_n79-n83 in table 7.3C.0.3.2.2-1. And updating the table format for table 7.3C.0.3.2.1-1 and 7.3C.0.3.2.2-1 - Adding CA_n41A_n79A-n83A in test case 7.3C.3, 7.6C.2_1.1 and 7.6C.3_1.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rx requirements for CA_n41A_n79A-n83A are absent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C.0.3, 7.3C.3, 7.6C.2_1.1, 7.6C.3_1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Changes>

#### 7.3C.0.3	ΔRIB,c for SUL

7.3C.0.3.1	General

For a UE supporting a SUL configuration, the ΔRIB,c applies for both SC and SUL operation.

7.3C.0.3.2	SUL band combination

For the UE which supports SUL band combination, the minimum requirement for reference sensitivity in subclause 7.3C.0 shall be increased by the amount given in ΔRIB,c defined in subclause 7.3C.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14] for the applicable operating bands.

7.3C.0.3.2.1	ΔRIB,c  for two bands

Table 7.3C.0.3.2.1-1: ΔRIB,c due to SUL (two bands)

| SUL_n78-n80 |  | 0.5 |  | N/A |  |
| --- | --- | --- | --- | --- | --- |
| SUL_n78-n81 |  | 0.5 |  | N/A |  |
| SUL_n78-n82 |  | 0.5 |  | N/A |  |
| SUL_n78-n83 |  | 0.5 |  | N/A |  |
| SUL_n78-n84 |  | 0.5 |  | N/A |  |
| SUL_n78-n86 |  | 0.5 |  | N/A |  |
| SUL_n79-n83 |  | 0.5 |  | N/A |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE 3: The component band order in the configuration should be listed by the order of NR band, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |  |  |  |

7.3C.0.3.2.2	ΔRIB,c for three bands

Table 7.3C.0.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | ΔRIB,c for NR bands (dB)2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |  |  |  |  |
| CA_n1_n78-n80 | 0.2 |  | 0.5 |  | N/A |  |
| CA_n1_n78-n84 | 0.2 |  | 0.5 |  | N/A |  |
| CA_n3_n78-n80 | 0.2 |  | 0.5 |  | N/A |  |
| CA_n3_n78-n84 | 0.2 |  | 0.5 |  | N/A |  |
| CA_n3_n79-n80 | - |  | 0.5 |  | N/A |  |
| CA_n28_n41-n83 | 0.2 |  | - |  | N/A |  |
| CA_n28_n79-n83 | 0.2 |  | 0.5 |  | N/A |  |
| CA_n41_n79-n83 | 0.5 |  | 0.5 |  | N/A |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE 3: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_n78-n80 the order of band is n1, n78 and n80. |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3C.2 and 7.3C.3.

### <Unchanged Skipped>

### 7.3C.3	Reference sensitivity power level for SUL (3CC)

Editor’s Note:

- No test points defined for Reference sensitivity power level testing for SUL with DL CA. This test case is covered by 7.3.2 and 7.3C.2.

7.3C.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under SUL and 2 DL CA operation and conditions of low signal level, ideal propagation and no added noise.

7.3C.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports SUL operation on the SUL bands with 2DL CA.

7.3C.3.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 7.3C.0.

7.3C.3.4	Test description

NOTE:	No testing needs to be performed since the testing has been covered in test case 7.3.2 and 7.3C.2.

For band combination CA_nX_nY-nZ, test the REFSENS of SUL configuration or NR band as listed in table 7.3C.3.4-1.

Table 7.3C.3.4-1: Test band combinations and configuration

| Band configuration | Verifying REFSENS of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n1A_n78A-n80A | SUL_n78A-n80A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n1 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n1A_n78A-n84A | SUL_n78A-n84A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n1 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n3A_n78A-n80A | SUL_n78A-n80A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n3 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n28A_n41A-n83A | SUL_n41A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n28 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n79C-n83A | SUL_n79A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
| CA_n28A_n79A-n83A | SUL_n79A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n28 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n41A_n79A-n83A | SUL_n79A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n41 | 7.3.2 | Table 7.3.2.4.1-1 |

7.3C.3.5	Test requirement

Same test requirement as clause 7.3.2 and 7.3C.2 for each band or band combinations listed in table 7.3C.3.4-1.

### <Unchanged Skipped>

#### 7.6C.2_1.1	Inband Blocking for SUL with 2 DL CA

### <Unchanged Skipped>

For SUL configuration with inter-band DL CA: No testing need to be performed since the testing has been covered in test case 7.6.2 and 7.6C.2. For band combination CA_nX_SUL_nY-nZ, test the inband blocking of SUL configuration or NR band as listed in Table 7.6C.2_1.1.4.1-2.

Table 7.6C.2_1.1.4.1-2: Test band combinations and configuration

| Band configuration | Verifying in-band blocking of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n80A | SUL_n78-n80 | 7.6C.2 | Table 7.6C.2.4-1 |
|  | n1 | 7.6.2 | Table 7.6.2.4.1-1 |
| CA_n1A_SUL_n78A-n84A | SUL_n78-n84 | 7.6C.2 | Table 7.6C.2.4-1 |
|  | n1 | 7.6.2 | Table 7.6.2.4.1-1 |
| CA_n3A_SUL_n78A-n80A | SUL_n78-n80 | 7.6C.2 | Table 7.6C.2.4-1 |
|  | n3 | 7.6.2 | Table 7.6.2.4.1-1 |
| CA_n28A_SUL_n41A-n83A | SUL_n41-n83 | 7.6C.2 | Table 7.6C.2.4-1 |
|  | n28 | 7.6.2 | Table 7.6.2.4.1-1 |
| CA_n28A_SUL_n79A-n83A | SUL_n79A-n83A | 7.6C.2 | Table 7.6C.2.4-1 |
|  | n28 | 7.6.2 | Table 7.6.2.4.1-1 |
| CA_n41A_n79A-n83A | SUL_n79A-n83A | 7.6C.2 | Table 7.6C.2.4-1 |
|  | n41 | 7.6.2 | Table 7.6.2.4.1-1 |

### <Unchanged Skipped>

#### 7.6C.3_1.1	Out-of-band Blocking for SUL with 2 DL CA

### <Unchanged Skipped>

For SUL configuration with inter-band DL CA: No testing need to be performed since the testing has been covered in test case 7.6.3 and 7.6C.3. For band combination CA_nX_SUL_nY-nZ, test the out-of-band blocking of SUL configuration or NR band as listed in Table 7.6C.3_1.1.4.1-2.

Table 7.6C.3_1.1.4.1-2: Test band combinations and configuration

| Band configuration | Verifying out-of-band blocking of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n80A | SUL_n78-n80 | 7.6C.3 | Table 7.6C.3.4-1 |
|  | n1 | 7.6.3 | Table 7.6.3.4.1-1 |
| CA_n1A_SUL_n78A-n84A | SUL_n78-n84 | 7.6C.3 | Table 7.6C.3.4-1 |
|  | n1 | 7.6.3 | Table 7.6.3.4.1-1 |
| CA_n3A_SUL_n78A-n80A | SUL_n78-n80 | 7.6C.3 | Table 7.6C.3.4-1 |
|  | n3 | 7.6.3 | Table 7.6.3.4.1-1 |
| CA_n28A_SUL_n41A-n83A | SUL_n41-n83 | 7.6C.3 | Table 7.6C.3.4-1 |
|  | n28 | 7.6.3 | Table 7.6.3.4.1-1 |
| CA_n28A_SUL_n79A-n83A | SUL_n79A-n83A | 7.6C.3 | Table 7.6C.3.4-1 |
|  | n28 | 7.6.3 | Table 7.6.3.4.1-1 |
| CA_n41A_n79A-n83A | SUL_n79A-n83A | 7.6C.3 | Table 7.6C.3.4-1 |
|  | n41 | 7.6.3 | Table 7.6.3.4.1-1 |

### <End of Changes>