3GPP TSG RAN5 Meeting #104	 	R5-244895

Maastricht, Netherlands, 19th - 23rd August 2024

Source:		Rohde & Schwarz

Title:	On the MU and TT for MIMO test cases

Agenda Item:		5.4.20

Document for:	Discussion and Endorsement

# Introduction

For several Rel-15 UL MIMO test cases the MU and TT has not been specified which blocks the progress of these test cases. In this contribution, we propose MU and TT for some of these test cases.

# Discussion

In this paper, we focus our analysis on PC3, CHBW ≤ 400 MHz, IFF with a maximum device size of 40cm.

## UL MIMO test cases

### Frequency error for UL MIMO

Based on the MU assessment of our test system, we propose to reuse the MU and TT of the SISO test case for the two layer measurement described in Annex F.10 of TS 38.101-2.

Proposal 1: We propose a MU of ±0.01 ppm and a TT of 0.005 ppm for the frequency error for UL MIMO TC.

### Occupied bandwidth for UL MIMO

Compared to the OBW SISO TC which applies DFT-s-OFDM QPSK, the MIMO test case uses CP-OFDM QPSK. In contrast to SISO, there is no MBR for MIMO. Considering MBR, MPR and T(MPR), PUmax for MIMO is 2.75 dB (CHBW≤200MHz) and 3.25 dB (CHBW=400MHz) lower than that for SISO. This results in a lower SNR and, thus, higher OBW uncertainties for the bandwidth and frequency ranges where the impact of SNR is significant.

Based on the characteristics of our test equipment, we propose the uncertainties summarized in Table 1 for PC3, IFF with a max device size ≤ 40 cm.

Proposal 2: We propose the values in Table 1 for MU of OBW with UL MIMO, PC3, IFF, max device size ≤ 40 cm.

Table 1: Proposed MU for OBW for UL MIMO with PC3

| Frequency Range | CHBW (MHz) | MU [%CBW] |
| --- | --- | --- |
| FR2a | 50 | ±0.4 |
|  | 100 | ±0.4 |
|  | 200 | ±1.2 |
|  | 400 | TBD |
| FR2b | 50 | ±0.4 |
|  | 100 | ±0.4 |
|  | 200 | ±1.3 |
|  | 400 | TBD |
| FR2c | 50 | ±0.65 |
|  | 100 | ±0.65 |
|  | 200 | TBD |
|  | 400 | TBD |

For the channel bandwidths and frequency ranges with defined MU, we propose a TT of 0 dB.

Proposal 3: For the channel bandwidths and frequency ranges with defined MU in Table 1, we propose a TT of 0 dB.

Accompanying CRs in Refs. [1] and [2] document the MU assessment in TR 38.903 [3] and TS 38.521-2 [4], respectively. CR in [5] updates the status of the associated EN-DC TCs in TS 38.521-3 [6]. The applicability in TS 38.522 [7] is updated with CR in Ref. [8].

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2 |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, we have assessed the uncertainty and make the following proposals:

Proposal 1: We propose a MU of ±0.01 ppm and a TT of 0.005 ppm for the frequency error for UL MIMO TC.

Proposal 2: We propose the values in Table 1 for MU of OBW with UL MIMO, PC3, IFF, max device size ≤ 40 cm.

Proposal 3: For the channel bandwidths and frequency ranges with defined MU in Table 1, we propose a TT of 0 dB.

# References

- R5-244896, Documentation of MU for UL MIMO, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #104, August 2024

- R5-244897, Update of MU and TT for MIMO, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #104, August 2024

- TR 38.903, V18.3.0, 3GPP TSG RAN WG5, June 2024

- TS 38.521-2, V18.3.0, 3GPP TSG RAN WG5, June 2024

- R5-244898, Update of status of FR2 CA and UL MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #104, August 2024

- TS 38.521-3, V18.3.0, 3GPP TSG RAN WG5, June 2024

- TS 38.522, V18.3.0, 3GPP TSG RAN WG5, June 2024

- R5-244899, Update of applicability of FR2 CA and MIMO test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #104, August 2024