3GPP TSG-RAN WG5 Meeting # 104	R5-244919

# Maastricht, Netherlands,  August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0935 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update reference sensitivity test cases for CA_n1A-n5A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update reference sensitivity test cases for three bands configuration CA_n1A-n5A-n78A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the reference sensitivity test cases per CA configuration (three bands) for CA_n1A-n5A-n78A in Table 4.1.3.1-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The updated information for CA_n1A-n5A-n78A will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR 2936 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

4.1.3	Test point analysis per NR CA configuration

#### 4.1.3.1	Reference Sensitivity test cases for FR1 NR CA

Editor's note:

-	Not all CA configurations completed in TS 38.521-1 refsens test cases are listed in Table 4.1.3.1-1 through Table 4.1.3.1-5.

This clause contains information on test point analysis, test frequency selection and status for FR1 NR CA test cases in TS 38.521-1 [2] clause 7. The analyses are performed per CA configuration in Table 4.1.3.1-1 through Table 4.1.3.1 4. The principles for selection of reference sensitivity test points for NR CA is given in Annex B including a test point analysis check list in B.9.

Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration (single band)

| CA config | Single band refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n2(2A) |  | NE | RAN5#100 |
| CA_n41C |  | NE | RAN5#89-e |
| CA_n48(2A) | - | NE | RAN5#102 |
| CA_n48B |  | NE | RAN5#94-e |
| CA_n48C |  | NE | RAN5#103 |
| CA_n66B |  | NE | RAN5#86-e |
| CA_n66(2A) |  | NE | RAN5#86-e |
| CA_n66(3A) |  | NE | RAN5#100 |
| CA_n71(2A) |  | NE | RAN5#94-e |
| CA_n77(2A) |  | NE | RAN5#100 |
| CA_n77C |  | NE | RAN5#96e |
| CA_n78C |  | NE | RAN5#87-e |
| CA_n78(2A) |  | NE | RAN5#101 |
| CA_n79C |  | NE | RAN5#103 |
| NOTE 1: Void. NOTE 2: Notations used: NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |

Table 4.1.3.1-2: Reference Sensitivity test cases per CA configuration (two bands)

| CA config | Two band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n1A-n3A | n1 (IMD3) n3(CBI) | IMD,CBI | RAN5#95-e |
| CA_n1A-n8A | n1 (IMD4) | IMD | RAN5#95-e |
| CA_n1A-n28A | n1 (HD3) | HD | RAN5#101 |
| CA_n1A-n41A | n1(CBI), n41(CBI) | CBI | RAN5#102 |
| CA_n1A-n77A | n77 (HD2) | HD | RAN5#89-e |
| CA_n1A-n78A | n1 (IMD4) | IMD | RAN5#94 |
| CA_n2A-n48A | n48 (HD2), n2(IMD4) | HD, IMD | RAN5#96-e |
| CA_n2A-n5A | - | NE | RAN5#97 |
| CA_n2A-n14A | - | NE | RAN5#100 |
| CA_n2A-n48A | n48 (HD2), n2(IMD4) | HD, IMD | RAN5#100 |
| CA_n2A-n48(2A) | (NOTE 2) | NE | RAN5#102 |
| CA_n2A-n48B | (NOTE 2) | NE | RAN5#101 |
| CA_n2A-n66A | n2(IMD3), n66(IMD5) | IMD | RAN5#96-e |
| CA_n2A-n77A PC3 | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4), n2(IMD5), n2(IMD7) | HD, HM, IMD, | RAN5#99 |
| CA_n2A-n77A PC2 | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4) n2(CBI) | HD, HM, IMD, CBI | RAN5#99 |
| CA_n2A-n77C PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n2A-n77C PC2 | (NOTE 2) | NE | RAN5#102 |
| CA_n3A-n5A | n3 (IMD4 ¦2*fn3-2*fn5¦) n5 (IMD2 ¦fn3-fn5¦) | IMD | RAN5#95-e |
| CA_n3A-n8A | n8(IMD4), n3(IMD5) | IMD | RAN5#100 |
| CA_n3A-n28A | - | NE | RAN5#101 |
| CA_n3A-n41A PC2 | n3 (IMD4), n3 (CBI) | IMD, CBI | RAN5#100 |
| CA_n3A-n41A PC3 | n3 (IMD4), n3 (CBI), n41 (CBI) | IMD, CBI | RAN5#100 |
| CA_n3A-n77A | n77 (HD2), n3 (IMD2), n3 (IMD4) | HD, IMD | RAN5#99 |
| CA_n3A-n77(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n3A-n78A PC3 | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2) | HD, IMD | RAN5#101 |
| CA_n3A-n78A PC2 | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2), n3 (HM) | HD, HM, IMD | RAN5#101 |
| CA_n5A-n48A | - | NE | RAN5#102 |
| CA_n5A-n48(2A) | - | NE | RAN5#102 |
| CA_n5A-n48B | - | NE | RAN5#102 |
| CA_n5A-n66A | n5(IMD2) | IMD | RAN5#96-e |
| CA_n5A-n77A PC3 | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4), n5(IMD5) | HD, HM, IMD | RAN5#99 |
| CA_n5A-n77A PC2 | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4) | HD, HM, IMD | RAN5#99 |
| CA_n5A-n77C PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n5A-n77C PC2 | (NOTE 2) | NE | RAN5#102 |
| CA_n5A-n78A | n78 (HD2) | HD | RAN5#94-e |
| CA_n7A-n78A | n7 and n78 (CBI) | CBI | RAN5#94-e |
| CA_n8A-n78A | n8 (IMD4 ¦fn78-3*fn8¦) n78 (HD4) | HD, IMD | RAN5#87-e |
| CA_n14A-n30A | - | NE | RAN5#100 |
| CA_n14A-n66A | - | NE | RAN5#100 |
| CA_n14A-n77A | n77 (HD5), n14 (HM), n14 (IMD5) | HD, HM, IMD | RAN5#100 |
| CA_n20A-n78A | n78 (HD4), n20 (IMD4) | HD, IMD | RAN5#100 |
| CA_n25A-n66A | n25 (IMD3), n66 (IMD3), n66 (IMD5) | IMD | RAN5#101 |
| CA_n25A-n77A | n77 (HD2), n25 (HM), n25 (IMD2), n25 (IMD4), n25 (IMD5) | HD, HM, IMD | RAN5#101 |
| CA_n25A-n77(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n25A-n78A | n78 (HD2), 25 (IMD2) | HD, IMD | RAN5#101 |
| CA_n25A-n78(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n26A-n66A | n26 (IMD2) | IMD | RAN5#94-e |
| CA_n26A-n70A | n26 (IMD2) | IMD | RAN5#94-e |
| CA_n28A-n40A | n28 (HM) | HM | RAN5#103 |
| CA_n28A-n41A |  | NE | RAN5#91-e |
| CA_n28A-n77A | n77 (HD5), n28 (IMD5) | HD, IMD | RAN5#99 |
| CA_n28A-n77(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n28A-n78A | n78 (HD5, HM) | HD, HM | RAN5#102 |
| CA_n28A-n79A |  | NE | RAN5#91-e |
| CA_n29A-n66A |  | NE | RAN5#94-e |
| CA_n29A-n70A |  | NE | RAN5#94-e |
| CA_n29A-n71A | n29 (CBI) | CBI | RAN5#95-e |
| CA_n39A-n41A | - | NE | RAN5#100 |
| CA_n39A-n41A4 | n39(HM), n41(HM), n39(CBI), n41(CBI) | HM, CBI | RAN5#103 |
| CA_n41A-n66A3 | n66 (CBI), n66 (IMD5) | CBI | RAN#97 |
| CA_n41A-n71A3 | n41(HD4), n71 (IMD4) | HD | RAN5#98 |
| CA_n41A-n77A | n41(HM), n77(IMD9), n41(CBI), n77(CBI) | HM, IMD, CBI | RAN5#102 |
| CA_n41A-n79A PC3 |  | NE | RAN5#83 |
| CA_n41A-n79A PC2 | n41 (CBI), n79 (CBI) | CBI | RAN5#100 |
| CA_n48A-n66A | n48 (HD2), n66 (IMD5) | HD, IMD | RAN5#94-e |
| CA_n48A-n66(2A) | (NOTE 2) | NE | RAN5#95-e |
| CA_n48A-n70A | n70 (IMD2) | IMD | RAN5#94-e |
| CA_n48A-n71A |  | NE | RAN5#94-e |
| CA_n48A-n71(2A) | - | NE | RAN5#95-e |
| CA_n48A-n77A PC3 | - | NE | RAN5#97-e |
| CA_n48A-n77A PC2 | - | NE | RAN5#102 |
| CA_n48A-n77C | - | NE | RAN5#102 |
| CA_n48B-n66A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48B-n70A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48B-n71A | - | NE | RAN5#95-e |
| CA_n48B-n77A | - | NE | RAN5#102 |
| CA_n48(2A)-n66A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48(2A)-n66(2A) | (NOTE 2) | NE | RAN5#96-e |
| CA_n48(2A)-n70A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48(2A)-n71A | - | NE | RAN5#95-e |
| CA_n48(2A)-n71(2A) | - | NE | RAN5#96-e |
| CA_n48(2A)-n77A | - | NE | RAN5#102 |
| CA_n66A-n70A |  | NE | RAN5#94-e |
| CA_n66A-n71A | n66 (IMD4) | IMD | RAN5#87-e |
| CA_n66A-n77A PC3 | n77 (HD2), n66(IMD2), n66(IMD5), n66(IMD7) | HD, IMD | RAN5#102 |
| CA_n66A-n77A PC2 | n77 (HD2), n66(IMD2), n66(IMD5), n66(CBI) | HD, IMD, CBI | RAN5#99 |
| CA_n66A-n77(2A) PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n66A-n77C PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n66A-n77C PC2 | (NOTE 2) | NE | RAN5#102 |
| CA_n66A-n78A | n78 (HD2), n66(IMD5) | HD, IMD | RAN5#101 |
| CA_n66A-n78(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n70A-n71A | n70 (HD3), n70 (IMD4) | HD, IMD | RAN5#85 |
| CA_n71A-n77A | n71 (IMD5) | IMD | RAN5#97-e |
| CA_n71A-n77(2A) | (NOTE 2) | NE | RAN5#102 |
| CA_n71A-n78A | n78 (HD5), n71 (IMD5) | HD, IMD | RAN5#102 |
| CA_n71A-n78(2A) | (NOTE 2) | NE | RAN5#102 |
| CA_n78A-n79A PC2 | n78 (CBI), n79(CBI) | CBI | RAN5#103 |
| CA_n78A-n79A PC3 | n78 (CBI), n79 (CBI) | CBI | RAN5#103 |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 NOTE 2: Exception for this band is tested with two carrier case. NOTE 3: Only single uplink analysed. IMD exception not yet covered. NOTE 4: Exception for this configuration is tested only when the UE reports supporting Simultaneous Rx/Tx capability. |  |  |  |

Table 4.1.3.1-3: Reference Sensitivity test cases per CA configuration (three bands)

| CA config | Three band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n1A-n3A-n28A |  | NE | RAN5#101 |
| CA_n1A-n3A-n77A | n1(IMD2), n3(IMD2), n78(IMD2) | IMD | RAN5#102 |
| CA_n1A-n3A-n78A | n3(IMD2), n78(IMD2, IMD4) | IMD | RAN5#101 |
| CA_n1A-n5A-n78A | n1(IMD3), n5(IMD5), n78(IMD3) | IMD | RAN5#104 |
| CA_n1A-n8A-n78A | n8(IMD5), n78(IMD3) | IMD | RAN5#102 |
| CA_n1A-n28A-n77A | n1(IMD3), n28(IMD5), n77(IMD3) | IMD | RAN5#102 |
| CA_n1A-n28A-n78A | n1(IMD3), n28(IMD5), n78(IMD3) | IMD | RAN5#102 |
| CA_n1A-n41A-n77A | n1(IMD4), n41(IMD4), n77(IMD3) | IMD | RAN5#102 |
| CA_n2A-n5A-n48A | n2(IMD3), n48(IMD3) | IMD | RAN5#100 |
| CA_n2A-n5A-n77A PC3 | n2(IMD3), n5 (IMD5), n77(IMD3) | IMD | RAN5#98 |
| CA_n2A-n5A-n77A PC2 | n2(IMD3), n5 (IMD5) | IMD | RAN5#100 |
| CA_n2A-n5A-n77C PC3 | (NOTE 2) | NE | RAN5#102 |
| CA_n2A-n48A-n66A | n48(IMD2), n66 (IMD4), n2(IMD2) | IMD | RAN5#100 |
| CA_n2A-n48A-n77A PC3 | - | NE | RAN5#100 |
| CA_n2A-n48A-n77A PC2 | - | NE | RAN5#102 |
| CA_n2A-n48A-n77C | - | NE | RAN5#102 |
| CA_n2A-n66A-n77A PC3 | n2(IMD2, IMD4, IMD5), n66 (IMD2, IMD4, IMD5), n77(IMD2, IMD4) | IMD | RAN5#98 |
| CA_n2A-n66A-n77A PC2 | n2(IMD2), n66 (IMD2) | IMD | RAN5#100 |
| CA_n2A-n66A-n77C PC3 | (NOTE 2) | NE | RAN5#102 |
| CA_n3A-n8A-n78A | n78(IMD3, IMD5), n3(IMD3) | IMD | RAN5#103 |
| CA_n3A-n28A-n41A | n3(IMD2), n28(IMD2), n41(IMD2, IMD3) | IMD | RAN5#101 |
| CA_n3A-n28A-n77A | n3(IMD3), n28(IMD3), n77(IMD3) | IMD | RAN5#101 |
| CA_n3A-n28A-n78A | n78(IMD3, IMD5) | IMD | RAN5#102 |
| CA_n3A-n41A-n77A | n3(IMD3), n41(IMD5), n77(IMD3) | IMD | RAN5#102 |
| CA_n5A-n48A-n66A | n48(IMD5) | IMD | RAN5#100 |
| CA_n5A-n48A-n77A PC3 | - | NE | RAN5#100 |
| CA_n5A-n48A-n77A PC2 | - | NE | RAN5#102 |
| CA_n5A-n48A-n77C | - | NE | RAN5#102 |
| CA_n5A-n66A-n77A PC3 | n66 (IMD3), n77(IMD3, IMD4, IMD5) | IMD | RAN5#98 |
| CA_n5A-n66A-n77A PC2 | n66 (IMD3) | IMD | RAN5#100 |
| CA_n5A-n66A-n77C PC3 | (NOTE 2) | NE | RAN5#102 |
| CA_n25A-n66A-n77 | n25 (IMD2, IMD3, IMD4, IMD5), n66 (IMD2, IMD3, IMD4, IMD5, IMD7) | IMD | RAN5#101 |
| CA_n25A-n66A-n77(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n25A-n66A-n78 | n78(IMD2) | IMD | RAN5#101 |
| CA_n25A-n66A-n78(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n26A-n66A-n70A |  | NE | RAN5#94-e |
| CA_n26A-n66(2A)-n70A |  | NE | RAN5#96-e |
| CA_n28A-n41A-n79A | n28(IMD3), n41 (IMD4), n79(IMD3) | IMD | RAN5#100 |
| CA_n29A-n66A-n70A |  | NE | RAN5#89-e |
| CA_n41A-n66A-n71A |  | NE | RAN5#99 |
| CA_n48A-n66A-n71(2A) | - | NE | RAN5#96-e |
| CA_n48A-n66A-n77A PC3 | - | NE | RAN5#100 |
| CA_n48A-n66A-n77A PC2 | - | NE | RAN5#102 |
| CA_n48A-n66A-n77C | - | NE | RAN5#102 |
| CA_n48A-n66(2A)-n70A | - | NE | RAN5#96-e |
| CA_n48A-n66(2A)-n71A | - | NE | RAN5#96-e |
| CA_n48A-n70A-n71(2A) | - | NE | RAN5#96-e |
| CA_n48B-n66A-n70A | - | NE | RAN5#96-e |
| CA_n48B-n66A-n71A | - | NE | RAN5#96-e |
| CA_n48B-n70A-n71A | - | NE | RAN5#96-e |
| CA_n48(2A)-n66A-n70A | - | NE | RAN5#96-e |
| CA_n48(2A)-n66A-n71A | - | NE | RAN5#96-e |
| CA_n48(2A)-n70A-n71A | - | NE | RAN5#96-e |
| CA_n66A-n70A-n71A |  | NE | RAN5#87-e |
| CA_n66A-n71A-n77A | n66 (IMD3), n71 (IMD3), n77 (IMD3) | IMD | RAN5#102 |
| CA_n66A-n71A-n77(2A) | (NOTE 2) | NE | RAN5#102 |
| CA_n66A-n71A-n78A | n66 (IMD3), n78 (IMD4) | IMD | RAN5#102 |
| CA_n66A-n71A-n78(2A) | (NOTE 2) | NE | RAN5#102 |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 NOTE 2: Exception for this band is tested with three carrier case. |  |  |  |

Table 4.1.3.1-4: Reference Sensitivity test cases per CA configuration (four bands)

| CA config | Four band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n1A-n3A-n28A-n78A |  | NE | RAN5#101 |
| CA_n2A-n5A-n48A-n66A |  | NE | RAN5#102 |
| CA_n2A-n5A-n48A-n77A |  | NE | RAN5#102 |
| CA_n2A-n5A-n66A-n77A |  | NE | RAN5#102 |
| CA_n2A-n48A-n66A-n77A |  | NE | RAN5#102 |
| CA_n3A-n28A-n41A-n77A | - | NE | RAN5#103 |
| CA_n5A-n48A-n66A-n77A |  | NE | RAN5#102 |
| NOTE 1: No exceptions can occur for four bands. NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. |  |  |  |

Table 4.1.3.1-5: Reference Sensitivity test cases per CA configuration (five bands)

| CA config | Five band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| TBD |  | TBD | TBD |
| NOTE 1: No exceptions can occur for five bands. |  |  |  |

For CA configurations affected by exceptions the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement, in addition to testing the case where the exception is avoided. This is indicated in Table 4.1.3.1-6. Exceptions in TS 38.101-1 [5] clause 7.3A.5 (2UL intermodulation) and 7.3A.6 (cross band isolation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-1 [5].

Table 4.1.3.1-6: Test frequency selection per band pair for bands and test points with exceptions avoidable by test frequency setting (UL harmonics, Rx mixing)

| Band pair (Note 2) | Frequency | Channel BW [MHz] | Comment |
| --- | --- | --- | --- |
| n1A / n28A | UL 713 MHz / DL 2139 MHz | 5/5 | Exception Note 3 of TS38.101 table 7.3A.4-1 |
|  | UL 713 MHz / DL 2139 MHz | 50/5 | Exception Note 3 of TS38.101 table 7.3A.4-1 |
| n1A / n77A | Mid / Low | Highest(50)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=2*50 MHz, FINT (550) ≥ (100+100)/2. |
|  | Mid / 3900 | 5/10 | Exception Note 2 of TS 38.101 table 7.3A.4-1 |
|  | Mid / 3900 | 20/100 | Exception Note 2 of TS 38.101 table 7.3A.4-1 |
|  | Mid / 3875 | 5/10 | Exception Note 6 of TS 38.101 table 7.3A.4-1 |
| n2A/n48A | Mid/Mid | Highest(40)/ Highest(40) | Non-exception. Not overlapping interference since BWINT=2*40 MHz, FINT (135) ≥ (80+40)/2. |
| n2A/n48A | UL 1860 MHz / DL 3690 MHz | 5/10 | Exception Note 6 of TS 38.521 Table 7.3A.0.4-1 |
| n2A/n77A | Mid / 3850 Mhz | 20/100 | Non-exception. Not overlapping interference since BWINT=2*20 MHz, FINT (90) ≥ (40+100)/2. |
|  | UL 1880 MHz / DL 3760 MHz | 5/10 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1880 MHz / DL 3760 MHz | 10/100 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1880 MHz / DL 3735 MHz | 5/10 | Exception Note 6 of TS 38.101 Table 7.3A.4-1 |
|  | UL 3920 MHz/ Mid | 10/5 | Exception Note 7 of TS 38.101 Table 7.3A.4-4 |
|  | UL 3920 MHz/ Mid | 20/20 | Exception Note 7 of TS 38.101 Table 7.3A.4-4 |
| n3A / n77A | Mid/High | Highest/Highest | Non-exception. |
|  | Mid/3495 MHz | 5/10 | Exception Note 2 of TS38.101 table 7.3A.4-1 |
|  | Mid/3495 MHz | 10/100 | Exception Note 2 of TS38.101 table 7.3A.4-1 |
|  | Mid/3470 MHz | 5/10 | Exception Note 6 of TS38.101 table 7.3A.4-1 |
| n3A / n78A | Mid/High | Highest(50)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=2*50 MHz, FINT (255) ≥ (100+100)/2. |
|  | Mid/3495 MHz | 5/10 | Exception Note 2 of TS38.101 table 7.3A.4-1 |
|  | Mid/3465 | 10/100 | Exception Note 2 of TS38.101 table 7.3A.4-1 |
|  | Mid/3470 MHz | 5/10 | Exception Note 6 of TS38.101 table 7.3A.4-1 |
| n5A/n77A | Mid/Mid | Highest(20)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=5*20 MHz, FINT (432.5) ≥ (100+100)/2 for HD5 and BWINT=4*20+100 MHz, FINT (224) ≥ (180+100)/2 for HM4. |
|  | UL 836.5 MHz / DL 3346 MHz | 5/10 | Exception Note 4 of TS 38.101-1 table 7.3A.4-1 |
|  | UL 846.5 MHz / DL 3386 MHz | 5/100 | Exception Note 4 of TS 38.101-1 table 7.3A.4-1 |
|  | UL 826.5 MHz / DL 4132.5 MHz | 5/10 | Exception Note 5 TS 38.101-1 table 7.3A.4-1 |
|  | UL 826.5 MHz / DL 4132.5 MHz | 5/100 | Exception Note 5 of TS 38.101-1 table 7.3A.4-1 |
| n5A/n77A | UL 3526 MHz / DL Mid | 20/20 | Exception Note 5 of TS 38.101 Table 7.3A.4-4 |
|  | UL 3526 MHz / DL Mid | 10/10 | Exception Note 5 of TS 38.101 Table 7.3A.4-4 |
| n5A / n78A | Mid/High | Highest(20)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=5*20 MHz, FINT (432.5) ≥ (100+100)/2 for HD5 and BWINT=4*20+100 MHz, FINT (224) ≥ (180+100)/2 for HM4. |
|  | Mid/3346 MHz | 5/10846.5 | Exception Note 4 of TS 38.101 [5] table 7.3A.4-1 |
|  | /3386 MHz | 5/100 | Exception Note 4 of TS 38.101 [5] table 7.3A.4-1 |
| n8A / n78A | Mid/High | Highest(20)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=4*20 MHz, FINT (160) ≥ (80+100)/2. |
|  | Mid/3590 MHz | 5/10 | Exception Note 4 of TS38.101 table 7.3A.4-1 |
|  | Mid/3590 MHz | 5/100 | Exception Note 4 of TS38.101 table 7.3A.4-1 |
| n20 / n78A | Mid/High | 5/10 | Exception Note 4 of TS38.101 table 7.3A.4-1 |
|  | Mid/3388 MHz | 5/100 | Exception Note 4 of TS38.101 table 7.3A.4-1 |
| n25A / n77A | Mid/High | Highest(45)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=2*45 MHz, FINT (385) ≥ (90+100)/2 for HD2. |
|  | UL 1882.5 MHz / DL 3765 MHz | 5/10 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1882.5 MHz / DL 3765 MHz | 10/100 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1852.5 MHz / DL 3680 MHz | 5/10 | Exception Note 6 of TS 38.101 Table 7.3A.4-1 |
| n25A / n77A | UL 3925 MHz/ DL 1962.5 MHz | 5/10 | Exception Note 7 of TS 38.101 Table 7.3A.4-4 |
|  | UL 3925 MHz/ DL 1962.5 MHz | 40/20 | Exception Note 7 of TS 38.101 Table 7.3A.4-4 |
| n25A / n78A | High/Low | Highest(45)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=2*45 MHz, FINT (435) ≥ (90+100)/2 for HD2. |
|  | UL 1882.5 MHz / DL 3765 MHz | 5/10 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1882.5 MHz / DL 3765 MHz | 10/100 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1852.5 MHz / DL 3680 MHz | 5/10 | Exception Note 6 of TS 38.101 Table 7.3A.4-1 |
| n28 / n77A | Low / Low | Highest/Highest | Non-exception |
|  | UL 705.5 MHz / DL 3527.5 MHz | 5/10 | Exception Note 5 of TS38.101 table 7.3A.4-1 |
|  | UL 705.5 MHz / DL 3527.5 MHz | 5/100 | Exception Note 5 of TS38.101 table 7.3A.4-1 |
| n28A / n78A | Mid/High | Highest(20)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=5*20 MHz, FINT (110) ≥ (100+100)/2 for HD5 |
|  | UL 705.5 MHz / DL 3527.5 MHz | 5/10 | Exception Note 5 of TS 38.101 [5] table 7.3A.4-1 |
|  | UL 705.5 MHz / DL 3527.5 MHz | 5/100 | Exception Note 5 of TS 38.101 [5] table 7.3A.4-1 |
|  | UL 705.5 MHz / DL 3527.5 MHz | 5/10 | Exception Note 5 of TS 38.101 [5] table 7.3A.4-4 |
| n41A/n71A | UL Low/DL Low | Highest/Highest | Non-exception. Not overlapping interference since BWINT=4*20 MHz, FINT (146) ≥ (80+100)/2 for HD4. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | UL Low / DL 2662 MHz | UL 5 / DL 10 | Exception Note 4 of TS 38.101 Table 7.3A.4-1 |
|  | UL Low / DL High | UL 5 / DL 100 | Exception Note 4 of TS 38.101 Table 7.3A.4-1 |
| n66A/n77A | Mid/Mid | Highest(45)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=2*45 MHz, FINT (260) ≥ (90+100)/2. |
|  | UL 1750 MHz / DL 3500 MHz | 5/10 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1750 MHz / DL 3500 MHz | 20/100 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1712.5 MHz / DL 3400 MHz | 5/10 | Exception Note 6 of TS 38.101 Table 7.3A.4-1 |
| n66A/n78A | Mid/Mid | Highest(45)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=2*45 MHz, FINT (330) ≥ (90+100)/2. |
|  | UL 1750 MHz / DL 3500 MHz | 5/10 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1750 MHz / DL 3500 MHz | 20/100 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1712.5 MHz / DL 3400 MHz | 5/10 | Exception Note 6 of TS 38.101 Table 7.3A.4-1 |
| n70A / n71A | Mid/High | Highest(25) /Highest(20) | Non-exception. Not overlapping interference since BWINT=3*20 MHz, FINT (56.5) ≥ (60+25)/2. |
|  | Low/Low | Highest / 10 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 20@10 |
|  | Low/Low | 5 / 5 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 8@10 |
| n71A/n78A | Mid/Mid | Highest(35)/ Highest(100) | Non-exception. Not overlapping interference since BWINT=2*35 MHz, FINT (147.5) ≥ (70+100)/2. |
|  | UL 680.5 MHz / DL 3402.5 MHz | 5/10 | Exception Note 5 of TS 38.101 Table 7.3A.4-1 |
| NOTE 1: This selection is used in test case 7.3A.1_1 unless otherwise stated. NOTE 2: Aggressor band in bold. NOTE 3: Non-exception test frequencies are required to be used in test cases 7.3A.1, 7.3A.2, 7.3A.3, 7.3A.4 which are also referred to in other Rx test cases under chapter 7 of TS38.521-1 [2] |  |  |  |

4.1.3.2	Spurious emissions test cases for FR1 UL CA

In this case, it is sufficient to verify the minimum requirements in frequency ranges affected by 2nd and 3rd order intermodulation products. The frequency ranges and UL RB allocations used in the test are calculated here.

The analyses are performed per CA configuration and are stored as zip-files as defined in annex A.

NOTE:	Compared to the test points in test configuration Table 6.5A.3.2.1.4.1-1 in TS 38.521-1[2], additional test points in the test point analysis for Spurious emissions for UE co-existence in "6.5A.3.2.3 co-existence" tab of the zip-files shown in Table 4.1.3.2-1 introduced before RAN5#102 shall be ignored.

Table 4.1.3.2-1: Frequency range analysis availability per CA configuration

| CA config | Number of test point | Justification | Comments |
| --- | --- | --- | --- |
| CA_n1A-n3A |  | 38.521-1_TpAnalysisSpur(CA_n1A-n3A).zip | RAN5#99 |
| CA_n1A-n8A | 1 | 38.521-1_TpAnalysisSpur(CA_n1A-n8A).zip | RAN5#98 |
| CA_n1A-n28A | 1 | 38.521-1_TpAnalysisSpur(CA_n1A-n28A).zip | RAN5#102 |
| CA_n1A-n78A | 3 | 38.521-1_TpAnalysisSpur(CA_n1A-n78A) v3.zip | RAN5#97 |
| CA_n1A-n79A | 4 | 38.521-1_TpAnalysisSpur(CA_n1A-n79A).zip | RAN5#97 |
| CA_n2A-n14A | 2 | 38.521-1_TpAnalysisSpur(CA_n2A-n14A).zip | RAN5#101 |
| CA_n2A-n30A | 6 | 38.521-1_TpAnalysisSpur(CA_n2A-n30A).zip | RAN5#103 |
| CA_n2A-n77A | 3 | 38.521-1_TpAnalysisSpur(CA_n2A-n77A)_v3.zip | RAN5#100 |
| CA_n2A-n5A | 3 | 38.521-1_TpAnalysisSpur(CA_n2A_n5A)_v1.zip | RAN5#100 |
| CA_n2A-n48A | 3 | 38.521-1_TpAnalysisSpur(CA_n2A_n48A)_v1.zip | RAN5#100 |
| CA_n3A-n8A | 3 | 38.521-1_TpAnalysisSpur(CA_n3A-n8A).zip | RAN5#100 |
| CA_n3A-n28A | 3 | 38.521-1_TpAnalysisSpur(CA_n3A-n28A).zip | RAN5#101 |
| CA_n3A-n41A | 6 | 38.521-1_TpAnalysisSpur(CA_n3A-n41A).zip | RAN5#97 |
| CA_n3A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n3A-n77A).zip | RAN5#100 |
| CA_n3A-n78A | 4 | 38.521-1_TpAnalysisSpur(CA_n3A-n78A) v1.zip | RAN5#97 |
| CA_n5A-n30A | 6 | 38.521-1_TpAnalysisSpur(CA_n5A-n30A).zip | RAN5#103 |
| CA_n5A-n48A | 7 | 38.521-1_TpAnalysisSpur(CA_n5A-n48A)_v1.zip | RAN5#100 |
| CA_n5A-n66A | 3 | 38.521-1_TpAnalysisSpur(CA_n5A-n66A).zip | RAN5#100 |
| CA_n5A-n77A | 7 | 38.521-1_TpAnalysisSpur(CA_n5A-n77A)_v3.zip | RAN5#100 |
| CA_n8A-n78A | 5 | 38.521-1_TpAnalysisSpur(CA_n8A-n78A) v1.zip | RAN5#97 |
| CA_n14A-n30A | 2 | 38.521-1_TpAnalysisSpur(CA_n14A-n30A).zip | RAN5#101 |
| CA_n14A-n66A | 2 | 38.521-1_TpAnalysisSpur(CA_n14A-n66A).zip | RAN5#101 |
| CA_n14A-n77A | 2 | 38.521-1_TpAnalysisSpur(CA_n14A-n77A).zip | RAN5#101 |
| CA_n20A-n78A | 5 | 38.521-1_TpAnalysisSpur(CA_n20A-n78A).zip | RAN5#100 |
| CA_n24A-n41A | 5 | 38.521-1_TpAnalysisSpur(CA_n24A-n41A).zip | RAN5#95e |
| CA_n24A-n48A | 2 | 38.521-1_TpAnalysisSpur(CA_n24A-n48A).zip | RAN5#95e |
| CA_n24A-n77A | 4 | 38.521-1_TpAnalysisSpur(CA_n24A-n77A).zip | RAN5#95e |
| CA_n25A-n66A | 5 | 38.521-1_TpAnalysisSpur(CA_n25A-n66A).zip | RAN5#101 |
| CA_n25A-n77A | 3 | 38.521-1_TpAnalysisSpur(CA_n25A-n77A).zip | RAN5#101 |
| CA_n25A-n78A | 1 | 38.521-1_TpAnalysisSpur(CA_n25A-n78A).zip | RAN5#101 |
| CA_n26A-n66A | 1 | 38.521-1_TpAnalysisSpur(CA_n26A-n66A).zip | RAN5#96e |
| CA_n26A-n70A | 1 | 38.521-1_TpAnalysisSpur(CA_n26A-n70A).zip | RAN5#96e |
| CA_n28A-n41A | 4 | 38.521-1_TpAnalysisSpur(CA_n28A-n41A) v1.zip | RAN5#97 |
| CA_n28A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n28A-n77A).zip | RAN5#100 |
| CA_n28A-n78A | 6 | 38.521-1_TpAnalysisSpur(CA_n28A-n78A).zip | RAN5#99 |
| CA_n28A-n79A | 1 | 38.521-1_TpAnalysisSpur(CA_n28A-n79A).zip | RAN5#100 |
| CA_n30A-n66A | 6 | 38.521-1_TpAnalysisSpur(CA_n30A-n66A).zip | RAN5#103 |
| CA_n30A-n77A | 7 | 38.521-1_TpAnalysisSpur(CA_n30A-n77A).zip | RAN5#103 |
| CA_n39A-n41A | 1 | 38.521-1_TpAnalysisSpur(CA_n39A-n41A).zip | RAN5#97 |
| CA_n41A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n41A-n77A).zip | RAN5#102 |
| CA_n41A-n79A | 8 | 38.521-1_TpAnalysisSpur(CA_n41A-n79A) v1.zip | RAN5#97 |
| CA_n48A-n66A | 1 | 38.521-1_TpAnalysisSpur(CA_n48A-n66A) v1.zip | RAN5#97 |
| CA_n48A-n70A | 2 | 38.521-1_TpAnalysisSpur(CA_n48A-n70A).zip | RAN5#96e |
| CA_n48A-n71A | 2 | 38.521-1_TpAnalysisSpur(CA_n48A-n71A)_v2.zip | RAN5#100 |
| CA_n66A-n71A | 1 | 38.521-1_TpAnalysisSpur(CA_n66A-n71A)_v1.zip | RAN5#99 |
| CA_n66A-n77A | 9 | 38.521-1_TpAnalysisSpur(CA_n66A-n77A) v2.zip | RAN5#97 |
| CA_n66A-n78A | 1 | 38.521-1_TpAnalysisSpur(CA_n66A-n78A).zip | RAN5#101 |
| CA_n70A-n71A | 1 | 38.521-1_TpAnalysisSpur(CA_n70A-n71A).zip | RAN5#96e |
| CA_n71A-n77A | 7 | 38.521-1_TpAnalysisSpur(CA_n71A-n77A)_v1.zip | RAN5#100 |
| CA_n71A-n78A | 0 | 38.521-1_TpAnalysisSpur(CA_n71A-n78A)_v1.zip | RAN5#102 |

==============================================================

<< End of changes >>