3GPP TSG-RAN WG5 Meeting # 104	R5-244927

# Maastricht, Netherlands,  August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2943 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on 6.2A for inter-band CA transmitter power for the Pcmax tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Pcmax lower tolerance requirements for inter-band CA have been corrected from 6dB to 5dB for the case of 20 ≤ PCMAX < 21 in the core spec due to the rule of the tolerance changes in a monotonic way with the variation of output power. The test requirements for the transmitter power for inter-band CA should be corrected accordingly for the case of 20 ≤ PCMAX < 21. In addition, some test requirements for A-MPR in Table 6.2A.3.1.5-3, Table 6.2A.3.1.5-9 and Table 6.2A.3.1.5-11 are incorrect or inaccurate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the Pcmax lower tolerance requirements for inter-band CA configurations for the case of 20 ≤ PCMAX < 21. - Correct the test requirements for MPR in Table 6.2A.2.1.5-1. - Correct the test requirements for A-MPR in Table 6.2A.3.1.5-3, Table 6.2A.3.1.5-9 and Table 6.2A.3.1.5-11. - Other editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The transmitter power for the Pcmax tolerance for inter-band CA will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.1.5, 6.2A.3.1.5, 6.2A.4.0.1.3, 6.2A.4.1.4.1, 6.2A.4.1.4.3, 6.2A.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2A.2.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.2.1.5-1 to Table 6.2A.2.1.5-3.

Table 6.2A.2.1.5-1: UE Output Power for inter-band CA (2 UL CA) of PC3 test requirements

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0.2 | 0.2 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 3 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 4 | 23 | 1.2 | 1.2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 6 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 0.5 | 0.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 9 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 12 | 23 | 1 | 1 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 13 | 23 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 14 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 15 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 16 | 23 | 2 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 17 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 222 | 52 | 25+TT | 17-TT2 |
| 18 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 222 | 52 | 25+TT | 17-TT2 |
| 19 | 23 | 2.5 | 2.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 20 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 52 | 25+TT | 15-TT2 |
| 21 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 52 | 25+TT | 15-TT2 |
| 22 | 23 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 25+TT | 16.5-TT |
| 23 | 23 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 24 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 25 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 26 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 27 | 23 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 23 (22.52) | 3 (52) | 25+TT | 20-TT (17.5-TT2) |
| 28 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 29 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 30 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 31 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 32 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 33 | 23 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 2 | 25+TT | 17.5-TT |
| 34 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 35 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 36 | 23 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 6.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1a: UE Output Power for intra-band contiguous 2 UL CA of PC3 (contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19.5-TT | 25+TT | 18-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 2 | 2.5 | 4 | 25+TT | 17.5-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 2 | 2 | 3.5 | 25+TT | 19-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 2 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1b: UE Output Power for intra-band contiguous 2 UL CA of PC3 (contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18.5-TT | 25+TT | 17.5-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 3 | 2.5 | 4 | 25+TT | 17-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 3 | 2 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 3 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1c: UE Output Power for intra-band contiguous 2 UL CA of PC3 (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 2 | 2.5 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 2 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 2 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 2 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1d: UE Output Power for intra-band contiguous 2 UL CA of PC3 (non-contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 3 | 2.5 | 2.5 | 25+TT | 17.5-TT | 25+TT | 17-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 3 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 3 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 3 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1e: UE Output Power for intra-band non-contiguous 2 UL CA of PC3 test requirements (MPRIM3 to meet -13dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 3 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 4 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 5 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 7,8 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 9,10 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 11 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 12 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 13 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 14 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15,16 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 17,18 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 19 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 20 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 21 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 23,24 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 25,26 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 27 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 29 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 30 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 31,32 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 33,34 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 35 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 36 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 37 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 38 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39,40 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 41,42 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 43 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 44 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 45 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 46 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47,48 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 49,50 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 51 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 52 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 53 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 54 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 55,56 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 57,58 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 59 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 60 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 61 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 62 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 63,64 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 65,66 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 67 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 68 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 70 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 71,72 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |

Table 6.2A.2.1.5-1f: UE Output Power for intra-band non-contiguous 2 UL CA of PC3 test requirements (MPRIM3 to meet - 30dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 3 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 4 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 5 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 6,7 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 8,9 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 10 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 11 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 12 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 13,14 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 15,16 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 17 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 18 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 19 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 20,21 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 22,23 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 24 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 25 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 26 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 27,28 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 29,30 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 31 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 32 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 33 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 34,35 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 36,37 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 38 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 39 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 40 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 41,42 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 43,44 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 45 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 46 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 47 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 48,49 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 50,51 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 52 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 53 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 54 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 55,56 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 57,58 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 59 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 60 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 61 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 62,63 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |

Table 6.2A.2.1.5-2: Test Tolerance forMaximum Power Reduction (MPR) for inter-band CA

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell |  | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | BW ≤ 40MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.2.1.5-3: Test Tolerance for Maximum Power Reduction (MPR) for intra-band CA

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

Table 6.2A.2.1.5-4: UE Output Power for intra-band contiguous 2 UL CA of PC2 (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 2 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 3 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 4 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 5 | 26 | 2.5 | 2.5 | 0 | 23.5 | 23.5 | 3 | 3 | 3 | 28+TT | 20.5-TT | 28+TT | 20.5-TT |
| 6 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 7 | 26 | 3.0 | 5.0 | 0 | 23 | 21.0 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 18.-TT |
| 8 | 26 | 4.5 | 7.0 | 0 | 21.5 | 19.0 | 3 | 2 | 3.5 | 25+TT | 18.5-TT | 25+TT | 15.5-TT |
| 9 | 26 | 5.5 | 7.0 | 0 | 20.5 | 19.0 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 15.5-TT |
| 10 | 26 | 6.0 | 7.5 | 0 | 20 | 18.5 | 3 | 2.5 | 4 | 25+TT | 17.-TT | 25+TT | 14.5-TT |
| 11 | 26 | 2.5 | 3.5 | 0 | 23.5 | 22.5 | 3 | 3 | 2 | 25+TT | 20.5-TT | 25+TT | 19.5-TT |
| 12 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 13 | 26 | 3.0 | 3.5 | 0 | 23 | 22.5 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 19.5-TT |
| 14 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 15 | 26 | 3.5 | 5.0 | 0 | 22.5 | 21.0 | 3 | 2 | 2 | 25+TT | 19.5-TT | 25+TT | 18.-TT |
| 16 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 17 | 26 | 6.5 | 7.0 | 0 | 19.5 | 19.0 | 3 | 3.5 | 3.5 | 25+TT | 16.-TT | 25+TT | 15.5-TT |
| 18 | 26 | 6.5 | 8.0 | 0 | 19.5 | 18.0 | 3 | 3.5 | 4 | 25+TT | 16.-TT | 25+TT | 14.-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-5: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

Table 6.2A.2.1.5-6: UE Output Power for intra-band contiguous 2 UL CA of PC2 (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 3.0 | 3 | 0 | 23.0 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 2 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 3 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 4 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 5 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 6 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 7 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 8 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 9 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 10 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 11 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 12 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 13 | 26 | 6.5 | 6.5 | 0 | 19.5 | 19.5 | 3 | 3.5 | 3.5 | 28+TT | 16.-TT | 28+TT | 16.-TT |
| 14 | 26 | 7.0 | 7.5 | 0 | 19 | 18.5 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.5-TT |
| 15 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 16 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 17 | 26 | 7.0 (5.0)1 | 8.0 (6.0)1 | 0 | 19 (21.0)1 | 18.0 (20.0)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 15.5-TT (18.0-TT)1 | 28+TT | 14.-TT (17.0-TT)1 |
| 18 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 19 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 20 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 21 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 22 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 23 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 24 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 25 | 26 | 7.5 | 7.5 | 0 | 18.5 | 18.5 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.5-TT |
| 26 | 26 | 7.5 | 8.0 | 0 | 18.5 | 18.0 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.-TT |
| 27 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-7: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (non-contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

Table 6.2A.2.1.5-8: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements (both bands supporting PC3)

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 6 | 26 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 7 | 26 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 8 | 26 | 0.5 | 0.5 | 0 | 0 | 3 | 25.5 | 3 | 28+TT | 22.5-TT |
| 9 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 10 | 26 | 1 | 1 | 1.52 | 1.52 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 11 | 26 | 1 | 1 | 1.52 | 1.52 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 12 | 26 | 1 | 1 | 0 | 0 | 3 | 25.0 | 3 | 28+TT | 22-TT |
| 13 | 26 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 25.0 (23.52) | 3 | 28+TT | 22-TT (20.5-TT2) |
| 14 | 26 | 2 | 2 | 1.52 | 1.52 | 3 | 22.5 | 5 | 28+TT | 17.5-TT |
| 15 | 26 | 2 | 2 | 1.52 | 1.52 | 3 | 22.5 | 5 | 28+TT | 17.5-TT |
| 16 | 26 | 2 | 2 | 0 | 0 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 17 | 26 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.0 | 5 | 28+TT | 17-TT2 |
| 18 | 26 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.0 | 5 | 28+TT | 17-T |
| 19 | 26 | 2.5 | 2.5 | 0 | 0 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 20 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 20.0 | 6 | 28+TT | 15-TT |
| 21 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 20.0 | 6 | 28+TT | 15-TT |
| 22 | 26 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 28+TT | 18.5-TT |
| 23 | 26 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 24.5 (232) | 3 | 28+TT | 21.5-TT (20-TT2) |
| 24 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 25 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 26 | 26 | 3 | 3 | 0 | 0 | 3 | 23.0 | 3 | 28+TT | 20-TT |
| 27 | 26 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 24.0 (22.52) | 3 (52) | 28+TT | 21-TT (17.5-TT2) |
| 28 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 29 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 30 | 26 | 3 | 3 | 0 | 0 | 3 | 23.0 | 3 | 28+TT | 20-TT |
| 31 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 21.0 | 5 | 28+TT | 16-TT |
| 32 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 21.0 | 5 | 28+TT | 16-TT |
| 33 | 26 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 | 28+TT | 19.5-TT |
| 34 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 18.0 | 5 | 28+TT | 13-TT |
| 35 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 18.0 | 5 | 28+TT | 13-TT |
| 36 | 26 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 37 | 26 | 0 | 6.5 | 0 | 0 | 3 | 23.9 | 3 | 28+TT | 20.9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-8a: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements (PCC supporting PC3 and SCC supporting PC2)

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 6 | 26 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 7 | 26 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 8 | 26 | 0.5 | 0.5 | 0 | 0 | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 9 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 10 | 26 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| 11 | 26 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| 12 | 26 | 1 | 1 | 0 | 0 | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 13 | 26 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (25.32) | 3 | 28+TT | 23-TT (22.3-TT2) |
| 14 | 26 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 15 | 26 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 16 | 26 | 2 | 2 | 0 | 0 | 3 | 25.8 | 3 | 28+TT | 22.8-TT |
| 17 | 26 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 28+TT | 20.1-TT |
| 18 | 26 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 28+TT | 20.1-TT |
| 19 | 26 | 2.5 | 2.5 | 0 | 0 | 3 | 25.3 | 3 | 28+TT | 22.3-TT |
| 20 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 28+TT | 16.8-TT |
| 21 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 28+TT | 16.8-TT |
| 22 | 26 | 4.5 | 4.5 | 0 | 0 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 23 | 26 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.82) | 3 | 28+TT | 23-TT (21.8-TT2) |
| 24 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 25 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 26 | 26 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 28+TT | 21.8-TT |
| 27 | 26 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 25.8 (24.32) | 3 | 28+TT | 22.8-TT (21.3-TT2) |
| 28 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 29 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 30 | 26 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 28+TT | 21.8-TT |
| 31 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 28+TT | 17.8-TT |
| 32 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 28+TT | 17.8-TT |
| 33 | 26 | 3.5 | 3.5 | 0 | 0 | 3 | 24.3 | 3 | 28+TT | 21.3-TT |
| 34 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 28+TT | 14.8-TT |
| 35 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 28+TT | 14.8-TT |
| 36 | 26 | 6.5 | 6.5 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 37 | 26 | 0 | 6.5 | 0 | 0 | 3 | 24.6 | 3 | 28+TT | 21.6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-9: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements with supporting higherPowerLimit-r17

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 27.8 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8-TT (23.3-TT2) |
| 6 | 27.8 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 29.8+TT | 21.0-TT |
| 7 | 27.8 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 29.8+TT | 21.0-TT |
| 8 | 27.8 | 0.5 | 0.5 | 0 | 0 | 3 | 27.3 | 3 | 29.8+TT | 24.3-TT |
| 9 | 27.8 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8-TT (23.3-TT2) |
| 10 | 27.8 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 29.8+TT | 20.8-TT |
| 11 | 27.8 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 29.8+TT | 20.8-TT |
| 12 | 27.8 | 1 | 1 | 0 | 0 | 3 | 26.8 | 3 | 29.8+TT | 23.8-TT |
| 13 | 27.8 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 26.8 (25.32) | 3 | 29.8+TT | 23.8-TT (22.3-TT2) |
| 14 | 27.8 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 29.8+TT | 20.3-TT |
| 15 | 27.8 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 29.8+TT | 20.3-TT |
| 16 | 27.8 | 2 | 2 | 0 | 0 | 3 | 25.8 | 3 | 29.8+TT | 22.8-TT |
| 17 | 27.8 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 29.8+TT | 20.1-TT |
| 18 | 27.8 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 29.8+TT | 20.1-TT |
| 19 | 27.8 | 2.5 | 2.5 | 0 | 0 | 3 | 25.3 | 3 | 29.8+TT | 22.3-TT |
| 20 | 27.8 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 29.8+TT | 16.8-TT |
| 21 | 27.8 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 29.8+TT | 16.8-TT |
| 22 | 27.8 | 4.5 | 4.5 | 0 | 0 | 3 | 23.3 | 3 | 29.8+TT | 20.3-TT |
| 23 | 27.8 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 26.3 (24.82) | 3 | 29.8+TT | 23.3-TT (21.8-TT2) |
| 24 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 25 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 26 | 27.8 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 29.8+TT | 21.8-TT |
| 27 | 27.8 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 25.8 (24.32) | 3 | 29.8+TT | 22.8-TT (21.3-TT2) |
| 28 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 29 | 27.8 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 29.8+TT | 17.9-TT |
| 30 | 27.8 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 29.8+TT | 21.8-TT |
| 31 | 27.8 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 29.8+TT | 17.8-TT |
| 32 | 27.8 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 29.8+TT | 17.8-TT |
| 33 | 27.8 | 3.5 | 3.5 | 0 | 0 | 3 | 24.3 | 3 | 29.8+TT | 21.3-TT |
| 34 | 27.8 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 29.8+TT | 14.8-TT |
| 35 | 27.8 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 29.8+TT | 14.8-TT |
| 36 | 27.8 | 6.5 | 6.5 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 37 | 27.8 | 0 | 6.5 | 0 | 0 | 3 | 24.6 | 3 | 29.8+TT | 21.6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Next changes >>

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance for Additional Maximum Power Reduction (A-MPR)

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1a: UE Power Class 2 with both bands supporting PC3 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23.9 (22.72) | 3 (52) | 28+TT | 20.9-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1b: UE Power Class 2 with PCC supporting PC3 and SCC supporting PC2 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 (52) | 28+TT | 21.6TT (20.6-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: UE Power Class 2 with supporting higherPowerLimit-r17 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 27.8 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8TT (23.3-TT2) |
| 2 | 27.8 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 3 | 27.8 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 | 29.8+TT | 21.6-TT (20.6+-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 1 | 0 | 2 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 7 | 0 | 0 | 0 | 3 | 19 | 5 | 25+TT | 14-TT |
| 3 | 23 | 1 | 0 | 7 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 1 | 0 | 2 | 0 | 0 | 0 | 3 | 25 | 3 | 28+TT | 20-TT |
| 2 | 26 | 1 | 6.5 | 7 | 0 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 1 | 0 | 7 | 0 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

Table 6.2A.3.1.5-3b: UE Power Class 2 test requirement with PCC supporting PC2 and SCC supporting PC3 (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 3.5 | 0 | 5 | 0 | 0 | 0 | 3 | 25 | 3 | 28+TT | 20-TT |
| 2 | 26 | 3.5 | 6.5 | 10 | 0 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 3.5 | 0 | 10 | 0 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

Table 6.2A.3.1.5-3c: UE Power Class 2 test requirement with PCC and SCC supporting PC2 (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 3.5 | 0 | 5 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 3.5 | 6.5 | 10 | 0 | 0 | 0 | 3 | 21 | 5 | 28+TT | 16-TT |
| 3 | 26 | 3.5 | 0 | 10 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |

Table 6.2A.3.1.5-4: Void

Table 6.2A.3.1.5-5: Void

Table 6.2A.3.1.5-6: Void

Table 6.2A.3.1.5-7: UE Power Class 3 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 2 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 3 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 4 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 5 | 23 | 2.5 | 3.5 | 3.5 | 0 | 2 | 19.5 | 3.5 | 25+TT | 16-TT |
| 6 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 7 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 8 | 23 | 5 | 4.5 | 5 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 9 | 23 | 7 | 6 | 7 | 0 | 2 | 16 | 5 | 25+TT | 11-TT |
| 10 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 11 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 12 | 23 | 5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 13 | 23 | 7 | 8 | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |

Table 6.2A.3.1.5-8: UE Power Class 2 not supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 2 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 1-TT |
| 3 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 4 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 14-TT |
| 5 | 26 | 2.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 17.5-TT |
| 6 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 7 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 8 | 26 | 5 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 7 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-8a: UE Power Class 2 supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 2 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 3 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 4 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 5 | 26 | 3.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 18-TT |
| 6 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 7 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 8 | 26 | 6 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 8 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7.5 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-9: UE Power Class 3 test requirement (network signalling value NS_17/NS_01) for CA_n28-n78 and CA_n28-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 16.5 | 5 | 25+TT | 11.5-TT |
| 3 | 23 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 20 | 5 | 25+TT | 15-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-10: UE Power Class 2 test requirement (network signalling value NS_17/NS_01) for CA_n28-n79

| Test ID | PPowerclass | PPowerclass,c |  | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW (PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 23 | 26 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 23 | 26 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 21.2 | 5 | 28+TT | 16.2-TT |
| 3 | 26 | 23 | 26 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |

Table 6.2A.3.1.5-11: UE Power Class 3 test requirement (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 25+TT | 14-TT |
| 3 | 23 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-11a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

Table 6.2A.3.1.5-11b: UE Power Class 2 test requirement with with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 3.5 | 0 | 5 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 3.5 | 0 | 10 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 3.5 | 0 | 10 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Next changes >>

6.2A.4	Configured output power for CA

6.2A.4.0	Minimum conformance requirements

6.2A.4.0.1	Configured transmitted power level

6.2A.4.0.1.1	Void

6.2A.4.0.1.2	Void

6.2A.4.0.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.3.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in subclause 6.2.2.3 and subclause 6.2.3.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same slot symbol pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c), pPowerClass,c/(MAX(mprc,a-mprc)·tC,c ·tIB,c·tRxSRS,c), pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c, PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [6];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.0.3-1 without taking into account the tolerance specified in the Table 6.2A.1.0.3-1; If the UE indicates higherPowerLimit-r17 for an eligible CA configuration as specified in Table 6.2A.1.3-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

-	pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1.3-1 according to ue-PowerClassPerBandPerBC-r17  if indicated or ue-PowerClass otherwise without taking into account the tolerance;

-	ΔPPowerClass, CA = 3 dB for a power class 2 UE when the requirements of default power class are applied as specified in sub-clause 6.2.A.1.3; otherwise ΔPPowerClass, CA = 0 dB;

-	mpr c and a-mpr c are the linear values of MPR c and A-MPR c as specified in subclause 6.2.2.3 and subclause 6.2.3.3, respectively;

-	pmprc is the linear value of P-MPR c;

-	∆tRxSRS,c  is the linear value of ∆TRxSRS,c;

-	tC,c is the linear value of TC,ctC,c = 1.41 when NOTE 2 in Table 6.2A.1.0.3-1 applies for a serving cell c, otherwise tC,c = 1;

-	tIB,c  is the linear value of the inter-band relaxation term TIB,c of the serving cell c as specified in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC; otherwise tIB,c In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4] for the applicable operating bands.

-	PEMAX,CA is p-UE-FR1 value signalled by RRC and defined in [6]

For uplink inter-band carrier aggregation with one serving cell c per operating band when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in subclause 6.2.4.3.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on different bands:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),i (p) + pCMAX_L,f,c(i),j (q)], PPowerClass}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,f,c(i),i (p) + pCMAX_H,f,c(i),j (q)], PPowerClass}

where pCMAX_L,f,c (i),i  and pCMAX_H,f,c(i),i  are the respective limits PCMAX_L,f,c (i),i and PCMAX_H,f,c(i),i expressed in linear scale and pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise without taking into account the tolerance; If the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

TREF and Teval are specified in Table 6.2A.4.0.1.3-0 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. PPowerClass shall not be exceeded by the UE during any period of time.

Table 6.2A.4.0.1.3-0: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for slots i and i + 1 applies for any overlapping portion of slots i and i + 1. PPowerClass shall not be exceeded by the UE during any period of time.

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.0.1.3-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.3-1 for inter-band carrier aggregation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or symbol pattern, shall be within the following range:

P’CMAX_L–  MAX{TL, TLOW (P’CMAX_L)} ≤  P’UMAX  ≤  P’CMAX_H + THIGH (P’CMAX_H)

P’UMAX = 10 log10 ∑ p’UMAX,c

where p’UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P’CMAX) and THIGH(P’CMAX) for applicable values of P’CMAX are specified in Table 6.2A.4.0.1.3-1 for inter-band carrier aggregation. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.0.3-1 for inter-band carrier aggregation.

where:

P’CMAX_L  = MIN{ MIN {10log10∑(pCMAX_L,f,c(i),i), PPowerClass} over all overlapping slots in TREF}

P’CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c, PPowerClass} over all overlapping slots in TREF}

If the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c

Table 6.2A.4.0.1.3-1: PCMAX tolerance for uplink inter-band CA (two bands)

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

<<Unchanged texts are omitted>>

6.2A.4.1	Configured transmitted power for CA (2UL CA)

6.2A.4.1.1	Test purpose

To verify that the total measured UE configured maximum output power PUMAX in two uplink carrier aggregation is within the specified bounds.

6.2A.4.1.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward that support NR 2 UL CA.

6.2A.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.2A.4.0.

6.2A.4.1.4	Test description

6.2A.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.4.1.4.1-1 to table 6.2A.4.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.4.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range for PCC and SCC (NOTE 3) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in 5.3.5-1 |  | Lowest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs (NOTE 2) | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for minimum output power test case | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

Table 6.2A.4.1.4.1-2: Intra-band contiguous CA (contiguous RB allocation) Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM Pi/2 BPSK |  | Inner Full |
| 2 |  | DFT-s-OFDM QPSK |  | Inner Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |

Table 6.2A.4.1.4.1-3: Intra-band non-contiguous CA when UE supporting IE dualPA-Architecture Test Configuration

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | One frequency chosen arbitrarily from Low range or High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |  |  |
| Test SCS as specified in 5.3.5-1 |  | Lowest |  |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |  |  |  |  |
|  |  | Modulation for all CCs (NOTE 2) | RB allocation (NOTE 1) |  |  |  |  |  |
|  |  |  | SCS 15 kHz |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  | PCC | SCC | PCC | SCC | PCC | SCC |
| 1 | N/A | DFT-s-OFDM Pi/2 BPSK | 45@0 | 45@0 | 24@0 | 24@0 | 12@0 | 12@0 |
| 2 |  | DFT-s-OFDM QPSK | 45@0 | 45@0 | 24@0 | 24@0 | 12@0 | 12@0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Tables 6.2A.4.1.4.1-1, 6.2A.4.1.4.1-2 and Table 6.2A.4.1.4.1-3 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.4.1.4.3.

6.2A.4.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.4.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.2A.4.1.4.1-1, 6.2A.4.1.4.1-2 and 6.2A.4.1.4.1-3 as appropriate on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD, only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

6.2A.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions.

Table 6.2A.4.1.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2A.4.1.4.3-1: FrequencyInfoUL: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2A.4.1.4.3-1a: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2A.4.1.4.3-2: FrequencyInfoUL: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2A.4.1.4.3-2a: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2A.4.1.4.3-3: FrequencyInfoUL: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 5 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-3a: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 5 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-4: FrequencyInfoUL: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-4a: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 | UL intra-band contiguous CA with contiguous RB allocation |  |

Table 6.2A.4.1.4.3-5: FrequencyInfoUL: Test point 5

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 9 | UL intra-band non-contiguous CA when MPRIM3 to meet -13dBm/MHz |  |

Table 6.2A.4.1.4.3-5a: FrequencyInfoUL-SIB: Test point 5

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 9 | UL intra-band non-contiguous CA when MPRIM3 to meet -13dBm/MHz |  |

6.2A.4.1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2A.4.1.5-1 to Table 6.2A.4.1.5-1b.

Table 6.2A.4.1.5-1: Configured UE Output Power for Inter-band CA

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 7 dBm - TT | 19 dBm + TT |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.4.1.5-2. |  |  |

Table 6.2A.4.1.5-1a: Configured UE Output Power for Intra-band contiguous CA of PC2 and PC3 (contiguous RB allocation)

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 8 dBm - TT | 18 dBm + TT |
| Measured UE output power test point 3 | 2 dBm - TT | 14 dBm + TT |
| Measured UE output power test point 4 | 14 dBm - TT | 22 dBm + TT |
| Note 1: TT for each intra-band UL CA configuration is in table 6.2A.4.1.5-3. |  |  |

Table 6.2A.4.1.5-1b: Configured UE Output Power for Intra-band non-contiguous CA

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 5 (Note 2) | 6 dBm - TT | 18 dBm + TT |
| Note 1: TT for each intra-band UL CA configuration is in table 6.2A.4.1.5-3. Note 2: Only applicable when MPRIM3 to meet -13dBm/MHz |  |  |

Table 6.2A.4.1.5-2: Test Tolerance for Configured transmitted power for inter-band CA

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.4.1.5-3: Test Tolerance for Configured transmitted power for intra-band CA

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

==============================================================

<< End of changes >>