3GPP TSG-RAN5 Meeting #104	R5-244968

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0068 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New Test Case 6.8.2 Message Store Function Folders |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no Message Store test cases for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Message Store test case 6.8.2 for Message Store Function Folders |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message Store Function Folders would not be tested for MCData |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.8.2	On-network / MCData Message Store / Folders / Discover a folder / Create a folder / Object and folder copy / Search a folder / Move a folder / Delete a folder

6.8.2.1	Test Purpose (TP)

(1)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to discover its main folder ID in the Message Store Function }

then { UE (MCData Client) requests to discover its main folder ID in the Message Store Function via an HTTP POST message }

}

(2)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to create a folder in the Message Store Function }

then { UE (MCData Client) requests to create a folder in the Message Store Function via an HTTP POST message }

}

(3)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to upload a file to the Message Store Function }

then { UE (MCData Client) uploads the file to the Message Store Function via an HTTP POST message }

}

(4)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to copy an object from one folder to another folder in the Message Store Function }

then { UE (MCData Client) copies an object from one folder to another folder in the Message Store Function via an HTTP POST message }

}

(5)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to search for information about the main folder in the  Message Store Function }

then { UE (MCData Client) searches for information about the main folder in the Message Store Function via an HTTP GET message }

}

(6)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to move a folder and its contents into another folder in the Message Store Function }

then { UE (MCData Client) moves the folder into another folder in the Message Store Function via an HTTP POST message }

}

(7)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to delete a folder from the Message Store Function }

then { UE (MCData Client) deletes the folder from the Message Store Function via an HTTP DELETE message }

}

6.8.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 21.2.11.1, 21.2.8.1, 21.2.15.1, 21.2.6.1, 21.2.18.1, 21.2.10.1, 21.2.7.1. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282 clause 21.2.11.1]

To search for information about a selected set of folder(s) in the message store, the message store client, acting as an HTTP client shall follow the procedure described in subclause 6.16 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.16.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send the HTTP POST request, which may include a SelectionCriteria, towards the message store function.

Upon receipt of a HTTP response, the message store client should follow the procedure as described in subclause 6.16.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.8.1]

To create a folder in message store using the message store function, the message store client, acting as an HTTP client shall follow the procedure described in subclause 6.13 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.13.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send towards the message store function the HTTP POST request identifying the target folder where the new folder is to be created.

Upon receipt of a HTTP response, the message store client should follow the procedure as described in subclause 6.13.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.15.1]

To upload the object(s) to the message store, the message store client acting as an HTTP client, shall follow the procedure described in subclause 6.1 for single upload and subclause 6.10 for bulk upload as specified in the OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.1.5 and 6.10.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send the HTTP POST request towards the message store function.

Upon receipt of an HTTP response, the message store client shall follow the procedure as described in subclause 6.1.2 for single upload and 6.10.2 for bulk upload as specified in the OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.6.1]

To copy object(s) and/or folder(s) to a destination folder in message store, the message store client, acting as an HTTP client, shall follow the procedure described in subclause 6.18 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.18.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send HTTP POST request identifying the target folder and the source objects(s) and/or folder(s) for copying operation towards the message store function.

Upon receipt of an HTTP response, the message store client should follow the procedure as described in subclause 6.18.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.18.1]

To list subfolders of a given folder identified by its folder ID in the message store using the message store function, the message store client, acting as an HTTP client shall follow the procedure described in clause 6.14 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with the following clarification:

1)	shall generate an HTTP GET request as specified in clause 6.14.3 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with the following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header;

c)	shall set the query string "listFilter" to:

i)	"Subfolders" if only a list of subfolders is to be returned;

ii)	"Objects" if only a list of objects is to be returned; or

iii)	"All" if a list all contents of the specified folder is to be returned; and

2)	shall send the HTTP GET request towards the message store function.

NOTE:	in order for the message store client to list the subfolders of the root folder, it first needs to discover its folder ID as described in clause 5.1.6 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] using Folder search procedure specified in clause 21.2.11 of the present document.

Upon receipt of an HTTP response, the message store client should follow the procedure as described in clause 6.14.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.10.1]

To move object(s) and/or folder(s) to a destination folder in the message store, the message store client, acting as an HTTP client shall follow the procedure described in subclause 6.19 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.19.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send the HTTP POST request, identifying source objects and/or folder(s) to be moved to the designated destination folder, towards the message store function.

Upon receipt of a HTTP response, the message store client shall follow the procedure as described in subclause 6.19.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.7.1]

To delete a folder in message store using the message store function, the message store client, acting as an HTTP client shall follow the procedure described in subclause 6.14 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP DELETE request as specified in subclause 6.14.6 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send the HTTP DELETE request identifying the folder to be deleted towards the message store function.

Upon receipt of a HTTP response, the message store client should follow the procedure as described in subclause 6.14.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

6.8.2.3	Test description

6.8.2.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.879-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.879-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.8.2.3.2	Test procedure sequence

Table 6.8.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) search for information about the main folder in the message store. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Retrieve using HTTP' as described in TS 36.579-1 [2] Table 5.3C.16.2-1 to search for information about the main folder in the message store? | - | - | 1 | P |
| 3 | Make the UE (MCData client) create folder in the the message store. (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Object Upload or Creation using HTTP' as described in TS 36.579-1 [2] Table 5.3C.14.2-1 to create a folder in the message store? | - | - | 2 | P |
| 5 | Make the UE (MCData client) upload an object, test file 1 (TS 36.579-7 A.2.1), to the message store in the new created folder. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Object Upload or Creation using HTTP' as described in TS 36.579-1 [2] Table 5.3C.14.2-1 to upload an object to the message store? | - | - | 3 | P |
| 7 | Make the UE (MCData client) create another folder in the the message store. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Object Upload or Creation using HTTP' as described in TS 36.579-1 [2] Table 5.3C.14.2-1 to create a folder in the message store? | - | - | 2 | P |
| 9 | Make the UE (MCData client) copy an object, test file 1 (TS 36.579-7 A.2.1), from Folder1 to Folder2 in the message store. (NOTE 1) | - | - | - | - |
| 10 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Post Request using HTTP' as described in TS 36.579-1 [2] Table 5.3C.17.2-1 to copy an object, test file 1 (TS 36.579-7 A.2.1), from Folder1 to Folder2 in the message store? | - | - | 4 | P |
| 11 | Make the UE (MCData client) search for information about the main folder in the message store. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Retrieve using HTTP' as described in TS 36.579-1 [2] Table 5.3C.16.2-1 to search for information about the main folder in the message store? | - | - | 5 | P |
| 13 | Make the UE (MCData client) move a folder and its contents, Folder2, into Folder1 in the message store. (NOTE 1) | - | - | - | - |
| 14 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Post Request using HTTP' as described in TS 36.579-1 [2] Table 5.3C.17.2-1 to move a folder and its contents, Folder2, into Folder1 in the message store? | - | - | 6 | P |
| 15 | Make the UE (MCData client) delete a folder, Folder1, including all the contained folders and objects. (NOTE 1) | - | - | - | - |
| 16 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Delete using HTTP' as described in TS 36.579-1 [2] Table 5.3C.15.2-1 to delete a folder in the message store? | - | - | 7 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.8.2.3.3	Specific message contents

Table 6.8.2.3.3-1: HTTP GET from the UE (Step 2, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.16.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "GET" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_UR | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.11.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| SelectionCriteria |  |  | RESTful Network API for NMS [40], clauses 6.8, 6.16 |  |
| maxEntries | 3 | the maximum number of entries in the response |  |  |
| searchCriteria |  |  |  |  |
| criterion |  |  |  |  |
| Date | Not present |  |  |  |
| Attribute | Not present |  |  |  |
| name | "root" |  |  |  |
| value | "Yes" |  |  |  |
| AllTextAttributes | Not present |  |  |  |
| Flag | Not present |  |  |  |
| WholeWord | Not present |  |  |  |
| VanishedObjects | Not present |  |  |  |
| CreatedObjects | Not present |  |  |  |
| PresetSearch | Not present |  |  |  |
| FileName | Not present |  |  |  |

Table 6.8.2.3.3-2: HTTP 200 (OK) from the SS (step 2, Table 6.8.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.16.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ETag |  |  |  |  |
| entity-tag | Any value as selected by the SS |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| folderList |  |  | RESTful Network API for NMS [40], clauses 6.8, 6.16 |  |
| folder |  |  |  |  |
| name | tsc_MCData_MsgSF_URI & "/userA" | The main folder of the MCData User |  |  |

Table 6.8.2.3.3-3: HTTP POST from the UE (Step 4, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.8.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Folder |  |  | RESTful Network API for NMS [40], clause 6.13 |  |
| parentFolderPath | “/userA” | The main folder of the MCData User |  |  |
| name | “Folder1” | The name of the new folder |  |  |

Table 6.8.2.3.3-4: HTTP 201 Created from the SS (step 4, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Reference |  |  | RESTful Network API for NMS [40], clause 6.13 |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder1" | URL identifying the location of the created folder |  |  |

Table 6.8.2.3.3-5: HTTP POST from the UE (Step 6, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.15.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | Folder | RESTful Network API for NMS [40], clause 6.1 |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | “application/xml” |  |  |  |
| MIME-part-body |  | tsc_MCData_MsgSF_URI & "/Folder1" |  | URL identifying the location of the created folder |  |  |  |  |  |
| MIME body part |  | File content |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/octet-stream" |  |  |  |
| MIME-part-body | binary data representing the file |  |  |  |

Table 6.8.2.3.3-6: HTTP 201 Created from the SS (step 6, Table 6.8.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Location |  |  | RESTful Network API for NMS [40], clause 6.1 |  |
| uri | tsc_MCData_MsgSF_URI & "/Folder1/file-location-1" | URL identifying the location of the stored file |  |  |

Table 6.8.2.3.3-7: HTTP POST from the UE (Step 8, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | "Bearer" |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.8.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Folder |  |  | RESTful Network API for NMS [40], clause 6.13 |  |
| parentFolderPath | "/userA" | The main folder of the MCData User |  |  |
| name | "Folder2" | The name of the new folder |  |  |

Table 6.8.2.3.3-8: HTTP 201 Created from the SS (step 8, Table 6.8.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Reference |  |  | RESTful Network API for NMS [40], clause 6.13 |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder2" | URL identifying the location of the created folder |  |  |

Table 6.8.2.3.3-9: HTTP POST from the UE (Step 10, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | "Bearer" |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.6.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| targetSourceRef |  |  | RESTful Network API for NMS [40], clause 6.18 |  |
| targetRef |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder1/file-location-1" | URL identifying the location of the stored file |  |  |
| sourceRefs |  |  |  |  |
| objects |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder2/" | URL identifying the location of the copied stored file |  |  |

Table 6.8.2.3.3-10: HTTP 200 (OK) Created from the SS (step 10, Table 6.8.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ETag |  |  |  |  |
| entity-tag | Any value as selected by the SS |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| bulkResponseList |  |  | RESTful Network API for NMS [40], clause 6.18 |  |
| allSuccess | "true" |  |  |  |
| Response |  |  |  |  |
| code | "200" |  |  |  |
| reason | "OK" |  |  |  |
| success |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder2/file-location-1" | URL identifying the location of the copied stored file |  |  |

Table 6.8.2.3.3-11: HTTP GET from the UE (Step 12, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.16.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "GET" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_UR | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.18.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| listFilter |  |  | RESTful Network API for NMS [40], clause 6.14 |  |
| media-type | “Subfolders” |  |  |  |

Table 6.8.2.3.3-12: HTTP 200 (OK) from the SS (step 12, Table 6.8.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.16.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ETag |  |  |  |  |
| entity-tag | Any value as selected by the SS |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Folder |  |  | RESTful Network API for NMS [40], clause 6.14 |  |
| parentFolder | tsc_MCData_MsgSF_URI & "/userA" | The main folder of the MCData User |  |  |
| subFolders |  |  |  |  |
| folderReference |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/userA/Folder1" |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/userA/Folder2" |  |  |  |

Table 6.8.2.3.3-13: HTTP POST from the UE (Step 14, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI | The absolute URI identifying the resource on a media storage function |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | "Bearer" |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.10.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| targetSourceRef |  |  | RESTful Network API for NMS [40], clause 6.19 |  |
| targetRef |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder1" | URL identifying the location of the target folder |  |  |
| sourceRefs |  |  |  |  |
| objects |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder2/" | URL identifying the location of the moved folder |  |  |

Table 6.8.2.3.3-14: HTTP 200 (OK) Created from the SS (step 14, Table 6.8.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ETag |  |  |  |  |
| entity-tag | Any value as selected by the SS |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| bulkResponseList |  |  | RESTful Network API for NMS [40], clause 6.19 |  |
| allSuccess | "true" |  |  |  |
| Response |  |  |  |  |
| code | "200" |  |  |  |
| reason | "OK" |  |  |  |
| success |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_URI & "/Folder2" | URL identifying the moved folder |  |  |

Table 6.8.2.3.3-15: HTTP DELETE from the UE (Step 16, Table 6.8.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.15.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.5-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "DELETE" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_URI & "/Folder1" |  |  |  |
| Authorization |  |  |  |  |
| authentication-scheme | “Bearer” |  |  |  |
| b64token | Access token as assigned to the UE by Token Response |  |  |  |
| Host |  |  |  |  |
| host | tsc_MCData_MsgSF_Hostname | hostname identifying the message store function | TS 24.282 [87], clause 21.2.7.1 |  |
| port | not present |  |  |  |

<END OF CHANGES>