3GPP TSG-RAN5 Meeting #104	R5-244985

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4610 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 12.3.1.2.2 Sidelink Relay / L2 U2N / Relay UE / 5G ProSe U2N Relay Discovery / Announcing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Added of new test case 12.3.1.2.2 Sidelink Relay / L2 U2N / Relay UE / 5G ProSe U2N Relay Discovery / Announcing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition new test case 12.3.1.2.2 Sidelink Relay / L2 U2N / Relay UE / 5G ProSe U2N Relay Discovery / Announcing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.2.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0504 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.3.1.2	Relay UE

12.3.1.2.1

12.3.1.2.2	Sidelink Relay / L2 U2N / Relay UE / 5G ProSe U2N Relay Discovery / Announcing

12.3.1.2.2.1	Test Purpose (TP)

(1)

with { UE being authorized for performing restricted 5G ProSe direct discovery model A announcing and having sent a DISCOVERY_REQUEST message }

ensure that {

when { the SS does not answer to the UE }

then { UE retransmits the DISCOVERY_REQUEST message }

}

(2)

with { UE being authorized for performing the announcing UE procedure for UE-to-network relay discovery }

ensure that {

when { UE is configured by upper layer to transmit NR sidelink discovery message and SIB12 includes sl-TxPoolSelectedNormal }

then { UE transmits PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement using the pools of resources indicated by sl-TxPoolSelectedNormal }

}

(3)

with { UE being authorized for performing the announcing UE procedure for UE-to-network relay discovery }

ensure that {

when { UE is configured by upper layer to transmit NR sidelink discovery message and SIB12 includes sl-TxPoolExceptional }

then { UE transmits PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement using the pools of resources indicated by sl-TxPoolExceptional }

}

(4)

with { UE being authorized for performing the announcing UE procedure for UE-to-network relay discovery }

ensure that {

when { UE is configured by upper layer to transmit NR sidelink discovery message and SIB12 includes sl-DiscTxPoolSelected }

then { UE transmits PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement using the pools of resources indicated by sl-DiscTxPoolSelected }

}

12.3.1.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.8.13.3, TS 23.304 clause 6.3.1.2, TS 23.304 clause 6.3.2.3.2, TS 24.554 clause 6.2.3.6.1 and TS 24.554 clause 8.2.1.2.2.2.

[TS 38.331, clause 5.8.13.3]

A UE capable of NR sidelink discovery that is configured by upper layer to transmit NR sidelink discovery message shall:

1>	if the frequency used for NR sidelink discovery is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message; or if the frequency used for NR sidelink discovery is included in sl-FreqInfoList within SIB12:

2>	if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message:

…

2>	else if the cell chosen for NR sidelink discovery transmission provides SIB12:

3>	if the UE is acting as NR sidelink U2N Relay UE and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Relay UE threshold conditions as specified in 5.8.14.2 are met based on sl-RelayUE-ConfigCommon in SIB12; or

3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

3>	if the UE is performing NR sidelink non-relay discovery:

4>	if SIB12 includes sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency, and a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in the sl-DiscTxPoolSelected for NR sidelink discovery transmission is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-AllowedResourceSelectionConfig, is selected:

5>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency in SIB12 as defined in TS 38.321 [3];

4>	else if SIB12 includes sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency, and a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in the sl-TxPoolSelectedNormal for NR sidelink discovery transmission is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-AllowedResourceSelectionConfig, is selected:

5>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency in SIB12 as defined in TS 38.321 [3];

4>	else if SIB12 includes sl-TxPoolExceptional for the concerned frequency:

5>	from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or

5>	if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]; or

5>	if sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency is not included in SIB12 and if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection (as defined in TS 38.321 [3]) using one of the pools of resources indicated by sl-TxPoolExceptional for NR sidelink discovery transmission on the concerned frequency;

…

[TS 23.304, clause 6.3.1.2]

Figure 6.3.1.2-1: Overall procedure for Model A 5G ProSe Direct Discovery

This procedure is applied for open and restricted 5G ProSe Direct Discovery when the ProSe enabled UE is served by NG-RAN.

1.	Service authorisation for 5G ProSe Direct Discovery services is performed for as defined in clause 6.2.

If the UE is authorised to announce:

2a.	When the UE is triggered to announce, then it sends a discovery request for announcing to the 5G DDNMF in HPLMN as defined in clause 6.3.1.4. In addition, for restricted 5G ProSe Direct Discovery, the 5G DDNMF further interacts with the ProSe Application server for the authorization of the discovery request.

3a.	If the request is successful and is provided with ProSe Application Code/ProSe Restricted Code, it starts announcing on PC5 interface.

For ProSe restricted discovery and UE requests "on demand" announcing, ProSe Restricted Code may be provided to UE after this procedure. In this case, UE waits for the ProSe Restricted Code allocation and starts to announce the ProSe Restricted Code on PC5 after receiving it in Announcing Alert procedure specified in clause 6.3.1.6.

NOTE 1:	More details on the Access Stratum protocol of this step are provided in RAN specifications.

…

[TS 23.304, clause 6.3.2.3.2]

Depicted in Figure 6.3.2.3.2-1 is the procedure for 5G ProSe UE-to-Network Discovery with Model A.

Figure 6.3.2.3.2-1: 5G ProSe UE-to-Network Relay Discovery with Model A

1.	The 5G ProSe UE-to-Network Relay sends a UE-to-Network Relay Discovery Announcement message. The UE-to-Network Relay Discovery Announcement message contains the Type of Discovery Message, Announcer Info and RSC, and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

For 5G ProSe Layer-3 UE-to-Network Relay, the 5G ProSe Layer-3 UE-to-Network Relay shall only include a RSC in the UE-to-Network Relay Discovery Announcement when the S-NSSAI associated with that RSC belongs to the Allowed NSSAI of the UE-to-Network Relay.

The 5G ProSe Remote UE (1 to 3) determines the Destination Layer-2 ID for signalling reception. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.4.1.

5G ProSe Remote UE (1 to 3) monitors announcement messages with the 5G ProSe UE-to-Network RSC corresponding to the desired services.

Optionally, the 5G ProSe UE-to-Network Relay may also send Relay Discovery Additional Information messages as defined in clause 6.5.1.3. The parameters contained in this message and the Source Layer-2 ID and Destination Layer-2 ID used for sending and receiving the message are described in clause 5.8.3.

The 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay based on the information received in step 1.

[TS 24.554, clause 6.2.3.6.1]

The following abnormal cases can be identified:

a)	Indication from the transport layer of transmission failure of DISCOVERY_REQUEST message (e.g., after TCP retransmission timeout)

The UE shall close the existing secure connection to the 5G DDNMF, establish a new secure connection and then restart the announce request procedure.

NOTE 1:	The UE can abort this procedure when detecting loss of NG-RAN coverage in its serving PLMN.

b)	No response from the 5G DDNMF after the DISCOVERY_REQUEST message has been successfully delivered (e.g., TCP ACK has been received for the DISCOVERY_REQUEST message)

The UE shall retransmit the DISCOVERY_REQUEST message.

NOTE 2:	The UE can abort this procedure when detecting loss of NG-RAN coverage in its serving PLMN.

NOTE 3:	The timer to trigger retransmission and the maximum number of allowed retransmissions are UE implementation specific.

…

[TS 24.554, clause 8.2.1.2.2.2]

The UE is authorised to perform the announcing UE procedure for UE-to-network relay discovery if:

a)	the UE is authorised to act as a UE-to-network relay in the PLMN indicated by the serving cell as specified in clause 5.2.5, and

1)	the UE is served by NG-RAN and the UE is authorised to perform 5G ProSe direct discovery in the PLMN as specified in clause 5; or

2)	the UE is authorised to perform 5G ProSe direct discovery when not served by NG-RAN as specified in clause 5 and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be announced as specified in clause 5.2.5 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe layer-3 UE-to-network relay UE,

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11]; and

2)	the User info ID for the UE-to-network relay discovery parameter as specified in clause 5.2.5;

c)	for 5G ProSe layer-3 UE-to-network relay UE, the UE is configured with PDU Session parameters which is used for relayed traffic for the associated relay service code, as specified in clause 5.2.5; and

d)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the announcing UE procedure for UE-to-network relay discovery.

Figure 8.2.1.2.2.2.1 illustrates the interaction of the UEs in the announcing UE procedure for UE-to-network relay discovery.

Figure 8.2.1.2.2.2.1: Announcing UE procedure for UE-to-network relay discovery

…

12.3.1.2.2.3	Test Description

12.3.1.2.2.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1.

-	System information combination NR-31 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used.

-	NR-SS-UE

-	NR-SS-UE1 operating as L2 U2N Remote UE.

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

- The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.4 and ProSeP using condition Announcing_Restricted_Model_A as per TS 38.508-1 [4] clause 4.7C.1.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

12.3.1.2.2.3.2	Test procedure sequence

Table 12.3.1.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1 to 8 of the generic procedure described in TS 38.508-1 table 4.9.43.2.2-1 is performed. | - | - | - | - |
| 9 | The UE transmits a DISCOVERY_REQUEST message over the PC3a interface. | --> | HTTP Request containing DISCOVERY_REQUEST | 1 | - |
| 10 | Check: Does the UE retransmit a DISCOVERY_REQUEST message over the PC3a interface in 30 seconds. | --> | HTTP Request containing DISCOVERY_REQUEST | 1 | P |
| 11 | The SS-NW transmits a DISCOVERY_RESPONSE message over the PC3a interface. | <-- | HTTP Response containing DISCOVERY_RESPONSE | - | - |
| 12 | Check: Does the UE transmit PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement using the pools of resources indicated in sl-TxPoolSelectedNormal? | --> | PC5-D: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Announcement) | 2 | P |
| 13 | The SS-NW transmits an RRCRelease message to release RRC connection and move to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |
| 14 | SS-NW resets SIB12 according to table 12.3.1.2.2.3.3-1, The ValueTag of SIB12 in the SI-SchedulingInfo of SIB1 is increased. | - | - | - | - |
| 15 | SS-NW notifies UE of the system information change by send Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 16 | Check: Does the UE transmit PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement using the pools of resources indicated in sl-TxPoolExceptional? | --> | PC5-D: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Announcement) | 3 | P |
| 17 | SS-NW resets SIB12 according to table 12.3.1.2.2.3.3-2, The ValueTag of SIB12 in the SI-SchedulingInfo of SIB1 is increased. | - | - | - | - |
| 18 | SS-NW notifies UE of the system information change by send Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 19 | Check: Does the UE transmit PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement using the pools of resources indicated in sl-DiscTxPoolSelected? | --> | PC5-D: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Announcement) | 4 | P |

12.3.1.2.2.3.3	Specific message contents

Table 12.3.1.2.2.3.3-1: DISCOVERY_REQUEST (Steps 9 and 10, Table 12.3.1.2.2.3.2-1)

Derivation Path: TS 38.508-1[4], Table 4.7C.4-1 with condition Announce AND Restricted AND Model A

Table 12.3.1.2.2.3.3-2: DISCOVERY_RESPONSE (Step 11, Table 12.3.1.2.2.3.2-1)

Derivation Path: TS 38.508-1[4], Table 4.7C.4-2 with condition Announce AND Restricted

Table 12.3.1.2.2.3.3-1: PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement (Steps 12, 16 and 19, Table 12.3.1.2.2.3.2-1)

Derivation Path: TS 38.508-1[4], Table 4.7C.2-8 with condition L2RelayUE

Table 12.3.1.2.2.3.3-2: SL-BWP-ConfigCommon (Step 14, Table 12.3.1.2.2.3.2-1)

| Derivation Path: TS 38.508-1[4], Table 4.6.6-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon with condition RXPOOL and EXCEPTIONAL |  |  |
| } |  |  |  |  |

Table 12.3.1.2.2.3.3-3: SL-BWP-ConfigCommon (Step 17, Table 12.3.1.2.2.3.2-1)

| Derivation Path: TS 38.508-1[4], Table 4.6.6-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon with condition RXPOOL |  |  |
| sl-BWP-DiscPoolConfigCommon-r17 |  | SL-BWP-DiscPoolConfigCommon with condition DISC_RXPOOL and DISC_SELECTED |  |  |
| } |  |  |  |  |

<End of Changed Section>