3GPP TSG-RAN WG5 Meeting #104	R5-245059

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	On testing correct identification of the start of the non-integer DRX cycle

Agenda item:	6.3.21.7

Document for:	Discussion

Introduction

The work plan for XR enhancements for NR was agreed in [1]. Among others, the following RAN protocol test purpose was agreed for non-integer DRX operation.

Agreements:

TS 38.321 5.7: Test Purpose 1: with { UE in RRC_CONNECTED state } ensure that { when { Non-Integer Long DRX cycle is configured and drx-TimeReferenceSFN is configured and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = floor([(drx-TimeReferenceSFN × 10) + drx-StartOffset] modulo (drx-NonIntegerLongCycle)) } then { UE starts the OnDurationTimer and monitors the PDCCH for OnDurationTimer PDCCH-Occasions} }

In this paper, we discuss the test objectives and procedure based on previous discussions and agreements in RAN WG1 and WG2.

Discussion

Non-integer operation of DRX long cycle can be configured in DRX-ConfigExt2 to a UE indicating support of DRX_NONINTEGER_CAPABILITY.

DRX-ConfigExt2-v1800 ::=                SEQUENCE {

drx-NonIntegerLongCycleStartOffset-r18  CHOICE {

ms1001over240                           INTEGER(0..3),

ms25over6                               INTEGER(0..3),

ms25over3                               INTEGER(0..7),

...

ms50over3                               INTEGER(0..15),

...

},

The parameter drx-NonIntegerLongCycleStartOffset indicates the length of the long DRX cycle (i.e., drx-NonIntegerLongCycle) as well as the subframe within the DRX cycle when the OnDurationTimer shall be triggered (i.e., drx-StartOffset). More specifically, the formula specified in TS 38.321 5.7 for non-integer DRX operation uses the parameter drx-NonIntegerLongCycleStartOffset to trigger the OnDurationTimer as follows:

1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset: 2> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3: 3> ... 2> else: 3> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

It can be observed that the formula contains a modulo operation where the dividend is a non-integer number (precisely a rational number). Depending on the implementation of the formula this may lead to rounding errors due to the numerical precision used for the representation of the fractional number. Therefore, the UE and network potentially trigger the OnDurationTimer at two different time instants/subframes.

During the RAN2 WI phase, the problem of rounding errors with non-integer DRX cycle was discussed in several contributions [2][3][4] and captured in chair’s notes. It was showed that an incorrect implementation of the formula specified in TS 38.321 5.7 results in rounding errors that cause the activation of the OnDurationTimer in a wrong subframe. In particular, an incorrect implementation may identify the beginning of the DRX cycle and trigger the OnDurationTimer a number of slots earlier or later than expected. Despite this problem, TS 38.321 5.7 does not indicate how non-integer DRX operations shall be executed to avoid such issue.

TS 38.321 clause 5.7: The MAC entity shall ensure no rounding error is generated when performing the modulus operation with drx-NonIntegerShortCycle or drx-NonIntegerLongCycle as the divisor.

During RAN2 WI phase the problem of rounding errors with non-integer DRX cycle was discussed and a note was added to TS 38.321 clause 5 indicating that MAC entity shall ensure no rounding error for non-integer DRX operations.

To better illustrate the problem statement, hereafter, we present a detailed analysis of the formula specification and potential issues if modulo operation is implemented as suggested in [4]. Without loss of generality, we describe the example for the long cycle case, and the target formula is as follows:

floor ([(SFN × 10) + subframe number] modulo (drx- NonIntegerLongCycle)) = drx-StartOffset

Now for the sake of convenience, let us define the following shorthand:

- A = [(SFN × 10) + subframe number]: an integer number.

- B / C = drx- NonIntegerLongCycle: a rational number represented by the ratio of two integers (B and C).

- Z = drx-StartOffset: an integer number.

Then the above formula reduced to

floorA modulo BC=Z

Now if we apply a formula like the one proposed in [4], we have:

floorA-BC×floorA×CB=Z                                                       (1)

Therefore, we analyze equation (1) as follows. Firstly, A×C is an integer since it is the product of two integers, and B is an integer, which results in that A×CB is a rational number as well. Thus, floorA×CB is an integer number. However, the fraction BC is still a rational number. Multiplying an integer floorA×CB by a fractional number BC result in different values depending on the numerical precision used to represent the fractional number in a given computation platform. To be more explicit, the rational number BC and recurrent decimal (e.g., 50/3 = 16.6666…) represented in machine can cause rounding errors. The numerical representation of the fractional number may cause the product to wrap up the nearest integer toward infinity. This is seen as the machine-dependent problem which cannot be overcome in the practical implementation.

- For example, let F be a fractional number and I be an integer number. The product of F×I can result in different values depending on the representation of the fractional F in a computer machine. Let I=50 and F=13 (note that this is the typical example 503=16.6666…):

- If the fractional number F is represented as F=13=0.3 by a computer machine, we get F×I=50×0.3=15, which result in floorF×I=floor50×0.3=15.

- If the fractional number F is represented as F=13=0.33 by a computer machine, we get F×I=50×0.33=16.5, which result in floorF×I=floor50×0.33=16.

- If the fractional number F is represented as F=13=0.333 by a computer machine, we get F×I=50×0.333=16.6, which result in floorF×I=floor50×0.333=16.

As discussed above, to avoid the numerical errors due to the numerical precision of fractional numbers by a computer machine, the DRX formula with rational numbers may adopt the following implementation:

floorA×C-B×floorA×CBC=Z                                                                  (2)

In this way, the rounding error caused by floor operation of a rational number BC multiplied by an integer number floorA×CB will be avoided.

To demonstrate the mismatch between two implementations (1) and (2) of the DRX formula, we evaluate the start of the DRX cycle with the following non-integer configuration:

- drx- NonIntegerLongCycle: ms50over3 (i.e., 50/3 ms: B = 50 ms and C = 3).

- drx-StartOffset: 0.

This configuration corresponds to a repeating pattern of 3 subsequent DRX long cycles with durations 17ms, 17ms, and 16ms. The OnDurationTimer is expected to be triggered in subframes 0, 17, 34, 50, 67, 84, 100, and so on. Figure 1 illustrates the subframes within each DRX cycle obtained with two formulae (1) and (2). It can be observed that up to the 15th DRX cycle, the two formulae provide the same result. However, the beginning of the 16th DRX cycle is identified as subframe 249 using equation (1) and 250 using equation (2).  This means that by using equation (1) the OnDurationTimer is triggered one subframe earlier than expected (note that  249≡-1 mod 50), namely the duration from the 16th DRX cycles is 16ms instead of 17ms. It can be further observed that the error repeats multiple times in this simple numerical evaluation.

Figure 1 – Numerical illustration of rounding errors by using equation (1) for non-integer operation of DRX cycle.

In general, these two cases might happen when the DRX formula is incorrectly implemented:

- The UE wakes up earlier than expected and the OnDurationTimer expires a number of slots earlier than expected.

- The UE wakes up later than expected and the OnDurationTimer is triggered a number of slots later than expected.

It is therefore evident the need of verifying that both the network and the UE implement a correct formula for non-integer DRX operation. This can be achieved by testing whether the UE correctly decodes the PDCCH on the boundaries of the OnDurationTimer when no other trigger extends the active time beyond the OnDurationTimer (i.e., in the first and last slots of the active time set by the OnDurationTimer).

With a wrong implementation of the DRX formula for non-integer operation, the UE may wake up a number of slots earlier or later than expected. Therefore, testing whether the UE correctly decodes the PDCCH on the boundaries of the OnDurationTimer over multiple DRX cycles is necessary for non-integer DRX operation.

Test whether the UE correctly decodes the PDCCH in the first and last slots of the UE OnDurationTimer over multiple DRX cycles.

Conclusion

As seen from the discussion part, it is necessary to test not only consecutive PDDCH occasion from few consecutive DRX cycles from the beginning, but certain DRX cycles which may occur after several cycles. This sets some requirements for the testing of the non-integer DRX regarding both testcases and test equipment. Tests must be long enough and certain key points must be carefully tested. On the other hand test equipment needs to implement non-integer DRX frame structure correctly and set PDCCH occasions according to that.

Observation 1: During RAN2 WI phase the problem of rounding errors with non-integer DRX cycle was discussed and a note was added to TS 38.321 clause 5 indicating that MAC entity shall ensure no rounding error for non-integer DRX operations.

Observation 2: With a wrong implementation of the DRX formula for non-integer operation, the UE may wake up a number of slots earlier or later than expected. Therefore, testing whether the UE correctly decodes the PDCCH on the boundaries of the OnDurationTimer over multiple DRX cycles is necessary for non-integer DRX operation.

Proposal 1: Test whether the UE correctly decodes the PDCCH in the first and last slots of the UE OnDurationTimer over multiple DRX cycles.

- References

- R5-243179, WP for XR (eXtended Reality) enhancements for NR plus CT1 aspects, Nokia, RAN5#103, Fukuoka, Japan, 20 – 24 May 2024.

- R2-2302514, DRX enhancements for XR, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, RAN2#121bis, Online, 17 – 26 April 2023.

- R2-2303861, DRX cycle alignment for XR, Nokia, Nokia Shanghai Bell, CMCC, China Unicom, DENSO CORPORATION, Ericsson, Intel, Google Inc., Huawei, HiSilicon, Samsung, Xiaomi, RAN2#121bis, Online, 17 – 26 April 2023.

- R2-2304709, DRX enhancements for XR, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, Meta, RAN2#122, Incheon, Korea, 22 – 26 May 2023.