3GPP TSG-RAN5 Meeting #104	R5-245141

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:				Discussion on beam correspondence in initial access

Source:			Samsung

Agenda Item:

Document for:			Discussion

# 1.	Background

RAN5 has extensively discussed the initial access beam corresponding test issue focusing on the beam lock function and beam refinement aspects, and the following WF [1] was endorsed:

After further discussion during the meeting, companies identified the key questions and come to following way forward: 1> How to activate/deactivate beam lock function in IDLE mode Option a: Activation in RRC_CONNECTED mode, then UE send the activation complete message and memorize the activation status when it enters RRC_IDLE mode. Beam lock function has to be sent per DL link polarization and per measurement grid point. Option a-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message. Option a-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point. Option a-3: UE automatically deactivate beam lock function when it enters RRC_CONNECTED mode. Option a-4: Other method. Option b: Activation in RRC_CONNECTED mode, UE memorizes the activation status when UE switches between RRC_CONNECTED mode and RRC_IDLE mode until: i) deactivation command is received, or ii) other agreed condition among Option b-1/b-2/b-3 is met. Beam is only locked in RRC_IDLE mode, and will be automatically unlocked in RRC_CONNECTED mode. Beam lock function is sent only once Option b-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message. Option b-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point. Option b-3: Other method. Conclusion: Continue discussion on all options. Method without test function is not precluded. 2> Beam refinement process Option a: TE sends paging as a trigger for UE beam refinement. UE needs to compare all the potential beams and decide the optimal one before the first preamble transmission. Beam is locked for future preamble transmissions. Option b: UE could perform beam refinement between the preamble transmissions, i.e. the basic RAN1/RAN2 process Conclusion: Option a. 3> Beam refinement time needed Option a: 3s as a starting point Option b: Further optimize beam refinement time to <3s Conclusion: Take option a as a starting point to evaluate the feasibility for preamble formats and preamble index, and the test time. 4> Whether the beam lock function could be extended to UE’s power setting behavior Option a: Extending the beam lock function to force UE to transmit at the Pcmax level. Option b: Don’t use beam lock function to control UE’s power setting Option b-1: With power ramping Option b-2: Without power ramping Conclusion: Continue discussion on all the options.

In this contribution, we share our views on the open issue mentioned in the above WF.

# 2	Discussion

# 2.1	How to activate/deactivate beam lock function in IDLE mode

Option a: Activation in RRC_CONNECTED mode, then UE send the activation complete message and memorize the activation status when it enters RRC_IDLE mode. Beam lock function has to be sent per DL link polarization and per measurement grid point. Option a-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message. Option a-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point. Option a-3: UE automatically deactivate beam lock function when it enters RRC_CONNECTED mode. Option a-4: Other method. Option b: Activation in RRC_CONNECTED mode, UE memorizes the activation status when UE switches between RRC_CONNECTED mode and RRC_IDLE mode until: i) deactivation command is received, or ii) other agreed condition among Option b-1/b-2/b-3 is met. Beam is only locked in RRC_IDLE mode, and will be automatically unlocked in RRC_CONNECTED mode. Beam lock function is sent only once Option b-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message. Option b-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point. Option b-3: Other method.

Option a and b are not thoroughly contradicted to each other, and there are many common points. After detailed comparison, we think Option a is better, because in Option a there is ACK process (“UE send the activation complete message”) but option b has no ACK. It is important that the UE send ACK message once it successfully received the beam lock activation command in RRC_Connected mode, otherwise the measurement in subsequent initial access may be tested with wrong configuration.

Observation 1: It is important that the UE send ACK message once it successfully received the beam lock activation command in RRC_Connected mode.

About deactivation, all the sub-options in Option a and b are feasible. Among them Option a-1 (Option a-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message) and Option a-3 (Option a-3: UE automatically deactivate beam lock function when it enters RRC_CONNECTED mode.) can be down-selected. In our view Option a-1 is a little better which is more robust. With such process, we can make sure the measurement in previous initial access state is done under beam locked mode. Option a-2 (Option a-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point) is also one function which must to be supported, because when UE is beam locked in initial access and UE position is changed, the UE may be always failed to enter into RRC_Connected state again due to beam direction is away from the correct direction. So the expiration scheme must be supported.

Observation 2: deactivation based on timer expiration scheme must be supported to avoid UE could not enter into RRC_Connected state again.

Based on above discussion, we support option a including option a-1 and a-2. i.e.

Proposal 1: Activation in RRC_CONNECTED mode, then UE send the activation complete message and memorize the activation status when it enters RRC_IDLE mode. Beam lock function has to be sent per DL link polarization and per measurement grid point.

- Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message.

- When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point

# 2.2	Beam refinement

The endorsed WF on beam refinement process may be not aligned with UE real behaviour. As mentioned in Huawei’s paper [1], when the DL RSRP of any SSB is higher than RRC configured rsrp-ThresholdSSB, UE could transmit preamble on the PRACH resource corresponds to this SSB. So the beam may not be the optimal beam.

2> Beam refinement process Option a: TE sends paging as a trigger for UE beam refinement. UE needs to compare all the potential beams and decide the optimal one before the first preamble transmission. Beam is locked for future preamble transmissions. Option b: UE could perform beam refinement between the preamble transmissions, i.e. the basic RAN1/RAN2 process Conclusion: Option a. 3> Beam refinement time needed Option a: 3s as a starting point Option b: Further optimize beam refinement time to <3s Conclusion: Take option a as a starting point to evaluate the feasibility for preamble formats and preamble index, and the test time.

The beam refinement in initial access triggered by paging from TE is assumed in above WF but we are not sure if UE will do so. Anyway we will change UE behaviour to find the optimal beam for UE, one alternative way may be to re-use the beam which is used in previous RRC_Connected state. In such way, not only optimal beam is guaranteed, but also beam refinement time can be saved.

Observation 3: The beam refinement in initial access triggered by paging from TE is assumed in the WF but we are not sure if UE will do so.

Proposal 2: One alternative way is to re-use the beam which is used in previous RRC_Connected state for initial access PRACH transmission, and the beam refinement time can be saved.

# 2.3	Whether the beam lock function could be extended to UE’s power setting behaviour

4> Whether the beam lock function could be extended to UE’s power setting behavior Option a: Extending the beam lock function to force UE to transmit at the Pcmax level. Option b: Don’t use beam lock function to control UE’s power setting Option b-1: With power ramping Option b-2: Without power ramping Conclusion: Continue discussion on all the options.

Generally it is good idea to force UE to transmit at Pcmax level directly. But on the other hand it conflicts with explicit meaning of the beam lock function, and it makes the usage of beam lock function to be only applicable for maximum output power transmission. In this sense, we think the safe way is to use power ramping. As long as we optimize the parameters well, the power ramping will be fast and does not waste much time.

Observation 4: As long as we optimize the parameters well, the power ramping will be fast and does not waste much time.

Proposal 3: Adopt the power ramping to enable maximum output power transmission for PRACH.

If most companies prefer to force UE to transmit at Pcmax directly, we think the it would be better to make this power configuration optional and configurable in beam lock function design. It existing beam lock function, TE can decide to lock UE’s Tx beam or Rx beam or both Tx & Rx beams. If power will be an additional parameter of beam lock function, it would be that the power item is also configurable, i.e. TE can configure if it is true or false when sending the beam lock function.

Proposal 4: If UE is forced to transmit at Pcmax directly in initial access, the power parameter (e.g. true or false) is better to be optional and configurable in beam lock design.

# 3.	Conclusion

Observation 1: It is important that the UE send ACK message once it successfully received the beam lock activation command in RRC_Connected mode.

Observation 2: deactivation based on timer expiration scheme must be supported to avoid UE could not enter into RRC_Connected state again.

Proposal 1: Activation in RRC_CONNECTED mode, then UE send the activation complete message and memorize the activation status when it enters RRC_IDLE mode. Beam lock function has to be sent per DL link polarization and per measurement grid point.

- Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message.

- When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point

Observation 3: The beam refinement in initial access triggered by paging from TE is assumed in the WF but we are not sure if UE will do so.

Proposal 2: One alternative way is to re-use the beam which is used in previous RRC_Connected state for initial access PRACH transmission, and the beam refinement time can be saved.

Observation 4: As long as we optimize the parameters well, the power ramping will be fast and does not waste much time.

Proposal 3: Adopt the power ramping to enable maximum output power transmission for PRACH.

Proposal 4: If UE is forced to transmit at Pcmax directly in initial access, the power parameter (e.g. true or false) is better to be optional and configurable in beam lock design.

# 4.	Reference

[1]	R5-243159r1,  Discussion on FR2 beam correspondence for initial access, Huawei, Hisilicon