3GPP TSG-RAN5 Meeting #104	R5-245143

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3391 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the position of core requirements for test case 7.5.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 38.533, minimum requirements are put under a separate section above the test cases. The core requirements of test case 7.5.14 are embedded inside the test case, which has inconsistent style with all other RRM tests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move core requirements into a separate section 7.5.14.0 and renumber the test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case 7.5.14 will keep unaligned style with other RRM tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.14.0(new), 7.5.14.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.5.14	PSCell activation and deactivation delay

7.5.14.0	Minimum conformance requirements

7.5.14.0.1	SCG Activation Delay Requirement

The requirements in this clause shall apply for the UE configured with one deactivated SCG in NR-DC and when PScell in one SCG is being activated.

The delay within which the UE shall be able to activate the deactivated SCG depends upon the specified conditions.

Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH or PUSCH towards PSCell no later than in slot n+Tactivation_timeNR slot length ,

where:

Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = 20ms. Otherwise, Tprocessing = 5 ms.	Tsearch is the time for AGC settling and PSS/SSS detection.

Tsearch is the time for AGC settling and PSS/SSS detection.

For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

For RACH-less based PSCell activation, if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR2 PScell. There are no requirements if PSCell is unknown.

T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms.

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH].

Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in SCG activation command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5.

In FR2, the PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

If the UE is configured to perform BFD while the SCG is deactivated

-	The TCI state is known if the following conditions are met:

-	During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated,

-	UE has not detected beam failure

-	Otherwise, the TCI state is unknown.

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 8.17.2.

7.5.14.0.2	SCG Deactivation Delay Requirement

The requirements in this clause shall apply for a UE which is configured with at least PCell and PScell.

Upon receiving RRC-based SCG deactivation command in subframe n, the UE shall accomplish the deactivation actions specified in TS 38.331 [13] no later than in slot n+TRRC_delayNR slot length:

where

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 8.17.3.

7.5.14.1	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- General test parameters are TBD since it’s missing in TS 38.133

7.5.14.1.1	Test purpose

The purpose of this test case is to test the activation PSCell delay for a UE configured with one deactivated SCG in NR-DC and when PScell in one SCG is being activated. The test also tests the deactivation delay. The test case tests the requirements within which the UE shall be able to activate the deactivated SCG in TS 38.133 [6] section 8.17.2 for RACH-less based conditions when PSCell and TCI state are known. The PCell is in NR FR1 and the PSCell is in NR FR2.

7.5.14.1.2	Test applicability

This test applies to all types of NR UE from Release 17 onwards and supporting NR-DC, and supporting activation and deactivation on SCG.

7.5.14.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.14.0.

The normative reference for this requirement is TS.38.133 [6] clause 8.17.3.

n+Tactivation_timeNR slot length

n+TRRC_delayNR slot length

7.5.14.1.4	Test description

7.5.14.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.14.1.4.1-1.

Table 7.5.14.1.4.1-1: Supported test configurations for FR2 PSCell activation case

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target PSCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target PSCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target PSCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.14.1.4.1-2.

Table 7.5.14.1.4.1-2: Initial conditions for FR2 PSCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.14.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.14.1.4.1-3.

2. Message contents are defined in clause 7.5.14.1.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is PSCell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.14.1.4.1-3: General test parameters for FR2 PSCell activation case

TBD

7.5.14.1.4.2	Test procedure

At the beginning of T1 the UE is configured with a PSCell which is activated. At T1 the PSCell is deactivated. PSCell is configured with bfd-and-RLM with value true.

An RRC message for activation of PSCell is sent by the test equipment 1s after the RRC message deactivating the PSCell, in a slot # denoted m. The point in time at which the RRC message for activation of PSCell is received at the UE defines the start of time period T2.

During T2, the test equipment monitors for SR from the UE on the PSCell. The time when test equipment receives a scheduling request from the UE is denoted as slot T3.

Time period T4 starts when an RRC message for deactivation of the PSCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of the PSCell, respectively.

The test equipment verifies the activation time by when the SR from the UE is received in the activated PSCell.

The test equipment verifies the deactivation time by counting the slots from the time when the PSCell deactivation command is sent until UL transmission from the PSCell is discontinued.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Tables 7.5.14.1.5-1 and 7.5.14.1.5-2. T1 starts.

3.	T1 starts. Immediately after, the SS shall transmit RRCReconfiguration message with condition DeactivatedSCG according to TS 38.508-1[14] Table 4.6.1-13 to deactivate NR cell (PSCell).

4.	The UE shall transmit RRCReconfigurationComplete message. After 1s, the SS should transmit RRCReconfiguration message to activate PSCell.

5.	The UE shall transmit an RRCConnectionReconfigurationComplete message. T2 starts.

6.	The UE shall send SR on PUCCH for PSCell in the first available uplink SR resource no later than T3, otherwise increase the number of failed iterations by one, switch off the UE and continue with step 11. T3 starts.

7.	During T3 the UE shall send at least one CSI report for PSCell with non-zero CQI index, otherwise increase the number of failed iterations by one, switch off the UE and continue to step 11.

8.	The SS shall transmit RRCReconfiguration message with condition DeactivatedSCG according to TS 38.508-1[14] Table 4.6.1-13 to deactivated NR cell (PSCell) after the UE has send at least one CQI report with non-zero CQI index for PSCell (Cell 2) during T3.

9.	The UE shall transmit an RRCReconfigurationComplete message. T4 starts.

10.	The UE shall stop sending PUSCH for PSCell no later than in slot n+TRRC_delayNR slot length into T4, if so increase the number of passed iterations by one otherwise increase the number of failed iterations by one and switch off the UE.

11.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for the next iteration of the test procedure loop.

12.	If the UE is not switched off, the SS shall transmit an RRCRelease message to release the RRC connection then, the SS transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State RRC_CONNECTED with generic parameter Connectivity NR-DC. If paging succeeds, go to step 14, otherwise switches off the UE.

13. Switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR-DC with Test Mode On according to TS 38.508-1 [14] clause 4.5.

14.	Repeat step 2-13 until a test verdict has been achieved.

7.5.14.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 7.5.14.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-19 in TS 38.508-1 [14] with condition RLM_BFD_PSCell Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with condition SSB.3 FR2 and SMTC.1 |

Table 7.5.14.1.4.3-2: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceId | SchedulingRequestResourceId |  |  |
| schedulingRequestID | SchedulingRequestId |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl80 | 9 | With SCS = kHz120 results in repetition every 10 ms | SCS120 |
| } |  |  |  |
| } |  |  |  |

7.5.14.1.5	Test requirement

Table 7.5.14.1.5-1 and Table 7.5.14.1.5-2 define the primary level settings including test tolerances.

Table 7.5.14.1.5-1: Cell specific test parameters for FR2 PSCell activation case

| ParameterNote 5 |  | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 |  | T1 | T2 | T3 | T4 |
| SSB ARFCN |  |  | Freq1 |  |  |  |  | Freq2 |  |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |  | TDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  | TDDConf.3.1 |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |
| Downlink initial BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration | Config 1,2,3 |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | N/A |  |  |  |  | TRS.2.1 TDD |  |  |  |
| TCI state | Config 1,2,3 |  | TCI.State.0 |  |  |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |  | 100: NRB,c = 66 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| Data RBs allocated | Config 1,2 |  | 52 | 66 | 66 | 52 |  | 66 | 52 | 52 | 66 |
|  | Config 3 |  | 106 |  |  | 106 |  |  | 106 | 106 |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  | - |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET Parameters | Config 1 |  | CR.1.1 FDD |  |  |  |  | - |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |  |  |  |
| Dedicated CORESET Parameters | Config 1 |  | CCR.1.1 FDD |  |  |  |  | - |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  | SSB.3 FR2 |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1~3 |  | N/A |  |  |  |  | CSI-RS.3.1 TDD |  |  |  |
| reportConfigType for CSI reporting |  |  | periodic |  |  |  |  | N/A |  |  |  |
| reportConfigType for L1-RSRP |  |  | periodic |  |  |  |  | N/A |  |  |  |
| reportQuantity for CSI reporting |  |  | cri-RI-PMI-CQI |  |  |  |  | N/A |  |  |  |
| reportQuantity for L1-RSRP |  |  | ssb-Index-RSRP |  |  |  |  | N/A |  |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |  |  | N/A |  |  |  |
|  | Config 3 |  | 10 |  |  |  |  |  |  |  |  |
| L1-RSRP reporting periodicity Note 7 | Config 1,2 | slot | 5 |  |  |  |  | N/A |  |  |  |
|  | Config 3 |  | 10 |  |  |  |  |  |  |  |  |
| CSI reporting offset | Config 1,2 | slot | 2 |  |  |  |  | N/A |  |  |  |
|  | Config 3 |  | 4 |  |  |  |  |  |  |  |  |
| L1-RSRP reporting offset | Config 1,2 | slot | 2 |  |  |  |  | N/A |  |  |  |
|  | Config 3 |  | 4 |  |  |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | N/A Link only, see clause A.3.7A |  |  |  | AWGN |  |  |  |  |
| Scheduling request resource periodicity |  | ms | N/A |  |  |  | 20 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: All parameters apply for configuration 1, 2 and 3 Note 6: Void. Note 7: L1-RSRP measurement and reporting are configured to the UE prior to the start of time period T1. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.5.14.1.5-2: OTA related test parameters for FR1 PCell with FR2 PSCell activation case

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  |  | N/A |  |  |  | According to clause A.3.15.1 |  |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  |  | Rough |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | Link only, see clause A.3.7A |  |  |  | -103.8 |  |  |  |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  |  | -94.8 |  |  |  |
|  | Config 1,2,3 | dB |  |  |  |  | 7 |  |  |  |
|  | Config 1,2,3 | dB |  |  |  |  | 7 |  |  |  |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  |  | -87.8 |  |  |  |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  |  | -58 |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

During T2 the UE shall send the first SR on PSCell in the first available uplink SR resource no later than T3 which is:

TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms

where

Tprocessing = 5ms (no RRC parameter has been modified),

Tsearch = 0ms (RACH-less based PSCell activation, with RLM and BFD are configured, PSCell and TCI state are known), and

T∆ = 20ms.

This allows T3 of [TRRC_delay + TIU + 27]ms.

During T4 the UE shall stop all transmissions on the PSCell no later than in slot n+TRRC_delayNR slot length as defined in 8.17.3.

During T2 the interruption of PCell during PSCell activation shall not happen outside the slot m + TRRC_delay.

During T4 the interruption of PCell during PSCell deactivation shall not happen outside the slot n + TRRC_delay.

The interruption duration on PCell due to activation and deactivation of PSCell shall not be more than the values specified for in Clause 8.17.2 and 8.17.3 in TS 38.133 [6].

<End of Changes>