3GPP TSG-RAN5 Meeting #104	R5-245229

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1484 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MO/MT access procedures for NB-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | x |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added timer T_DLOffset expiry procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5A.2, 8.1.5A.3, 8.1.5A.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s240441, R5s240230 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.5A.2	Test procedure to check UE response to Paging for Control Plane CIoT MT access

##### 8.1.5A.2.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2.

User Equipment:

- 	The UE is in RRC-IDLE.

##### 8.1.5A.2.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.2.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, Condition ATTACH_WITHOUT_PDN.

##### 8.1.5A.2.3	Procedure

Table 8.1.5A.2.3-1: UE response to Paging for Control Plane CIoT MT access

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging-NB | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest-NB message "mt-access"? | --> | RRC: RRCConnectionRequest-NB | - | P |
| 3 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| - | EXCEPTION: Steps 4a1a1 to 4a1b3b1 describe behaviour which depends on UE implementation; the "lower case letter" identifies a step sequence which takes place depending on whether the UE is configured or not to establish PDN upon establishing RRC connection and whether UE has pending data to transmit. | - | - | - | - |
| 4a1a1 | IF UE is configured to establish PDN upon establishing RRC connection and UE has pending data to transmit THEN Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message containing a ESM DATA TRANSPORT message in the ESM message container IE? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | P |
| 4a1b1 | ELSE Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message not containing user data (there is not yet a PDN) not containing a PDN CONNECTIVITY REQUEST message? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST | - | P |
| 4a1b2 | SS starts wait timer 2 sec. Note: The Service request procedure success is guarded in the UE by timer T3417 = 5 sec after which expiry without any DL action the UE will consider the procedure as failed. | - | - | - | - |
| - | EXCEPTION: Steps 4a1b3a1 to 4a1b3a4 describe behaviour which depends on UE implementation; the "lower case letter" identifies a step sequence which takes place depending on whether the UE is configured or not to establish PDN in RRC-CONNECTED after it sends the CONTROL PLANE SERVICE REQUEST message. | - | - | - | - |
| 4a1b3a1 | The UE transmits a PDN CONNECTIVITY REQUEST message. | --> | RRC: ULInformationTransfer-NB NAS: PDN CONNECTIVITY REQUEST | - | - |
| 4a1b3a2 | Stop timer wait timer 2 sec. | - | - | - | - |
| 4a1b3a3 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: IF pc_IP_PDN AND not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 4a1b3a3 TNEN In parallel to the events described in steps 4a1b3a4 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - |
| 4a1b3a4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer-NB NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 4a1b3b1 | Timer 2 sec expires | - | - | - | - |
| 5 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: SERVICE ACCEPT | - | - |
| 6 | For NTN access, SS waits for timer T_DLOffset expiry (Note 1) | - | - | - | - |
| Note 1: T_DLOffset is equal to DLoffset. DLoffset is specified in specified in TS 36.523-3 [20] clause 7A.14.2.3). |  |  |  |  |  |

##### 8.1.5A.2.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 4.7 with the exceptions below.

Table 8.1.5A.2.4-1: Message RRCConnectionRequest-NB (step 2, Table 8.1.5A.2.3-1)

| Derivation path: Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| establishmentCause-r13 | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.2.4-2: Message RRCConnectionSetupComplete-NB (steps 4a1a1, 4a1b1, Table 8.1.5A.2.3-1)

| Derivation path: Table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | the same value as included in the RRCConnectionSetup-NB message received from SS |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| attachWithoutPDN-Connectivity-r13 | True |  | px_DoAttachWithoutPDN |
|  | Not Present |  | NOT px_DoAttachWithoutPDN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.2.4-3: PDN CONNECTIVITY REQUEST (steps 4a1a1, 4a1b3a1, Table 8.1.5A.2.3-1)

| Derivation path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not Present or Any allowed value |  |  |
| Protocol configuration options | Not Present |  |  |
| Header compression configuration | Any allowed value |  | pc_HCCPCIoT AND pc_IP_PDN |
| Extended protocol configuration options | Not Present or Any allowed value |  |  |

Table 8.1.5A.2.4-4: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 4a1b3a3, Table 8.1.5A.2.3-1)

| Derivation path: Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Header compression configuration | 0000H | No Compression profile NOTE: For the purposes of CIoT (NAS) testing regardless of the Compression files supported by the UE and indicated in the PDN CONNECTIVITY REQUEST, the SS does not agree header compression to be applied. | pc_HCCPCIoT AND pc_IP_PDN |
| Control plane only indication | '0001'B | PDN connection can be used for control plane CIoT EPS optimization only |  |
| Extended protocol configuration options |  | The content of the IE below uses the same fields and Conditions (and their meaning) as those defined in TS 36.508 [18] for the IE 'Protocol configuration options' |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents | IPv4 address | DNS IPv4 Address |  |
| Serving PLMN rate control | '11111111 11111111'B | Unrestricted Max number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send via a PDN connection per 6 minute interval |  |

Table 8.1.5A.2.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 4a1b3a4, Table 8.1.5A.2.3-1)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Extended protocol configuration options | Not present or any allowed value |  |  |

#### 8.1.5A.3	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer non-SMS transport

##### 8.1.5A.3.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2.

User Equipment:

- 	The UE is in RRC-IDLE.

##### 8.1.5A.3.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.3.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, condition ATTACH_WITHOUT_PDN.

##### 8.1.5A.3.3	Procedure

Table 8.1.5A.3.3-1: Control Plane CIoT MO user data transfer non-SMS transport

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | Check: Does the UE transmit an RRCConnectionRequest-NB message? | --> | RRC: RRCConnectionRequest-NB | - | P |
| 2 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| - | EXCEPTION: Steps 3a1 to 3b1 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - | - | - |
| 3a1a1 | Void. | - | - | - | - |
| 3a1b1 | IF px_DoAttachWithoutPDN THEN Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST | - | P |
| 3a1b2 | Check: Does the UE transmit a PDN CONNECTIVITY REQUEST message? | --> | RRC: ULInformationTransfer-NB NAS: PDN CONNECTIVITY REQUEST | - | P |
| 3a2 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: IF pc_IP_PDN AND not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 3a2 TNEN In parallel to the events described in steps 3a3 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - |
| 3a3 | Check: Does the UE transmit an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer-NB NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | P |
| 3a4 | Check: Does the UE send an ESM DATA TRANSPORT message containing user data? | --> | RRC: ULInformationTransfer-NB NAS: ESM DATA TRANSPORT | - | P |
| 3b1 | ELSE (NOT px_DoAttachWithoutPDN) Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message containing user data encapsulated in an ESM DATA TRANSPORT message included in the ESM message container IE? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | P |
| 4 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: SERVICE ACCEPT | - | - |
| 5 | For NTN access, SS waits for timer T_DLOffset expiry (Note 1) | - | - | - | - |
| Note 1: T_DLOffset is equal to DLoffset. DLoffset is specified in specified in TS 36.523-3 [20] clause 7A.14.2.3). |  |  |  |  |  |

##### 8.1.5A.3.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 4.7 with the exceptions below.

Table 8.1.5A.3.4-1: Message RRCConnectionRequest-NB (step 1, Table 8.1.5A.3.3-1)

| Derivation path: Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| establishmentCause-r13 | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3.4-2: Message RRCConnectionSetupComplete-NB (steps 3a1a1, 3a1b1, Table 8.1.5A.3.3-1)

| Derivation path: Table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | the same value as included in the RRCConnectionSetup-NB message received from SS |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| attachWithoutPDN-Connectivity-r13 | True |  | px_DoAttachWithoutPDN |
|  | Not Present |  | NOT px_DoAttachWithoutPDN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3.4-3: Message CONTROL PLANE SERVICE REQUEST (step 3b1, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.2-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Data service type |  |  |  |
| ..Data service type value | mobile originating request |  |  |
| ESM message container | ESM DATA TRANSPORT message, User data container not checked | If the ESM DATA TRANSPORT message 'User data container' IE needs to be checked and then its content shall be specified in TC |  |

Table 8.1.5A.3.4-4: PDN CONNECTIVITY REQUEST (steps 3a1a1, 3a1b2, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not Present or Any allowed value |  |  |
| Protocol configuration options | Not Present |  |  |
| Header compression configuration | Any allowed value |  | pc_HCCPCIoT AND pc_IP_PDN |
| Extended protocol configuration options | Not Present or Any allowed value |  |  |

Table 8.1.5A.3.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 3a2, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Header compression configuration | 0000H | No Compression profile NOTE: For the purposes of CIoT (NAS) testing regardless of the Compression files supported by the UE and indicated in the PDN CONNECTIVITY REQUEST, the SS does not agree header compression to be applied. | pc_HCCPCIoT AND pc_IP_PDN |
| Control plane only indication | '0001'B | PDN connection can be used for control plane CIoT EPS optimization only |  |
| Extended protocol configuration options |  | The content of the IE below uses the same fields and Conditions (and their meaning) as those defined in TS 36.508 [18] for the IE 'Protocol configuration options' |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents | IPv4 address | DNS IPv4 Address |  |
| Serving PLMN rate control | '11111111 11111111'B | Unrestricted Max number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send via a PDN connection per 6 minute interval |  |

Table 8.1.5A.3.4-6: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 3a3, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Extended protocol configuration options | Not present or any allowed value |  |  |

#### 8.1.5A.3A	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer SMS transport

##### 8.1.5A.3A.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2.

User Equipment:

- 	The UE is in RRC-IDLE.

##### 8.1.5A.3A.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.3A.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, condition ATTACH_WITHOUT_PDN.

##### 8.1.5A.3A.3	Procedure

Table 8.1.5A.3A.3-1: Control Plane CIoT MO user data transfer SMS transport

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | Check: Does the UE transmit an RRCConnectionRequest-NB message? | --> | RRC: RRCConnectionRequest-NB | - | P |
| 2 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 3a1 | Void. | - | - | - | - |
| 3a2 | Void. | - | - | - | - |
| 3a3 | Void. | - | - | - | - |
| 3b1 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message , data service type="mobile originating request", integrity protected and partially ciphered and including the SMS in the NAS message container IE, and, not containing a PDN CONNECTIVITY REQUEST message? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST | - | P |
| 3b2 | SS starts wait timer 2 sec. Note: The Service request procedure success is guarded in the UE by timer T3417 = 5 sec after which expiry without any DL action the UE will consider the procedure as failed. | - | - | - | - |
| - | EXCEPTION: Steps 3b3a1 to 3b3b1 describe behaviour which depends on UE implementation; the "lower case letter" identifies a step sequence which takes place depending on whether the UE is configured or not to establish PDN in RRC-CONNECTED after it sends the CONTROL PLANE SERVICE REQUEST message. | - | - | - | - |
| 3b3a1 | IF px_nonSMSTransport_CP_CIoT AND px_DoAttachWithoutPDN THEN The UE transmits a PDN CONNECTIVITY REQUEST message. | --> | RRC: ULInformationTransfer-NB NAS: PDN CONNECTIVITY REQUEST | - | - |
| 3b3a2 | Stop timer wait timer 2 sec. | - | - | - | - |
| 3b3a3 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: IF pc_IP_PDN AND not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 3b3a3 TNEN In parallel to the events described in steps 3b3a4 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - |
| 3b3a4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer-NB NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 3b3b1 | Timer 2 sec expires | - | - | - | - |
| 4 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: SERVICE ACCEPT | - | - |
| 5 | For NTN access, SS waits for timer T_DLOffset expiry (Note 1) | - | - | - | - |
| Note 1: T_DLOffset is equal to DLoffset. DLoffset is specified in specified in TS 36.523-3 [20] clause 7A.14.2.3). |  |  |  |  |  |

##### 8.1.5A.3A.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 4.7 with the exceptions below.

Table 8.1.5A.3A.4-1: Message RRCConnectionRequest-NB (step 1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| establishmentCause-r13 | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3A.4-2: Message RRCConnectionSetupComplete-NB (steps 3a1, 3b1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | the same value as included in the RRCConnectionSetup-NB message received from SS |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| attachWithoutPDN-Connectivity-r13 | True |  | px_DoAttachWithoutPDN |
|  | Not Present |  | NOT px_DoAttachWithoutPDN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3A.4-3: Message CONTROL PLANE SERVICE REQUEST (steps 3a1, 3b1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.2-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Data service type |  |  |  |
| ..Data service type value | mobile originating request |  |  |
| NAS message container | SMS - content not checked | If the SMS needs to be checked and then its content shall be specified in TC |  |

Table 8.1.5A.3A.4-4: PDN CONNECTIVITY REQUEST (steps 3a1, 3b3a1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not Present or Any allowed value |  |  |
| Protocol configuration options | Not Present |  |  |
| Header compression configuration | Any allowed value |  | pc_HCCPCIoT AND pc_IP_PDN |
| Extended protocol configuration options | Not Present or Any allowed value |  |  |

Table 8.1.5A.3A.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (steps 3a2, 3b3a3, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Header compression configuration | 0000H | No Compression profile NOTE: For the purposes of CIoT (NAS) testing regardless of the Compression files supported by the UE and indicated in the PDN CONNECTIVITY REQUEST, the SS does not agree header compression to be applied. | pc_HCCPCIoT AND pc_IP_PDN |
| Control plane only indication | '0001'B | PDN connection can be used for control plane CIoT EPS optimization only |  |
| Extended protocol configuration options |  | The content of the IE below uses the same fields and Conditions (and their meaning) as those defined in TS 36.508 [18] for the IE 'Protocol configuration options' |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents | IPv4 address | DNS IPv4 Address |  |
| Serving PLMN rate control | '11111111 11111111'B | Unrestricted Max number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send via a PDN connection per 6 minute interval |  |

Table 8.1.5A.3A.4-6: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (steps 3a3, 3b3a4, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Extended protocol configuration options | Not present or any allowed value |  |  |