3GPP TSG-RAN5 Meeting #104	R5-245297

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:	5.3.31.9

Source: 	Qualcomm Inc

Title: 	UE beam correspondence for initial access

Document for:	Discussion and Endorsement

## 1.	Introduction

In response to the RAN4 LS [1] to RAN5 for the FR2 UE beam correspondence for initial access and RRC_inactive requirements, there has been several discussions in previous RAN5 meetings. The latest WF is captured in [2]. This paper discusses the alternate method without requiring beam lock test function.

## 2.	Discussion

TS 38.101-2 [3] has defined the FR2 beam correspondence requirements for RACH. Intention is to evaluate UE’s spherical coverage EIRP metric during RACH.

In order to test this requirement, the discussions so far have primarily centered on sending a beamlock function to the UE so that UE can lock its beam before sending RACH for EIRP measurement.

This approach has several complications. Notably, beamlock function has to be sent while UE is in RRC connected mode and then the beamlock settings need to be carried over while the call is released and UE goes to RRC_Idle mode. This beamlock function has no practical purpose in real deployment.

For the alternate method without the need for beamlock function, we considered the RRM random access test case as the reference configuration.

Below are the RACH config parameters for this test case.

As per the RAN4 LS [1] to RAN5,

Enabling multiple PRACH transmissions

Modify the preambleTransMax IE value to a large value to allow multiple PRACH transmissions. Max allowed value as per TS 38.331 is n200.

For each received PRACH transmission, TE can either hold off on sending RAR or send an RAR containing Random access preamble identifier that do not match the transmitted random access preamble.

UE computed power reaches Pcmax before PRACH EIRP measurement

Modify the preambleReceivedTargetPower IE to a large value such that UE computed Tx power reaches Pcmax. Max allowed value as per TS 38.331 is value -60.

Accumulated PRACH power measurement period is atleast 1ms

prach-ConfigurationIndex value 190 corresponds to PRACH format C2. An even longer applicable PRACH format B4 can be configured for the test to reduce the number of PRACH needed to fulfil the 1ms PRACH measurement period.

# of SSB and SSB power level

Below snippets from TS 38.101-2 provide info on the SSB configuration

For PRACH EIRP measurement, only SSB needs to be configured. SSB power level to be configured as per Table 6.6.4.3.1-1. The requirements do not refer to the number of SSB to be configured.

Configuring a single SSB for this test will ensure UE has only one SSB to measure and send its PRACH towards the direction of the SSB.

With the above mentioned changes on top of the RRM random access test case, it is feasible to enable

PRACH EIRP spherical coverage measurement without the use of any beamlock function.

Proposal 1: Configure the FR2 beam correspondence for initial access as per the above mentioned changes using RRM random access test case as baseline configuration.

## 3.	Conclusion

Proposal 1: Configure the FR2 beam correspondence for initial access as per the above mentioned changes using RRM random access test case as baseline configuration.

## 4.		References

[1]		R4-2317764, LS on testability issues of beam correspondence requirement for initial access, 	Huawei

[2]		R5-243159, Discussion on FR2 beam correspondence for initial access, Huawei

[3]	TS 38.101-2 v18.6.0