3GPP TSG-RAN5 Meeting #104	R5-245308

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2983 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Reference sensitivity power level for eRedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Refsens TC for eRedCap UE is currently missing in the TS38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new TC 7.3I.3 for eRedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eRedCap will be still missing from UE RF Rx TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3I.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

7.3I.3	Reference sensitivity power level for RedCap

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- The test description is incomplete.

- Test requirement is incomplete.

- Annex F MU/TT is FFS.

7.3I.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3I.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR eRedCap.

7.3I.3.3	Minimum conformance requirements

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, the reference sensitivity requirements for 5 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply. These reference sensitivity requirements for 5 MHz channel bandwidth apply also for 10, 15 and 20 MHz channel bandwidths with 15 kHz SCS. In case the reference sensitivity requirements for 5 MHz channel bandwidth are not defined, the reference sensitivity requirements for 10 MHz channel bandwidth apply with the reference sensitivity level reduced by 3.2 dB, and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4  and 25 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 25RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 25 RBs for 15 kHz SCS.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, for 30 kHz SCS, the reference sensitivity requirements defined for 10 MHz channel bandwidth in clause 7.3I.2 apply with reference sensitivity level reduced by 3.0 dB and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4 and 12 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 12RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths with 30 kHz SCS. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS.

NOTE:	It is not necessary to repeat verification when same requirement applies for multiple channel bandwidths.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3I.3.

7.3I.3.4	Test description

7.3I.3.4.1	Initial conditions

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for eRedCap.

For eRedCap UE not supporting eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

-	The downlink RB allocation in test configuration table refers to Table 7.3I.3.4.1-1

-	The uplink RB allocation in test configuration table refers to Table 7.3I.3.4.1-2 and Table 7.3I.3.4.1-3 for FD-FDD and HD-FDD eRedCap UE respectively.

Table 7.3I.3.4.1-1  Downlink Configuration of each RB allocation of eRedCap UE not supporting eRedCapNotReducedBB-BW-r18

| Channel Bandwidth | SCS(kHz) | LCRBmax | Outer RB allocation / Normal RB allocation |
| --- | --- | --- | --- |
| 5MHz | 15 | 25 | 25@0 |
|  | 30 | 12 | 12@0 |
| 10MHz | 15 | 25 | 25@14 |
|  | 30 | 12 | 12@6 |
| 15MHz | 15 | 25 | 25@27 |
|  | 30 | 12 | 12@13 |
| 20MHz | 15 | 25 | 12@41 |
|  | 30 | 12 | 12@20 |

Table 7.3I.3.4.1-2  Uplink configuration for reference sensitivity of eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

FFS

Table 7.3I.3.4.1-3  Uplink configuration for reference sensitivity of HD-FDD eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

FFS

7.3I.3.4.2	Test procedure

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3.2.4.2.

-	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.2.5-1, 7.3I.2.5-2 and 7.3I.2.5-5 if 2Rx antennas connected or Tables 7.3I.2.5-3, 7.3I.2.5-4 and 7.3I.2.5-6 if 1Rx antennas connected.

For eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3.2.4.2 with the following exceptions on step 3.

-	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.3.5-1, 7.3I.3.5-2 and 7.3I.3.5-5 if 2Rx antennas connected or Tables 7.3I.3.5-3, 7.3I.3.5-4 and 7.3I.3.5-6 if 1Rx antennas connected.

7.3I.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions.

FFS

7.3I.3.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.2.5-1 and Table 7.3I.2.5-2 for eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18 with 2 Rx antenna port, Tables 7.3I.2.5-3 and Table 7.3I.2.5-4 for eRedCap UE supporting eRedCapNotReducedBB-BW-r18 with single antenna port, Table 7.3I.2.5-5 for HD-FDD eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18 with 2 Rx antenna port, Table 7.3I.2.5-6 for HD-FDD eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18 with single antenna port, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3I.2.4.1-1.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.3.5-1 and Table 7.3I.3.5-2 for eRedCap UE not supporting eRedCapNotReducedBB-BW-r18 with 2 Rx antenna port, Tables 7.3I.3.5-3 and Table 7.3I.3.5-4 for eRedCap UE not supporting eRedCapNotReducedBB-BW-r18 with single antenna port, Table 7.3I.3.5-5 for HD-FDD eRedCap UE not supporting eRedCapNotReducedBB-BW-r18 with 2 Rx antenna port, Table 7.3I.3.5-6 for HD-FDD eRedCap UE not supporting eRedCapNotReducedBB-BW-r18 with single antenna port, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, Table 7.3I.3.4.1-1, Table 7.3I.3.4.1-2 and Table 7.3I.3.4.1-3.

Table 7.3I.3.5-1: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | FDD |
|  | 30 |  | -97.1-3 +TT | -97.1-3 +TT | -97.1-3 +TT |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n3 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n5 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n7 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n8 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n12 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT |  | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT |  |  |
| n14 | 15 | -97.0 +TT | -97.0 +TT |  |  | FDD |
|  | 30 |  | -94.1 -3+TT |  |  |  |
| n20 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n24 | 15 | -100.0 +TT | -100.0 +TT |  |  | FDD |
|  | 30 |  | -97.1 -3+TT |  |  |  |
| n25 | 15 | -96.5 +TT | -96.5 +TT | -96.5 +TT | -96.5 +TT | FDD |
|  | 30 |  | -93.6 -3+TT | -93.6 -3+TT | -93.6 -3+TT |  |
| n26 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | FDD |
|  | 30 |  | -94.8 -3+TT | -94.8 -3+TT | -94.8 -3+TT |  |
| n28 | 15 | -98.5 +TT | -98.5 +TT | -98.5 +TT | -98.5 +TT | FDD |
|  | 30 |  | -95.6 -3 +TT | -95.6 -3 +TT | -95.6 -3 +TT |  |
| n30 | 15 | -99.0 +TT | -99.0 +TT |  |  | FDD |
|  | 30 |  | -96.1 -3+TT |  |  |  |
| n65 | 15 | -99.5+TT | -99.5+TT | -99.5+TT | -99.5+TT | FDD |
|  | 30 |  | -96.6 -3+TT | -96.6 -3+TT | -96.6 -3+TT |  |
| n66 | 15 | -99.5 +TT | -99.5 +TT | -99.5 +TT | -99.5 +TT | FDD |
|  | 30 |  | -96.6 -3+TT | -96.6 -3+TT | -96.6 -3+TT |  |
| n70 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | FDD |
|  | 30 |  | -97.1 -3+TT | -97.1 -3+TT | -97.1 -3+TT |  |
| n71 | 15 | -97.2 +TT | -97.2 +TT | -97.2 +TT | -97.2 +TT | FDD |
|  | 30 |  | -94.3 -3+TT | -94.3 -3+TT | -94.3 -3+TT |  |
| n74 | 15 | -99.53 +TT | -99.53 +TT | -99.53 +TT | -99.53 +TT | FDD |
|  | 30 |  | -96.63 -3+TT | -96.63 -3+TT | -96.63 -3+TT |  |
| n85 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT |  | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT |  |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |

Table 7.3I.3.5-2: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 +TT | TDD |
|  | 30 | 10, 15 | -97.1 -3 +TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 -3 +TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n411 | 15 | 5, 10, 15, 20 | -94.8+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 -3+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 +TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 -3+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 +TT | TDD |
|  | 30 | 10 | -97.1-3+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 -3 +TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -96.-3 +TT |  |
| n791 | 15 | 10, 20, | -95.8 -3.2+TT | TDD |
|  | 30 | 10, 20, | -96.1 -3 +TT |  |
| n91 | 15 | 5 | -100+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 +TT | FDD |
|  | 30 | 10,15,20 | -97.1 -3+TT |  |
| n93 | 15 | 5 | -100+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 -3 +TT | FDD |
|  | 30 | 10,15,20 | -97.1 -3+TT |  |
| n101 | 15 | 5, 10 | -100 +TT | TDD |
|  | 30 | 10 | -97.1 -3+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |

Table 7.3I.3.5-3: Single antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  | Duplex Mode |  |
| n1 | 15 | -100.0+2.5 +TT |  | -100.0+2.5 +TT |  | -100.0+2.5 +TT |  | -100.0+2.5 +TT |  | FDD |  |
|  | 30 |  |  | -97.1 +TT |  | -97.1 +TT |  | -97.1 +TT |  |  |  |
| n2 | 15 | -98.0 +2.5+TT |  | -98.0 +2.5+TT |  | -98.0 +2.5+TT |  | -98.0 +2.5+TT |  | FDD |  |
|  | 30 |  |  | -95.1 +TT |  | -95.1 +TT |  | -95.1 +TT |  |  |  |
| n3 | 15 | -97.0+2.5 +TT |  | -97.0+2.5 +TT |  | -97.0+2.5 +TT |  | -97.0+2.5 +TT |  | FDD |  |
|  | 30 |  |  | -94.1 +TT |  | -94.1 +TT |  | -94.1 +TT |  |  |  |
| n5 | 15 | -98.0+2.5 +TT |  | -98.0+2.5 +TT |  | -98.0+2.5 +TT |  | -98.0+2.5 +TT |  | FDD |  |
|  | 30 |  |  | -95.1 +TT |  | -95.1 +TT |  | -95.1 +TT |  |  |  |
| n7 | 15 | -98.0+2.5 +TT |  | -98.0+2.5 +TT |  | -98.0+2.5 +TT |  | -98.0+2.5 +TT |  | FDD |  |
|  | 30 |  |  | -95.1 +TT |  | -95.1 +TT |  | -95.1 +TT |  |  |  |
| n8 | 15 | -97.0+2.5 +TT |  | -97.0+2.5 +TT |  | -97.0+2.5 +TT |  | -97.0+2.5 +TT |  | FDD |  |
|  | 30 |  |  | -94.1 +TT |  | -94.1 +TT |  | -94.1 +TT |  |  |  |
| n12 | 15 | -97.0 +2.5+TT |  | -97.0 +2.5+TT |  | -97.0 +2.5+TT |  |  |  | FDD |  |
|  | 30 |  |  | -94.1 +TT |  | -94.1 +TT |  |  |  |  |  |
| n14 | 15 | -97.0+2.5 +TT |  | -97.0+2.5 +TT |  |  |  |  |  | FDD |  |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 +2.5+TT |  | -97.0 +2.5+TT |  | -97.0 +2.5+TT |  | -97.0 +2.5+TT |  | FDD |  |
|  | 30 |  |  | -94.1 +TT |  | -94.1 +TT |  | -94.1 +TT |  |  |  |
| n24 | 15 | -100.0 +2.5+TT |  | -100.0 +2.5+TT |  |  |  |  |  | FDD |  |
|  | 30 |  |  | -97.1 +TT |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 +2.5+TT |  | -96.5 +2.5+TT |  | -96.5 +2.5+TT |  | -96.5 +2.5+TT |  | FDD |  |
|  | 30 |  |  | -93.6 +TT |  | -93.6 +TT |  | -93.6 +TT |  |  |  |
| n26 | 15 | -97.5+2.5 +TT |  | -97.5+2.5 +TT |  | -97.5+2.5 +TT |  | -97.5+2.5 +TT |  | FDD |  |
|  | 30 |  |  | -94.8 +TT |  | -94.8 +TT |  | -94.8 +TT |  |  |  |
| n28 | 15 | -98.5 +2.5+TT |  | -98.5 +2.5+TT |  | -98.5 +2.5+TT |  | -98.5 +2.5+TT |  | FDD |  |
|  | 30 |  |  | -95.6 +TT |  | -95.6 +TT |  | -95.6 +TT |  |  |  |
| n30 | 15 | -99.0 +2.5+TT |  | -99.0 +2.5+TT |  |  |  |  |  | FDD |  |
|  | 30 |  |  | -96.1 +TT |  |  |  |  |  |  |  |
| n65 | 15 | -99.5 +2.5+TT |  | -99.5 +2.5+TT |  | -99.5 +2.5+TT |  | -99.5 +2.5+TT |  | FDD |  |
|  | 30 |  |  | -96.6 +TT |  | -96.6 +TT |  | -96.6 +TT |  |  |  |
| n66 | 15 | -99.5 +2.5+TT |  | -99.5 +2.5+TT |  | -99.5 +2.5+TT |  | -99.5 +2.5+TT |  | FDD |  |
|  | 30 |  |  | -96.6 +TT |  | -96.6 +TT |  | -96.6 +TT |  |  |  |
| n70 | 15 | -100.0 +2.5 +TT |  | -100.0 +2.5 +TT |  | -100.0 +2.5 +TT |  | -100.0 +2.5 +TT |  | FDD |  |
|  | 30 |  |  | -97.1 +TT |  | -97.1 +TT |  | -97.1 +TT |  |  |  |
| n71 | 15 | -97.2+2.5 +TT |  | -97.2+2.5 +TT |  | -97.2+2.5 +TT |  | -97.2+2.5 +TT |  | FDD |  |
|  | 30 |  |  | -94.3 +TT |  | -94.3 +TT |  | -94.3 +TT |  |  |  |
| n74 | 15 | -99.53 +2.5 +TT |  | -99.53 +2.5 +TT |  | -99.53 +2.5 +TT |  | -99.53 +2.5 +TT |  | FDD |  |
|  | 30 |  |  | -96.63 +TT |  | -96.63 +TT |  | -96.63 +TT |  |  |  |
| n85 |  | 15 | -97+2.5 +TT |  | -97+2.5 +TT |  | -97+2.5 +TT |  |  |  | FDD |  |
|  |  | 30 |  |  | -94.1 +TT |  | -94.1 +TT |  |  |  |  |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: That the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |  |  |  |  |

Table 7.3I.3.5-4: Single antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS/Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15 | -97.1 +2.5 -3+TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n411 | 15 | 10, 15, 20 | -94.8 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 +2.5 -3+TT |  |
| n481 | 15 | 5, 10, 15, 20 | -99 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1+2.5 -3+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n51 | 15 | 5 | -100 +2.5+TT | TDD |
| n53 | 15 | 5, 10 | -100 +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5 -3+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 +2.5 -3.2 +TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 +2.5 -3 +TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 +2.5 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 +2.5 -3 +TT |  |
| n791 | 15 | 10, 20, | -95.8 +2.5+TT | TDD |
|  | 30 | 10, 20, | -96.1 +2.5 -3+TT |  |
| n91 | 15 | 5 | -100+2.5+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 +2.5 -3+TT |  |
| n93 | 15 | 5 | -100 +2.5+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 +2.5 -3+TT |  |
| n101 | 15 | 5, 10 | -100 +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5 -3+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |

Table 7.3I.3.5-5: Two antenna port reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n2 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n3 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n5 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n7 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n8 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n12 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT |  |  |
| n13 | 15 | -97.8 +TT | -97.8 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT |  |  |  |
| n14 | 15 | -97.8 +TT | -97.8 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT |  |  |  |
| n18 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT |  | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT |  |  |
| n20 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n24 | 15 | -100.0 +TT | -100.0 +TT |  |  | HD-FDD |
|  | 30 |  | -97.2 -3+TT |  |  |  |
| n25 | 15 | -97.3 +TT | -97.3 +TT | -97.3 +TT | -97.3 +TT | HD-FDD |
|  | 30 |  | -94.5 -3+TT | -94.5 -3+TT | -94.5 -3+TT |  |
| n26 | 15 | -98.3 +TT | -98.3 +TT | -98.3 +TT | -98.3 +TT | HD-FDD |
|  | 30 |  | -95.5 -3+TT | -95.5 -3+TT | -95.5 -3+TT |  |
| n28 | 15 | -99.3 +TT | -99.3 +TT | -99.3 +TT | -99.3 +TT | HD-FDD |
|  | 30 |  | -96.5 -3+TT | -96.5 -3+TT | -96.5 -3+TT |  |
| n30 | 15 | -99.5 +TT | -99.5 +TT |  |  | HD-FDD |
|  | 30 |  | -96.7 -3+TT |  |  |  |
| n65 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n66 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n70 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n71 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | HD-FDD |
|  | 30 |  | -95.2 -3+TT | -95.2 -3+TT | -95.2 -3+TT |  |
| n74 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n85 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT |  |  |
| n91 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n92 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n93 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n94 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |

Table 7.3I.3.5-6: Single antenna port Reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT | -94.7 -3+TT |  |
| n2 | 15 |  | -96.3 +TT |  | -96.3 +TT |  | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  |  |  | -93.5 -3+TT |  | -93.5 -3+TT | -93.5 -3+TT |  |
| n3 | 15 |  | -95.3 +TT |  | -95.3 +TT |  | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  |  |  | -92.5 -3+TT |  | -92.5 -3+TT | -92.5 -3+TT |  |
| n5 | 15 |  | -96.3 +TT |  | -96.3 +TT |  | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  |  |  | -93.5 -3+TT |  | -93.5 -3+TT | -93.5 -3+TT |  |
| n7 | 15 |  | -96.3 +TT |  | -96.3 +TT |  | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  |  |  | -93.5 -3+TT |  | -93.5 -3+TT | -93.5 -3+TT |  |
| n8 | 15 |  | -95.3 +TT |  | -95.3 +TT |  | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  |  |  | -92.5 -3+TT |  | -92.5 -3+TT | -92.5 -3+TT |  |
| n12 | 15 |  | -95.3 +TT |  | -95.3 +TT |  | -95.3 +TT |  | HD-FDD |
|  | 30 |  |  |  | -92.5 -3+TT |  | -92.5 -3+TT |  |  |
| n13 | 15 |  | -95.3 +TT |  | -95.3 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -92.5 -3+TT |  |  |  |  |
| n14 | 15 |  | -95.3 +TT |  | -95.3 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -92.5 -3+TT |  |  |  |  |
| n18 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT |  | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT |  |  |
| n20 | 15 |  | -95.3 +TT |  | -95.3 +TT |  | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  |  |  | -92.5 -3+TT |  | -92.5 -3+TT | -92.5 -3+TT |  |
| n24 | 15 |  | -97.5 +TT |  | -97.5 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  |  |  |  |
| n25 | 15 |  | -94.8 +TT |  | -94.8 +TT |  | -94.8 +TT | -94.8 +TT | HD-FDD |
|  | 30 |  |  |  | -92.0 -3+TT |  | -92.0 -3+TT | -92.0 -3+TT |  |
| n26 | 15 |  | -95.8 +TT |  | -95.8 +TT |  | -95.8 +TT | -95.8 +TT | HD-FDD |
|  | 30 |  |  |  | -93.0 -3+TT |  | -93.0 -3+TT | -93.0 -3+TT |  |
| n28 | 15 |  | -96.8 +TT |  | -96.8 +TT |  | -96.8 +TT | -96.8 +TT | HD-FDD |
|  | 30 |  |  |  | -94.0 -3+TT |  | -94.0 -3+TT | -94.0 -3+TT |  |
| n30 | 15 |  | -97.0 +TT |  | -97.0 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -94.2 -3+TT |  |  |  |  |
| n65 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT | -94.7 -3+TT |  |
| n66 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT | -94.7 -3+TT |  |
| n70 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT | -94.7 -3+TT |  |
| n71 | 15 |  | -95.5 +TT |  | -95.5 +TT |  | -95.5 +TT | -95.5 +TT | HD-FDD |
|  | 30 |  |  |  | -92.7 -3+TT |  | -92.7 -3+TT | -92.7 -3+TT |  |
| n74 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT | -94.7 -3+TT |  |
| n85 | 15 | -95.3 +TT |  | -95.3 +TT |  | -95.3 +TT |  |  | HD-FDD |
|  | 30 |  |  | -92.5 -3+TT |  | -92.5 -3+TT |  |  |  |
| n91 | 15 |  | -97.5 +TT |  |  |  |  |  | HD-FDD |
| n92 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT | -94.7 -3+TT |  |
| n93 | 15 |  | -97.5 +TT |  |  |  |  |  | HD-FDD |
| n94 | 15 |  | -97.5 +TT |  | -97.5 +TT |  | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 -3+TT |  | -94.7 -3+TT | -94.7 -3+TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |  |  |  |

Table 7.3I.3.5-7: Test Tolerance (TT) for RX sensitivity level for RedCap UE

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| FFS | FFS |

<End of changes>