3GPP TSG-RAN5 Meeting #104	R5-245568

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5325 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition to IoT NTN enhancement test case 22.3.1.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New IoT NTN enhancement test case 22.3.1.14 need to be added according to work plan of the WI IoT_NTN_enh_plus_CT1-UEConTest. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New IoT NTN enhancement test case 22.3.1.14 was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.14 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-2 CR 1456 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-244300 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.3.1.14	NB-IoT / NTN / Correct HARQ process / HARQ disabling

22.3.1.14.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state with DRB established and has 2 DL HARQ processes and the second HARQ process feedback is disabled by configuring downlinkHARQ-FeedbackDisabled }

ensure that {

when { UE receives MAC PDUs for these two HARQ processes }

then { UE doesn’t transmits ACK/NACK for the second HARQ process while ACK is transmitted for the first HARQ process }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state with DRB established and has 2 UL HARQ processes and the second HARQ process feedback is disabled by configuring HARQ Mode B }

ensure that {

when { UE transmits MAC PDUs but does not receive an ACK for these 2 HARQ processes }

then { UE re-transmits the MAC PDU for the first HARQ process but does not re-transmit for the second HARQ process }

}

22.3.1.14.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.321, clause 5.3.2.1, 5.3.2.2, 5.4.2.1 and 5.4.2.2. Unless otherwise stated these are Rel-18 requirements.

[TS 36.321, clause 5.3.2.1]

There is one HARQ entity at the MAC entity for each Serving Cell which maintains a number of parallel HARQ processes. Each HARQ process is associated with a HARQ process identifier. The HARQ entity directs HARQ information and associated TBs received on the DL-SCH to the corresponding HARQ processes (see clause 5.3.2.2).

The number of DL HARQ processes per HARQ entity is specified in TS 36.213 [2], clause 7.

When the physical layer is configured for downlink spatial multiplexing, as specified in TS 36.213 [2], one or two TBs are expected per TTI and they are associated with the same HARQ process. Otherwise, one TB is expected per TTI.

For NB-IoT UEs or BL UEs or UEs in enhanced coverage, the parameter DL_REPETITION_NUMBER provides the number of transmissions repeated in a bundle. For each bundle, DL_REPETITION_NUMBER is set to a value provided by lower layers. Within a bundle, after the initial (re)transmission, DL_REPETITION_NUMBER-1 HARQ retransmissions follow. The HARQ feedback is transmitted for the bundle and a downlink assignment corresponding to a new transmission or a retransmission of the bundle is received after the last repetition of the bundle. A retransmission of a bundle is also a bundle. HARQ feedback may be disabled per HARQ process by configuring downlinkHARQ-FeedbackDisabled and/or by indication from lower layers.

[TS 36.321, clause 5.3.2.2]

For each TTI where a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

-	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

-	if the HARQ process is equal to the broadcast process and if this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

-	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

-	consider this transmission to be a new transmission.

-	else:

-	consider this transmission to be a retransmission.

The MAC entity then shall:

-	if this is a new transmission:

-	attempt to decode the received data.

-	else if this is a retransmission:

-	if the data for this TB has not yet been successfully decoded:

-	combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

-	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

-	if the data for this TB was successfully decoded before:

-	if the HARQ process is equal to the broadcast process:

-	deliver the decoded MAC PDU to upper layers.

-	else if this is the first successful decoding of the data for this TB:

-	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

-	generate a positive acknowledgement (ACK) of the data in this TB.

-	else:

-	replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

-	generate a negative acknowledgement (NACK) of the data in this TB.

-	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or

-	if the HARQ process is equal to the broadcast process; or

-	if the HARQ process is not associated with a transmission indicated with a PUR-RNTI and the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired; or

-	if the HARQ feedback is disabled for the corresponding HARQ process:

-	if harq-FeedbackEnablingforSPSactive is configured and the transmission is the first SPS PDSCH transmission after SPS activation:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

-	else:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

[TS 36.321, clause 5.4.2.1]

There is one HARQ entity at the MAC entity for each Serving Cell with configured uplink, which maintains a number of parallel HARQ processes allowing transmissions to take place continuously while waiting for the HARQ feedback on the successful or unsuccessful reception of previous transmissions.

The number of parallel HARQ processes per HARQ entity is specified in TS 36.213 [2], clause 8. NB-IoT has one or two UL HARQ processes.

22.3.1.14.3	Test description

22.3.1.14.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE shall be in State 2B-NB with test loop mode G (CP CIoT Optimisation) according to TS 36.508 [18].

22.3.1.14.3.2	Test procedure sequence

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU’s in preamble on bearer to be used, then any sequence number ‘n’ in test procedure maps as:

UL SQN n maps to SQN X+n MOD 1024 &

DL SQN n maps to SQN Y+n MOD 1024.

Table 22.3.1.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a MAC PDU containing a MAC SDU with a RLC SDU of 38 bytes in an AMD PDU with polling field ‘P’ set to ‘0’ on HARQ process 0. (NOTE 1) | <-- | NPDCCH (DCI N1) MAC PDU (R/R/E/LCID MAC sub-header (E=’0’, LCID= ‘00011’), 40 bytes MAC SDU) (RLC SN=0) | - | - |
| 2 | Check: Does the UE send a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 3 | The SS transmits a MAC PDU containing a MAC SDU with a RLC SDU of 38 bytes in an AMD PDU with polling field ‘P’ set to ‘0’ on HARQ process 1. (NOTE 1) | <-- | NPDCCH (DCI N1) MAC PDU (R/R/E/LCID MAC sub-header (E=’0’, LCID= ‘00011’), 40 bytes MAC SDU) (RLC SN=1) | - | - |
| - | EXCEPTION: Steps 4a1-4b1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on UE support of disabled HARQ feedback for downlink transmission. | - | - | - | - |
| 4a1 | IF pc_NB_ntn_DL_HARQ_disable_RRC_singleTB THEN Check: Does the UE send a HARQ ACK? | - | - | 1 | F |
| 4b1 | ELSE The UE sends a HARQ ACK. | --> | HARQ ACK | - | - |
| 5 | The SS allocates an UL grant on HARQ process 0, allowing the UE to return the RLC SDU as received in step 1. | <-- | Uplink Grant | - | - |
| 6 | UE transmits a MAC PDU corresponding to UL grant in step 5. | --> | MAC PDU(RLC SN=0) | - | - |
| 7 | The SS allocates an UL grant on HARQ process 1, allowing the UE to return the RLC SDU as received in step 3. | <-- | Uplink Grant | - | - |
| 8 | UE transmit a MAC PDU corresponding to UL grant in step 7. | --> | MAC PDU(RLC SN=1) | - | - |
| 9 | The SS allocates an UL grant on HARQ process 0 with NDI not toggled, allowing the UE to retransmit the RLC SDU in step 6. | <-- | Uplink Grant | - | - |
| 10 | Check: Does the UE retransmit the MAC PDU in step 6? | --> | MAC PDU(RLC SN=0) | 2 | P |
| 11 | The SS allocates an UL grant on HARQ process 1 with NDI not toggled, allowing the UE to retransmit the RLC SDU in step 8. | <-- | Uplink Grant | - | - |
| - | EXCEPTION: Steps 12a1-12b1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on UE support of disabled HARQ feedback for uplink transmission. | - | - | - | - |
| 12a1 | IF pc_NB_ntn_UL_HARQ_MODE_B_singleTB THEN Check: Does the UE retransmit the MAC PDU in step 8? | - | - | 2 | F |
| 12b1 | ELSE The UE retransmit the MAC PDU in step 8. | --> | MAC PDU(RLC SN=1) | - | - |
| NOTE 1: RLC SDU with size n contains a DLInformationTransfer-NB with ESM_DATA_TRANSPORT: 18 or 26 bits DLInformationTransfer-NB(PER encode: ‘0000000000’B + 8 bits ( length of DedicatedInfoNAS <= 127 bytes ) or 16 bits length ( 128 bytes <= length of DedicatedInfoNAS < 16383 bytes )) 1/2 byte Protocol discriminator ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Security header type ( SECURITY PROTECTED NAS MESSAGE ) 4 bytes Message authentication code ( SECURITY PROTECTED NAS MESSAGE ) 1 byte Sequence number ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Protocol discriminator ( ESM_DATA_TRANSPORT ) 1/2 byte EPS bearer identity ( ESM_DATA_TRANSPORT ) 1 byte Procedure transaction identity ( ESM_DATA_TRANSPORT ) 1 byte ESM data transport message identity ( ESM_DATA_TRANSPORT ) 2 bytes length indicator of the user data container ( ESM_DATA_TRANSPORT ) N bytes User data container contents ( UL RLC SDU ) 6 bits Aligned bits ( PER encode ) n = N+14 or N+15 bytes DL RLC SDU |  |  |  |  |  |

22.3.1.14.3.3	Specific message contents

Table 22.3.1.14.3.3-1: RRCConnectionSetup-NB (preamble: Table 22.3.1.14.3.2-1)

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| srb-ToAddModList-r13 SEQUENCE (SIZE (1)) OF SEQUENCE { | 1 entry |  |  |
| SRB-ToAddMod-NB-r13 SEQUENCE { |  |  |  |
| rlc-Config-v1430 SEQUENCE { |  |  |  |
| t-Reordering-r14 | ms1600-v1310 |  |  |
| } |  |  |  |
| rlc-Config-v1700 SEQUENCE { |  |  |  |
| t-ReorderingExt-r17 CHOICE { |  |  |  |
| setup | ms3200 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| physicalConfigDedicated-r13 SEQUENCE { |  |  |  |
| twoHARQ-ProcessesConfig-r14 | true |  |  |
| npdsch-ConfigDedicated-v1800 SEQUENCE { |  |  |  |
| downlinkHARQ-FeedbackDisabledBitmap-NB-r18 CHOICE { |  |  | pc_NB_ntn_DL_HARQ_disable_RRC_singleTB |
| setup | 10’01’B |  |  |
| } |  |  |  |
| } |  |  |  |
| npusch-ConfigDedicated-v1800 SEQUENCE { |  |  |  |
| uplinkHARQ-mode-r18 CHOICE { |  |  | pc_NB_ntn_UL_HARQ_MODE_B_singleTB |
| setup | ’10’B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]