3GPP TSG-RAN5 Meeting #104	R5-245677

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0069 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New Test Case 6.8.3 Message Store Function Subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no Message Store test cases for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Message Store test case 6.8.3 for Message Store Function Subscription |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message Store Function Subscription would not be tested for MCData |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-244969 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.8.3	On-network / MCData Message Store / Subscription / Create a subscription to notifications / Update a subscription to notifications / Delete a subscription to notifications

6.8.3.1	Test Purpose (TP)

(1)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to create a subscription to notifications about changes in the message store using the Message Store Function }

then { UE (MCData Client) requests create a subscription to notifications about changes in the message store using the Message Store Function via an HTTP POST message }

}

(2)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to update a subscription to notifications about changes in the message store using the Message Store Function }

then { UE (MCData Client) requests to update a subscription to notifications about changes in the message store using the Message Store Function via an HTTP POST message }

}

(3)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to delete a subscription to notifications about changes in the message store using to the Message Store Function }

then { UE (MCData Client) requests to delete a subscription to notifications about changes in the message store using the Message Store Function via an HTTP DELETE message }

}

6.8.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 21.2.12A.1, 21.2.14A.1, 21.2.13A.1. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282 clause 21.2.12A.1]

In order for the message store client to keep its local store in sync with the MCData message store, it needs to receive notifications about changes in the message store. For this purpose, the message store client would need to subscribe to notification from the message store, Synchronization using subscriptions and notifications is described in clause 5.1.5.1 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

To create a subscription to notifications about changes in the message store using the message store function, the message store client, acting as an HTTP client shall follow the procedure described in clause 6.20 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with the following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.20.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with the following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function; and

b)	shall include a valid MCData access token in the HTTP Authorization header; and

2)	shall send the HTTP POST request towards the message store function.

Upon receipt of an HTTP response, the message store client should follow the procedure as described in clause 6.20.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.14A.1]

A client may update its subscription to notification in order to:

1)	extend the life of the subscription;

2)	restart the notification stream from where it left off.

Synchronization using subscriptions and notifications is described in clause 5.1.5.1 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66]

To update a subscription to notifications about changes in the message store using the message store function, the message store client, acting as an HTTP client shall follow the procedure described in clause 6.21 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] the with following clarification:

1)	shall generate an HTTP POST request as specified in clause 6.21.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with the following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

2)	shall send the HTTP POST request towards the message store function.

Upon receipt of an HTTP response, the message store client should follow the procedure described in clause 6.21.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.13A.1]

To delete / cancel a subscription and stop corresponding notifications about changes in the message store using the message store function, the message store client, acting as an HTTP client shall follow the procedure described in clause 6.21 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with the following clarification:

1)	shall generate an HTTP DELETE request as specified in subclause 6.21.6 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with the following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

2)	shall send the HTTP DELETE request identifying the subscription to be deleted towards the message store function.

Upon receipt of an HTTP response, the message store client should follow the procedure as described in clause 6.21.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

6.8.3.3	Test description

6.8.3.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.879-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.879-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.8.3.3.2	Test procedure sequence

Table 6.8.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) create a subscription to notifications about changes in the message store function. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Object Upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.14.2-1 to create a subscription to notifications about changes in the message store function? | - | - | 1 | P |
| 3 | Make the UE (MCData client) update the duration of the subscription created at step 2 to 10800 seconds. (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Post Request using HTTP' as described in TS 36.579-1 [2] Table 5.3C.17.2-1 to update a subscription to notifications about changes in the message store function? | - | - | 2 | P |
| 5 | Make the UE (MCData client) delete the subscription created at step 2. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Delete using HTTP' as described in TS 36.579-1 [2] Table 5.3C.15.2-1 to delete a subscription to notifications about changes in the message store function? | - | - | 3 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.8.3.3.3	Specific message contents

Table 6.8.3.3.3-1: HTTP POST from the UE (Step 2, Table 6.8.3.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "subscriptions" |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS nmsSubscription |  |  | RESTful Network API for NMS [40], clauses 6.20 |  |
| callbackReference |  |  |  |  |
| notifyURL | any valid absolute http URI with hostname and path | URL provided by the client to be used in HTTP POST requests of notifications; e.g. "http://nmsClient.3gpp.org/nms/notifications" |  |  |
| callbackData | any value if present |  |  |  |
| notificationFormat | "XML" ifpresent | XML if not present |  |  |
| duration | any value if present |  |  |  |
| filter | Not present | (NOTE 1) |  |  |
| clientCorrelator | any value if present |  |  |  |
| resourceURL | Not present | shall not be included in POST requests by the client |  |  |
| index | Not present | shall not be included in requests by the client |  |  |
| restartToken | any value if present |  |  |  |
| maxEvents | any value if present |  |  |  |
| objectAttributeNames | Not present |  |  |  |
| inlineImdn | false ifpresent |  |  |  |
| NOTE 1 It is assumed that the MMI at step 1 does not require/provide any filter |  |  |  |  |

Table 6.8.3.3.3-2: HTTP 201 Created from the SS (step 2, Table 6.8.3.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  |  |  |
| uri | tsc_MCData_MsgSF_AbsoluteBaseURL & "/subscriptions/subscriptId0001" | "subscriptId0001" arbitrarily selected |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS Subscription |  |  | RESTful Network API for NMS [40], clauses 6.20 |  |
| callbackReference |  |  |  |  |
| notifyURL | same value as provided by the client in the POST request |  |  |  |
| callbackData | Same value as provided by the client if present in the POST request otherwise not present |  |  |  |
| notificationFormat | Same value as provided by the client if present in the POST request otherwise not present | Either "XML" or not present |  |  |
| duration | Same value as provided by the client if present in the POST request otherwise "7200" | 7200 seconds if no other value has been provided by the client |  |  |
| filter | Not present |  |  |  |
| clientCorrelator | Same value as provided by the client if present in the POST request otherwise not present |  |  |  |
| resourceURL | tsc_MCData_MsgSF_AbsoluteBaseURL & "/subscriptions/subscriptId0001" | same as in Location |  |  |
| index | 1 |  |  |  |
| restartToken | Same value as provided by the client if present in the POST request otherwise not present |  |  |  |
| maxEvents | Same value as provided by the client if present in the POST request otherwise "5" | "5" arbitrarily selected |  |  |
| objectAttributeNames | Not present |  |  |  |
| inlineImdn | Same value as provided by the client if present in the POST request otherwise not present | Either "false" or not present |  |  |

Table 6.8.3.3.3-3: HTTP POST from the UE (Step 4, Table 6.8.3.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "subscriptions/subscriptId0001" |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS nmsSubscriptionUpdate |  |  | RESTful Network API for NMS [40], clauses 6.21 |  |
| duration | "10800" |  |  |  |
| restartToken | Not present |  |  |  |

Table 6.8.3.3.3-4: HTTP 200 (OK) from the SS (step 4, Table 6.8.3.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS nmsSubscription | Same nmsSubscription as specified in Table 6.8.3.3.3-2 with modification of the <duration> element as shown below |  | RESTful Network API for NMS [40], clauses 6.21 |  |
| duration | "10800" |  |  |  |

Table 6.8.3.3.3-5: HTTP DELETE from the UE (Step 6, Table 6.8.3.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.15.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.5-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "subscriptions/subscriptId0001" | The subscription to be deleted |  |  |

<END OF CHANGES>