3GPP TSG-RAN5 Meeting #104	R5-245746

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0041 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TT in NTN demod case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt., ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in R5-244333 proposal 2, additional SNR uncertainty should be added due to finite test time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SNR uncertainty due to finite test time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MU TT analysis remain uncompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.4, F.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR denpends on R5-244333. This is the outcome of R5-244335 with 1 revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-244335r1: - Add ROHDE & SCHWARZ as co-source - Add more details about MUTT analysis - Add MUTT analysis for TC 8.3.1.1.2 |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

F.1.4	Measurement of performance requirements

Table F.1.4-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1.1 PDSCH in standalone mode for UE category M1 under NTN fading conditions | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 8.3.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 under NTN fading conditions | ± 0.8 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to- noise ratio uncertainty2 + Fading profile power uncertainty2 + (0.25 x AWGN flatness and signal flatness)2 + variation due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.15 dB for Test 1 and ±0.2 dB for Test 2. |
| 8.3.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 | Same as clause 8.3.1.1.1. | Same as clause 8.3.1.1.1 with following exception, Result variation due to finite test time ±0.35 dB for Test 1 and ±0.3 dB for Test 2. |
| 8.3.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 | Same as clause 8.12.1.1.3 in TS 36.521-1 [14]. |  |
| 8.3.1.2.1 Demodulation of NPDCCH single-antenna performance for category NB1 and NB2 | Same as clause 8.12.2.1.1 in TS 36.521-1 [14]. |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

<<< END OF CHANGES 1>>>

<<< START OF CHANGES 2>>>

F.3.4	Measurement of performance requirements

Table F.3.4-1: Derivation of Test Requirements (performance tests)

| Test | Minimum Requirement in TS 36.102 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 8.2.1.1.1 PDSCH in standalone mode for UE category M1 under NTN fading conditions | SNRs as specified in clause 8.2.1.1.1 | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 under NTN fading conditions | SNRs as specified in clause 8.3.1.1.1 | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 | SNRs as specified in clause 8.3.1.1.2 | Same as clause 8.3.1.1.1 | Same as clause 8.3.1.1.1 |
| 8.3.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 | SNRs as specified in clause 8.12.1.1.3 in TS 36.521-1 [14] | Same as clause 8.12.1.1.3 in TS 36.521-1 [14] | Same as clause 8.12.1.1.3 in TS 36.521-1 [14] |
| 8.3.1.2.1 Demodulation of NPDCCH single-antenna performance for category NB1 and NB2 | SNRs as specified in clause 8.12.2.1.1 in TS 36.521-1 [14] | Same as clause 8.12.2.1.1 in TS 36.521-1 [14] | Same as clause 8.12.2.1.1 in TS 36.521-1 [14] |

<<< END OF CHANGES 2>>>