3GPP TSG-RAN5 Meeting #104	R5-245996

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2988 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Reference sensitivity power level test for XR UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Benelux B.V. - Belgium |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The core specification has introduced minimum conformance requirements for REFSENS for XR UEs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new test case for REFSENS power level for XR UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will not be aligned with core specification which specifies REFSENS requirement for XR UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3K |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 modifies all the clause and sub-clause numbering along with edits and corrections within the minimum conformance section. R5-245450r1->R5-245996 |  |  |  |  |  |  |  |  |

<Start of changes>

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for 2 Rx antenna port, Tables 7.3.2.5-2 a and Tables 7.3.2.5-2b for 4 Rx antenna port, Table 7.3.2.5-2c and Table 7.3.2.5-2d for PC2 UE on FDD bands, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3.2.4.1-3.

Table 7.3.2.5-1a: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands for PC3

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n1 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT |  | -90.6 +TT | -90.1 +TT | -89.6 +TT | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT |  | -90.7 +TT | -90.2 +TT | -89.7 +TT |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT |  | -90.9 +TT | -90.3 +TT | -89.7 +TT |  |
| n2 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -84.1 +TT |  | -81.5 +TT |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -84.2 +TT |  | -81.6 +TT |  |  |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -90.9 +TT | -84.3 +TT |  | -81.7 +TT |  |  |  |
| n3 | 15 |  | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -86.2+TT | -82.3 +TT | -81.3+TT | -79.7 +TT | FDD |
|  | 30 |  |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -86.3+TT | -82.4 +TT | -81.4+TT | -79.8 +TT |  |
|  | 60 |  |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -86.4+TT | -82.6 +TT | -81.5+TT | -79.9 +TT |  |
| n5 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -86.8 +TT | -84.8 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -88.6 +TT | -84.9 +TT |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -89.9 +TT |  | -88.6 +TT |  | -81.5 +TT | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -90.0 +TT |  | -88.7 +TT |  | -81.5 +TT |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -91.0 +TT | -90.1 +TT |  | -88.9 +TT |  | -81.5 +TT |  |
| n8 | 15 |  | -97.0 +TT | -93.8 +TT | -91.4 +TT | -85.8 +TT |  |  | -78.4+TT |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.7 +TT | -87.2 +TT |  |  | -78.5+TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | -89.3 +TT | -82.2 +TT | -81.7 +TT | -79.5 +TT | -77.6+TT |  | FDD |
|  | 30 |  |  | -93.6 +TT | -91.6 +TT | -90.5 +TT | -89.4 +TT | -82.3 +TT | -81.8 +TT | -79.6 +TT | -77.7 +TT |  |  |
|  | 60 |  |  | -94.0 +TT | -91.9 +TT | -90.7 +TT | -89.6 +TT | -82.4 +TT | -81.9 +TT | -79.7 +TT | -77.8 +TT |  |  |
| n26 | 15 | -99.7 +TT | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |
| n28 | 15 | -100.2 +TT | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  |  |  |  | FDD |
|  | 30 |  |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 |  | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |
| n66 | 15 |  | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT |  | -90.1 +TT |  |  | FDD |
|  | 30 |  |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT |  | -90.2 +TT |  |  |  |
|  | 60 |  |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT |  | -90.4 +TT |  |  |  |
| n70 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |
| n71 | 15 |  | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT | -84.17 +TT -74.88+TT | -82.57 +TT -67.18+TT | -80.77 +TT -64.08 +TT |  |  |  | FDD |
|  | 30 |  |  | -94.3 +TT | -91.9 +TT | -87.4 +TT | -84.27 +TT -74.98 +TT | -82.67 +TT -67.28 +TT | -80.87 +TT -64.18 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 |  | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |
| n85 | 15 | -99.2 +TT | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
| n100 | 15 | -102.2 +TT | -100.0 +TT |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | -99.2 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 7: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 8: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands for PC3, PC2, PC1.5

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 10, 20, 40, 50 | -95.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 20, 40, 50, 60, 80, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 20, 40, 50, 60, 80, 100 | -96.5 + 10log10(NRB/11) +TT |  |
| n91 | 15 | 5 | -100+TT | FDD |
| n92 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
| n93 | 15 | 5 | -100+TT | FDD |
| n94 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24)+TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

Table 7.3.2.5-2a: Four antenna port Reference sensitivity QPSK PREFSENS FDD bands for PC3

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT |  | -93.3 +TT | -92.8 +TT | -92.3 +TT | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT |  | -93.4 +TT | -92.9 +TT | -92.4 +TT |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT |  | -93.6 +TT | -93 +TT | -92.4 +TT |  |
| n2 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -86.8 +TT |  | -84.2 +TT |  |  | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -86.9 +TT |  | -83.3 +TT |  |  |  |
|  | 60 |  | -98.2 +TT | -96.1 +TT | -94.9 +TT | -93.6 +TT | -87.0 +TT |  | -84.4 +TT |  |  |  |
| n3 | 15 | -99.7 +TT | -96.5 +TT | -94.7 +TT | -93.5 +TT | -92.4 +TT | -91.6 +TT | -88.9+TT | -90.3 +TT | -84.0+TT | -82.4 +TT | FDD |
|  | 30 |  | -96.8 +TT | -94.8 +TT | -93.7 +TT | -92.5 +TT | -91.7 +TT | -90.0+TT | -90.4 +TT | -84.1+TT | -82.5 +TT |  |
|  | 60 |  | -97.2 +TT | -95.1 +TT | -93.9 +TT | -92.7 +TT | -91.8 +TT | -90.1+TT | -90.6 +TT | -84.2+TT | -82.6 +TT |  |
| n71 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -92.6 +TT |  | -91.3 +TT |  | -84.2 +TT | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -92.7 +TT |  | -91.4 +TT |  | -84.2 +TT |  |
|  | 60 |  | -98.2 +TT | -97.1 +TT | -94.9 +TT | -93.7 +TT | -92.8 +TT |  | -91.6 +TT |  | -84.2 +TT |  |
| n8 | 15 | -99.7 +TT | -96.5 +TT | -94.1 +TT | -88.5 +TT |  |  | -81.1 +TT |  |  |  | FDD |
|  | 30 |  | -96.8 +TT | -94.4 +TT | -89.9 +TT |  |  | -81.2 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -101.7 +TT | -98.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -98.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -102.2 +TT | -99.0 +TT | -97.2 +TT | -96.0 +TT | -94.9 +TT | -94.1 +TT |  | -92.8 +TT |  |  | FDD |
|  | 30 |  | -99.3 +TT | -97.3 +TT | -96.2 +TT | -95.0 +TT | -94.2 +TT |  | -92.9 +TT |  |  |  |
|  | 60 |  | -99.7 +TT | -97.6 +TT | -96.4 +TT | -95.2 +TT | -94.3 +TT |  | -93.1 +TT |  |  |  |
| n70 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT |  |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for above listed operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2 The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 3: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2b: Four antenna port Reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands for PC3, PC2, PC1.5

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+-2.7 TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) -2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/52)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)-2.7 +TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)-2.2+TT |  |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)-2.2 +TT |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| n791 | 15 | 10, 20, 40, 50 | -95.8 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 20, 40, 50, 60, 80, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 20, 40, 50, 60, 80, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

Table 7.3.2.5-2c: Reference Sensitivity for PC2 UE on FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | - | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 |
| n3 | REFSENS_n3 +0.5 | REFSENS_n3+ 0.5 | REFSENS_n3+ 0.5 | REFSENS_n3+ 0.5 | REFSENS_n3+ 0.6 | REFSENS_n3+ 0.8 | REFSENS_n3+ 1.1 | REFSENS_n3+ 1.5 | REFSENS_n3+ 2.3 | REFSENS_n3+ 2.8 |
| n8 | REFSENS_n8 +0.5 | REFSENS_n8 +0.7 | REFSENS_n8 +0.8 | REFSENS_n8 +2.3 | - | - | REFSENS_n8 +3.1 | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: REFSENS_n1 refers to the two antenna port and four antenna port Reference Sensitivity of n1 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 3: REFSENS_n3 refers to the two antenna port and four antenna port Reference Sensitivity of n3 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 4: REFSENS_n8 refers to the two antenna port and four antenna port Reference Sensitivity of n8 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2d: Reference Sensitivity for PC2 UE on FDD bands for UE supporting Tx Diversity operation bands

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | - | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 |
| n3 | REFSENS_n3+ 1.4 | REFSENS_n3+ 1.5 | REFSENS_n3+ 1.5 | REFSENS_n3+ 1.5 | REFSENS_n3+ 1.6 | REFSENS_n3+ 1.7 | REFSENS_n3+ 2.8 | REFSENS_n3+ 5 | REFSENS_n3+ 5.5 | REFSENS_n3+ 6.0 |
| n8 | REFSENS_n8 +1.3 | REFSENS_n8 +1.4 | REFSENS_n8 +2.1 | REFSENS_n8 +5.8 | - | - | REFSENS_n8 +7.0 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: REFSENS_n1 refers to the two antenna port and four antenna port Reference Sensitivity of n1 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 3: REFSENS_n3 refers to the two antenna port and four antenna port Reference Sensitivity of n3 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 4: REFSENS_n8 refers to the two antenna port and four antenna port Reference Sensitivity of n8 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-3: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

For the UE which supports CA, SUL or DC band combination, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 a and Table 7.3.2.5-1b shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.3 for the applicable operating bands.

### 7.3.2_1	Reference sensitivity power level for XR

7.3.2_1.1	Test purpose

The test purpose is to verify the ability of the XR UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2_1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that indicate support for supportOf2RxXR-r18.

7.3.2_1.3	Minimum conformance requirements

For a XR UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a and Table 7.3.2.3-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a XR UE equipped with 2 Rx antenna ports shall be adjusted by the value specified in Table [TBD] met with uplink transmission bandwidth less than or equal to that specified in Table [TBD] and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For two Rx antenna port XR UE(s) indicating UE capability supportOf2RxXR-r18, reference sensitivity for two Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRXR,2R in Table 7.3.2_1-1 for the applicable operating bands.

Table 7.3.2_1-1: Two antenna port XR UE reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41, n48, n77, n78, n79 | -1.0 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.2-1a, Table 7.3.2-1b along with the modification specified in Table 7.3.2_1-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.2.

7.3.2_1.4	Test description

7.3.2_1.4.1	Initial conditions

FFS

7.3.2_1.4.2	Test procedure

FFS

7.3.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

7.3.2_1. 5	Test requirement

FFS

<End of changes>