3GPP TSG-RAN WG5 Meeting # 104	R5-246032

# Maastricht, Netherlands,  August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2949 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on 6.2 for delta TIB description for single carrier transmitter power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The texts for how to handle the ΔTIB,c for the cases when the UE supports more than one of band combination have been duplicated for many times in different sub-clauses in the spec. It is suggested to move the corresponding texts to the “General” sub-clause for the test of single carrier transmitter power. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the text for how to handle the ΔTIB,c for the cases when the UE supports more than one of band combination to the “General” sub-clause (new added) for single carrier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The texts for how to handle the ΔTIB,c for the cases when the UE supports more than one of band combination will be redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.0 (new), 6.2.1.5, 6.2.2.5, 6.2.3.5, 6.2.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: A new clause 6.0 is created and referred by the sections of test requirements. |  |  |  |  |  |  |  |  |

<< Start of changes >>

6	Transmitter characteristics

6.0	General test requirements for ΔTIB,c

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

For UEs that do not indicate IE dualPA-Architecture, transmitter requirements for CA operation apply only when the DMRS initialization parameters (including the case when the UE applies cell ID as DMRS scrambling ID) are different across all CCs. The UE may use higher MPR values outside this limitation.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

The transmitter requirements other than UL MIMO (suffix D), Tx Diversity (suffix G) and Carrier Aggregation with UL MIMO (suffix H) shall be measured on a single UL antenna connector for each configured UL band. In case of UL CA or NR-DC (suffix A, suffix B) the requirements shall be measured on a single UL antenna connector for each UL band. In case the UE supports dualPA-Architecture according to the corresponding IE, the requirements are measured on the corresponding antenna connector for each carrier, except for the Output RF spectrum emissions (clause 6.5A) which are measured across all antenna connectors for a given band.

Uplink RB allocations given in Table 6.1-1 are used throughout this section, unless otherwise stated by the test case.

<<Unchanged sections are omitted>>

6.2	Transmitter power

6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

<<Unchanged sections are omitted>>

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-2 for Power Class 2 as defined in TS 38.306.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n85 |  |  |  |  | 23 | ±(23+TT) |
| n91 |  |  |  |  | 23 | ±(23, 5+TT) |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n106 |  |  |  |  | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n8 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n75 | 31 | +2+TT/-3-TT |  |  |  |  |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n415 | 31 | +2+TT/-3-TT |  |  |  |  |
| n785 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3. NOTE 5: Generally, PC1 UEis not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

Table 6.2.1.5-2b: Void

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2.2	UE maximum output power reduction

6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed.

6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-2, Table 6.2.2.3-1, Table 6.2.2.3-4b and Table 6.2.2.3-5, respectively for channel bandwidths ≤ 100 MHz.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth is larger than 4% for TDD bands or 3% for FDD bands, the ∆MPR is defined in Table 6.2.2.3-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high).

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

<<Unchanged sections are omitted>>

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1,4 Table 6.2.2.5-r.

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n85, n91, n92, n93, n94, n100, n101, n106) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 6 | 23 | 0 | 0.55 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.55 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | (19.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25 n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: For bands n91, n92, n93 and n94 the maximum power reduction requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-1a: UE Power Class test requirements Band 24 for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 0.53 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 0.53 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2a: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | 25.8 | 2.0 | 3 | 28.0 + TT | 22.8 - TT |
| 2 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 3 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 4 | 23 | -3 | 1.2 | 0 | 24.8 | 2.0 | 3 | 28.0 + TT | 21.8 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements (for Bands n1, n3, n34, n39, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 2.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n3 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4a: UE Power Class test requirements for Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 2 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 3 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 4 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 5 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 6 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 7 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 8 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 9 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 10 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 11 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 12 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 13 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 14 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 15 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 16 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 17 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 18 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 19 | 31 | 0 | 1.5 | 0 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 — TT |
| 20 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 21 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 22 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 23 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 24 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 25 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 26 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 27 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 28 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 29 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 30 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 31 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 32 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 33 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 34 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 35 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4b: Void

Table 6.2.2.5-4c: UE Power Class test requirements for bands other than Band n14 (bands n7, n78) (bands n41, n100 and n101) for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <50 MHz CBW | ≥50 MHz CBW |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 2 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 3 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 4 | 31 | 0 | 0.5 | 0.5 | 0 | 30.5 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 - TT | 27.5 - TT |
| 5 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 6 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 7 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 8 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 9 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 10 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 11 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 12 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 13 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 14 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 15 | 31 | 0 | 2.5 | 2.5 | 0 | 28.5 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 - TT | 25.5 - TT |
| 16 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 17 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 18 | 31 | 0 | 4.5 | 4.5 | 0 | 26.5 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 - TT | 23.5 - TT |
| 19 | 31 | 0 | 1.5 | 1.5 | 0 | 29.5 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 - TT | 26.5 - TT |
| 20 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 21 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 22 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 23 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 24 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 25 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 26 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 27 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 28 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 29 | 31 | 0 | 3.5 | 3.5 | 0 | 27.5 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 - TT | 24.5 - TT |
| 30 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 31 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 32 | 31 | 0 | 6.5 | 6.5 | 0 | 24.5 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 - TT | 21.5 - TT |
| 33 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 34 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 35 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n85, n91, n92, n93, n94, n100, n101, n106) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 25.0 + TT | 18 - TT | 15.5 - TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 25.0 + TT | 16 - TT | 14 - TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 25.0 + TT | 14 - TT | 11.5 - TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 25.0 + TT | 10 - TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 25.0 + TT | 10.5 - TT | 9 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-6a: Void

Table 6.2.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 2.5 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-7a: Void

Table 6.2.2.5-7b: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5 - TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 5 | 2.5 | 25.0 + TT | 9.5 - TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 3 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 3 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 3 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8a: Void

Table 6.2.2.5-9: UE Power Class test requirements (for Bands n1, n3, n34, n39, n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 1.5 | 1.5 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18 - TT2 |
| 15 | 26 | 0 | 1.5 | 1 | 0 | 1.52 | 23.5 | 222 | 2 | 22 | 3 | 28.0 + TT | 20.5 - TT | 19.0 - TT2 |
| 26 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 27 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 34 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 212 | 2 | 22 | 3 | 28.0 + TT | 19.5 - TT | 18.0 - TT2 |
| 35 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18.5 - TT2 |
| 46 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 47 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 56 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21 | 19.52 | 2 | 3.52 | 3 | 28.0 + TT | 18 - TT | 16.0 - TT2 |
| 57 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 66 | 26 | 0 | 6.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 3 | 28.0 + TT | 14 - TT | 11.5 - TT2 |
| 67 | 26 | 0 | 6.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 3 | 28.0 + TT | 14.5 - TT | 12.0 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n3 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-9a: Void

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

<<Unchanged sections are omitted>>

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-38. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

<<Unchanged texts are omitted>>

Table 6.2.3.5-39: UE Power Class 3 test requirements (NS_50) for band n39 (Step 3)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0- TT |  |
| 2 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 3 | 23 | 0 | 1 | 0 | 0 |  | 22 |  | 2.0 |  | 2 | 25.0 + TT | 20.0 - TT |  |
| 4 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 5 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 6 | 23 | 0 | 1 | 0 | 0 |  | 23 |  | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT |  |
| 7 | 23 | 0 | 1 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 8 | 23 | 0 | 1 | 3 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 9 | 23 | 0 | 1 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT |  |
| 10 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 11 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 12 | 23 | 0 | 4.5 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 13 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 14 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5- TT |  |
| 15 | 23 | 0 | 4.5 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 16 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 17 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 18 | 23 | 0 | 4.5 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 19 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 21 | 23 | 0 | 3 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT |  |
| 22 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 23 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 24 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 – TT |  |
| 25 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 26 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 27 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 28 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 29 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 – TT |  |
| 30 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 2 | 25.0 + TT | 5.0 - TT |  |
| 31 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 32 | 23 | 0 | 3 | 3.5 | 0 |  | 19.5 |  | 3.5 |  | 2 | 25.0 + TT | 16 - TT |  |
| 33 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 34 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 35 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 36 | 23 | 0 | 6.5 | 6 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 37 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 38 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 39 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 40 | 23 | 0 | 6.5 | 12 | 0 |  | 11 |  | 6.0 |  | 2 | 25.0 + TT | 5.0 – TT |  |
| 41 | 23 | 0 | 6.5 | 4..5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 – TT |  |
| 42 | 23 | 0 | 6.5 | 8 | 0 |  | 15 |  | 5.0 |  | 2 | 25.0 + TT | 10.0 – TT |  |
| 43 | 23 | 0 | 6.5 | 3.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 – TT |  |
| 44 | 23 | 0 | 6.5 | 5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2.4	Configured transmitted power

6.2.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c is within the specified bounds.

6.2.4.2	Test applicability

This test case applies to all types of NR Power Class 1 release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.2.4.3	Minimum conformance requirements

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1.3-1 without taking into account the tolerance specified in the Table 6.2.1.3-1;

When the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass =

-	3 dB for a power class 2 UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3 dB for a power class 1.5 UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1.3 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’;

-	0 dB otherwise;

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for V2X operating bands for concurrent operation, CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL in this specification and 6.2B.4.2 in TS 38.101-3 [4] for EN-DC, truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL in this specification and 6.2B.4.2 in TS 38.101-3 [4] for EN-DC, for the applicable operating bands.

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1.3-1 in 38.521-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2.3 and clause 6.2.3.3, respectively;

∆MPRc for serving cell c is specified in clause 6.2.2.3.

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2' or 't1r1-t1r2'

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4' or 't1r1-t1r2-t1r4' or, 't1r1-t1r2-t2r2-t1r4-t2r4'

c)	UE transmits SRS from the second SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4', or

d)	UE transmits SRS to a DL-only carrier

The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16..

The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self defence requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4.3-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4.3-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c – MAX{TL,c, T(PCMAX_L,f,c)} ≤ PUMAX,f,c ≤ PCMAX_H,f,c + T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4.3-2. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1.3-1.

Table 6.2.4.3-2: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.4.

6.2.4.4	Test description

6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: A power class 3 UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.4.4.3.

6.2.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE reaches the Pumax level of the test point.

3.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2.4.5-1 according to the test configuration from table 6.2.4.4.1-1. The period of measurement shall be at least the continuous duration of one active slot and in the uplink symbols. For TDD slots with transient periods are not under test.

6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.2.4.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2.4.4.3-4: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2.4.4.3-5: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | Test ID 1, 2 |
|  | 1 |  | Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2.4.5-1.

Table 6.2.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2.4.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2.4.5-1b. |  |  |

Table 6.2.4.5-1a: Measured UE output power test point 4 for Test ID 1,2

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 20 dBm ±(2.5+TT) |
| n2 | 20 dBm ±(2.5+TT) |
| n3 | 20 dBm ±(2.5+TT) |
| n5 | 20 dBm ±(2.5+TT) |
| n7 | 20 dBm ±(2.5+TT) |
| n8 | 20 dBm ±(2.5+TT) |
| n12 | 20 dBm ±(2.5+TT) |
| n14 | 20 dBm ±(2.5+TT) |
| n20 | 20 dBm ±(2.5+TT) |
| n24 | 20 dBm + 2.5+TT/-3-TT |
| n25 | 20 dBm ±(2.5+TT) |
| n26 | 20 dBm ±(2.51+TT) |
| n28 | 20 dBm ±(2.5+TT) |
| n30 | 20 dBm ±(2.5+TT) |
| n34 | 20 dBm ±(2.5+TT) |
| n38 | 20 dBm ±(2.5+TT) |
| n39 | 20 dBm ±(2.5+TT) |
| n40 | 20 dBm ±(2.5+TT) |
| n41 | 20 dBm ±(2.5+TT) |
| n50 | 20 dBm ±(2.5+TT) |
| n51 | 20 dBm ±(2.5+TT) |
| n65 | 20 dBm ±(2.5+TT) |
| n66 | 20 dBm ±(2.5+TT) |
| n70 | 20 dBm ±(2.5+TT) |
| n71 | 20 dBm ±(2.5+TT) |
| n74 | 20 dBm ±(2.5+TT) |
| n77 | 20 dBm + 2.5+TT/-3-TT |
| n78 | 20 dBm + 2.5+TT/-3-TT |
| n79 | 20 dBm + 2.5+TT/-3-TT |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-1b: Measured UE output power test point 4 for Test ID 3

| NR band | Tolerance (dB) |
| --- | --- |
| n40 | 23 dBm +2+TT/-2-TT |
| n41 | 23 dBm +2+TT/-2-TT |
| n77 | 23 dBm +2+TT/-3-TT |
| n78 | 23 dBm +2+TT/-3-TT |
| n79 | 23 dBm +2+TT/-3-TT |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-2: Test Tolerance (Configured transmitted power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

==============================================================

<< End of changes >>