3GPP TSG-RAN5 Meeting #105	R5-246177

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5350 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 13.4.3.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Solutions, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current Prose, in parallel to Steps 23 – 30, the Steps specified in Table 13.4.3.18.3.2-3 &13.4.3.18.3.2-4 take place, which are IMS signalling steps. If a UE does not support IMS over UTRAN, these steps will never get executed. This can cause a conformant UE to fail the test case and needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to check whether the UE supports IMS over UTRAN or not, and based on this the Steps in Table 13.4.3.18.3.2-3, 13.4.3.18.3.2-4 will be called. Since Table 13.4.3.18.3.2-3 is also referenced before Step 14, it is proposed to add a new Table 13.4.3.18.3.2-6.The EXCEPTION before Step 23 is updated with the new table. This table follows the same sequence as 13.4.3.18.3.2-3 but will only be executed if the UE supports IMS over UTRAN. For the IMS over UTRAN dependency, a new PICS pc_IMS_UTRAN is introduced in CR R5-246312. Based on this PICS, an EXCEPTION has been introduced before Step 1 in table 13.4.3.18.3.2-4 & 13.4.3.18.3.2-6 where it is mentioned that the relevant Steps will be executed only if pc_IMS_UTRAN = TRUE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2... CR R5-246312.. |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of change>

#### 13.4.3.18	Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / bSRVCC / MO call

13.4.3.18.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message and an MO IMS voice call is in before alerting state and an UTRA PS RB + Speech combination is configured for an UTRA cell }

then { UE transmits a HANDOVER TO UTRAN COMPLETE message on the UTRA cell }

}

(2)

with { UE having transmitted a HANDOVER TO UTRAN COMPLETE message }

ensure that {

when { UE receives a CONNECT message on the utra cell }

then { UE transmits a CONNECT ACKNOWLEDGE message on the utra cell }

}

13.4.3.18.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.216 clause 6.2.2.2,  TS 24.237 clauses 12.1, 12.2.3B.1, 12.2.3B.1A, 12.2.3B.2, clause 12.2.3B.3.3 and clause 12.2.4.2. Unless otherwise stated these are Rel-12 requirements.

[TS 23.216, clause 6.2.2.2]

Depicted in figure 6.2.2.2-1 is a call flow for SRVCC from E-UTRAN to UTRAN or GERAN with DTM HO support, including the handling of the non-voice component. The flow requires that eNB can determine that either the target is UTRAN with PS HO or the target is GERAN with DTM support and the UE is supporting DTM.

Figure 6.2.2.2-1: SRVCC from E-UTRAN to UTRAN with PS HO or GERAN with DTM HO support

1.	UE sends measurement reports to E-UTRAN.

2.	Based on UE measurement reports the source E-UTRAN decides to trigger an SRVCC handover to UTRAN/GERAN.

[TS 24.237, clause 12.1]

In order to fulfil the requirements for PS-CS access transfer in SR-VCC for calls in alerting state, the SC UE needs to be engaged in a session with speech media component in early dialog state according to the following conditions before SR-VCC access transfer is performed:

-	a SIP 180 (Ringing) response for the initial SIP INVITE request to establish this session has been sent or received; and

-	a SIP final response for the initial SIP INVITE request to establish this session has not been sent or received.

[TS 24.237, clause 12.2.3B.1]

The SC UE shall apply the procedures in subclauses 12.2.3B.3.3 if one of the following is true:

1)	there are zero, one or more dialogs supporting a session with speech media component and a SIP INVITE request was sent by SC UE such that:

A)	all dialogs are early dialogs created by a SIP response to the SIP INVITE request;

B)	a final SIP response to the SIP INVITE request has not been received yet;

C)	a SIP 180 (Ringing) response to the SIP INVITE request has not been received yet in any existing early dialog created by a SIP response to the SIP INVITE request;

D)	the SC UE included in the SIP INVITE request a Contact header field containing the g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag as described in annex C; and

E)	a SIP 1xx response to the SIP INVITE request was received where the SIP 1xx response contained a Feature-Caps header field with the g.3gpp.ps2cs-srvcc-orig-pre-alerting feature-capability indicator as described in annex C; or

NOTE:	UE can have zero dialogs if all the early dialogs were terminated by 199 (Early Dialog Terminated) as described in RFC 6228 [80].

2)	there are one or more dialogs supporting a session with speech media component such that:

A)	there are zero, one or more early dialogs and the remaining dialogs are confirmed dialogs;

B)	all the confirmed dialogs support sessions with inactive speech media component;

C)	the UE does not apply the MSC server assisted mid-call feature according to subclause 12.2.3A;

D)	a SIP INVITE request was sent by SC UE such that:

a)	all early dialogs are created by a SIP response to the SIP INVITE request;

b)	a final SIP response to the SIP INVITE request has not been received yet;

c)	a SIP 180 (Ringing) response to the SIP INVITE request has not been received yet in any existing early dialog created by a SIP response to the SIP INVITE request;

d)	the SC UE included in the SIP INVITE request a Contact header field containing the g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag as described in annex C; and

e)	a SIP 1xx response to the SIP INVITE request was received where the SIP 1xx response contained a Feature-Caps header field with the g.3gpp.ps2cs-srvcc-orig-pre-alerting feature-capability indicator as described in annex C.

[TS 24.237, clause 12.2.3B.1A]

2)	there are one or more dialogs supporting sessions with speech media component according to the following conditions:

A)	there are zero, one or more early dialogs and the remaining dialog(s) are confirmed dialog(s);

B)	all the confirmed dialogs support sessions with inactive speech media component;

C)	the UE applies the MSC server assisted mid-call feature according to subclause 12.2.3A;

D)	a SIP INVITE request was sent by SC UE such that:

a)	all the early dialogs are created by a SIP response to the SIP INVITE request;

b)	a final SIP response to the SIP INVITE request has not been received yet;

c)	a SIP 180 (Ringing) response to the SIP INVITE request has not been received yet in any existing early dialog created by a SIP response to the SIP INVITE request;

d)	the SC UE included in the SIP INVITE request a Contact header field containing the g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag as described in annex C; and

e)	a SIP 1xx response to the SIP INVITE request was received where the SIP 1xx response contained a Feature-Caps header field with the g.3gpp.ps2cs-srvcc-orig-pre-alerting feature-capability indicator as described in annex C.

[TS 24.237, clause 12.2.3B.2]

If the SC UE applies the procedures in subclause 12.2.3B.3 and the SC UE only has a single call:

-	in alerting phase following access transfer; or

-	in pre-alerting phase and the SC UE supports the PS to CS SRVCC for originating calls in pre-alerting phase;

the SC UE shall associate this session with transaction identifier value and TI flag as described in 3GPP TS 24.008 [8].

If the SC UE applies the procedures in subclause 12.2.3B.4 and the SC UE has an established session and an additional session in alerting phase or pre-alerting phase following access transfer, then the SC UE shall associate the transferred session that was in alerting phase or pre-alerting phase with CS call with transaction identifier 1 and TI flag value as in mobile terminated call.

NOTE:	For the procedures in subclause 12.2.3B.4.2, the held transaction identifier value is described in subclause 12.2.3A as for single inactive session transfer and the active session transaction identifier value is described in 3GPP TS 24.008 [8]

[TS 24.237, clause 12.2.3B.3.3]

If the SC UE supports the PS to CS SRVCC for originating calls in pre-alerting phase and this subclause is invoked according to the conditions in subclause 12.2.3B.1 and the SC UE successfully performs access transfer to the CS domain, then the UE continues the call in the CS domain in the "Mobile originating call proceeding" (U3) call state as described in 3GPP TS 24.008 [8].

If the SC UE has generated and rendered the locally generated communication progress information before the access transfer to the CS domain, the UE keeps generating and rending the locally generated communication progress information after the access transfer to the CS domain.

If the SC UE has rendered received early media before the access transfer to the CS domain, the UE attaches the user connection, as specified in 3GPP TS 24.008 [8].

[TS 24.237, clause 12.2.4.2]

If the SC UE is engaged in a session in early dialog state and:

-	receives a SM NOTIFICATION message containing an "SRVCC handover cancelled, IMS session re-establishment required" as described in 3GPP TS 24.008 [8] or 3GPP TS 24.301 [52] depending on the access in use; or

...

then if the SC UE the SC UE shall send a SIP UPDATE request containing:

1)	an SDP offer, including the media characteristics as used in the existing dialog; and

2)	a Reason header field containing protocol "SIP" and reason parameter "cause" with value "487" as specified in IETF RFC 3326 [57], and with reason-text set to either "handover cancelled" or "failure to transition to CS domain";

by following the rules of 3GPP TS 24.229 [2] in each transferred session.

13.4.3.18.3	Test description

13.4.3.18.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

13.4.3.18.3.2	Test procedure sequence

Table 13.4.3.18.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 13.4.3.18.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that entering conditions for event B2 are not satisfied. |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -88 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -88 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -84 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -64 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -64 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Non-suitable “Off” | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -64 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -64 |  |

Table 13.4.3.18.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures UTRA cell 5 to reference configuration according TS 36.508 [18] Table 4.8.3-1, condition UTRA PS RB + Speech. | - | - | - | - |
| - | EXCEPTION: The following messages are to be observed on Cell 1 unless explicitly stated otherwise. | - | - | - | - |
| 2-13 | Steps 1 to 12 of the generic test procedure for IMS MO speech call (TS 36.508 [18], 4.5A.6.3-1). | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 14 to 15 the steps specified in Table 13.4.3.18.3.2-3 should take place. | - | - | - | - |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT | - | - |
| 16 | The SS transmits an RRCConnectionReconfiguration message to setup inter RAT measurement and reporting for event B2. | <-- | RRCConnectionReconfiguration | - | - |
| 17 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 18 | The SS changes the power level for Cell 1 and Cell 5 according to the row "T1" in Table 13.4.3.18.3.2-1 | - | - | - | - |
| 19 | The UE transmits a MeasurementReport message to report event B2 for Cell 5. | --> | MeasurementReport | - | - |
| 20 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E-UTRA and UTRA. | <-- | UECapabilityEnquiry | - | - |
| 21 | The UE transmits a UECapabilityInformation message on Cell 1. NOTE: The start-PS and start-CS values received, should be used to configure ciphering on Cell 5. | --> | UECapabilityInformation | - | - |
| 22 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| - | EXCEPTION: In parallel to the events described in step 23 to 30 the steps specified in Table 13.4.3.18.3.2-6 take place. | - | - | - | - |
| 23 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message on Cell 5? | --> | HANDOVER TO UTRAN COMPLETE | 1 | P |
| 24 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 25 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 26 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 27 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 28 | The SS transmits a TMSI REALLOCATION COMMAND message. | <-- | TMSI REALLOCATION COMMAND | - | - |
| 29 | The UE transmits a TMSI REALLOCATION COMPLETE message. | --> | TMSI REALLOCATION COMPLETE | - | - |
| - | EXCEPTION: In parallel to the events described in step 29A to 33A and depending on the UE implementation the steps defined in Table 13.4.3.18.3.2-4 take place. | - | - | - | - |
| 29A | Check Does the UE transmit a ROUTING AREA UPDATE REQUEST message? | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 30 | The SS transmits a SECURITY MODE COMMAND message for the PS domain on Cell 5. | <-- | SECURITY MODE COMMAND | - | - |
| 31 | The UE transmits a SECURITY MODE COMPLETE message on Cell 5. | --> | SECURITY MODE COMPLETE | - | - |
| 32 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 33 | The UE transmits a ROUTING AREA UPDATE COMPLETE message on Cell 5. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 33A | Wait for 2 seconds to provide enough time to finish up optional IMS deregistration. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 33A to 37 the steps specified in Table 13.4.3.18.3.2-5 take place. | - | - | - | - |
| 34 | The SS transmits a CC_ALERTING message | <-- | CC_ALERTING | - | - |
| 35 | The SS transmits a CONNECT message on Cell 5. | <-- | CONNECT | - | - |
| 36 | Check: Does the UE transmit a CONNECT ACKNOWLEDGE message on Cell 5? | --> | CONNECT ACKNOWLEDGE | 2 | P |
| 37 | SS adjusts cell levels according to row T2 of Table 13.4.3.18.3.2-1. | - | - | - | - |
| - | The UE is in end state UTRA CS call (U5). | - | - | - | - |

Table 13.4.3.18.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-4 | Steps 5-8 expected sequence defined in annex C.21 of TS 34.229-1 [35]. NOTE: IMS MO speech call establishment gets to pre-alerting phase. | - | - | - | - |

Table 13.4.3.18.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: If pc_IMS_UTRAN=True, Step 1 takes place |  |  |  |  |
| 1 | Generic test procedure for mobile initiated IMS SIP re-registration or de-registration defined in Annex C.46 or C.30, respectively, of TS 34.229-1 [35] can take place. | - | - | - | - |

Table 13.4.3.18.3.2-5: Parallel behaviour

| - | EXCEPTION: Step 1a1-1a2 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a certain action. | - | - | - | - |
| --- | --- | --- | --- | --- | --- |
| 1a1 | The UE transimts PDP CONTEXT Deactivaiton message | <-- | DEACTIVATE PDP CONTEXT REQUEST | - | - |
| 1a2 | The UE transimts PDP CONTEXT Deactivaiton message | <-- | DEACTIVATE PDP CONTEXT ACCEPT | - | - |

Table 13.4.3.18.3.2-6: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: If pc_IMS_UTRAN=True, Steps 1-4 take place. |  |  |  |  |
| 1-4 | Steps 5-8 expected sequence defined in annex C.21 of TS 34.229-1 [35]. NOTE: IMS MO speech call establishment gets to pre-alerting phase. | - | - | - | - |

13.4.3.18.3.3	Specific message contents

Table 13.4.3.18.3.3-0: Conditions for specific message contents
in Table 13.4.3.18.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 13.4.3.18.3.3-1: ATTACH REQUEST (preamble)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported |  |  |
| Mobile station classmark 2 | Any allowed value |  |  |
| Supported Codecs | Any allowed value |  |  |

Table 13.4.3.18.3.3-2: RRCConnectionReconfiguration (step 16, Table 13.4.3.18.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 13.4.3.18.3.3-3: MeasConfig (step 16, Table 13.4.3.18.3.3-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 with condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measurementConfiguration ::= SEQUENCE { |  |  |  |
| measObjectToAddModifyList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f8 |  |  |
| measObject[1] | MeasObjectUTRA-GENERIC(f8) |  |  |
| measObjectId[2] | IdMeasObject-f1 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModifyList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-UTRA |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-UTRA (-72, -76) |  |  |
| } |  |  |  |
| measIdToAddModifyList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f8 |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-UTRA |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE {} |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.18.3.3-4: MeasurementReport (step 19, Table 13.4.3.18.3.2-2)

| Derivation Path: 36.508, table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| utra-RSCP | (-5..91) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.18.3.3-5: MobilityFromEUTRACommand (step 22, Table 13.4.3.18.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| cs-FallbackIndicator | False |  |  |
| purpose CHOICE{ |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | Utra |  |  |
| targetRAT-MessageContainer | HANDOVER TO UTRAN COMMAND(UTRA RRC message) |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.18.3.3-6: HANDOVER TO UTRAN COMMAND (step 23, Table 13.4.3.18.3.3-5)

Derivation Path: 36.508, Table 4.7B.1-1, condition UTRA PS RB + Speech

Table 13.4.3.18.3.3-7: UECapabilityEnquiry (step 20, Table 13.4.3.18.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 2 entry |  |  |
| RAT-Type[1] | eutra |  |  |
| RAT-Type[2] | utra |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.18.3.3-7A: ROUTING AREA UPDATE ACCEPT (step 32, Table 13.4.3.18.3.2-2)

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 0 ‘ follow-on proceed’ |  |  |
| PDP context status | ‘0010000000000000’B | NSAPI 5 |  |

Table 13.4.3.18.3.3-8: CONNECT (step 35, Table 13.4.3.18.3.2-2)

| Derivation Path: TS 24.008 Table 9.59 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Transaction identifier |  |  |  |
| TI flag | '0'B | The message is sent from the side that originates the TI |  |
| TIO | '000'B | TI value 0 |  |

Table 13.4.3.18.3.3-9: CONNECT ACKNOWLEDGE (step 36, Table 13.4.3.18.3.2-2)

| Derivation Path: TS 24.008 Table 9.60 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Transaction identifier |  |  |  |
| TI flag | '1'B | The message is sent to the side that originates the TI |  |
| TIO | '000'B | TI value 0 |  |

#### <End of change>