3GPP TSG-RAN5 Meeting #105	R5-246230

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	5.4.20

Source:	Keysight Technologies

Title:	PC7 FR2 MU - Discussion on non-antenna related MU factors

Document for:	Discussion and Endorsement

# Introduction

Despite Rel-17 WI NR_redcap_plus_ARCH-UEConTest was closed, MU/TT analysis of FR2 PC7 for RedCap is still missing. This contribution will focus on defining the PC7 FR2 MU factors not related to the UE antenna array assumption. Those related to the UE antenna array will be specifically covered in [1].

# Discussion

In TR 38.903 [2], all MU factors not related with the UE antenna array assumption are the same for all power classes except for the influence of noise. Therefore, there is no reason to not follow the same approach for PC7.

Observation 1. All MU factors not related with the UE antenna array assumption are the same for all power classes except for the influence of noise.

Proposal 1. For all MU factors not related with the UE antenna array assumption except for the influence of noise, apply for PC7 the same value already defined for other power classes.

Influence of noise is a MU factor that needs to be calculated per test case, band and power class considering the expected UE power level, typically the lowest expected value, and the assumed test system noise floor in those conditions.

In this section, the minimum requirements described in TS 38.101-2 [3] for PC7 will be analysed and compared to those applicable to other power classes. In TR 38.903 [2],  MU factors are already defined for PC1, PC3, PC5 and PC6. However, due to the scope of RedCap devices, not targeting FWA (PC1, PC5) or HST scenarios (PC6), analysis for PC7 will be focused on comparison with PC3.

In addition, considering the associated RAN5 WI is Rel-17, the latest Rel-17 version of TS 38.101-2 [3] will be used in this analysis.

Observation 2. Due to the scope of RedCap devices, from power classes with MU factors already defined in TR 38.903 [2], MU analysis for PC7 will be based on comparison with PC3 over latest Rel-17 version of TS 38.101-2 [3].

## Frequency range

As it can be observed in TS 38.101-2 [3], requirements for PC7 are only defined for FR2a bands.

Observation 3. MU analysis for PC7 is limited to FR2a bands.

## Channel bandwidth (CBW)

As it can be observed in TS 38.101-2 [3], requirements for PC7 are only defined for up to 100MHz CBW. Noting also that carrier aggregation (CA) does not apply either for PC7, 100MHz is the maximum BW a RedCap FR2 Rel-17 UE can support.

Observation 4. MU analysis for PC7 is limited to CBW ≤ 100MHz.

## UE Power requirements

As it can be observed in TS 38.101-2 [3], all UE power requirements explicitly defined and applicable to PC7 are the same as defined for PC3 with the following exceptions for the applicable frequency range:

- Min peak EIRP is 16.4dBm, that is 6dB lower than 22.4dBm defined for PC3.

- Min EIRP at 50 %-tile CDF (spherical coverage) is 5.5dBm, that is 6dB lower than 11.5dBm defined for PC3.

Observation 5. For PC7 in FR2a, all power requirements are the same as those defined for PC3 except for Min Peak EIRP and Min EIRP for Spherical coverage.

In order to analyse the influence of noise when measuring an UL signal, for a given UL signal power, the worst case is for the highest CBW because the greater the measurement BW, the lower the SNR given more noise is captured. In case of PC3 the greatest CBW for which MU has been analysed is 400MHz. In case of PC7 the greatest CBW to consider is 100MHz. That means that the lower power levels applicable for PC7, 6dB lower than those for PC3, drive however into the same SNR as in case of PC3 and hence into the same influence of noise because total noise captured power will also be 6dB lower. Therefore, for test cases applicable to both PC3 and PC7, influence of noise defined for PC3 must be applicable to PC7.

Observation 6. In worst case, for all transmitter characteristics test cases applicable to PC7, the resulting SNR at the measurement equipment and so the influence of noise MU factor will be the same as estimated for PC3.

Proposal 2. For test cases applicable to both PC3 and PC7, influence of noise defined for PC3 in FR2a must be applicable to PC7.

Proposal 3. For test cases applicable to both PC3 and PC7, if relaxations are defined for PC3 due to the influence of noise for CBW up to 100MHz, the same relaxations must be applicable to PC7.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. All MU factors not related with the UE antenna array assumption are the same for all power classes except for the influence of noise.

Observation 2. Due to the scope of RedCap devices, from power classes with MU factors already defined in TR 38.903 [2], MU analysis for PC7 will be based on comparison with PC3 over latest Rel-17 version of TS 38.101-2 [3].

Observation 3. MU analysis for PC7 is limited to FR2a bands.

Observation 4. MU analysis for PC7 is limited to CBW ≤ 100MHz.

Observation 5. For PC7 in FR2a, all power requirements are the same as those defined for PC3 except for Min Peak EIRP and Min EIRP for Spherical coverage.

Observation 6. In worst case, for all transmitter characteristics test cases applicable to PC7, the resulting SNR at the measurement equipment and so the influence of noise MU factor will be the same as estimated for PC3.

Proposal 1. For all MU factors not related with the UE antenna array assumption except for the influence of noise, apply for PC7 the same value already defined for other power classes.

Proposal 2. For test cases applicable to both PC3 and PC7, influence of noise defined for PC3 in FR2a must be applicable to PC7.

Proposal 3. For test cases applicable to both PC3 and PC7, if relaxations are defined for PC3 due to the influence of noise for CBW up to 100MHz, the same relaxations must be applicable to PC7.

# References

- R5-246537, “PC7 MUs based on antenna array assumptions”, Keysight, RAN5#105

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v18.4.0 (2024-09)

- TS 38.101-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v17.15.0 (2024-09)