3GPP TSG-RAN WG5 Meeting #105                                                                  R5-246379

# Orlando, USA, 18th Nov 2024 – 22nd Nov 2024

Source:	MediaTek Inc.

Title:		Discussion paper on UE capabilities depending on band or band combination

Agenda item:	6.4.7

# Document for: Discussion and Decision

- Introduction

In UE Capability transfer test cases 8.1.5.1.1, 8.2.1.1.1 and 8.2.1.1.2, there are some UE capability checks in FeatureSetDownlink, FeatureSetUplink or CA-ParametersNR depend on UE reported band or band combinations. For UE capabilities defined per feature set, per feature set per component carrier, per band or per band combination in TS 38.306, the UE reports the supported band combinations and feature set combinations based on IE frequencyBandListFilter (configured by px_NR_PrimaryBand in current TTCN) as specified in TS 38.331 clause 5.6.1.4. It is possible that the filtered band combinations and feature set combinations not including the UE capabilities.

- Discussion

For some UE capabilities which are optional and can be reported independently, but only one PICS item defined in TS 38.508-2, it is proposed to have separate PICS items. For example, pc_ul_FullPwrMode2_r16 for ULFPTX mode 2, but there are three UE capabilities.

Table 8.1.5.1.1.3.3-7: FeatureSetUplink-v1610 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplink-v1610 ::= SEQUENCE { |  |  |  |
| pusch-RepetitionTypeB-r16 | Checked |  | pc_pusch_RepetitionTypeB_r16 |
| … |  |  |  |
| ul-FullPwrMode2-MaxSRS-ResInSet-r16 | Not checked |  |  |
| … | Not checked |  |  |
| ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 | Not checked |  |  |
| ul-FullPwrMode2-TPMIGroup-r16 | Not checked |  |  |
| } |  |  |  |

Observation 1: There are some UE capabilities, but only one PICS item defined.

Proposal 1: Split the one specific PICS item into corresponding PICS items as done in CR [1].

When UE supports a feature with the relevant PICS set to true on dedicate bands or band combinations, but the filtered band combinations and feature set combinations don`t include this feature. This will cause the case failure as the reported UE capability doesn`t match the PICS setting.

For example, pc_ul_FullPwrMode_r16 and pc_ul_FullPwrMode1_r16 are checked in Table 8.1.5.1.1.3.3-7:

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplink-v1610 ::= SEQUENCE { |  |  |  |
| pusch-RepetitionTypeB-r16 | Checked |  | pc_pusch_RepetitionTypeB_r16 |
| … |  |  |  |
| ul-FullPwrMode-r16 | Checked |  | pc_ul_FullPwrMode_r16 |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| ul-FullPwrMode1-r16 | Checked |  | pc_ul_FullPwrMode1_r16 |
| ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 | Not checked |  |  |
| ul-FullPwrMode2-TPMIGroup-r16 | Not checked |  |  |
| } |  |  |  |

And these two capabilities are defined per FS in TS 38.306:

| ul-FullPwrMode-r16 Indicates the UE support of UL full power transmission mode of fullpower as specified in clause 7.1 of TS 38.213 [11]. If the UE indicates this capability the UE also indicates the support of codebook based PUSCH MIMO transmission using mimo-CB-PUSCH and the support of PUSCH codebook coherency subset using pusch-TransCoherence. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ul-FullPwrMode1-r16 Indicates the UE support of UL full power transmission mode of fullpowerMode1. If the UE indicates this capability the UE also indicates the support of codebook based PUSCH MIMO transmission using mimo-CB-PUSCH and the support of PUSCH codebook coherency subset using pusch-TransCoherence. | FS | No | N/A | N/A |

If UE supports ULFPTX and mode 1 in any band or band combination (e.g., n41 or CA_n41C), these two PICS shall be set to True. But if the px_NR_PrimaryBand (frequencyBandListFilter) is filled as FDD band n2 and the sorted our band combinations, such as CA_n2(2A) doesn`t support full power mode, the ul-FullPwrMode-r16 and ul-FullPwrMode1-r16 will not be reported. Therefore, the PICS check in table  8.1.5.1.1.3.3-7 will fail.

In general, we can have two ways to pass these UE cap cases:

- Don`t modify the PIXIT, but change the PICS to false: the issue is that actually UE support full power and mode 1 but will never be checked.

- Don`t modify the PICS, but change the PIXIT to a band included in any BC which supports full power mode and mode 1: the problem is that usually the test engineer doesn`t know which band supports full power mode and mode 1 without guidance from chipset vendor or OEM.

Obviously these two solutions cann`t fix the issue perfectly and the existing Notes in Table 8.1.5.1.1.3.3-3 are not suitable for this issue.

Observation 2: There are some UE capabilities which are reported depends on the band under test.

Proposal 2: Add a meaningful note as done in the CR [1] to the impacted PICS in TS 38.508-2 to check whether the band under test is correctly set.

3.	Proposal

In this discussion paper, it`s proposed that RAN5 to discuss and agree on below proposals.

Proposal 1: Split the one specific PICS item into corresponding PICS items as done in CR [1].

Proposal 2: Add a meaningful note as done in the CR [1] to the impacted PICS in TS 38.508-2 to check whether the band under test is correctly set.

4.	References

[1]	R5-246367: Addition of note for PICS depending on band or band combination, MediaTek Inc.