3GPP TSG-RAN5 Meeting #105	R5-246391

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

Source:	MediaTek Inc., CMCC, China Telecom, SRTC, Sateliot, Inmarsat, ViaSat, EchoStar, Novamint, Thales, ESA

Title:		Discussion paper on adding new test cases of UE coarse location reporting for NB-IoT NTN

Agenda item:	6.5.6

# Document for: Discussion and Decision

- Introduction

As in TS 23.401 clause 4.13.4, there is a requirement for NB-IoT NTN to provide UE location for regulation purpose under Rel-17 WI LTE_NBIOT_eMTC_NTN / IoT_SAT_ARCH_EPS.

4.13.4	Verification of UE location

In order to ensure that the regulatory requirements are met, the network may be configured to enforce that the selected PLMN is allowed to operate in the current UE location by verifying the UE location during EMM and ESM procedures. In this case, when the MME receives the User Location Information (ULI) for a UE using satellite access for Cellular IoT, the MME may decide to verify the UE location.

Regarding how to provide such UE coarse location, SA2 provided some guidance in the reply LS [1] in April 2022.

For a UE using CP CIoT EPS optimization only, RAN3 understands that the UE might not be able to provide UE location information to eNB, on the assumption that such information can only be sent after AS security activation. Therefore, the CGI provided in the ULI to MME may only refer to a quite coarse-grained location (e.g., based on the satellite beam footprint) and might not be sufficient to determine the country where the UE is located (and also not close to TN level granularity, as previously requested for NR NTN).

RAN3 would like thus to ask SA2 and SA3 whether such a coarse-grained location is sufficient for UEs using only CP CIoT EPS optimization in release 17. RAN3 assumes that no such issue will arise for UEs using UP CIoT EPS optimization.

[SA2 answer]: SA2 does not see any issue with providing coarse-grained location for UE using only CP CIoT EPS optimisation because MME has anyway the ability to trigger LCS procedure to retrieve a more fine grained location if needed.

However, after a long discussion between SA2 and CT1 there was an agreement [2] within WI TEI18 /  TEI18, IoT_SAT_ARCH_EPS in May 2024 to introduce the mechanism for the UE in NB-S1 mode to provide a coarse location during attach and TAU procedure.

- Discussion

Considering that the verification of UE location is regulatory requirement, and there is strong industrial requirement for this feature, it is proposed to add below new signalling test cases to verify that a NB-IoT NTN UE can report its coarse location during the attach, normal and periodic tracking area updating procedure.

| TC ID | Title | Release |
| --- | --- | --- |
| 22.5.25 | NB-IoT / NTN / Attach Procedure / Coarse location information reporting | Rel-18 |
| 22.5.26 | NB-IoT / NTN / Normal tracking area update / Coarse location information reporting | Rel-18 |
| 22.5.27 | NB-IoT / NTN / Periodic tracking area update / Coarse location information reporting | Rel-18 |

Below CRs [3][4][5][6] have been submitted for this meeting:

| CR | Title | Spec |
| --- | --- | --- |
| R5-246387 | Addition of NB-IoT NTN TC 22.5.25 | 36.523-1 |
| R5-246388 | Addition of NB-IoT NTN TC 22.5.26 | 36.523-1 |
| R5-246389 | Addition of NB-IoT NTN TC 22.5.27 | 36.523-1 |
| R5-246390 | Addition of applicability for new NB-IoT NTN TC | 36.523-2 |

3.	Proposal

Proposal:	RAN5 to accept the proposal of adding new test cases of UE coarse location reporting for NB-IoT NTN.

4.	References

[1]         S2-2203064: Reply LS on opens issues for NB-IoT and eMTC support for NTN, 3GPP SA2.

[2]         C1-243950: UE location reporting for NB-IoT satellite access, Ericsson, MediaTek Inc., vivo, Nokia, Samsung, Apple, Gatehouse Satcom, Novamint, Sateliot, Qualcomm Incorporated, EchoStar, Inmarsat, Viasat, Skylo, DISH Network, ZTE, Huawei, HiSilicon.

[3]         R5-246387: Addition of NB-IoT NTN TC 22.5.25, MediaTek Inc., CMCC, China Telecom, SRTC, Sateliot, Inmarsat, ViaSat, EchoStar, Novamint, Thales, ESA

[4]         R5-246388: Addition of NB-IoT NTN TC 22.5.26, MediaTek Inc., CMCC, China Telecom, SRTC, Sateliot, Inmarsat, ViaSat, EchoStar, Novamint, Thales, ESA

[5]         R5-246389: Addition of NB-IoT NTN TC 22.5.27, MediaTek Inc., CMCC, China Telecom, SRTC, Sateliot, Inmarsat, ViaSat, EchoStar, Novamint, Thales, ESA

[6]         R5-246390: Addition of applicability for new NB-IoT NTN TC, MediaTek Inc., CMCC, China Telecom, SRTC, Sateliot, Inmarsat, ViaSat, EchoStar, Novamint, Thales, ESA