3GPP TSG RAN WG5 Meeting #105	R5-246498

# Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3030 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new test case 7.6E.3.1 Out-of-band blocking for V2X non-concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 7.6E.3.1 Out-of-band blocking for V2X / non-concurrent operation is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 7.6E.3.1 Out-of-band blocking for V2X / non-concurrent operation was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7.6E.3.1 will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6E.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - Test points analysis in R5-246503. - Depends on RAN4 CR R4-2417641. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.6E.3.1	Out-of-band blocking for V2X / non-concurrent operation

7.6E.3.1.1	Test Purpose

Out-of-band blocking for V2X is defined for an unwanted CW interfering signal falling more than 30 MHz below or above the V2X band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels in aggregated signals.

For the first 30 MHz below or above the V2X band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5E and sub-clause 7.6E.2 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6E.3.1.2	Test Applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

7.6E.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6E.3.0.1.

7.6E.3.1.4	Test Description

7.6E.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on V2X operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.6E.3.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.2 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 7.6E.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1[5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Receive |  |  |
| Test ID | Modulation |  | PSSCH RB allocation |
| 1 | CP-OFDM QPSK |  | Full RB (NOTE 1) |
| NOTE 1: The specific configuration is defined in Table 7.3E.2.4.1-2. |  |  |  |

1.	Connect the SS to the UE antenna connectors and connect the GNSS simulator to the UE GNSS RX antenna connector as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1a for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 7.6E.3.1.4.3.

3.	The V2X reference measurement channel is set according to Table 7.6E.3.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Receive Mode according to TS 38.508-1 [5] clause 4.5.

7.	Trigger the UE to reset UTC time. (NOTE: The UTC time reset may be performed by MMI or AT command (+CUTCR).)

8.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

7.6E.3.1.4.2	Test procedure

1.	The UE starts to perform the NR V2X sidelink communication according to SL-V2X-Preconfiguration and to schedule the V2X RMC according to Table 7.6E.3.1.4.1-1. Set the downlink signal level according to Table 7.6E.3.1.5-1.

2.	Set the parameters of the CW signal generator for an interfering signal below the wanted signal according to Table 7.6E.3.1.5-2. The frequency step size is 1MHz.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2. Record the frequencies for which the throughput doesn't meet the requirements.

4.	Repeat step 2 and 3, using an interfering signal above the wanted signal at step 2.

7.6E.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

7.6E.3.1.5	Test Requirement

Except for the spurious response frequencies recorded at the final step of test procedure, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Tables 7.6E.3.1.5-1 and 7.6E.3.1.5-2.

For Table 7.6E.3.1.5-2 in frequency range 1, 2 and 3, the number of spurious response frequencies recorded in the final step of test procedure shall not exceed in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7E.1 spurious response are applicable.

Table 7.6E.3.1.5-1: Out-of-band blocking parameters for NR V2X

| RX parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in transmission bandwidth configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |
|  | dB | 6 | 9 | 11 | 12 |
| NOTE: Reference measurement channel is A.7.2. |  |  |  |  |  |

Table 7.6E.3.1.5-2: Out of-band blocking for NR V2X

| NR band | Parameter | Units | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n47 | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4400 MHz. |  |  |  |  |  |

<< End of changes >>