3GPP TSG-RAN5 Meeting #105	R5-246617

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3043 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 6.3J.3.4 SRS time mask for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 6.3J.3.4 SRS time mask for ATG still missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.3J.3.4 SRS time mask for ATG was introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_ATGUEContest work item cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J.3.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

#### 6.3J.3.4	SRS time mask for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Whether requirement apply per connector or to the sum power of all connectors is FFS

- MU and TT are FFS

6.3J.3.4.1	Test purpose

Same test purpose as in 6.3.3.6.1.

6.3J.3.4.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

6.3J.3.4.3	Minimum conformance requirements

The SRS time mask requirements as specified in clause 6.3.3.6.3 are applicable for ATG UE at each transmit antenna connector or each TAB connector.

not apply during this transient period.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.3.

6.3J.3.4.4	Test description

Same test description as in clause 6.3.3.6.4 with following exceptions:

-	The power is measured as sum of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

6.3J.3.4.5	Test requirement

The measured SRS time mask power shall not exceed the values specified in Table 6.3J.3.4.5-1.

Table 6.3J.3.4.5-1: SRS time mask

|  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  | Same as Table 6.2.1.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.3.6.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3J.3.4.5-2: Test Tolerance (Transmit OFF power and SRS time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |
| 40MHz < BW ≤ 100MHz | FFS | FFS |

<< END OF CHANGES >>