3GPP TSG-RAN5 Meeting #105	R5-246631

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3365 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NTN satellite information clause 6.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Tables 6.3.4.1-1, 6.3.4.2-1 contains ephemeris and other parameters related to satellite but table naming incorrectly says Satellite ephemeris information in multi-cell NTN GSO/NGSO scenario. 2. IE names are incorrect for Koffset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Updated the titles of Tables 6.3.4.1-1, 6.3.4.2-1. 2. Corrected Koffset parameter naming. 3. Updated wording in the clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous GSO, NGSO test environment remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 6.3.4	Satellite assistance information for NTN

#### 6.3.4.1	GSO scenario

The position of GSO satellite is defined in Table 4.6.3-56C for NTN with below assumption:

-	Inclination: 0

-	Eccentricity: 0

-	Latitude: 0

-	Longitude: 121.56076999

-	Altitude: 35786 km

-	Speed: 0 km/s

In case of multi-cell NTN GSO scenario, a maximum of 3 NTN cells can be used. The relevant assistance information for satellite positioning and timing adjustement calculations is defined in Table 6.3.4.1-1 unless otherwise specified in a test case.

Table 6.3.4.1-1: Satellite assistance information in multi-cell NTN GSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX/Y/Z | nta-Common | cellSpecificKoffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | -16976014 | 27636499 | 0 | 0 | 65896378 | 550 |
| Second | -23550424 | 22301093 | 0 | 0 | 66485757 | 550 |
| Third | -14502736 | 29010891 | 0 | 0 | 66485757 | 550 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 38.523-1[12] TC 9.4.1.1 running in GSO scenario, NGC Cell A is the serving cell in the preamble, so it uses the satellite assistance information of First Cell, while NGC Cell G uses the satellite assistance information of Second Cell. |  |  |  |  |  |  |

#### 6.3.4.2	NGSO scenario

The position of NGSO satellite is defined in Table 4.6.3-56C for NTN with below assumption:

-	Inclination: 90

-	Eccentricity: 0

-	Longitude: 121.56076999

-	Altitude: 600 km

-	Speed: 7.56 km/s

The UE position defined in clause 4.5.C is the projection position from one of the satellite information to the ground.

All NTN test cases defined in TS 38.523-1 [12], unless otherwise stated, shall use the first row of Table 6.3.4.2-1 as the assistance information for configuration of the serving cell with a specific location, corresponding to zero Doppler and constant delay for NGSO scenario, keeping such information constant throughout the duration of each measurement in the test.

In case of multi-cell NTN NGSO scenario, a maximum of 3 NTN cells can be used. The satellite assistance information for satellite positioning and timing adjustement calculations is defined in Table 6.3.4.2-1 unless otherwise specified in a test case, keeping such information constant throughout the duration of each measurement in the test.

Table 6.3.4.2-1: Satellite assistance information in multi-cell NTN NGSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX | velocityVY | velocityVZ | kMac | cellSpecifickOffset | Fd (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 3 | 5 | 0.013 |
| Second | -2654549 | 4386991 | 1594205 | 14581 | -34487 | 120182 | 4 | 8 | 19.83 |
| Third | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 4 | 8 | -19.95 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and Third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 38.523-1[12] TC 9.4.1.1 running in NGSO scenario, NGC Cell A is the serving cell in the preamble, so it uses the satellite assistance information of First Cell, while NGC Cell G uses the satellite assistance information of Second Cell. Note 2: Fd is the frequency Doppler in the service link, unit is ppm. The values are not broadcasted but resemble those that are calculated at the UE based on the corresponding cell satellite assistance information. They need to be considered for UL & DL transmission in SS side. |  |  |  |  |  |  |  |  |  |

<End of modified section>