3GPP TSG-RAN5 Meeting #105	R5-247043

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3385 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test frequencies for 5 MHz BW for n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test frequencies for 5 MHz BW needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for 5 MHz BW added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies for 5 MHz BW missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 4.3.1.1.1.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

###### Reference test frequencies for NR operating band n41

Table 4.3.1.1.1.41-1: Test frequencies for NR operating band n41, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2498.505 | 499701 | 2496.255 | 499251 | 0 | 15 | 6246 | 499710 | 9 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2572.395 | 514479 | 102 |  | 6483 | 518670 | 5 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2687.49 | 537498 | 2594.52 | 518904 | 504 |  | 6720 | 537630 | 2 | 2 | 2 (4) | 510 |
| 10 | 52 | Downlink | Low | 2501.01 | 500202 | 2496.33 | 499266 | 0 | 15 | 6246 | 499710 | 4 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2569.965 | 513993 | 102 |  | 6477 | 518190 | 7 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
| 15 | 79 | Downlink | Low | 2503.5 | 500700 | 2496.39 | 499278 | 0 | 15 | 6246 | 499710 | 0 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2567.535 | 513507 | 102 |  | 6471 | 517710 | 9 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2682.495 | 536499 | 2584.665 | 516933 | 504 |  | 6693 | 535470 | 11 | 0 | 0 (0) | 504 |
| 20 | 106 | Downlink | Low | 2506.005 | 501201 | 2496.465 | 499293 | 0 | 15 | 6246 | 499710 | 7 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2565.105 | 513021 | 102 |  | 6465 | 517230 | 11 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2679.99 | 535998 | 2579.73 | 515946 | 504 |  | 6681 | 534510 | 8 | 1 | 0 (0) | 505 |
| 30 | 160 | Downlink | Low | 2511 | 502200 | 2496.6 | 499320 | 0 | 15 | 6246 | 499710 | 10 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2560.245 | 512049 | 102 |  | 6453 | 516270 | 3 | 1 | 2 (4) | 107 |
|  |  | Uplink | High | 2674.995 | 534999 | 2569.875 | 513975 | 504 |  | 6657 | 532590 | 1 | 1 | 1 (2) | 507 |
| 40 | 216 | Downlink | Low | 2516.01 | 503202 | 2496.57 | 499314 | 0 | 15 | 6246 | 499710 | 0 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2555.205 | 511041 | 102 |  | 6441 | 515310 | 7 | 2 | 2 (4) | 108 |
|  |  | Uplink | High | 2670 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
| 50 | 270 | Downlink | Low | 2521.005 | 504201 | 2496.705 | 499341 | 0 | 15 | 6246 | 499710 | 3 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2550.345 | 510069 | 102 |  | 6426 | 514110 | 3 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2664.99 | 532998 | 2549.97 | 509994 | 504 |  | 6606 | 528510 | 4 | 0 | 0 (0) | 504 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-2: Test frequencies for NR operating band n41, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2501.01 | 500202 | 2496.69 | 499338 | 0 | 30 | 6252 | 500190 | 20 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2551.95 | 510390 | 102 |  | 6483 | 518670 | 0 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (0) | 1008 |
| 15 | 38 | Downlink | Low | 2503.5 | 500700 | 2496.66 | 499332 | 0 | 30 | 6252 | 500190 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2549.43 | 509886 | 102 |  | 6474 | 517950 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2682.48 | 536496 | 2494.2 | 498840 | 504 |  | 6699 | 535950 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 2506.02 | 501204 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2547.09 | 509418 | 102 |  | 6471 | 517710 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2679.99 | 535998 | 2489.37 | 497874 | 504 |  | 6687 | 534990 | 12 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2511 | 502200 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2542.23 | 508446 | 102 |  | 6456 | 516510 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2674.98 | 534996 | 2479.5 | 495900 | 504 |  | 6663 | 533070 | 6 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 | 6252 | 500190 | 4 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2537.19 | 507438 | 102 |  | 6444 | 515550 | 16 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (0) | 1008 |
| 50 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 | 6252 | 500190 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2592.99 | 518598 | 2532.33 | 506466 | 102 |  | 6432 | 514590 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  | 6612 | 528990 | 20 | 0 | 0 (0) | 1008 |
| 60 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2527.11 | 505422 | 102 |  | 6420 | 513630 | 0 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  | 6588 | 527070 | 14 | 0 | 2 (2) | 1012 |
| 70 | 189 | Downlink | Low | 2531.01 | 506202 | 2496.99 | 499398 | 0 | 30 | 6252 | 500190 | 0 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2522.25 | 504450 | 102 |  | 6408 | 512670 | 4 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2655 | 531000 | 2439.54 | 487908 | 504 |  | 6564 | 525150 | 6 | 0 | 3 (3) | 1014 |
| 80 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2517.21 | 503442 | 102 |  | 6396 | 511710 | 20 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  | 6537 | 522990 | 4 | 0 | 1 (1) | 1010 |
| 90 | 245 | Downlink | Low | 2541 | 508200 | 2496.9 | 499380 | 0 | 30 | 6252 | 500190 | 6 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2512.17 | 502434 | 102 |  | 6381 | 510510 | 4 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2644.98 | 528996 | 2419.44 | 483888 | 504 |  | 6513 | 521070 | 10 | 0 | 2 (2) | 1012 |
| 100 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 | 6252 | 500190 | 8 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2507.13 | 501426 | 102 |  | 6369 | 509550 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  | 6486 | 518910 | 6 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-3: Test frequencies for NR operating band n41, SCS 60 kHz and ΔFRaster 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2501.01 | 500202 | 2497.05 | 499410 | 0 | 15 | - | 499770 |
|  |  | & | Mid | 2593.005 | 518601 | 2515.605 | 503121 | 102 |  | - | 518169 |
|  |  | Uplink | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
| 15 | 18 | Downlink | Low | 2503.5 | 500700 | 2497.02 | 499404 | 0 | 15 | - | 499764 |
|  |  | & | Mid | 2593.005 | 518601 | 2513.085 | 502617 | 102 |  | - | 517665 |
|  |  | Uplink | High | 2682.495 | 536499 | 2313.135 | 462627 | 504 |  | - | 535563 |
| 20 | 24 | Downlink | Low | 2506.005 | 501201 | 2497.365 | 499473 | 0 | 15 | - | 499833 |
|  |  | & | Mid | 2593.005 | 518601 | 2510.925 | 502185 | 102 |  | - | 517233 |
|  |  | Uplink | High | 2679.99 | 535998 | 2308.47 | 461694 | 504 |  | - | 534630 |
| 30 | 38 | Downlink | Low | 2511 | 502200 | 2497.32 | 499464 | 0 | 15 | - | 499824 |
|  |  | & | Mid | 2593.005 | 518601 | 2505.885 | 501177 | 102 |  | - | 516225 |
|  |  | Uplink | High | 2674.995 | 534999 | 2298.435 | 459687 | 504 |  | - | 532623 |
| 40 | 51 | Downlink | Low | 2516.01 | 503202 | 2497.65 | 499530 | 0 | 15 | - | 499890 |
|  |  | & | Mid | 2593.005 | 518601 | 2501.205 | 500241 | 102 |  | - | 515289 |
|  |  | Uplink | High | 2670 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
| 50 | 65 | Downlink | Low | 2521.005 | 504201 | 2497.605 | 499521 | 0 | 15 | - | 499881 |
|  |  | & | Mid | 2593.005 | 518601 | 2496.165 | 499233 | 102 |  | - | 514281 |
|  |  | Uplink | High | 2664.99 | 532998 | 2278.71 | 455742 | 504 |  | - | 528678 |
| 60 | 79 | Downlink | Low | 2526 | 505200 | 2497.56 | 499512 | 0 | 15 | - | 499872 |
|  |  | & | Mid | 2593.005 | 518601 | 2491.125 | 498225 | 102 |  | - | 513273 |
|  |  | Uplink | High | 2659.995 | 531999 | 2268.675 | 453735 | 504 |  | - | 526671 |
| 70 | 93 | Downlink | Low | 2531.01 | 506202 | 2497.53 | 499506 | 0 | 15 | - | 499866 |
|  |  | & | Mid | 2593.005 | 518601 | 2486.085 | 497217 | 102 |  | - | 512265 |
|  |  | Uplink | High | 2655 | 531000 | 2258.64 | 451728 | 504 |  | - | 524664 |
| 80 | 107 | Downlink | Low | 2536.005 | 507201 | 2497.485 | 499497 | 0 | 15 | - | 499857 |
|  |  | & | Mid | 2593.005 | 518601 | 2481.045 | 496209 | 102 |  | - | 511257 |
|  |  | Uplink | High | 2649.99 | 529998 | 2248.59 | 449718 | 504 |  | - | 522654 |
| 90 | 121 | Downlink | Low | 2541 | 508200 | 2497.44 | 499488 | 0 | 15 | - | 499848 |
|  |  | & | Mid | 2593.005 | 518601 | 2476.005 | 495201 | 102 |  | - | 510249 |
|  |  | Uplink | High | 2644.995 | 528999 | 2238.555 | 447711 | 504 |  | - | 520647 |
| 100 | 135 | Downlink | Low | 2546.01 | 509202 | 2497.41 | 499482 | 0 | 15 | - | 499842 |
|  |  | & | Mid | 2593.005 | 518601 | 2470.965 | 494193 | 102 |  | - | 509241 |
|  |  | Uplink | High | 2640 | 528000 | 2228.52 | 445704 | 504 |  | - | 518640 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.42 to 4.3.1.1.1.45	FFS

##### << End of changes>>