3GPP TSG-RAN5 Meeting #105	R5-247062

# Orlando, USA, 18 Nov – 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3092 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.5E.1 NR V2X Occupied bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR V2X Occupied bandwidth test cases are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5E.1.0(new), 6.5E.1.1(new), 6.5E.1.1D(new), 6.5E.1.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0953 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5E	Output RF spectrum emissions for V2X

6.5E.1	Occupied bandwidth for V2X

6.5E.1.0	Minimum conformance requirements

6.5E.1.0.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.1.3 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in Table 6.5.1.3-1. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.3.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

6.5E.1.0.2	Occupied bandwidth for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.5.1.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.5E.1.0.1 shall apply for the sidelink in licensed band or Band n47.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5E.1

6.5E.1.1	Occupied bandwidth for V2X / non-concurrent operation

6.5E.1.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits.

6.5E.1.1.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication.

6.5E.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.1.0.

6.5E.1.1.4	Test description

6.5E.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.5E.1.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.5E.1.1.4.1-1: Test Configuration Table for contiguous PSCCH and PSSCH allocation for PC3

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Mid range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | 10MHz, 20MHz, 30MHz, 40MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | QPSK |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.5E.1.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.5E.1.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

6.5E.1.1.4.2	Test procedure

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

2.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is at least 1ms over consecutive active uplink slots.

3.	Calculate the total power within the range of all frequencies measured in step 3 and save this value as “Total power”.

4.	Identify the measurement window whose centre is aligned on the centre of the channel for which the sum of the power measured is 99% of the “Total power”.

5.	The “Occupied Bandwidth” is the width of the measurement window obtained in step 5.

6.5E.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

Table 6.5E.1.1.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

6.5E.1.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.5E.1.1.5-1.

Table 6.5E.1.1.5-1: Occupied channel bandwidth

|  | NR channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Occupied channel bandwidth (MHz) | 10 | 20 | 30 | 40 |

6.5E.1.1D	Occupied bandwidth for V2X / non-concurrent operation / SL-MIMO

6.5E.1.1D.1	Test purpose

Same test purpose as in 6.5E.1.1.1.

6.5E.1.1D.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.5E.1.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.1.0.

6.5E.1.1D.4	Test description

Same test description as in 6.5E.1.1.4 with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

6.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

For test procedures:

Steps 2 in test procedures is replaced by:

2.	Measure the power spectrum distribution at each of UE’s transmit antenna connectors respectively within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is at least 1ms over consecutive active uplink slots.

6.5E.1.1D.5	Test requirement

The measured Occupied Bandwidth on each antenna connector shall not exceed values in Table 6.5E.1.1D.5-1.

Table 6.5E.1.1D.5-1: Occupied channel bandwidth

|  | NR channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Occupied channel bandwidth (MHz) | 10 | 20 | 30 | 40 |

6.5E.1.2	Occupied bandwidth for V2X / concurrent operation

6.5E.1.2.1	Test purpose

Same test purpose as in 6.5E.1.1.1 for concurrent operation.

6.5E.1.2.2	Test applicability

No test case details are specified for inter-band concurrent operation. The Occupied bandwidth requirements for concurrent operation apply and are tested in clauses 6.5.1 for NR Uu operation and 6.5E.1.1 for NR sidelink option.

No test case details are specified for intra-band concurrent operation since no additional emission requirement is specified in this release.

6.5E.2	Out of band emission for V2X

< End of Changes >