3GPP TSG-RAN5 Meeting #105	R5-247201

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3392 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 4.9.42 Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The configuration of RadioBearerConfig and CellGroupConfig for SRB2 and DRBs are missing at Step 21a1 in 4.9.42, The configuration of PC5 RLC relay channel for remote UE is missing as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Table 4.9.42.3-2 and adding RRCReconfigurationSidelink procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.42 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.9.42	Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side

4.9.42.1	Scope

The purpose of this procedure is to test remote UE behaviour during 5G ProSe Layer-2 U2N Relay Initial Access procedure.

4.9.42.2	Procedure description

4.9.42.2.1	Initial conditions

UE is authorised to perform NR sidelink communication.

4.9.42.2.2	Procedure sequence

Table 4.9.42.2.2-1: Procedure for 5G ProSe Layer-2 U2N Relay initial access / Remote UE side

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-2 | Steps 1a1 to 1A of the generic procedure in Table 4.5.7.2-1 are performed. | - | - | - | - |
| 3 | Steps 11a3a1 to 11a3b2 of the procedure for 5G ProSe U2N Relay Discovery specified in subclause 4.9.43 is performed. | - | - | - | - |
| - | EXCEPTION: Steps 4a1 and 4b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 4a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.39 is performed to establish unicast mode ProSe Direct communication between the UE and the NR-SS-UE. | - | - | - | - |
| 4b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.40 is performed to establish unicast mode ProSe Direct communication between the UE and the NR-SS-UE. | - | - | - | - |
| 5 | The UE transmit an RemoteUEInformationSidelink message including sl-RequestedSIB-List-r17 to setup? | --> | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 6 | The NR-SS-UE1 transmits a UuMessageTransferSidelink message to deliver SIBs requested at step 5. | <-- | PC5 RRC: UuMessageTransferSidelink | - | - |
| 7 | The UE transmits an RRCSetupRequest message. | --> | PC5 Relay RLC Channel: RRCSetupRequest | - | - |
| 8 | The NR-SS-UE transmits an RRCSetup message. | <-- | PC5 Relay RLC Channel: RRCSetup | - | - |
| - | EXCEPTION: Steps 9 and 10 can be performed in any order. | - | - | - | - |
| 9 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message. | --> | PC5 Relay RLC Channel: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 10 | The UE transmit an RemoteUEInformationSidelink message with sl-RequestedSIB-List-r17 to the value release? | --> | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 11 | The NR-SS-UE transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | PC5 Relay RLC Channel: DLInformationTransfer 5GMM: AUTHENTICATION REQUEST | - | - |
| 12 | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | PC5 Relay RLC Channel: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE | - | - |
| 13 | The NR-SS-UE transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | PC5 Relay RLC Channel: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 14 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | PC5 Relay RLC Channel: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | - |
| 15 | The NR-SS-UE transmits a SecurityModeCommand message. | <-- | PC5 Relay RLC Channel: SecurityModeCommand | - | - |
| 16 | The UE transmits a SecurityModeComplete message. | --> | PC5 Relay RLC Channel: SecurityModeComplete | - | - |
| 17 | The NR-SS-UE transmits a UECapabilityEnquiry message. | <-- | PC5 Relay RLC Channel: UECapabilityEnquiry | - | - |
| 18 | The UE transmits a UECapabilityInformation message. | --> | PC5 Relay RLC Channel: UECapabilityInformation | - | - |
| 19 | The NR-SS-UE transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | PC5 Relay RLC Channel: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 20 | The UE transmits a ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | PC5 Relay RLC Channel: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| - | EXCEPTION: Steps 21a1-21a3 describe behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 21a1 | IF pc_noOf_PDUsSameConnection_Relay > 0 THEN the generic procedure for UE requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection_Relay. | - | - | - | - |
| 21a2 | The NR-SS-UE transmits an RRCReconfigurationSidelink message to add a PC5 Relay RLC channel. | <-- | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 21a3 | The UE sends an RRCReconfigurationCompleteSidelink message. | --> | PC5 RRC: RRCReconfigurationCompleteSidelink | - | - |
| - | EXCEPTION: Steps 22a1-22a2 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 22a1 | IF connected without release is not present THEN, the NR-SS-UE transmits an RRCRelease message. | <-- | PC5 Relay RLC Channel: RRCRelease | - | - |
| 22a2 | The UE transmit an RemoteUEInformationSidelink message including sl-PagingInfo-RemoteUE to setup with ng-5G-S-TMSI? | --> | PC5 RRC: RemoteUEInformationSidelink | - | - |

4.9.42.3	Specific message contents

All specific message contents shall be according subclause 4.6 with the exceptions below.

Table 4.9.42.3-1: RRCSetup (Step 8, Table 4.9.42.2.2-1)

Derivation path: Table 4.6.1-21 with condition L2RemoteUE

Table 4.9.42.3-2: RRCReconfiguration (Step 21a1, Table 4.9.42.2.2-1)

| Derivation path: Table 4.6.1-13 with condition NR and L2RemoteUE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  | NOT IMS_PDU |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition SRB2_DRB1 |  | NOT IMS_PDU |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} | DedicatedNAS-Message |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.9.42.3-2A: RRCReconfigurationSidelink (Step 21a2, Table 4.9.42.2.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition RX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-DRX-ConfigUC-PC5-r17 | Not Present |  |  |
| sl-LatencyBoundIUC-Report-r17 | Not Present |  |  |
| sl-RLC-ChannelToReleaseListPC5-r17 | Not Present |  |  |
| sl-RLC-ChannelToAddModListPC5-r17 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfigPC5-r17{ |  |  |  |
| SL-RLC-ChannelConfigPC5-r17[1]{ |  |  |  |
| sl-RLC-ChannelID-PC5-r17 | 3 | SRB2 |  |
| sl-RLC-ConfigPC5-r17 CHOICE { |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |
| sl-SN-FieldLengthAM-r16 | size12 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r17 SEQUENCE { |  |  |  |
| sl-LogicalChannelIdentity-r16 | LogicalChannelIdentity with condition SRB2 |  |  |
| } |  |  |  |
| } |  |  |  |
| SL-RLC-ChannelConfigPC5-r17[2]{ |  |  |  |
| sl-RLC-ChannelID-PC5-r17 | 5 | DRB1 |  |
| sl-RLC-ConfigPC5-r17 CHOICE { |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |
| sl-SN-FieldLengthAM-r16 | size12 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r17 SEQUENCE { |  |  |  |
| sl-LogicalChannelIdentity-r16 | LogicalChannelIdentity with condition DRB1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.9.42.3-2B: RRCReconfigurationCompleteSidelink (Step 21a3, Table 4.9.42.2.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-4 with condition TX

Table 4.9.42.3-3: RRCRelease (Step 22a1, Table 4.9.42.2.2-1)

Derivation path: Table 4.6.1-16 with condition L2RemoteUE

<End of Changed Section>