3GPP TSG-RAN WG5 Meeting #105	R5-247247

# Orlando, USA,  18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3397 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test channel bandwidths and test frequencies for band n8, n39, n98 and n109 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test CBWs and test frequencies need to be specified for NR band n8, n39, n98 and n109 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses updated to add test frequencies for band n8, n39, n98 and n109. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test CBWs and test frequencies for band n8, n39, n98 and n109 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3.1	Test frequencies

Editor's Note: n65 (High: 50MHz), n66 (High: 45MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

#### 4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8], TS 38.101-3 [9] and TS 38.101-5 [56].

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1, Table 4.3.1.0A-2 and Table 4.3.1.0A-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap and eRedCap UE Mid Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 25 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 20 | 15 |
| n12 | 10 | 10 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 2511 | 15 |
| n26 | 15 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n31 | 5 | 5 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 25 | 15 |
| n40 | 50 | 15 |
| n41 | 50 | 15 |
| n46 | 604, 405 | N/A |
| n48 | 2011 | 15 |
| n50 | 404, 205,11 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n54 | 5 | 5 |
| n65 | 15 | 15 |
| n66 | 20 | 15 |
| n70 | 152, 103,12 | 152, 103 |
| n71 | 153,12, 202 | 15 |
| n72 | 5 | 5 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 50 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n85 | 10 | 10 |
| n86 | 203 | [15] |
| n91 | 52,12, 103 | [52,103] |
| n92 | 15 | [15] |
| n93 | 52,12, 103 | [52,103] |
| n94 | 15 | [15] |
| n95 | 103 | [10] |
| n96 | 60 | N/A |
| n97 | 503 | [15] |
| n98 | 253 | [153] |
| n99 | 103 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n106 | 3 | N/A |
| n109 | 25 | 15 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 (most updated version of latest release available) [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: This Mid test channel bandwidth is chosen since it is more commonly used. Note 11: This is an exception to Note 1 in this table to avoid not allowed asymmetric channel bandwidths. Note 12: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 200 | 100 |
| n258 | 200 | 100 |
| n259 | 200 | N/A |
| n260 | 200 | N/A |
| n261 | 200 | 100 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-2 [15] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. |  |  |

Table 4.3.1.0A-3: Mid Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 15 |
| n255 | 15 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-5 [57] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in TS 38.306 clause 4.2.7.2 [55] for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

#### 4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1, Table 4.3.1.0B-2 and Table 4.3.1.0B-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band | UE Low Test Channel bandwidth [MHz]1, 1a, 1b | RedCap and eRedCap UE Low Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 5 | 5 |
| n2 | 5 | 5 |
| n3 | 5 | 5 |
| n5 | 5 | 5 |
| n7 | 5 | 5 |
| n8 | 5 | 5 |
| n12 | 5 | 5 |
| n13 | 5 | 5 |
| n14 | 5 | 5 |
| n20 | 5 | 5 |
| n24 | 5 | 5 |
| n25 | 5 | 5 |
| n26 | 36,5 | 5 |
| n28 | 36,5 | 5 |
| n29 | 52 | N/A |
| n30 | 5 | 5 |
| n31 | 3 | 5 |
| n34 | 5 | 5 |
| n38 | 5 | 5 |
| n39 | 5 | 5 |
| n40 | 54,105 | 10 |
| n41 | 10 | 10 |
| n46 | 104,205 | N/A |
| n48 | 54, 105 | 10 |
| n50 | 54,105 | 10 |
| n51 | 5 | 5 |
| n53 | 5 | 5 |
| n54 | 5 | 5 |
| n65 | 5 | 5 |
| n66 | 5 | 5 |
| n70 | 5 | 5 |
| n71 | 5 | 5 |
| n72 | 3 | 5 |
| n74 | 5 | 5 |
| n75 | 52 | N/A |
| n76 | 52 | N/A |
| n77 | 10 | 10 |
| n78 | 10 | 10 |
| n79 | 10 | 10 |
| n80 | 53 | [5] |
| n81 | 53 | [5] |
| n82 | 53 | [5] |
| n83 | 53 | [5] |
| n84 | 53 | [5] |
| n85 | 36,5 | 5 |
| n86 | 53 | [5] |
| n91 | 5 | [5] |
| n92 | 5 | [5] |
| n93 | 5 | [5] |
| n94 | 5 | [5] |
| n95 | 53 | [5] |
| n96 | 20 | N/A |
| n97 | 53 | [5] |
| n98 | 53 | [53] |
| n99 | 53 | [5] |
| n100 | 5 | 5 |
| n101 | 36,5 | 5 |
| n106 | 3 | N/A |
| n109 | 5 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as SCell in CA or SCell in DC configuration. Note 5: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 6: This UE channel bandwidth is optional in Release 18. |  |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR Band | UE Low Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 50 | 50 |
| n258 | 50 | 50 |
| n259 | 50 | N/A |
| n260 | 50 | N/A |
| n261 | 50 | 50 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. |  |  |

Table 4.3.1.0B-3: Low Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 5 |
| n255 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

#### 4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1, Table 4.3.1.0C-2 and Table 4.3.1.0C-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz]10, 11, 12 | RedCap and eRedCap UE High Test Channel bandwidth [MHz] 10, 11, 12 |
| --- | --- | --- |
| n1 | 50 | 20 |
| n2 | 40 | 20 |
| n3 | 50 | 20 |
| n5 | 201,14, 252 | 20 |
| n7 | 50 | 20 |
| n8 | 201,14, 352 | 20 |
| n12 | 15 | 15 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 20 | 20 |
| n24 | 10 | 10 |
| n25 | 401,14, 452 | 20 |
| n26 | 20 | 20 |
| n28 | 30 | 20 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n31 | 5 | 5 |
| n34 | 15 | 15 |
| n38 | 25, 4013 | 15 |
| n39 | 40 | 20 |
| n40 | 100 | 20 |
| n41 | 100 | 20 |
| n46 | 80 | N/A |
| n48 | 403, 1004 | 20 |
| n50 | 601,14, 802 | 20 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n54 | 5 | 5 |
| n65 | 20 | 20 |
| n66 | 40 | 20 |
| n70 | 151,14, 252 | 151, 202 |
| n71 | 201,14, 352 | 20 |
| n72 | 5 | 5 |
| n74 | 20 | 20 |
| n75 | 202 | N/A |
| n76 | 52 | N/A |
| n77 | 100 | 20 |
| n78 | 100 | 20 |
| n79 | 100 | 20 |
| n80 | 301 | [20] |
| n81 | 201 | [20] |
| n82 | 201 | [20] |
| n83 | 301 | [20] |
| n84 | 501 | [20] |
| n85 | 15 | 15 |
| n86 | 401 | [20] |
| n91 | 101, 52,14 | [10] |
| n92 | 20 | [20] |
| n93 | 101, 52,14 | [10] |
| n94 | 20 | [20] |
| n95 | 151 | [15] |
| n96 | 80 | N/A |
| n97 | 1001 | [20] |
| n98 | 401 | [201] |
| n99 | 101 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n106 | 3 | N/A |
| n109 | 30, 502 | 20 |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-1 [7] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 11: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-1 [7] is supported. However, these channel bandwidths (except for 100MHz of bands n41, n48, n77, n78, n79) are mandatory with capability signalling as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL and channelBWs-DL-v1590/channelBWs-UL-v1590 parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Channel bandwidth 100MHz of bands n41, n48, n77, n78, and n79 is mandatory without capability signalling (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Note 12: For CA, DC, SDL and SUL, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA, DC, SDL or SUL, testing could be performed with one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth instead of the values described in the Table. Note 13: These UE channel bandwidths are applicable to sidelink operation. Note 14: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

NOTE 1: Void.

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 400 | 100 |
| n258 | 400 | 100 |
| n259 | 400 | N/A |
| n260 | 400 | N/A |
| n261 | 400 | 100 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-2 [8] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-2 [8] is mandatory with capability bit mandated to be set to 1 (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Note 3: For CA and DC, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA and DC, testing could be performed with one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth instead of the values described in the Table. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination), testing for Rel-15 and Rel-16 could be performed using the value signalled by the UE in single carrier operation instead of the values described in Table 4.3.1.0C-2.

Table 4.3.1.0C-3: High Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 20 |
| n255 | 20 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-5 [56] is mandatory without capability parameter (i.e., purely mandatory) as defined in TS 38.306 clauses 4.2.7.6/4.2.7.8 [55] for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). |  |

#### 4.3.1.0D	Bandwidth part

The value of locationAndBandwidth in BWP for FR1 is given in Table 4.3.1.0D-1. The value of locationAndBandwidth in BWP for FR2 is given in Table 4.3.1.0D-2.

Table 4.3.1.0D-1: locationAndBandwidth in BWP for FR1

| BW (MHz) | SCS (kHz) | LRBs (Note 2) | locationAndBandwidth (Note 1) |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 3850 |
| 3 | 30 | N/A | N/A |
| 3 | 60 | N/A | N/A |
| 5 | 15 | 25 | 6600 |
| 5 | 30 | 11 | 2750 |
| 5 | 60 | N/A | N/A |
| 10 | 15 | 52 | 14025 |
| 10 | 30 | 24 | 6325 |
| 10 | 60 | 11 | 2750 |
| 15 | 15 | 79 | 21450 |
| 15 | 30 | 38 | 10175 |
| 15 | 60 | 18 | 4675 |
| 20 | 15 | 106 | 28875 |
| 20 | 30 | 51 | 13750 |
| 20 | 60 | 24 | 6325 |
| 25 | 15 | 133 | 36300 |
| 25 | 30 | 65 | 17600 |
| 25 | 60 | 31 | 8250 |
| 30 | 15 | 160 | 32174 |
| 30 | 30 | 78 | 21175 |
| 30 | 60 | 38 | 10175 |
| 35 | 15 | 188 | 24474 |
| 35 | 30 | 92 | 25025 |
| 35 | 60 | 44 | 11825 |
| 40 | 15 | 216 | 16774 |
| 40 | 30 | 106 | 28875 |
| 40 | 60 | 51 | 13750 |
| 45 | 15 | 242 | 9624 |
| 45 | 30 | 119 | 32450 |
| 45 | 60 | 58 | 15675 |
| 50 | 15 | 270 | 1924 |
| 50 | 30 | 133 | 36300 |
| 50 | 60 | 65 | 17600 |
| 60 | 15 | N/A | N/A |
| 60 | 30 | 162 | 31624 |
| 60 | 60 | 79 | 21450 |
| 70 | 15 | N/A | N/A |
| 70 | 30 | 189 | 24199 |
| 70 | 60 | 93 | 25300 |
| 80 | 15 | N/A | N/A |
| 80 | 30 | 217 | 16499 |
| 80 | 60 | 107 | 29150 |
| 90 | 15 | N/A | N/A |
| 90 | 30 | 245 | 8799 |
| 90 | 60 | 121 | 33000 |
| 100 | 15 | N/A | N/A |
| 100 | 30 | 273 | 1099 |
| 100 | 60 | 135 | 36850 |
| Note 1: The value for locationAndBandwidth parameter is calculated as the RIV value in accordance to TS 38.214 [21] with = 275, =0 and for each bandwidth and subcarrier spacing. Note 2: = Max NRB is the default configuration, unless explicitly specified. |  |  |  |

Table 4.3.1.0D-2: locationAndBandwidth in BWP for FR2

| BW (MHz) | SCS (kHz) | LRBs (Note 2) | locationAndBandwidth (Note 1) |
| --- | --- | --- | --- |
| 50 | 60 | 66 | 17875 |
| 50 | 120 | 32 | 8525 |
| 100 | 60 | 132 | 36025 |
| 100 | 120 | 24(Note 3) | 6325 |
| 100 | 120 | 48(Note 3) | 12925 |
| 100 | 120 | 66 | 17875 |
| 200 | 60 | 264 | 3574 |
| 200 | 120 | 132 | 36025 |
| 400 | 60 | N/A | N/A |
| 400 | 120 | 264 | 3574 |
| Note 1: The value for locationAndBandwidth parameter is calculated as the RIV value in accordance to TS 38.214 [21] with = 275 and =0 for each bandwidth and subcarrier spacing. Note 2: = Max NRB is the default configuration, unless explicitly specified. Note 3: For configurations where the is explicitly specified to be < Max NRB. |  |  |  |

#### 4.3.1.0E	Void

#### 4.3.1.1	Test frequencies for NR operating bands in FR1

##### 4.3.1.1.1	NR operating bands in FR1

###### 4.3.1.1.1.1	Reference test frequencies for NR operating band n1

Table 4.3.1.1.1.1-1: Test frequencies for NR operating band n1 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2119.39 | 423878 | 102 |  | 5350 | 427970 | 8 | 1 | 0 (0) | 103 |
|  |  |  | High | 2167.5 | 433500 | 2074.53 | 414906 | 504 |  | 5418 | 433470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2116.96 | 423392 | 102 |  | 5344 | 427490 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 2165 | 433000 | 2069.6 | 413920 | 504 |  | 5405 | 432490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2114.53 | 422906 | 102 |  | 5338 | 427010 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 2162.5 | 432500 | 2064.67 | 412934 | 504 |  | 5395 | 431570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2112.1 | 422420 | 102 |  | 5332 | 426530 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 2160 | 432000 | 2059.74 | 411948 | 504 |  | 5382 | 430590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2109.67 | 421934 | 102 |  | 5326 | 426050 | 4 | 0 | 1 (2) | 104 |
|  |  |  | High | 2157.5 | 431500 | 2054.81 | 410962 | 504 |  | 5369 | 429610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.53 | 384106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1847.31 | 369462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1954.45 | 390890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2107.24 | 421448 | 102 |  | 5320 | 425570 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 2155 | 431000 | 2049.88 | 409976 | 504 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.6 | 384120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1844.88 | 368976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1949.52 | 389904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2102.2 | 420440 | 102 |  | 5308 | 424610 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 2150 | 430000 | 2039.84 | 407968 | 504 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.56 | 384112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1839.84 | 367968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1939.48 | 387896 | 6 |  | - | - | - | - | - | - |
| 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2099.86 | 419972 | 102 |  | 5302 | 424130 | 6 | 1 | 1(2) | 105 |
|  |  |  | High | 2147.5 | 429500 | 2035 | 407000 | 504 |  | 5320 | 425570 | 10 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1920.72 | 384144 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1837.5 | 367500 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1934.64 | 386928 | 6 |  | - | - | - | - | - | - |
| 50 | 270 | Downlink | Low | 2135 | 427000 | 2110.7 | 422140 | 0 | 15 | 5282 | 422650 | 2 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 2140 | 428000 | 2097.34 | 419468 | 102 |  | 5296 | 423650 | 2 | 0 | 2 (4) | 106 |
|  |  |  | High | 2145 | 429000 | 2029.98 | 405996 | 504 |  | 5307 | 424590 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1945 | 389000 | 1920.7 | 384140 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1834.98 | 366996 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1929.62 | 385924 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-2: Test frequencies for NR operating band n1 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2098.96 | 419792 | 102 |  | 5350 | 427970 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 2165 | 433000 | 1979.24 | 395848 | 504 |  | 5411 | 432970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2140 | 428000 | 2096.44 | 419288 | 102 |  | 5344 | 427490 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2162.5 | 432500 | 1974.22 | 394844 | 504 |  | 5401 | 432050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2094.1 | 418820 | 102 |  | 5338 | 427010 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 2160 | 432000 | 1969.38 | 393876 | 504 |  | 5388 | 431070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2091.58 | 418316 | 102 |  | 5332 | 426530 | 2 | 0 | 2 (7) | 218 |
|  |  |  | High | 2157.5 | 431500 | 1964.36 | 392872 | 504 |  | 5375 | 430090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.8 | 384160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1756.86 | 351372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1953.64 | 390728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2089.24 | 417848 | 102 |  | 5326 | 426050 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2155 | 431000 | 1959.52 | 391904 | 504 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.96 | 384192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1754.52 | 350904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1948.8 | 389760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2084.2 | 416840 | 102 |  | 5314 | 425090 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 2150 | 430000 | 1949.48 | 389896 | 504 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.92 | 384184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1749.48 | 349896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1938.76 | 387752 | 6 |  | - | - | - | - | - | - |
| 45 | 119 | Downlink | Low | 2132.5 | 426500 | 2111.08 | 422216 | 0 | 15 | 5290 | 423170 | 6 | 0 | 3(8) | 16 |
|  |  |  | Mid | 2140 | 428000 | 2081.86 | 416372 | 102 |  | 5308 | 424610 | 10 | 0 | 2(7) | 218 |
|  |  |  | High | 2147.5 | 429500 | 1944.64 | 388928 | 504 |  | 5326 | 426050 | 14 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.08 | 384216 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1747.14 | 349428 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1933.92 | 386784 | 6 |  | - | - | - | - | - | - |
| 50 | 133 | Downlink | Low | 2135 | 427000 | 2111.06 | 422212 | 0 | 15 | 5288 | 423130 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2140 | 428000 | 2079.34 | 415868 | 102 |  | 5302 | 424130 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 2145 | 429000 | 1939.62 | 387924 | 504 |  | 5313 | 425070 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.06 | 384212 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1744.62 | 348924 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1928.9 | 385780 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-3: Test frequencies for NR operating band n1 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  | Mid | 2140 | 428000 | 2062.6 | 412520 | 102 |  | - | 427568 |
|  |  |  | High | 2165 | 433000 | 1798.16 | 359632 | 504 |  | - | 432568 |
|  |  | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |  | - | - |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  | Mid | 2140 | 428000 | 2060.08 | 412016 | 102 |  | - | 427064 |
|  |  |  | High | 2162.5 | 432500 | 1793.14 | 358628 | 504 |  | - | 431564 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |  | - | - |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  | Mid | 2140 | 428000 | 2057.92 | 411584 | 102 |  | - | 426632 |
|  |  |  | High | 2160 | 432000 | 1788.48 | 357696 | 504 |  | - | 430632 |
|  |  | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |  | - | - |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  | Mid | 2140 | 428000 | 2055.4 | 411080 | 102 |  | - | 426128 |
|  |  |  | High | 2157.5 | 431500 | 1783.46 | 356692 | 504 |  | - | 429628 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1921.34 | 384268 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1575.96 | 315192 | 504 |  | - | - |
|  |  |  | High | 1967.5 | 393500 | 1952.02 | 390404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  | Mid | 2140 | 428000 | 2052.88 | 410576 | 102 |  | - | 425624 |
|  |  |  | High | 2155 | 431000 | 1778.44 | 355688 | 504 |  | - | 428624 |
|  |  | Uplink | Low | 1935 | 387000 | 1921.32 | 384264 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1573.44 | 314688 | 504 |  | - | - |
|  |  |  | High | 1965 | 393000 | 1947 | 389400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  | Mid | 2140 | 428000 | 2048.2 | 409640 | 102 |  | - | 424688 |
|  |  |  | High | 2150 | 430000 | 1768.76 | 353752 | 504 |  | - | 426688 |
|  |  | Uplink | Low | 1940 | 388000 | 1921.64 | 384328 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1568.76 | 313752 | 504 |  | - | - |
|  |  |  | High | 1960 | 392000 | 1937.32 | 387464 | 6 |  | - | - |
| 45 | 58 | Downlink | Low | 2132.5 | 426500 | 2111.62 | 422324 | 0 | 15 | - | 422684 |
|  |  |  | Mid | 2140 | 428000 | 2045.68 | 409136 | 102 |  | - | 424184 |
|  |  |  | High | 2147.5 | 429500 | 1763.74 | 352748 | 504 |  | - | 425684 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.62 | 384324 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1566.24 | 313248 | 504 |  | - | - |
|  |  |  | High | 1957.5 | 391500 | 1932.3 | 386460 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 2135 | 427000 | 2111.6 | 422320 | 0 | 15 | - | 422680 |
|  |  |  | Mid | 2140 | 428000 | 2043.16 | 408632 | 102 |  | - | 423680 |
|  |  |  | High | 2145 | 429000 | 1758.72 | 351744 | 504 |  | - | 424680 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.6 | 384320 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1563.72 | 312744 | 504 |  | - | - |
|  |  |  | High | 1955 | 391000 | 1927.28 | 385456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.2	 Reference test frequencies for NR operating band n2

Table 4.3.1.1.1.2-1: Test frequencies for NR operating band n2 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1932.5 | 386500 | 1930.25 | 386050 | 0 | 15 | 4829 | 386410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1939.39 | 387878 | 102 |  | 4900 | 391970 | 8 | 1 | 0 (0) | 103 |
|  |  |  | High | 1987.5 | 397500 | 1894.53 | 378906 | 504 |  | 4968 | 397470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1852.5 | 370500 | 1850.25 | 370050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1787.03 | 357406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1904.17 | 380834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1935 | 387000 | 1930.32 | 386064 | 0 | 15 | 4830 | 386430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1936.96 | 387392 | 102 |  | 4894 | 391490 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1985 | 397000 | 1889.6 | 377920 | 504 |  | 4955 | 396490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.32 | 370064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1784.6 | 356920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1899.24 | 379848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1937.5 | 387500 | 1930.39 | 386078 | 0 | 15 | 4831 | 386450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1934.53 | 386906 | 102 |  | 4888 | 391010 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 1982.5 | 396500 | 1884.67 | 376934 | 504 |  | 4945 | 395570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.39 | 370078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1782.17 | 356434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1894.31 | 378862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1940 | 388000 | 1930.46 | 386092 | 0 | 15 | 4832 | 386650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1932.1 | 386420 | 102 |  | 4882 | 390530 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1980 | 396000 | 1879.74 | 375948 | 504 |  | 4932 | 394590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.46 | 370092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1779.74 | 355948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1889.38 | 377876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1942.5 | 388500 | 1930.53 | 386106 | 0 | 15 | 4833 | 386670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1929.67 | 385934 | 102 |  | 4876 | 390050 | 4 | 0 | 1 (2) | 104 |
|  |  |  | High | 1977.5 | 395500 | 1874.81 | 374962 | 504 |  | 4919 | 393610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.53 | 370106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1777.31 | 355462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1884.45 | 376890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1945 | 389000 | 1930.6 | 386120 | 0 | 15 | 4834 | 386690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1927.24 | 385448 | 102 |  | 4870 | 389570 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 1975 | 395000 | 1869.88 | 373976 | 504 |  | 4909 | 392690 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.6 | 370120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1774.88 | 354976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1879.52 | 375904 | 6 |  | - | - | - | - | - | - |
| 35 | 188 | Downlink | Low | 1947.5 | 389500 | 1930.58 | 386116 | 0 | 15 | 4832 | 386650 | 10 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 1960 | 392000 | 1924.72 | 384944 | 102 |  | 4864 | 389090 | 2 | 1 | 1 (2) | 105 |
|  |  |  | High | 1972.5 | 394500 | 1864.86 | 372972 | 504 |  | 4896 | 391710 | 6 | 2 | 2 (4) | 510 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.58 | 370116 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1772.36 | 354472 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892.5 | 378500 | 1874.5 | 374900 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1950 | 390000 | 1930.56 | 386112 | 0 | 15 | 4833 | 386670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1922.2 | 384440 | 102 |  | 4858 | 388610 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 1970 | 394000 | 1859.84 | 371968 | 504 |  | 4880 | 390490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.56 | 370112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1769.84 | 353968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1890 | 378000 | 1869.48 | 373896 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.2-2: Test frequencies for NR operating band n2 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1935 | 387000 | 1930.68 | 386136 | 0 | 15 | 4836 | 386910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1918.96 | 383792 | 102 |  | 4900 | 391970 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 1985 | 397000 | 1799.24 | 359848 | 504 |  | 4961 | 396970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.68 | 370136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1694.24 | 338848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1898.52 | 379704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1937.5 | 387500 | 1930.66 | 386132 | 0 | 15 | 4837 | 386930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1960 | 392000 | 1916.44 | 383288 | 102 |  | 4894 | 391490 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1982.5 | 396500 | 1794.22 | 358844 | 504 |  | 4951 | 396050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.66 | 370132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1691.72 | 338344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1893.5 | 378700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1940 | 388000 | 1930.82 | 386164 | 0 | 15 | 4835 | 386890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1914.1 | 382820 | 102 |  | 4888 | 391010 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1980 | 396000 | 1789.38 | 357876 | 504 |  | 4938 | 395070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.82 | 370164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1689.38 | 337876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1888.66 | 377732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1942.5 | 388500 | 1930.8 | 386160 | 0 | 15 | 4836 | 386910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1911.58 | 382316 | 102 |  | 4882 | 390530 | 2 | 0 | 2 (7) | 218 |
|  |  |  | High | 1977.5 | 395500 | 1784.36 | 356872 | 504 |  | 4925 | 394090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.8 | 370160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1686.86 | 337372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1883.64 | 376728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1945 | 389000 | 1930.96 | 386192 | 0 | 15 | 4837 | 386930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1909.24 | 381848 | 102 |  | 4876 | 390050 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1975 | 395000 | 1779.52 | 355904 | 504 |  | 4912 | 392930 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.96 | 370192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1684.52 | 336904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1878.8 | 375760 | 6 |  | - | - | - | - | - | - |
| 35 | 92 | Downlink | Low | 1947.5 | 389500 | 1930.94 | 386188 | 0 | 15 | 4838 | 387130 | 2 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1960 | 392000 | 1906.72 | 381344 | 102 |  | 4870 | 389570 | 6 | 0 | 2 (7) | 218 |
|  |  |  | High | 1972.5 | 394500 | 1774.5 | 354900 | 504 |  | 4899 | 391950 | 14 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.94 | 370188 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1682 | 336400 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892.5 | 378500 | 1873.78 | 374756 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1950 | 390000 | 1930.92 | 386184 | 0 | 15 | 4836 | 386910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1904.2 | 380840 | 102 |  | 4864 | 389090 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 1970 | 394000 | 1769.48 | 353896 | 504 |  | 4886 | 390970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.92 | 370184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1679.48 | 335896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1890 | 378000 | 1868.76 | 373752 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.2-3: Test frequencies for NR operating band n2 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1935 | 387000 | 1931.04 | 386208 | 0 | 15 | - | 386568 |
|  |  |  | Mid | 1960 | 392000 | 1882.6 | 376520 | 102 |  | - | 391568 |
|  |  |  | High | 1985 | 397000 | 1618.16 | 323632 | 504 |  | - | 396568 |
|  |  | Uplink | Low | 1855 | 371000 | 1851.04 | 370208 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1513.16 | 302632 | 504 |  | - | - |
|  |  |  | High | 1905 | 381000 | 1896.72 | 379344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1937.5 | 387500 | 1931.02 | 386204 | 0 | 15 | - | 386564 |
|  |  |  | Mid | 1960 | 392000 | 1880.08 | 376016 | 102 |  | - | 391064 |
|  |  |  | High | 1982.5 | 396500 | 1613.14 | 322628 | 504 |  | - | 395564 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1851.02 | 370204 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1510.64 | 302128 | 504 |  | - | - |
|  |  |  | High | 1902.5 | 380500 | 1891.7 | 378340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1940 | 388000 | 1931.36 | 386272 | 0 | 15 | - | 386632 |
|  |  |  | Mid | 1960 | 392000 | 1877.92 | 375584 | 102 |  | - | 390632 |
|  |  |  | High | 1980 | 396000 | 1608.48 | 321696 | 504 |  | - | 394632 |
|  |  | Uplink | Low | 1860 | 372000 | 1851.36 | 370272 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1508.48 | 301696 | 504 |  | - | - |
|  |  |  | High | 1900 | 380000 | 1887.04 | 377408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1942.5 | 388500 | 1931.34 | 386268 | 0 | 15 | - | 386628 |
|  |  |  | Mid | 1960 | 392000 | 1875.4 | 375080 | 102 |  | - | 390128 |
|  |  |  | High | 1977.5 | 395500 | 1603.46 | 320692 | 504 |  | - | 393628 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1851.34 | 370268 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1505.96 | 301192 | 504 |  | - | - |
|  |  |  | High | 1897.5 | 379500 | 1882.02 | 376404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1945 | 389000 | 1931.32 | 386264 | 0 | 15 | - | 386624 |
|  |  |  | Mid | 1960 | 392000 | 1872.88 | 374576 | 102 |  | - | 389624 |
|  |  |  | High | 1975 | 395000 | 1598.44 | 319688 | 504 |  | - | 392624 |
|  |  | Uplink | Low | 1865 | 373000 | 1851.32 | 370264 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1503.44 | 300688 | 504 |  | - | - |
|  |  |  | High | 1895 | 379000 | 1877 | 375400 | 6 |  | - | - |
| 35 | 44 | Downlink | Low | 1947.5 | 389500 | 1931.66 | 386332 | 0 | 15 | - | 386692 |
|  |  |  | Mid | 1960 | 392000 | 1870.72 | 374144 | 102 |  | - | 389192 |
|  |  |  | High | 1972.5 | 394500 | 1593.78 | 318756 | 504 |  | - | 391692 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1851.66 | 370332 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1501.28 | 300256 | 504 |  | - | - |
|  |  |  | High | 1892.5 | 378500 | 1872.34 | 374468 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1950 | 390000 | 1931.64 | 386328 | 0 | 15 | - | 386688 |
|  |  |  | Mid | 1960 | 392000 | 1868.2 | 373640 | 102 |  | - | 388688 |
|  |  |  | High | 1970 | 394000 | 1588.76 | 317752 | 504 |  | - | 390688 |
|  |  | Uplink | Low | 1870 | 374000 | 1851.64 | 370328 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1498.76 | 299752 | 504 |  | - | - |
|  |  |  | High | 1890 | 378000 | 1867.32 | 373464 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.3	Reference test frequencies for NR operating band n3

Table 4.3.1.1.1.3-1: Test frequencies for NR operating band n3 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1807.5 | 361500 | 1805.25 | 361050 | 0 | 15 | 4518 | 361470 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1842.5 | 368500 | 1821.89 | 364378 | 102 |  | 4604 | 368410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1877.5 | 375500 | 1784.53 | 356906 | 504 |  | 4693 | 375410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1654.53 | 330906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1782.5 | 356500 | 1779.17 | 355834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1810 | 362000 | 1805.32 | 361064 | 0 | 15 | 4519 | 361490 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1842.5 | 368500 | 1819.46 | 363892 | 102 |  | 4598 | 367930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 1875 | 375000 | 1779.6 | 355920 | 504 |  | 4680 | 374430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1652.1 | 330420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1780 | 356000 | 1774.24 | 354848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1812.5 | 362500 | 1805.39 | 361078 | 0 | 15 | 4517 | 361450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1817.03 | 363406 | 102 |  | 4592 | 367450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 1872.5 | 374500 | 1774.67 | 354934 | 504 |  | 4667 | 373450 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1649.67 | 329934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1777.5 | 355500 | 1769.31 | 353862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1815 | 363000 | 1805.46 | 361092 | 0 | 15 | 4518 | 361470 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1814.6 | 362920 | 102 |  | 4586 | 366970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 1870 | 374000 | 1769.74 | 353948 | 504 |  | 4657 | 372530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1647.24 | 329448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1775 | 355000 | 1764.38 | 352876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1817.5 | 363500 | 1805.53 | 361106 | 0 | 15 | 4519 | 361490 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1812.17 | 362434 | 102 |  | 4580 | 366490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 1867.5 | 373500 | 1764.81 | 352962 | 504 |  | 4644 | 371550 | 4 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1644.81 | 328962 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1772.5 | 354500 | 1759.45 | 351890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1820 | 364000 | 1805.6 | 361120 | 0 | 15 | 4520 | 361690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1809.74 | 361948 | 102 |  | 4574 | 366010 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 1865 | 373000 | 1759.88 | 351976 | 504 |  | 4631 | 370570 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1642.38 | 328476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1770 | 354000 | 1754.52 | 350904 | 6 |  | - | - | - | - | - | - |
| 35 | 188 | Downlink | Low | 1822.5 | 364500 | 1805.58 | 361116 | 0 | 15 | 4521 | 361710 | 6 | 2 | 2 (4) | 6 |
|  |  |  | Mid | 1842.5 | 368500 | 1807.22 | 361444 | 102 |  | 4568 | 365530 | 6 | 1 | 0 (0) | 103 |
|  |  |  | High | 1862.5 | 372500 | 1754.86 | 350972 | 504 |  | 4621 | 369650 | 10 | 0 | 2(4) | 508 |
|  |  | Uplink | Low | 1727.5 | 345500 | 1710.58 | 342116 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1639.86 | 327972 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1767.5 | 353500 | 1749.5 | 349900 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1825 | 365000 | 1805.56 | 361112 | 0 | 15 | 4519 | 361490 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1804.7 | 360940 | 102 |  | 4562 | 365050 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1860 | 372000 | 1749.84 | 349968 | 504 |  | 4608 | 368670 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1637.34 | 327468 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1765 | 353000 | 1744.48 | 348896 | 6 |  | - |  | - | - | - | - |
| 45 | 242 | Downlink | Low | 1827.5 | 365500 | 1805.72 | 361144 | 0 | 15 | 4520 | 361690 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1802.36 | 360472 | 102 |  | 4556 | 364570 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1857.5 | 371500 | 1745 | 349000 | 504 |  | 4595 | 367690 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1732.5 | 346500 | 1710.72 | 342144 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1635 | 327000 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1762.5 | 352500 | 1739.64 | 347928 | 6 |  | - |  | - | - | - | - |
| 50 | 270 | Downlink | Low | 1830 | 366000 | 1805.7 | 361140 | 0 | 15 | 4521 | 361710 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1799.84 | 359968 | 102 |  | 4550 | 364090 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 1855 | 371000 | 1739.98 | 347996 | 504 |  | 4582 | 366530 | 10 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1735 | 347000 | 1710.7 | 342140 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1632.48 | 326496 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1760 | 352000 | 1734.62 | 346924 | 6 |  | - |  | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.3-2: Test frequencies for NR operating band n3 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1810 | 362000 | 1805.68 | 361136 | 0 | 15 | 4525 | 361970 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1801.46 | 360292 | 102 |  | 4604 | 368410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 1875 | 375000 | 1689.24 | 337848 | 504 |  | 4686 | 374910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1561.74 | 312348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1780 | 356000 | 1773.52 | 354704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1812.5 | 362500 | 1805.66 | 361132 | 0 | 15 | 4523 | 361930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1798.94 | 359788 | 102 |  | 4598 | 367930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1872.5 | 374500 | 1684.22 | 336844 | 504 |  | 4673 | 373930 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1559.22 | 311844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1777.5 | 355500 | 1768.5 | 353700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1815 | 363000 | 1805.82 | 361164 | 0 | 15 | 4524 | 361950 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1796.6 | 359320 | 102 |  | 4592 | 367450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 1870 | 374000 | 1679.38 | 335876 | 504 |  | 4663 | 373010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1556.88 | 311376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1775 | 355000 | 1763.66 | 352732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1817.5 | 363500 | 1805.8 | 361160 | 0 | 15 | 4525 | 361970 | 6 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1794.08 | 358816 | 102 |  | 4586 | 366970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 1867.5 | 373500 | 1674.36 | 334872 | 504 |  | 4650 | 372030 | 2 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1554.36 | 310872 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1772.5 | 354500 | 1758.64 | 351728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1820 | 364000 | 1805.96 | 361192 | 0 | 15 | 4523 | 361930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1791.74 | 358348 | 102 |  | 4580 | 366490 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1865 | 373000 | 1669.52 | 333904 | 504 |  | 4637 | 371050 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1552.02 | 310404 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1770 | 354000 | 1753.8 | 350760 | 6 |  | - | - | - | - | - | - |
| 35 | 92 | Downlink | Low | 1822.5 | 364500 | 1805.94 | 361188 | 0 | 15 | 4524 | 361950 | 14 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1789.22 | 357844 | 102 |  | 4574 | 366010 | 10 | 0 | 1 (6) | 216 |
|  |  |  | High | 1862.5 | 372500 | 1664.5 | 332900 | 504 |  | 4627 | 370130 | 2 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1727.5 | 345500 | 1710.94 | 342188 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1549.5 | 309900 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1767.5 | 353500 | 1748.78 | 349756 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1825 | 365000 | 1805.92 | 361184 | 0 | 15 | 4525 | 361970 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1786.7 | 357340 | 102 |  | 4568 | 365530 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1860 | 372000 | 1659.48 | 331896 | 504 |  | 4611 | 368910 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1546.98 | 309396 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1765 | 353000 | 1743.76 | 348752 | 6 |  | - |  | - | - | - | - |
| 45 | 119 | Downlink | Low | 1827.5 | 365500 | 1806.08 | 361216 | 0 | 15 | 4526 | 362170 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1842.5 | 368500 | 1784.36 | 356872 | 102 |  | 4562 | 365050 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 1857.5 | 371500 | 1654.64 | 330928 | 504 |  | 4601 | 368170 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1732.5 | 346500 | 1711.08 | 342216 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1544.64 | 308928 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1762.5 | 352500 | 1738.92 | 347784 | 6 |  | - |  | - | - | - | - |
| 50 | 133 | Downlink | Low | 1830 | 366000 | 1806.06 | 361212 | 0 | 15 | 4524 | 361950 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1781.84 | 356368 | 102 |  | 4556 | 364570 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1855 | 371000 | 1649.62 | 329924 | 504 |  | 4588 | 367010 | 2 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1735 | 347000 | 1711.06 | 342212 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1542.12 | 308424 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1760 | 352000 | 1733.9 | 346780 | 6 |  | - |  | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.3-3: Test frequencies for NR operating band n3 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1810 | 362000 | 1806.04 | 361208 | 0 | 15 | - | 361568 |
|  |  |  | Mid | 1842.5 | 368500 | 1765.1 | 353020 | 102 |  | - | 368068 |
|  |  |  | High | 1875 | 375000 | 1508.16 | 301632 | 504 |  | - | 374568 |
|  |  | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1380.66 | 276132 | 504 |  | - | - |
|  |  |  | High | 1780 | 356000 | 1771.72 | 354344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1812.5 | 362500 | 1806.02 | 361204 | 0 | 15 | - | 361564 |
|  |  |  | Mid | 1842.5 | 368500 | 1762.58 | 352516 | 102 |  | - | 367564 |
|  |  |  | High | 1872.5 | 374500 | 1503.14 | 300628 | 504 |  | - | 373564 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1378.14 | 275628 | 504 |  | - | - |
|  |  |  | High | 1777.5 | 355500 | 1766.7 | 353340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1815 | 363000 | 1806.36 | 361272 | 0 | 15 | - | 361632 |
|  |  |  | Mid | 1842.5 | 368500 | 1760.42 | 352084 | 102 |  | - | 367132 |
|  |  |  | High | 1870 | 374000 | 1498.48 | 299696 | 504 |  | - | 372632 |
|  |  | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1375.98 | 275196 | 504 |  | - | - |
|  |  |  | High | 1775 | 355000 | 1762.04 | 352408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1817.5 | 363500 | 1806.34 | 361268 | 0 | 15 | - | 361628 |
|  |  |  | Mid | 1842.5 | 368500 | 1757.9 | 351580 | 102 |  | - | 366628 |
|  |  |  | High | 1867.5 | 373500 | 1493.46 | 298692 | 504 |  | - | 371628 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1373.46 | 274692 | 504 |  | - | - |
|  |  |  | High | 1772.5 | 354500 | 1757.02 | 351404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1820 | 364000 | 1806.32 | 361264 | 0 | 15 | - | 361624 |
|  |  |  | Mid | 1842.5 | 368500 | 1755.38 | 351076 | 102 |  | - | 366124 |
|  |  |  | High | 1865 | 373000 | 1488.44 | 297688 | 504 |  | - | 370624 |
|  |  | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1370.94 | 274188 | 504 |  | - | - |
|  |  |  | High | 1770 | 354000 | 1752 | 350400 | 6 |  | - | - |
| 35 | 44 | Downlink | Low | 1822.5 | 364500 | 1806.66 | 361332 | 0 | 15 | - | 361692 |
|  |  |  | Mid | 1842.5 | 368500 | 1753.22 | 350644 | 102 |  | - | 365692 |
|  |  |  | High | 1862.5 | 372500 | 1483.78 | 296756 | 504 |  | - | 369692 |
|  |  | Uplink | Low | 1727.5 | 345500 | 1711.66 | 342332 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1368.78 | 273756 | 504 |  | - | - |
|  |  |  | High | 1767.5 | 353500 | 1747.34 | 349468 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1825 | 365000 | 1806.64 | 361328 | 0 | 15 | - | 361688 |
|  |  |  | Mid | 1842.5 | 368500 | 1750.7 | 350140 | 102 |  | - | 365188 |
|  |  |  | High | 1860 | 372000 | 1478.76 | 295752 | 504 |  | - | 368688 |
|  |  | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1366.26 | 273252 | 504 |  | - | - |
|  |  |  | High | 1765 | 353000 | 1742.32 | 348464 | 6 |  | - | - |
| 45 | 58 | Downlink | Low | 1827.5 | 365500 | 1806.62 | 361324 | 0 | 15 | - | 361684 |
|  |  |  | Mid | 1842.5 | 368500 | 1748.18 | 349636 | 102 |  | - | 364684 |
|  |  |  | High | 1857.5 | 371500 | 1473.74 | 294748 | 504 |  | - | 367684 |
|  |  | Uplink | Low | 1732.5 | 346500 | 1711.62 | 342324 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1363.74 | 272748 | 504 |  | - | - |
|  |  |  | High | 1762.5 | 352500 | 1737.3 | 347460 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 1830 | 366000 | 1806.6 | 361320 | 0 | 15 | - | 361680 |
|  |  |  | Mid | 1842.5 | 368500 | 1745.66 | 349132 | 102 |  | - | 364180 |
|  |  |  | High | 1855 | 371000 | 1468.72 | 293744 | 504 |  | - | 366680 |
|  |  | Uplink | Low | 1735 | 347000 | 1711.6 | 342320 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1361.22 | 272244 | 504 |  | - | - |
|  |  |  | High | 1760 | 352000 | 1732.28 | 346456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.4	FFS

###### 4.3.1.1.1.5	Reference test frequencies for NR operating band n5

Table 4.3.1.1.1.5-1: Test frequencies for NR operating band n5, SCS 15 kHz and ΔFRaster 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 871.5 | 174300 | 869.25 | 173850 | 0 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 860.89 | 172178 | 102 |  | 2203 | 176210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 826.5 | 165300 | 824.25 | 164850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 743.53 | 148706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 874 | 174800 | 869.32 | 173864 | 0 | 15 | 2179 | 174290 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 858.46 | 171692 | 102 |  | 2197 | 175730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 829 | 165800 | 824.32 | 164864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 741.1 | 148220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 876.5 | 175300 | 869.39 | 173878 | 0 | 15 | 2177 | 174250 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 856.03 | 171206 | 102 |  | 2191 | 175250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.39 | 164878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 738.67 | 147734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 879 | 175800 | 869.46 | 173892 | 0 | 15 | 2178 | 174270 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 853.6 | 170720 | 102 |  | 2185 | 174770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 834 | 166800 | 824.46 | 164892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 736.24 | 147248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 881.5 | 176300 | 869.53 | 173906 | 0 | 15 | 2179 | 174290 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 836.5 | 167300 | 826.96 | 165392 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. Note 4: 20/25 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.5-2: Test frequencies for NR operating band n5, SCS 30 kHz and ΔFRaster 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 874 | 174800 | 869.68 | 173936 | 0 | 30 | 2185 | 174770 | 14 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 840.46 | 168092 | 102 |  | 2203 | 176210 | 18 | 0 | 0 (0) | 204 |
|  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  | 2224 | 177890 | 6 | 0 | 3 (3) | 1014 |
|  |  | Uplink | Low | 829 | 165800 | 824.68 | 164936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 650.74 | 130148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 837.52 | 167504 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 876.5 | 175300 | 869.66 | 173932 | 0 | 30 | 2183 | 174730 | 2 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 837.94 | 167588 | 102 |  | 2197 | 175730 | 2 | 0 | 1 (1) | 206 |
|  |  |  | High | 886.5 | 177300 | 698.22 | 139644 | 504 |  | 2208 | 176670 | 6 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.66 | 164932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 648.22 | 129644 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 832.5 | 166500 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 879 | 175800 | 869.82 | 173964 | 0 | 30 | 2184 | 174750 | 22 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 835.6 | 167120 | 102 |  | 2191 | 175250 | 22 | 0 | 0 (0) | 204 |
|  |  |  | High | 884 | 176800 | 693.38 | 138676 | 504 |  | 2195 | 175690 | 2 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 834 | 166800 | 824.82 | 164964 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 645.88 | 129176 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 827.66 | 165532 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 881.5 | 176300 | 869.8 | 173960 | 0 | 30 | 2185 | 174770 | 6 | 0 | 1 (1) | 2 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 836.54 | 167300 | 827.32 | 165464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. Note 4: 20/25 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.6	FFS

###### 4.3.1.1.1.7	Reference test frequencies for NR operating band n7

Table 4.3.1.1.1.7-1: Test frequencies for NR operating band n7 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2622.5 | 524500 | 2620.25 | 524050 | 0 | 15 | 6554 | 524410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2634.39 | 526878 | 102 |  | 6636 | 530910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 2687.5 | 537500 | 2594.53 | 518906 | 504 |  | 6718 | 537410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2502.5 | 500500 | 2500.25 | 500050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2442.03 | 488406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2567.5 | 513500 | 2564.17 | 512834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2625 | 525000 | 2620.32 | 524064 | 0 | 15 | 6555 | 524430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2631.96 | 526392 | 102 |  | 6630 | 530430 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.32 | 500064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2439.6 | 487920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2559.24 | 511848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2627.5 | 525500 | 2620.39 | 524078 | 0 | 15 | 6556 | 524450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2629.53 | 525906 | 102 |  | 6624 | 529950 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 2682.5 | 536500 | 2584.67 | 516934 | 504 |  | 6692 | 535450 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.39 | 500078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2437.17 | 487434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2554.31 | 510862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2630 | 526000 | 2620.46 | 524092 | 0 | 15 | 6557 | 524650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2627.1 | 525420 | 102 |  | 6618 | 529470 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 2680 | 536000 | 2579.74 | 515948 | 504 |  | 6682 | 534530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.46 | 500092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2434.74 | 486948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2549.38 | 509876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 2632.5 | 526500 | 2620.53 | 524106 | 0 | 15 | 6558 | 524670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2624.67 | 524934 | 102 |  | 6612 | 528990 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 2677.5 | 535500 | 2574.81 | 514962 | 504 |  | 6669 | 533550 | 4 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2500.53 | 500106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2432.31 | 486462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2557.5 | 511500 | 2544.45 | 508890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 2635 | 527000 | 2620.6 | 524120 | 0 | 15 | 6559 | 524690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2622.24 | 524448 | 102 |  | 6606 | 528510 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 2675 | 535000 | 2569.88 | 513976 | 504 |  | 6656 | 532570 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2515 | 503000 | 2500.6 | 500120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2429.88 | 485976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2555 | 511000 | 2539.52 | 507904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 2640 | 528000 | 2620.56 | 524112 | 0 | 15 | 6558 | 524670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2617.2 | 523440 | 102 |  | 6594 | 527550 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 2670 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 2520 | 504000 | 2500.56 | 500112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2424.84 | 484968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2550 | 510000 | 2529.48 | 505896 | 6 |  | - | - | - | - | - | - |
| 50 | 270 | Downlink | Low | 2645 | 529000 | 2620.7 | 524140 | 0 | 15 | 6557 | 524650 | 2 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 2655 | 531000 | 2612.34 | 522468 | 102 |  | 6582 | 526590 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 2665 | 533000 | 2549.98 | 509996 | 504 |  | 6607 | 528530 | 10 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2525 | 505000 | 2500.7 | 500140 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2419.98 | 483996 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2545 | 509000 | 2519.62 | 503924 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-2: Test frequencies for NR operating band n7 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2625 | 525000 | 2620.68 | 524136 | 0 | 15 | 6561 | 524910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2613.96 | 522792 | 102 |  | 6636 | 530910 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.68 | 500136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2349.24 | 469848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2558.52 | 511704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2627.5 | 525500 | 2620.66 | 524132 | 0 | 15 | 6562 | 524930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2655 | 531000 | 2611.44 | 522288 | 102 |  | 6630 | 530430 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2682.5 | 536500 | 2494.22 | 498844 | 504 |  | 6698 | 535930 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.66 | 500132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2346.72 | 469344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2553.5 | 510700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2630 | 526000 | 2620.82 | 524164 | 0 | 15 | 6560 | 524890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2609.1 | 521820 | 102 |  | 6624 | 529950 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 2680 | 536000 | 2489.38 | 497876 | 504 |  | 6688 | 535010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.82 | 500164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2344.38 | 468876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2548.66 | 509732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 2632.5 | 526500 | 2620.8 | 524160 | 0 | 15 | 6561 | 524910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2606.58 | 521316 | 102 |  | 6618 | 529470 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 2677.5 | 535500 | 2484.36 | 496872 | 504 |  | 6675 | 534030 | 2 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2500.8 | 500160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2341.86 | 468372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2557.5 | 511500 | 2543.64 | 508728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 2635 | 527000 | 2620.96 | 524192 | 0 | 15 | 6562 | 524930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2604.24 | 520848 | 102 |  | 6612 | 528990 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2675 | 535000 | 2479.52 | 495904 | 504 |  | 6662 | 533050 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2515 | 503000 | 2500.96 | 500192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2339.52 | 467904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2555 | 511000 | 2538.8 | 507760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 2640 | 528000 | 2620.92 | 524184 | 0 | 15 | 6561 | 524910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2599.2 | 519840 | 102 |  | 6600 | 528030 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2520 | 504000 | 2500.92 | 500184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2334.48 | 466896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2550 | 510000 | 2528.76 | 505752 | 6 |  | - | - | - | - | - | - |
| 50 | 133 | Downlink | Low | 2645 | 529000 | 2621.06 | 524212 | 0 | 15 | 6563 | 525130 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2655 | 531000 | 2594.34 | 518868 | 102 |  | 6588 | 527070 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2665 | 533000 | 2459.62 | 491924 | 504 |  | 6613 | 529010 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2525 | 505000 | 2501.06 | 500212 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2329.62 | 465924 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2545 | 509000 | 2518.9 | 503780 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-3: Test frequencies for NR operating band n7 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2625 | 525000 | 2621.04 | 524208 | 0 | 15 | - | 524568 |
|  |  |  | Mid | 2655 | 531000 | 2577.6 | 515520 | 102 |  | - | 530568 |
|  |  |  | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
|  |  | Uplink | Low | 2505 | 501000 | 2501.04 | 500208 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2168.16 | 433632 | 504 |  | - | - |
|  |  |  | High | 2565 | 513000 | 2556.72 | 511344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2627.5 | 525500 | 2621.02 | 524204 | 0 | 15 | - | 524564 |
|  |  |  | Mid | 2655 | 531000 | 2575.08 | 515016 | 102 |  | - | 530064 |
|  |  |  | High | 2682.5 | 536500 | 2313.14 | 462628 | 504 |  | - | 535564 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2501.02 | 500204 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2165.64 | 433128 | 504 |  | - | - |
|  |  |  | High | 2562.5 | 512500 | 2551.7 | 510340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2630 | 526000 | 2621.36 | 524272 | 0 | 15 | - | 524632 |
|  |  |  | Mid | 2655 | 531000 | 2572.92 | 514584 | 102 |  | - | 529632 |
|  |  |  | High | 2680 | 536000 | 2308.48 | 461696 | 504 |  | - | 534632 |
|  |  | Uplink | Low | 2510 | 502000 | 2501.36 | 500272 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2163.48 | 432696 | 504 |  | - | - |
|  |  |  | High | 2560 | 512000 | 2547.04 | 509408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 2632.5 | 526500 | 2621.34 | 524268 | 0 | 15 | - | 524628 |
|  |  |  | Mid | 2655 | 531000 | 2570.4 | 514080 | 102 |  | - | 529128 |
|  |  |  | High | 2677.5 | 535500 | 2303.46 | 460692 | 504 |  | - | 533628 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2501.34 | 500268 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2160.96 | 432192 | 504 |  | - | - |
|  |  |  | High | 2557.5 | 511500 | 2542.02 | 508404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 2635 | 527000 | 2621.32 | 524264 | 0 | 15 | - | 524624 |
|  |  |  | Mid | 2655 | 531000 | 2567.88 | 513576 | 102 |  | - | 528624 |
|  |  |  | High | 2675 | 535000 | 2298.44 | 459688 | 504 |  | - | 532624 |
|  |  | Uplink | Low | 2515 | 503000 | 2501.32 | 500264 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2158.44 | 431688 | 504 |  | - | - |
|  |  |  | High | 2555 | 511000 | 2537 | 507400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 2640 | 528000 | 2621.64 | 524328 | 0 | 15 | - | 524688 |
|  |  |  | Mid | 2655 | 531000 | 2563.2 | 512640 | 102 |  | - | 527688 |
|  |  |  | High | 2670 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
|  |  | Uplink | Low | 2520 | 504000 | 2501.64 | 500328 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2153.76 | 430752 | 504 |  | - | - |
|  |  |  | High | 2550 | 510000 | 2527.32 | 505464 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 2645 | 529000 | 2621.6 | 524320 | 0 | 15 | - | 524680 |
|  |  |  | Mid | 2655 | 531000 | 2558.16 | 511632 | 102 |  | - | 526680 |
|  |  |  | High | 2665 | 533000 | 2278.72 | 455744 | 504 |  | - | 528680 |
|  |  | Uplink | Low | 2525 | 505000 | 2501.6 | 500320 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2148.72 | 429744 | 504 |  | - | - |
|  |  |  | High | 2545 | 509000 | 2517.28 | 503456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.8	Reference test frequencies for NR operating band n8

Table 4.3.1.1.1.8-1: Test frequencies for NR operating band n8 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 927.5 | 185500 | 925.25 | 185050 | 0 | 15 | 2318 | 185530 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 921.89 | 184378 | 102 |  | 2354 | 188410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 957.5 | 191500 | 864.53 | 172906 | 504 |  | 2393 | 191530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 930 | 186000 | 925.32 | 185064 | 0 | 15 | 2319 | 185550 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 919.46 | 183892 | 102 |  | 2348 | 187930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 955 | 191000 | 859.6 | 171920 | 504 |  | 2383 | 190610 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 52 | Uplink | Low | 892.5 | 178500 | 887.82 | 177564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.74 | 179348 | 6 |  | - | - | - | - | - | - |
| 10/30 | 160 | Downlink | Low | 940 | 188000 | 925.6 | 185120 | 0 | 15 | 2320 | 185570 | 6 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 909.74 | 181948 | 102 |  | 2327 | 186250 | 6 | 3 | 2 (4) | 109 |
|  |  |  | High | 945 | 189000 | 839.88 | 167976 | 504 |  | 2331 | 186510 | 10 | 0 | 0 (0) | 504 |
|  | 52 | Uplink | Low | 895 | 179000 | 890.32 | 178064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 900 | 180000 | 894.24 | 178848 | 6 |  | - | - | - | - | - | - |
| 10/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 897.5 | 179500 | 892.82 | 178564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 79 | Downlink | Low | 932.5 | 186500 | 925.39 | 185078 | 0 | 15 | 2320 | 185570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 917.03 | 183406 | 102 |  | 2342 | 187450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 952.5 | 190500 | 854.67 | 170934 | 504 |  | 2370 | 189630 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 15/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 79 | Uplink | Low | 892.5 | 178500 | 885.39 | 177078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 894.31 | 178862 | 6 |  | - | - | - | - | - | - |
| 15/30 | 160 | Downlink | Low | 940 | 188000 | 925.6 | 185120 | 0 | 15 | 2320 | 185570 | 6 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 909.74 | 181948 | 102 |  | 2327 | 186250 | 6 | 3 | 2 (4) | 109 |
|  |  |  | High | 945 | 189000 | 839.88 | 167976 | 504 |  | 2331 | 186510 | 10 | 0 | 0 (0) | 504 |
|  | 79 | Uplink | Low | 895 | 179000 | 887.89 | 177578 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 900 | 180000 | 891.81 | 178362 | 6 |  | - | - | - | - | - | - |
| 15/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 | Uplink | Low | 897.58 | 179500 | 890.398 | 178078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 106 | Downlink | Low | 935 | 187000 | 925.46 | 185092 | 0 | 15 | 2318 | 185530 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 914.6 | 182920 | 102 |  | 2336 | 186970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 950 | 190000 | 849.74 | 169948 | 504 |  | 2357 | 188650 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 937.5 | 187500 | 925.53 | 185106 | 0 | 15 | 2319 | 185550 | 4 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 912.17 | 182434 | 102 |  | 2330 | 186490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 947.5 | 189500 | 844.81 | 168962 | 504 |  | 2344 | 187490 | 8 | 0 | 0 (0) | 504 |
|  | 106 | Uplink | Low | 892.5 | 178500 | 882.96 | 176592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.88 | 178376 | 6 |  | - | - | - | - | - | - |
| 20/30 | 160 | Downlink | Low | 940 | 188000 | 925.6 | 185120 | 0 | 15 | 2320 | 185570 | 6 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 909.74 | 181948 | 102 |  | 2327 | 186250 | 6 | 3 | 2 (4) | 109 |
|  |  |  | High | 945 | 189000 | 839.88 | 167976 | 504 |  | 2331 | 186510 | 10 | 0 | 0 (0) | 504 |
|  | 106 | Uplink | Low | 895 | 179000 | 885.46 | 177092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 900 | 180000 | 889.38 | 177876 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 897.5 | 179500 | 887.96 | 177592 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10/35 MHz, 15/35 MHz and 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.8-2: Test frequencies for NR operating band n8 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 930 | 186000 | 925.68 | 185136 | 0 | 15 | 2325 | 186030 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 901.46 | 180292 | 102 |  | 2354 | 188410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 955 | 191000 | 769.24 | 153848 | 504 |  | 2389 | 191090 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 24 | Uplink | Low | 892.5 | 178500 | 888.18 | 177636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 896.02 | 179204 | 6 |  | - | - | - | - | - | - |
| 10/30 | 78 | Downlink | Low | 940 | 188000 | 925.96 | 185192 | 0 | 15 | 2326 | 186050 | 22 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 891.74 | 178348 | 102 |  | 2333 | 186730 | 10 | 4 | 0 (5) | 222 |
|  |  |  | High | 945 | 189000 | 749.52 | 149904 | 504 |  | 2337 | 186990 | 2 | 1 | 0 (5) | 1020 |
|  | 24 | Uplink | Low | 895 | 179000 | 890.68 | 178136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 900 | 180000 | 893.52 | 178704 | 6 |  | - | - | - | - | - | - |
| 10/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 | Uplink | Low | 897.56 | 179500 | 893.18 | 178636 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 38 | Downlink | Low | 932.5 | 186500 | 925.66 | 185132 | 0 | 15 | 2326 | 186050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 898.94 | 179788 | 102 |  | 2348 | 187930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 952.5 | 190500 | 764.22 | 152844 | 504 |  | 2373 | 189870 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 15/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 38 | Uplink | Low | 892.5 | 178500 | 885.66 | 177132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 893.5 | 178700 | 6 |  | - | - | - | - | - | - |
| 15/30 | 78 | Downlink | Low | 940 | 188000 | 925.96 | 185192 | 0 | 15 | 2326 | 186050 | 22 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 891.74 | 178348 | 102 |  | 2333 | 186730 | 10 | 4 | 0 (5) | 222 |
|  |  |  | High | 945 | 189000 | 749.52 | 149904 | 504 |  | 2337 | 186990 | 2 | 1 | 0 (5) | 1020 |
|  | 38 | Uplink | Low | 895 | 179000 | 888.16 | 177632 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 900 | 180000 | 891 | 178200 | 6 |  | - | - | - | - | - | - |
| 15/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38 | Uplink | Low | 897.5 | 179500 | 890.66 | 178132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/20 | 51 | Downlink | Low | 935 | 187000 | 925.82 | 185164 | 0 | 15 | 2324 | 186010 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 896.6 | 179320 | 102 |  | 2342 | 187450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 950 | 190000 | 759.38 | 151876 | 504 |  | 2360 | 188890 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 937.5 | 187500 | 925.8 | 185160 | 0 | 15 | 2325 | 186030 | 2 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 894.08 | 178816 | 102 |  | 2336 | 186970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 947.5 | 189500 | 754.36 | 150872 | 504 |  | 2350 | 187970 | 6 | 0 | 1 (6) | 1020 |
|  | 51 | Uplink | Low | 892.5 | 178500 | 883.32 | 176664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 902.5 | 180500 | 891.16 | 178232 | 6 |  | - | - | - | - | - | - |
| 20/30 | 78 | Downlink | Low | 940 | 188000 | 925.96 | 185192 | 0 | 15 | 2326 | 186050 | 22 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 891.74 | 178348 | 102 |  | 2333 | 186730 | 10 | 4 | 0 (5) | 222 |
|  |  |  | High | 945 | 189000 | 749.52 | 149904 | 504 |  | 2337 | 186990 | 2 | 1 | 0 (5) | 1020 |
|  | 51 | Uplink | Low | 895 | 179000 | 885.82 | 177164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 900 | 180000 | 888.66 | 177732 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 897.5 | 179500 | 888.32 | 177664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10/35 MHz, 15/35 MHz and 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.9 to 4.3.1.1.1.11	FFS

###### 4.3.1.1.1.12	Reference test frequencies for NR operating band n12

Table 4.3.1.1.1.12-1: Test frequencies for NR operating band n12 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 731.5 | 146300 | 729.25 | 145850 | 0 | 15 | 1828 | 146210 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 737.5 | 147500 | 716.89 | 143378 | 102 |  | 1843 | 147410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 743.5 | 148700 | 650.53 | 130106 | 504 |  | 1858 | 148610 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 701.5 | 140300 | 699.25 | 139850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 614.53 | 122906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 713.5 | 142700 | 710.17 | 142034 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 734 | 146800 | 729.32 | 145864 | 0 | 15 | 1829 | 146410 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 737.5 | 147500 | 714.46 | 142892 | 102 |  | 1837 | 146930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 741 | 148200 | 645.6 | 129120 | 504 |  | 1845 | 147630 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 704 | 140800 | 699.32 | 139864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 612.1 | 122420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 705.24 | 141048 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 736.5 | 147300 | 729.39 | 145878 | 0 | 15 | 1830 | 146430 | 4 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 737.5 | 147500 | 712.03 | 142406 | 102 |  | 1831 | 146450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 738.5 | 147700 | 640.67 | 128134 | 504 |  | 1832 | 146650 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 706.5 | 141300 | 699.39 | 139878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 609.67 | 121934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 700.31 | 140062 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.12-2: Test frequencies for NR operating band n12 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 734 | 146800 | 729.68 | 145936 | 0 | 15 | 1835 | 146890 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 737.5 | 147500 | 696.46 | 139292 | 102 |  | 1843 | 147410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 741 | 148200 | 555.24 | 111048 | 504 |  | 1851 | 148110 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 704 | 140800 | 699.68 | 139936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 521.74 | 104348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 704.52 | 140904 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 736.5 | 147300 | 729.66 | 145932 | 0 | 15 | 1833 | 146670 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 737.5 | 147500 | 693.94 | 138788 | 102 |  | 1837 | 146930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 738.5 | 147700 | 550.22 | 110044 | 504 |  | 1838 | 147130 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 706.5 | 141300 | 699.66 | 139932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 519.22 | 103844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 699.5 | 139900 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.13	Reference test frequencies for NR operating band n13

Table 4.3.1.1.1.13-1: Test frequencies for NR operating band n13 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 748.5 | 149700 | 746.25 | 149250 | 0 | 15 | 1872 | 149790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 751 | 150200 | 730.39 | 146078 | 102 |  | 1879 | 150290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 753.5 | 150700 | 660.53 | 132106 | 504 |  | 1883 | 150730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 779.5 | 155900 | 777.25 | 155450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 782 | 156400 | 689.03 | 137806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 784.5 | 156900 | 781.17 | 156234 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 751 | 150200 | 746.32 | 149264 | 0 | 15 | 1873 | 149810 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 782 | 156400 | 777.32 | 155464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcchConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.13-2: Test frequencies for NR operating band n13 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 751 | 150200 | 746.68 | 149336 | 0 | 15 | 1879 | 150290 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 782 | 156400 | 777.68 | 155536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcchConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.14	Reference test frequencies for NR operating band n14

Table 4.3.1.1.1.14-1: Test frequencies for NR operating band n14 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 763 | 152600 | 742.39 | 148478 | 102 |  | 1909 | 152690 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 765.5 | 153100 | 672.53 | 134506 | 504 |  | 1913 | 153130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 790.5 | 158100 | 788.25 | 157650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 793 | 158600 | 700.03 | 140006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 795.5 | 159100 | 792.17 | 158434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.32 | 157664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcchConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.14-2: Test frequencies for NR operating band n14 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.68 | 157736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcchConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.15 to 4.3.1.1.1.17	FFS

###### 4.3.1.1.1.18	Reference test frequencies for NR operating band n18

Table 4.3.1.1.1.18-1: Test frequencies for NR operating band n18 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 862.5 | 172500 | 860.25 | 172050 | 0 | 15 | 2157 | 172590 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 867.5 | 173500 | 846.89 | 169378 | 102 |  | 2168 | 173530 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 872.5 | 174500 | 779.53 | 155906 | 504 |  | 2182 | 174530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 817.5 | 163500 | 815.25 | 163050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 729.53 | 145906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 827.5 | 165500 | 824.17 | 164834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 865 | 173000 | 860.32 | 172064 | 0 | 15 | 2158 | 172610 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 867.5 | 173500 | 844.46 | 168892 | 102 |  | 2162 | 173050 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 870 | 174000 | 774.6 | 154920 | 504 |  | 2169 | 173550 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 820 | 164000 | 815.32 | 163064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 727.1 | 145420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 825 | 165000 | 819.24 | 163848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 867.5 | 173500 | 860.39 | 172078 | 0 | 15 | 2156 | 172570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 822.5 | 164500 | 815.39 | 163078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  | High |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.18-2: Test frequencies for NR operating band n18 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 865 | 173000 | 860.68 | 172136 | 0 | 15 | 2164 | 173090 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 867.5 | 173500 | 826.46 | 165292 | 102 |  | 2168 | 173530 | 10 | 0 | 2 (7) | 218 |
|  |  |  | High | 870 | 174000 | 684.24 | 136848 | 504 |  | 2175 | 174030 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 820 | 164000 | 815.68 | 163136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 636.74 | 127348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 825 | 165000 | 818.52 | 163704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 867.5 | 173500 | 860.66 | 172132 | 0 | 15 | 2162 | 173050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 822.5 | 164500 | 815.66 | 163132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  | High |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.19	FFS

###### 4.3.1.1.1.20	Reference test frequencies for NR operating band n20

Table 4.3.1.1.1.20-1: Test frequencies for NR operating band n20 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 793.5 | 158700 | 791.25 | 158250 | 0 | 15 | 1983 | 158670 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 806 | 161200 | 785.39 | 157078 | 102 |  | 2015 | 161290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 818.5 | 163700 | 725.53 | 145106 | 504 |  | 2047 | 163730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 796 | 159200 | 791.32 | 158264 | 0 | 15 | 1984 | 158690 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 806 | 161200 | 782.96 | 156592 | 102 |  | 2009 | 160810 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 816 | 163200 | 720.6 | 144120 | 504 |  | 2034 | 162750 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 798.5 | 159700 | 791.39 | 158278 | 0 | 15 | 1982 | 158650 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 806 | 161200 | 780.53 | 156106 | 102 |  | 2003 | 160330 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 813.5 | 162700 | 715.67 | 143134 | 504 |  | 2021 | 161770 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 801 | 160200 | 791.46 | 158292 | 0 | 15 | 1983 | 158670 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 806 | 161200 | 778.1 | 155620 | 102 |  | 1997 | 159850 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 811 | 162200 | 710.74 | 142148 | 504 |  | 2011 | 160850 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.20-2: Test frequencies for NR operating band n20 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 796 | 159200 | 791.68 | 158336 | 0 | 15 | 1990 | 159170 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 806 | 161200 | 764.96 | 152992 | 102 |  | 2015 | 161290 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 816 | 163200 | 630.24 | 126048 | 504 |  | 2040 | 163230 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 798.5 | 159700 | 791.66 | 158332 | 0 | 15 | 1988 | 159130 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 806 | 161200 | 762.44 | 152488 | 102 |  | 2006 | 160570 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 813.5 | 162700 | 625.22 | 125044 | 504 |  | 2027 | 162250 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 801 | 160200 | 791.82 | 158364 | 0 | 15 | 1989 | 159150 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 806 | 161200 | 760.1 | 152020 | 102 |  | 2000 | 160090 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 811 | 162200 | 620.38 | 124076 | 504 |  | 2014 | 161090 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.21 to 4.3.1.1.1.23	FFS

###### 4.3.1.1.1.24	Reference test frequencies for NR operating band n24

Table 4.3.1.1.1.24-1a: Test frequencies for NR operating band n24,SCS 15 kHz and ΔFRaster 15 kHz

| UL/DL CBW [MHz] |  | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 |  | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1531 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  |  | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1632.5 | 326500 | 1539.53 | 307906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |  | - | - | - | - | - | - |
| 10/5) |  | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1531 | 306200 | 1510.39 | 302078 | 102 |  | 3829 | 306290 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  | 52 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low Mid | 1531.0 | 306200 | 1526.32 | 305264 | 0 | 15 | 3823 | 305810 | 2 | 1 | 2 (4) | 5 |  |
|  |  |  | High | 1531.0 | 306200 | 1435.6 | 287120 | 504 |  | 3823 | 305810 | 2 | 1 | 2 (4) | 509 |  |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |  |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |  | - | - | - | - | - | - |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. Note 4: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-2a: Test frequencies for NR operating band n24, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low Mid | 1531.0 | 306200 | 1526.68 | 305336 | 0 | 30 | 3829 | 306290 | 6 | 0 | 3 (3) | 6 |
|  |  |  | High | 1531.0 | 306200 | 1345.24 | 269048 | 504 |  | 3829 | 306290 | 6 | 0 | 3 (3) | 1014 |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1628.18 | 325636 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1645.02 | 329004 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-3a: Test frequencies for NR operating band n24, SCS 60 kHz and ΔFRaster 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low Mid | 1531.0 | 306200 | 1527.04 | 305408 | 0 | 15 | - | 305768 |
|  |  |  | High | 1531.0 | 306200 | 1164.16 | 232832 | 504 |  | - | 305768 |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1628.54 | 325708 | 0 | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |  | - | - |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: Void. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.25	Reference test frequencies for NR operating band n25

Table 4.3.1.1.1.25-1: Test frequencies for NR operating band n25 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1932.5 | 386500 | 1930.25 | 386050 | 0 | 15 | 4829 | 386410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1941.89 | 388378 | 102 |  | 4904 | 392410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1992.5 | 398500 | 1899.53 | 379906 | 504 |  | 4979 | 398410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1852.5 | 370500 | 1850.25 | 370050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1789.53 | 357906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1912.5 | 382500 | 1909.17 | 381834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 1935 | 387000 | 1930.32 | 386064 | 0 | 15 | 4830 | 386430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1939.46 | 387892 | 102 |  | 4898 | 391930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 1990 | 398000 | 1894.6 | 378920 | 504 |  | 4969 | 397490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.32 | 370064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1787.1 | 357420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1904.24 | 380848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 1937.5 | 387500 | 1930.39 | 386078 | 0 | 15 | 4831 | 386450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1937.03 | 387406 | 102 |  | 4892 | 391450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 1987.5 | 397500 | 1889.67 | 377934 | 504 |  | 4956 | 396510 | 0 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.39 | 370078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1784.67 | 356934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1899.31 | 379862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 1940 | 388000 | 1930.46 | 386092 | 0 | 15 | 4832 | 386650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1934.6 | 386920 | 102 |  | 4886 | 390970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 1985 | 397000 | 1884.74 | 376948 | 504 |  | 4943 | 395530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.46 | 370092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1782.24 | 356448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1894.38 | 378876 | 6 |  | - | - | - | - | - | - |
| 25/25 | 133 | Downlink | Low | 1942.5 | 388500 | 1930.53 | 386106 | 0 | 15 | 4833 | 386670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1932.17 | 386434 | 102 |  | 4880 | 390490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 1982.5 | 396500 | 1879.81 | 375962 | 504 |  | 4933 | 394610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.53 | 370106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1779.81 | 355962 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1889.45 | 377890 | 6 |  | - | - | - | - | - | - |
| 30/30 | 160 | Downlink | Low | 1945 | 389000 | 1930.6 | 386120 | 0 | 15 | 4834 | 386690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1929.74 | 385948 | 102 |  | 4874 | 390010 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 1980 | 396000 | 1874.88 | 374976 | 504 |  | 4920 | 393630 | 2 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.6 | 370120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1777.38 | 355476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1884.52 | 376904 | 6 |  | - | - | - | - | - | - |
| 35/35 | 188 | Downlink | Low | 1947.5 | 389500 | 1930.58 | 386116 | 0 | 15 | 4832 | 386650 | 10 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 1962.5 | 392500 | 1927.22 | 385444 | 102 |  | 4868 | 389530 | 6 | 1 | 0 (0) | 103 |
|  |  |  | High | 1977.5 | 395500 | 1869.86 | 373972 | 504 |  | 4907 | 392650 | 10 | 0 | 0 (0) | 508 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.58 | 370116 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1774.86 | 354972 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1879.5 | 375900 | 6 |  | - | - | - | - | - | - |
| 40/40 | 216 | Downlink | Low | 1950 | 390000 | 1930.56 | 386112 | 0 | 15 | 4833 | 386670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1924.7 | 384940 | 102 |  | 4862 | 389050 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1975 | 395000 | 1864.84 | 372968 | 504 |  | 4894 | 391490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.56 | 370112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1874.48 | 374896 | 6 |  | - | - | - | - | - | - |
| 40/45 | 242 | Downlink | Low | 1952.5 | 390500 | 1930.72 | 386144 | 0 | 15 | 4834 | 386690 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1922.36 | 384472 | 102 |  | 4856 | 388570 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1972.5 | 394500 | 1860 | 372000 | 504 |  | 4881 | 390510 | 2 | 0 | 0 (0) | 504 |
|  | 216 | Uplink | Low | 1872.5 | 374500 | 1853.06 | 370612 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892,5 | 378500 | 1871.98 | 374396 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-2: Test frequencies for NR operating band n25 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 1935 | 387000 | 1930.68 | 386136 | 0 | 15 | 4836 | 386910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1921.46 | 384292 | 102 |  | 4904 | 392410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 1990 | 398000 | 1804.24 | 360848 | 504 |  | 4975 | 397970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.68 | 370136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1696.74 | 339348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1903.52 | 380704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 1937.5 | 387500 | 1930.66 | 386132 | 0 | 15 | 4837 | 386930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1962.5 | 392500 | 1918.94 | 383788 | 102 |  | 4898 | 391930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1987.5 | 397500 | 1799.22 | 359844 | 504 |  | 4962 | 396990 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.66 | 370132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1694.22 | 338844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1898.5 | 379700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 1940 | 388000 | 1930.82 | 386164 | 0 | 15 | 4835 | 386890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1916.6 | 383320 | 102 |  | 4892 | 391450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 1985 | 397000 | 1794.38 | 358876 | 504 |  | 4949 | 396010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.82 | 370164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1691.88 | 338376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1893.66 | 378732 | 6 |  | - | - | - | - | - | - |
| 25/25 | 65 | Downlink | Low | 1942.5 | 388500 | 1930.8 | 386160 | 0 | 15 | 4836 | 386910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1914.08 | 382816 | 102 |  | 4886 | 390970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 1982.5 | 396500 | 1789.36 | 357872 | 504 |  | 4939 | 395090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.8 | 370160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1689.36 | 337872 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1888.64 | 377728 | 6 |  | - | - | - | - | - | - |
| 30/30 | 78 | Downlink | Low | 1945 | 389000 | 1930.96 | 386192 | 0 | 15 | 4837 | 386930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1911.74 | 382348 | 102 |  | 4880 | 390490 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1980 | 396000 | 1784.52 | 356904 | 504 |  | 4926 | 394110 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.96 | 370192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1687.02 | 337404 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1883.8 | 376760 | 6 |  | - | - | - | - | - | - |
| 35/35 | 92 | Downlink | Low | 1947.5 | 389500 | 1930.94 | 386188 | 0 | 15 | 4838 | 387130 | 2 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1962.5 | 392500 | 1909.22 | 381844 | 102 |  | 4874 | 390010 | 10 | 0 | 1 (6) | 216 |
|  |  |  | High | 1977.5 | 395500 | 1779.5 | 355900 | 504 |  | 4913 | 393130 | 2 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1850.94 | 370188 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1684.5 | 336900 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1878.78 | 375756 | 6 |  | - | - | - | - | - | - |
| 40/40 | 106 | Downlink | Low | 1950 | 390000 | 1930.92 | 386184 | 0 | 15 | 4836 | 386910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1906.7 | 381340 | 102 |  | 4868 | 389530 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1975 | 395000 | 1774.48 | 354896 | 504 |  | 4900 | 391970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.92 | 370184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1681.98 | 336396 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1873.76 | 374752 | 6 |  | - | - | - | - | - | - |
| 40/45 | 119 | Downlink | Low | 1952.5 | 390500 | 1931.08 | 386216 | 0 | 15 | 4840 | 387170 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 1962.5 | 392500 | 1904.36 | 380872 | 102 |  | 4862 | 389050 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 1972.5 | 394500 | 1769.64 | 353928 | 504 | - | 4887 | 390990 | 18 | 0 | 0 (5) | 1018 |
|  | 106 | Uplink | Low | 1872.5 | 374500 | 1853.42 | 370684 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1681.98 | 336396 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1892.5 | 378500 | 1871.26 | 374252 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-3: Test frequencies for NR operating band n25 and SCS 60 kHz without CORESET#0

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 11 | Downlink | Low | 1935 | 387000 | 1931.04 | 386208 | 0 | 15 | - | 386568 |
|  |  |  | Mid | 1962.5 | 392500 | 1885.1 | 377020 | 102 |  | - | 392068 |
|  |  |  | High | 1990 | 398000 | 1623.16 | 324632 | 504 |  | - | 397568 |
|  |  | Uplink | Low | 1855 | 371000 | 1851.04 | 370208 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1515.66 | 303132 | 504 |  | - | - |
|  |  |  | High | 1910 | 382000 | 1901.72 | 380344 | 6 |  | - | - |
| 15/15 | 18 | Downlink | Low | 1937.5 | 387500 | 1931.02 | 386204 | 0 | 15 | - | 386564 |
|  |  |  | Mid | 1962.5 | 392500 | 1882.58 | 376516 | 102 |  | - | 391564 |
|  |  |  | High | 1987.5 | 397500 | 1618.14 | 323628 | 504 |  | - | 396564 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1851.02 | 370204 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1513.14 | 302628 | 504 |  | - | - |
|  |  |  | High | 1907.5 | 381500 | 1896.7 | 379340 | 6 |  | - | - |
| 20/20 | 24 | Downlink | Low | 1940 | 388000 | 1931.36 | 386272 | 0 | 15 | - | 386632 |
|  |  |  | Mid | 1962.5 | 392500 | 1880.42 | 376084 | 102 |  | - | 391132 |
|  |  |  | High | 1985 | 397000 | 1613.48 | 322696 | 504 |  | - | 395632 |
|  |  | Uplink | Low | 1860 | 372000 | 1851.36 | 370272 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1510.98 | 302196 | 504 |  | - | - |
|  |  |  | High | 1905 | 381000 | 1892.04 | 378408 | 6 |  | - | - |
| 25/25 | 31 | Downlink | Low | 1942.5 | 388500 | 1931.34 | 386268 | 0 | 15 | - | 386628 |
|  |  |  | Mid | 1962.5 | 392500 | 1877.9 | 375580 | 102 |  | - | 390628 |
|  |  |  | High | 1982.5 | 396500 | 1608.46 | 321692 | 504 |  | - | 394628 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1851.34 | 370268 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1508.46 | 301692 | 504 |  | - | - |
|  |  |  | High | 1902.5 | 380500 | 1887.02 | 377404 | 6 |  | - | - |
| 30/30 | 38 | Downlink | Low | 1945 | 389000 | 1931.32 | 386264 | 0 | 15 | - | 386624 |
|  |  |  | Mid | 1962.5 | 392500 | 1875.38 | 375076 | 102 |  | - | 390124 |
|  |  |  | High | 1980 | 396000 | 1603.44 | 320688 | 504 |  | - | 393624 |
|  |  | Uplink | Low | 1865 | 373000 | 1851.32 | 370264 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1505.94 | 301188 | 504 |  | - | - |
|  |  |  | High | 1900 | 380000 | 1882 | 376400 | 6 |  | - | - |
| 35/35 | 44 | Downlink | Low | 1947.5 | 389500 | 1931.66 | 386332 | 0 | 15 | - | 386692 |
|  |  |  | Mid | 1962.5 | 392500 | 1873.22 | 374644 | 102 |  | - | 389692 |
|  |  |  | High | 1977.5 | 395500 | 1598.78 | 319756 | 504 |  | - | 392692 |
|  |  | Uplink | Low | 1867.5 | 373500 | 1851.66 | 370332 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1503.78 | 300756 | 504 |  | - | - |
|  |  |  | High | 1897.5 | 379500 | 1877.34 | 375468 | 6 |  | - | - |
| 40/40 | 51 | Downlink | Low | 1950 | 390000 | 1931.64 | 386328 | 0 | 15 | - | 386688 |
|  |  |  | Mid | 1962.5 | 392500 | 1870.7 | 374140 | 102 |  | - | 389188 |
|  |  |  | High | 1975 | 395000 | 1593.76 | 318752 | 504 |  | - | 391688 |
|  |  | Uplink | Low | 1870 | 374000 | 1851.64 | 370328 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1501.26 | 300252 | 504 |  | - | - |
|  |  |  | High | 1895 | 379000 | 1872.32 | 374464 | 6 |  | - | - |
| 40/45 | 58 | Downlink | Low | 1952.5 | 390500 | 1931.62 | 386324 | 0 | 15 | - | 386684 |
|  |  |  | Mid | 1962.5 | 392500 | 1868.18 | 373636 | 102 |  | - | 388684 |
|  |  |  | High | 1972.5 | 394500 | 1588.74 | 317748 | 504 |  | - | 390684 |
|  | 51 | Uplink | Low | 1872.5 | 374500 | 1854.14 | 370828 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1501.26 | 300252 | 504 |  | - | - |
|  |  |  | High | 1892.5 | 378500 | 1869.82 | 373964 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.26	Reference test frequencies for NR operating band n26

Table 4.3.1.1.1.26-1: Test frequencies for NR operating band n26 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 860.5 | 172100 | 859.15 | 171830 | 0 | 15 | 30938 | 172070 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 876.5 | 175300 | 856.79 | 171358 | 102 |  | 31017 | 175270 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 892.5 | 178500 | 800.43 | 160086 | 504 |  | 31099 | 178530 | 4 | 0 | 3 (2) | 506 |
|  |  | Uplink | Low | 815.5 | 163100 | 814.15 | 162830 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 739.43 | 147886 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 847.5 | 169500 | 845.07 | 169014 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 861.5 | 172300 | 859.25 | 171850 | 0 | 15 | 2153 | 172330 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 855.89 | 171178 | 102 |  | 2189 | 175210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 816.5 | 163300 | 814.25 | 162850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 738.53 | 147706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 864 | 172800 | 859.32 | 171864 | 0 | 15 | 2154 | 172350 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 853.46 | 170692 | 102 |  | 2183 | 174730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 819 | 163800 | 814.32 | 162864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 736.1 | 147220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 866.5 | 173300 | 859.39 | 171878 | 0 | 15 | 2155 | 172370 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 851.03 | 170206 | 102 |  | 2177 | 174250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 821.5 | 164300 | 814.39 | 162878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 733.67 | 146734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 869 | 173800 | 859.46 | 171892 | 0 | 15 | 2153 | 172330 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 876.5 | 175300 | 848.6 | 169720 | 102 |  | 2171 | 173770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 824 | 164800 | 814.46 | 162892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 731.24 | 146248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.26-2: Test frequencies for NR operating band n26 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 864 | 172800 | 859.68 | 171936 | 0 | 15 | 2160 | 172830 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 876.5 | 175300 | 835.46 | 167092 | 102 |  | 2189 | 175210 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  | 2224 | 177890 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 819 | 163800 | 814.68 | 162936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 645.74 | 129148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 837.52 | 167504 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 866.5 | 173300 | 859.66 | 171932 | 0 | 15 | 2161 | 172850 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 876.5 | 175300 | 832.94 | 166588 | 102 |  | 2183 | 174730 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 886.5 | 177300 | 698.22 | 139644 | 504 |  | 2208 | 176670 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 821.5 | 164300 | 814.66 | 162932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 643.22 | 128644 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 832.5 | 166500 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 869 | 173800 | 859.82 | 171964 | 0 | 15 | 2159 | 172810 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 876.5 | 175300 | 830.6 | 166120 | 102 |  | 2177 | 174250 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 884 | 176800 | 693.38 | 138676 | 504 |  | 2195 | 175690 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 824 | 164800 | 814.82 | 162964 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 640.88 | 128176 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 827.66 | 165532 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.27	FFS

###### 4.3.1.1.1.28	Reference test frequencies for NR operating band n28

Table 4.3.1.1.1.28-1: Test frequencies for NR operating band n28 and SCS 15 kHz when not used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 759.5 | 151900 | 758.15 | 151630 | 0 | 15 | 30432 | 151870 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 780.5 | 156100 | 760.79 | 152158 | 102 |  | 30537 | 156070 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 801.5 | 160300 | 709.43 | 141886 | 504 |  | 30642 | 160270 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 704.5 | 140900 | 703.15 | 140630 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 633.43 | 126686 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 746.5 | 149300 | 744.07 | 148814 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 759.89 | 151978 | 102 |  | 1949 | 156010 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 800.5 | 160100 | 707.53 | 141506 | 504 |  | 2002 | 160130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 757.46 | 151492 | 102 |  | 1943 | 155530 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 798 | 159600 | 702.6 | 140520 | 504 |  | 1989 | 159150 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 765.5 | 153100 | 758.39 | 151678 | 0 | 15 | 1901 | 152170 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 780.5 | 156100 | 755.03 | 151006 | 102 |  | 1937 | 155050 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 795.5 | 159100 | 697.67 | 139534 | 504 |  | 1976 | 158170 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 768 | 153600 | 758.46 | 151692 | 0 | 15 | 1902 | 152190 | 10 | 1 | 1 (2) | 3 |
| (Note 2) |  |  | Mid | 783 | 156600 | 755.1 | 151020 | 102 |  | 1938 | 155070 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 793 | 158600 | 692.74 | 138548 | 504 |  | 1966 | 157250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 627.74 | 125548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low Mid | 773 | 154600 | 758.6 | 151720 | 0 | 15 | 1901 | 152170 | 6 | 0 | 1 (2) | 2 |
| (Note 4) |  |  | High | 788 | 157600 | 682.88 | 136576 | 504 |  | 1940 | 155290 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low Mid | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-1A: Test frequencies for NR operating band n28 and SCS 15 kHz when used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] (Note 5) | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low Mid | 785.5 | 157100 | 783.25 | 156650 | 0 | 15 | 1963 | 157010 | 0 | 0 | 0 (0) | 0 |
|  |  |  | High | 790.5 | 158100 | 697.53 | 139506 | 504 |  | 1977 | 158190 | 0 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low Mid | 730.5 | 146100 | 728.25 | 145650 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 735.5 | 147100 | 732.17 | 146434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low Mid High | 788 | 157600 | 764.96 | 152992 | 102 | 15 | 1964 | 157210 | 2 | 1 | 2 (4) | 107 |
|  |  | Uplink | Low Mid High | 733 | 146600 | 637.6 | 127520 | 504 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. Note 5: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. (Reference: TS 38.101-3 [9], Table 5.5B.4.1-1 for DC_21A_n28 and Note 17) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-2: Test frequencies for NR operating band n28 and SCS 30 kHz when not used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 780.5 | 156100 | 739.46 | 147892 | 102 |  | 1949 | 156010 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 798 | 159600 | 612.24 | 122448 | 504 |  | 1995 | 159630 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 539.74 | 107948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 765.5 | 153100 | 758.66 | 151732 | 0 | 15 | 1907 | 152650 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 780.5 | 156100 | 736.94 | 147388 | 102 |  | 1943 | 155530 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 795.5 | 159100 | 607.22 | 121444 | 504 |  | 1982 | 158650 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 768 | 153600 | 758.82 | 151764 | 0 | 15 | 1908 | 152670 | 14 | 0 | 2 (7) | 14 |
| (Note 2) |  |  | Mid | 783 | 156600 | 737.1 | 147420 | 102 |  | 1944 | 155550 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 793 | 158600 | 602.38 | 120476 | 504 |  | 1969 | 157490 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 537.38 | 107476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low Mid | 773 | 154600 | 758.96 | 151792 | 0 | 15 | 1907 | 152650 | 22 | 0 | 1 (6) | 12 |
| (Note 4) |  |  | High | 788 | 157600 | 592.52 | 118504 | 504 |  | 1943 | 155530 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low Mid | 718 | 143600 | 703.96 | 140792 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-2A: Test frequencies for NR operating band n28 and SCS 30 kHz when used in EN-DC configuration DC_21A_n28A or in any EN-DC configuration of a higher order EN-DC configuration including DC_21A_n28A

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 5 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low Mid High | 788 | 157600 | 746.96 | 149392 | 102 | 15 | 1970 | 157690 | 6 | 0 | 3 (8) | 220 |
|  |  | Uplink | Low Mid High | 733 | 146600 | 547.24 | 109448 | 504 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. Note 5: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. (Reference: TS 38.101-3 [9], Table 5.5B.4.1-1 for DC_21A_n28 and Note 17) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.29	Reference test frequencies for NR operating band n29 (SDL)

Table 4.3.1.1.1.29-1: Test frequencies for NR operating band n29 and SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 719.5 | 143900 | 717.25 | 143450 | 0 | 15 | - | 143810 |
|  |  |  | Mid | 722.5 | 144500 | 701.89 | 140378 | 102 |  | - | 144410 |
|  |  |  | High | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | 145010 |
| 10 | 52 | Downlink | Low | 722 | 144400 | 717.32 | 143464 | 0 | 15 | - | 143824 |
|  |  |  | Mid | 722.5 | 144500 | 699.46 | 139892 | 102 |  | - | 143924 |
|  |  |  | High | 723 | 144600 | 627.6 | 125520 | 504 |  | - | 144024 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.29-2: Test frequencies for NR operating band n29 and SCS 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 722 | 144400 | 717.68 | 143536 | 0 | 15 | - | 143896 |
|  |  |  | Mid | 722.5 | 144500 | 681.46 | 136292 | 102 |  | - | 143996 |
|  |  |  | High | 723 | 144600 | 537.24 | 107448 | 504 |  | - | 144096 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.30	Reference test frequencies for NR operating band n30

Table 4.3.1.1.1.30-1: Test frequencies for NR operating band n30 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2352.5 | 470500 | 2350.25 | 470050 | 0 | 15 | 5879 | 470410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2355 | 471000 | 2334.39 | 466878 | 102 |  | 5886 | 470910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 2357.5 | 471500 | 2264.53 | 452906 | 504 |  | 5893 | 471410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2307.5 | 461500 | 2305.25 | 461050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2310 | 462000 | 2217.03 | 443406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2312.5 | 462500 | 2309.17 | 461834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2355 | 471000 | 2350.32 | 470064 | 0 | 15 | 5880 | 470430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 2310 | 462000 | 2305.32 | 461064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.30-2: Test frequencies for NR operating band n30 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2355 | 471000 | 2350.68 | 470136 | 0 | 15 | 5886 | 470910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 2310 | 462000 | 2305.68 | 461136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.31	Reference test frequencies for NR operating band n31

Table 4.3.1.1.1.31-1: Test frequencies for NR operating band n31 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 464 | 92800 | 462.65 | 92530 | 0 | 15 | 28956 | 92830 | 4 | 0 | 3 (2) | 2 |
|  |  |  | Mid | 465 | 93000 | 445.29 | 89058 | 102 |  | 28960 | 92970 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 466 | 93200 | 373.93 | 74786 | 504 |  | 28967 | 93230 | 4 | 0 | 3 (2) | 506 |
|  |  | Uplink | Low | 454 | 90800 | 452.65 | 90530 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 455 | 91000 | 362.93 | 72586 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 456 | 91200 | 453.57 | 90714 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 465 | 93000 | 462.75 | 92550 | 0 | 15 | 1162 | 92930 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 455 | 91000 | 452.75 | 90550 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.32 to 4.3.1.1.1.33	FFS

###### 4.3.1.1.1.34	Reference test frequencies for NR operating band n34

Table 4.3.1.1.1.34-1: Test frequencies for NR operating band n34 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2012.5 | 402500 | 2010.25 | 402050 | 0 | 15 | 5032 | 402530 | 4 | 1 | 1 (2) | 3 |
|  |  | & | Mid | 2017.5 | 403500 | 1996.89 | 399378 | 102 |  | 5043 | 403470 | 8 | 1 | 0 (0) | 103 |
|  |  | Uplink | High | 2022.5 | 404500 | 1929.53 | 385906 | 504 |  | 5054 | 404410 | 0 | 0 | 0 (0) | 504 |
| 10 | 52 | Downlink | Low | 2015 | 403000 | 2010.32 | 402064 | 0 | 30 | 5036 | 402970 | 2 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2017.5 | 403500 | 1994.46 | 398892 | 102 |  | 5043 | 403470 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2020 | 404000 | 1924.6 | 384920 | 504 |  | 5050 | 403970 | 2 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 2017.5 | 403500 | 2010.39 | 402078 | 0 | 30 | 5037 | 402990 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.34-2: Test frequencies for NR operating band n34 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2015 | 403000 | 2010.68 | 402136 | 0 | 30 | 5036 | 402970 | 14 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2017.5 | 403500 | 1976.46 | 395292 | 102 |  | 5043 | 403470 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2020 | 404000 | 1834.24 | 366848 | 504 |  | 5050 | 403970 | 14 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 2017.5 | 403500 | 2010.66 | 402132 | 0 | 30 | 5037 | 402990 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.34-3: Test frequencies for NR operating band n34 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2015 | 403000 | 2011.04 | 402208 | 0 | 15 | - | 402568 |
|  |  | & | Mid | 2017.5 | 403500 | 1940.1 | 388020 | 102 |  | - | 403068 |
|  |  | Uplink | High | 2020 | 404000 | 1653.16 | 330632 | 504 |  | - | 403568 |
| 15 | 18 | Downlink | Low | 2017.5 | 403500 | 2011.02 | 402204 | 0 | 15 | - | 402564 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.35 to 4.3.1.1.1.37	FFS

###### 4.3.1.1.1.38	Reference test frequencies for NR operating band n38

Table 4.3.1.1.1.38-1: Test frequencies for NR operating band n38 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2572.5 | 514500 | 2570.25 | 514050 | 0 | 15 | 6432 | 514590 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2592.5 | 518500 | 2571.89 | 514378 | 102 |  | 6479 | 518410 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2617.5 | 523500 | 2524.53 | 504906 | 504 |  | 6543 | 523470 | 8 | 1 | 0 (0) | 505 |
| 10 | 52 | Downlink | Low | 2575 | 515000 | 2570.32 | 514064 | 0 | 30 | 6439 | 515090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 2595 | 519000 | 2571.96 | 514392 | 102 |  | 6486 | 518910 | 6 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2615 | 523000 | 2519.6 | 503920 | 504 |  | 6536 | 522970 | 2 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 2577.5 | 515500 | 2570.39 | 514078 | 0 | 30 | 6437 | 515050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2569.53 | 513906 | 102 |  | 6480 | 518430 | 8 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2612.5 | 522500 | 2514.67 | 502934 | 504 |  | 6526 | 522050 | 0 | 1 | 1 (6) | 511 |
| 20 | 106 | Downlink | Low | 2580 | 516000 | 2570.46 | 514092 | 0 | 30 | 6438 | 515070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2567.1 | 513420 | 102 |  | 6474 | 517950 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2610 | 522000 | 2509.74 | 501948 | 504 |  | 6513 | 521070 | 2 | 1 | 1 (6) | 511 |
| 25 | 133 | Downlink | Low | 2582.5 | 516500 | 2570.53 | 514106 | 0 | 30 | 6439 | 515090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2564.67 | 512934 | 102 |  | 6468 | 517470 | 0 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 2607.5 | 521500 | 2504.81 | 500962 | 504 |  | 6500 | 520090 | 4 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 2585 | 517000 | 2570.6 | 514120 | 0 | 30 | 6437 | 515050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2595 | 519000 | 2562.24 | 512448 | 102 |  | 6462 | 516990 | 2 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 2605 | 521000 | 2499.88 | 499976 | 504 |  | 6487 | 518930 | 6 | 0 | 0 (2) | 506 |
| 40 | 216 | Downlink | Low | 2590 | 518000 | 2570.56 | 514112 | 0 | 30 | 6439 | 515090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2557.2 | 511440 | 102 |  | 6450 | 516030 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2600 | 520000 | 2489.84 | 497968 | 504 |  | 6461 | 516970 | 10 | 1 | 0 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 for SSB_SCS = 15kHz and Table 13-3 for SSB_SCS=30kHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.38-2: Test frequencies for NR operating band n38 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2575 | 515000 | 2570.68 | 514136 | 0 | 30 | 6439 | 515090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2595 | 519000 | 2553.96 | 510792 | 102 |  | 6486 | 518910 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2615 | 523000 | 2429.24 | 485848 | 504 |  | 6536 | 522970 | 14 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 2577.5 | 515500 | 2570.66 | 514132 | 0 | 30 | 6437 | 515050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2551.44 | 510288 | 102 |  | 6480 | 518430 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2612.5 | 522500 | 2424.22 | 484844 | 504 |  | 6526 | 522050 | 18 | 0 | 2 (2) | 1012 |
| 20 | 51 | Downlink | Low | 2580 | 516000 | 2570.82 | 514164 | 0 | 30 | 6438 | 515070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2549.1 | 509820 | 102 |  | 6474 | 517950 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2610 | 522000 | 2419.38 | 483876 | 504 |  | 6513 | 521070 | 14 | 0 | 2 (2) | 1012 |
| 25 | 65 | Downlink | Low | 2582.5 | 516500 | 2570.8 | 514160 | 0 | 30 | 6439 | 515090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2546.58 | 509316 | 102 |  | 6468 | 517470 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2607.5 | 521500 | 2414.36 | 482872 | 504 |  | 6500 | 520090 | 22 | 0 | 2 (2) | 1012 |
| 30 | 78 | Downlink | Low | 2585 | 517000 | 2570.96 | 514192 | 0 | 30 | 6437 | 515050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2595 | 519000 | 2544.24 | 508848 | 102 |  | 6462 | 516990 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2605 | 521000 | 2409.52 | 481904 | 504 |  | 6487 | 518930 | 6 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 2590 | 518000 | 2570.92 | 514184 | 0 | 30 | 6439 | 515090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2539.2 | 507840 | 102 |  | 6450 | 516030 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2600 | 520000 | 2399.48 | 479896 | 504 |  | 6461 | 516970 | 22 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.38-3: Test frequencies for NR operating band n38 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2575 | 515000 | 2571.04 | 514208 | 0 | 30 | - | 514928 |
|  |  | & | Mid | 2595 | 519000 | 2517.6 | 503520 | 102 |  | - | 518928 |
|  |  | Uplink | High | 2615 | 523000 | 2248.16 | 449632 | 504 |  | - | 522928 |
| 15 | 18 | Downlink | Low | 2577.5 | 515500 | 2571.02 | 514204 | 0 | 30 | - | 514924 |
|  |  | & | Mid | 2595 | 519000 | 2515.08 | 503016 | 102 |  | - | 518424 |
|  |  | Uplink | High | 2612.5 | 522500 | 2243.14 | 448628 | 504 |  | - | 521924 |
| 20 | 24 | Downlink | Low | 2580 | 516000 | 2571.36 | 514272 | 0 | 30 | - | 514992 |
|  |  | & | Mid | 2595 | 519000 | 2512.92 | 502584 | 102 |  | - | 517992 |
|  |  | Uplink | High | 2610 | 522000 | 2238.48 | 447696 | 504 |  | - | 520992 |
| 25 | 31 | Downlink | Low | 2582.5 | 516500 | 2571.34 | 514268 | 0 | 30 | - | 514988 |
|  |  | & | Mid | 2595 | 519000 | 2510.4 | 502080 | 102 |  | - | 517488 |
|  |  | Uplink | High | 2607.5 | 521500 | 2233.46 | 446692 | 504 |  | - | 519988 |
| 30 | 38 | Downlink | Low | 2585 | 517000 | 2571.32 | 514264 | 0 | 30 | - | 514984 |
|  |  | & | Mid | 2595 | 519000 | 2507.88 | 501576 | 102 |  | - | 516984 |
|  |  | Uplink | High | 2605 | 521000 | 2228.44 | 445688 | 504 |  | - | 518984 |
| 40 | 51 | Downlink | Low | 2590 | 518000 | 2571.64 | 514328 | 0 | 15 | - | 514688 |
|  |  | & | Mid | 2595 | 519000 | 2503.2 | 500640 | 102 |  | - | 515688 |
|  |  | Uplink | High | 2600 | 520000 | 2218.76 | 443752 | 504 |  | - | 516688 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.39	Reference test frequencies for NR operating band n39

Table 4.3.1.1.1.39-1: Test frequencies for NR operating band n39 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1882.5 | 376500 | 1880.25 | 376050 | 0 | 15 | 4707 | 376590 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 1899.1 | 379820 | 1878.49 | 375698 | 102 |  | 4747 | 379730 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 1917.5 | 383500 | 1824.53 | 364906 | 504 |  | 4793 | 383530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 1885 | 377000 | 1880.32 | 376064 | 0 | 30 | 4714 | 377090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 1900 | 380000 | 1876.96 | 375392 | 102 |  | 4750 | 379970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1915 | 383000 | 1819.6 | 363920 | 504 |  | 4789 | 383090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 1887.5 | 377500 | 1880.39 | 376078 | 0 | 30 | 4712 | 377050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1874.53 | 374906 | 102 |  | 4744 | 379490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1912.5 | 382500 | 1814.67 | 362934 | 504 |  | 4773 | 381870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 1890 | 378000 | 1880.46 | 376092 | 0 | 30 | 4713 | 377070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1872.1 | 374420 | 102 |  | 4738 | 379010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1910 | 382000 | 1809.74 | 361948 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 1892.5 | 378500 | 1880.53 | 376106 | 0 | 30 | 4714 | 377090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1869.67 | 373934 | 102 |  | 4732 | 378530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1907.5 | 381500 | 1804.81 | 360962 | 504 |  | 4750 | 379970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 1895 | 379000 | 1880.6 | 376120 | 0 | 30 | 4712 | 377050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 1900 | 380000 | 1867.24 | 373448 | 102 |  | 4726 | 378050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1905 | 381000 | 1799.88 | 359976 | 504 |  | 4737 | 378990 | 2 | 2 | 0 (2) | 508 |
| 35 | 188 | Downlink | Low | 1897.5 | 379500 | 1880.58 | 376116 | 0 | 30 | 4713 | 377070 | 6 | 4 | 0 (2) | 6 |
|  |  | & | Mid | 1900 | 380000 | 1864.72 | 372944 | 102 |  | 4720 | 377570 | 6 | 4 | 0 (2) | 108 |
|  |  | Uplink | High | 1902.5 | 380500 | 1794.86 | 358972 | 504 |  | 4724 | 378010 | 10 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 1900 | 380000 | 1880.56 | 376112 | 0 | 30 | 4714 | 377090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 for SSB_SCS = 15kHz and Table 13-3 for SSB_SCS=30kHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-2: Test frequencies for NR operating band n39 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1885 | 377000 | 1880.68 | 376136 | 0 | 30 | 4714 | 377090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 1900 | 380000 | 1858.96 | 371792 | 102 |  | 4750 | 379970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1915 | 383000 | 1729.24 | 345848 | 504 |  | 4789 | 383090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 1887.5 | 377500 | 1880.66 | 376132 | 0 | 30 | 4712 | 377050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1856.44 | 371288 | 102 |  | 4744 | 379490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1912.5 | 382500 | 1724.22 | 344844 | 504 |  | 4773 | 381870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 1890 | 378000 | 1880.82 | 376164 | 0 | 30 | 4713 | 377070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1854.1 | 370820 | 102 |  | 4738 | 379010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1910 | 382000 | 1719.38 | 343876 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 1892.5 | 378500 | 1880.8 | 376160 | 0 | 30 | 4714 | 377090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1851.58 | 370316 | 102 |  | 4732 | 378530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1907.5 | 381500 | 1714.36 | 342872 | 504 |  | 4750 | 379970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 1895 | 379000 | 1880.96 | 376192 | 0 | 30 | 4712 | 377050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1900 | 380000 | 1849.24 | 369848 | 102 |  | 4726 | 378050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1905 | 381000 | 1709.52 | 341904 | 504 |  | 4737 | 378990 | 2 | 0 | 1 (1) | 1010 |
| 35 | 92 | Downlink | Low | 1897.5 | 379500 | 1880.94 | 376188 | 0 | 30 | 4713 | 377070 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1846.72 | 369344 | 102 |  | 4720 | 377570 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1902.5 | 380500 | 1704.5 | 340900 | 504 |  | 4724 | 378010 | 10 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 1900 | 380000 | 1880.92 | 376184 | 0 | 30 | 4714 | 377090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-3: Test frequencies for NR operating band n39 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1885 | 377000 | 1881.04 | 376208 | 0 | 30 | - | 376928 |
|  |  | & | Mid | 1900 | 380000 | 1822.6 | 364520 | 102 |  | - | 379928 |
|  |  | Uplink | High | 1915 | 383000 | 1548.16 | 309632 | 504 |  | - | 382928 |
| 15 | 18 | Downlink | Low | 1887.5 | 377500 | 1881.02 | 376204 | 0 | 30 | - | 376924 |
|  |  | & | Mid | 1900 | 380000 | 1820.08 | 364016 | 102 |  | - | 379424 |
|  |  | Uplink | High | 1912.5 | 382500 | 1543.14 | 308628 | 504 |  | - | 381924 |
| 20 | 24 | Downlink | Low | 1890 | 378000 | 1881.36 | 376272 | 0 | 30 | - | 376992 |
|  |  | & | Mid | 1900 | 380000 | 1817.92 | 363584 | 102 |  | - | 378992 |
|  |  | Uplink | High | 1910 | 382000 | 1538.48 | 307696 | 504 |  | - | 380992 |
| 25 | 31 | Downlink | Low | 1892.5 | 378500 | 1881.34 | 376268 | 0 | 30 | - | 376988 |
|  |  | & | Mid | 1900 | 380000 | 1815.4 | 363080 | 102 |  | - | 378488 |
|  |  | Uplink | High | 1907.5 | 381500 | 1533.46 | 306692 | 504 |  | - | 379988 |
| 30 | 38 | Downlink | Low | 1895 | 379000 | 1881.32 | 376264 | 0 | 30 | - | 376984 |
|  |  | & | Mid | 1900 | 380000 | 1812.88 | 362576 | 102 |  | - | 377984 |
|  |  | Uplink | High | 1905 | 381000 | 1528.44 | 305688 | 504 |  | - | 378984 |
| 35 | 44 | Downlink | Low | 1897.5 | 379500 | 1881.66 | 376332 | 0 | 30 | - | 377052 |
|  |  | & | Mid | 1900 | 380000 | 1810.72 | 362144 | 102 |  | - | 377552 |
|  |  | Uplink | High | 1902.5 | 380500 | 1523.78 | 304756 | 504 |  | - | 378052 |
| 40 | 51 | Downlink | Low | 1900 | 380000 | 1881.64 | 376328 | 0 | 30 | - | 377048 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.40	Reference test frequencies for NR operating band n40

Table 4.3.1.1.1.40-1: Test frequencies for NR operating band n40 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2302.5 | 460500 | 2300.25 | 460050 | 0 | (Note 3) | - | 460770 | - | - | - | - |
|  |  | & | Mid | 2350 | 470000 | 2329.39 | 465878 | 102 |  | - | 470270 | - | - | - | - |
|  |  | Uplink | High | 2397.5 | 479500 | 2304.53 | 460906 | 504 |  | - | 479770 | - | - | - | - |
| 10 | 52 | Downlink | Low | 2305 | 461000 | 2300.32 | 460064 | 0 | 30 | 5764 | 461090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 2350 | 470000 | 2326.96 | 465392 | 102 |  | 5875 | 469970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2395 | 479000 | 2299.6 | 459920 | 504 |  | 5989 | 479090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 2307.5 | 461500 | 2300.39 | 460078 | 0 | 30 | 5762 | 461050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2324.53 | 464906 | 102 |  | 5869 | 469490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2392.5 | 478500 | 2294.67 | 458934 | 504 |  | 5973 | 477870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 2310 | 462000 | 2300.46 | 460092 | 0 | 30 | 5763 | 461070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2322.1 | 464420 | 102 |  | 5863 | 469010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2390 | 478000 | 2289.74 | 457948 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 2312.5 | 462500 | 2300.53 | 460106 | 0 | 30 | 5764 | 461090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2319.67 | 463934 | 102 |  | 5857 | 468530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2387.5 | 477500 | 2284.81 | 456962 | 504 |  | 5950 | 475970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 2315 | 463000 | 2300.6 | 460120 | 0 | 30 | 5762 | 461050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2317.24 | 463448 | 102 |  | 5851 | 468050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2385 | 477000 | 2279.88 | 455976 | 504 |  | 5937 | 474990 | 2 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 2320 | 464000 | 2300.56 | 460112 | 0 | 30 | 5764 | 461090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2312.2 | 462440 | 102 |  | 5839 | 467090 | 2 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2380 | 476000 | 2269.84 | 453968 | 504 |  | 5914 | 473090 | 2 | 1 | 1 (6) | 511 |
| 50 | 270 | Downlink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 | 30 | 5763 | 461070 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2307.34 | 461468 | 102 |  | 5827 | 466130 | 6 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2375 | 475000 | 2259.98 | 451996 | 504 |  | 5888 | 471130 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-2: Test frequencies for NR operating band n40 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2305 | 461000 | 2300.68 | 460136 | 0 | 30 | 5764 | 461090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2350 | 470000 | 2308.96 | 461792 | 102 |  | 5875 | 469970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2395 | 479000 | 2209.24 | 441848 | 504 |  | 5989 | 479090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 2307.5 | 461500 | 2300.66 | 460132 | 0 | 30 | 5762 | 461050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2306.44 | 461288 | 102 |  | 5869 | 469490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2392.5 | 478500 | 2204.22 | 440844 | 504 |  | 5973 | 477870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 | 30 | 5763 | 461070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2304.1 | 460820 | 102 |  | 5863 | 469010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2390 | 478000 | 2199.38 | 439876 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 2312.5 | 462500 | 2300.8 | 460160 | 0 | 30 | 5764 | 461090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2301.58 | 460316 | 102 |  | 5857 | 468530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2387.5 | 477500 | 2194.36 | 438872 | 504 |  | 5950 | 475970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2315 | 463000 | 2300.96 | 460192 | 0 | 30 | 5762 | 461050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2299.24 | 459848 | 102 |  | 5851 | 468050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2385 | 477000 | 2189.52 | 437904 | 504 |  | 5937 | 474990 | 2 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 2320 | 464000 | 2300.92 | 460184 | 0 | 30 | 5764 | 461090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2294.2 | 458840 | 102 |  | 5839 | 467090 | 14 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2380 | 476000 | 2179.48 | 435896 | 504 |  | 5914 | 473090 | 14 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 | 30 | 5763 | 461070 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2289.34 | 457868 | 102 |  | 5827 | 466130 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2375 | 475000 | 2169.62 | 433924 | 504 |  | 5888 | 471130 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 2330 | 466000 | 2300.84 | 460168 | 0 | 30 | 5762 | 461050 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2284.12 | 456824 | 102 |  | 5812 | 464930 | 14 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2370 | 474000 | 2159.4 | 431880 | 504 |  | 5862 | 468990 | 10 | 0 | 1 (1) | 1010 |
| 70 | 189 | Downlink | Low | 2335 | 467000 | 2300.98 | 460196 | 0 | 30 | 5764 | 461090 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2279.26 | 455852 | 102 |  | 5800 | 463970 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2365 | 473000 | 2149.54 | 429908 | 504 |  | 5839 | 467090 | 10 | 0 | 2 (2) | 1012 |
| 80 | 217 | Downlink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 | 30 | 5763 | 461070 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2274.22 | 454844 | 102 |  | 5788 | 463010 | 10 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2360 | 472000 | 2139.5 | 427900 | 504 |  | 5813 | 465130 | 2 | 0 | 3 (3) | 1014 |
| 90 | 245 | Downlink | Low | 2345 | 469000 | 2300.9 | 460180 | 0 | 30 | 5762 | 461050 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2269.18 | 453836 | 102 |  | 5776 | 462050 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2355 | 471000 | 2129.46 | 425892 | 504 |  | 5787 | 462990 | 6 | 0 | 1 (1) | 1010 |
| 100 | 273 | Downlink | Low | 2350 | 470000 | 2300.86 | 460172 | 0 | 30 | 5764 | 461090 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2264.14 | 452828 | 102 |  | 5764 | 461090 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2350 | 470000 | 2119.42 | 423884 | 504 |  | 5764 | 461090 | 18 | 0 | 2 (2) | 1012 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-3: Test frequencies for NR operating band n40 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2305 | 461000 | 2301.04 | 460208 | 0 | 30 | - | 460928 |
|  |  | & | Mid | 2350 | 470000 | 2272.6 | 454520 | 102 |  | - | 469928 |
|  |  | Uplink | High | 2395 | 479000 | 2028.16 | 405632 | 504 |  | - | 478928 |
| 15 | 18 | Downlink | Low | 2307.5 | 461500 | 2301.02 | 460204 | 0 | 30 | - | 460924 |
|  |  | & | Mid | 2350 | 470000 | 2270.08 | 454016 | 102 |  | - | 469424 |
|  |  | Uplink | High | 2392.5 | 478500 | 2023.14 | 404628 | 504 |  | - | 477924 |
| 20 | 24 | Downlink | Low | 2310 | 462000 | 2301.36 | 460272 | 0 | 30 | - | 460992 |
|  |  | & | Mid | 2350 | 470000 | 2267.92 | 453584 | 102 |  | - | 468992 |
|  |  | Uplink | High | 2390 | 478000 | 2018.48 | 403696 | 504 |  | - | 476992 |
| 25 | 31 | Downlink | Low | 2312.5 | 462500 | 2301.34 | 460268 | 0 | 30 | - | 460988 |
|  |  | & | Mid | 2350 | 470000 | 2265.4 | 453080 | 102 |  | - | 468488 |
|  |  | Uplink | High | 2387.5 | 477500 | 2013.46 | 402692 | 504 |  | - | 475988 |
| 30 | 38 | Downlink | Low | 2315 | 463000 | 2301.32 | 460264 | 0 | 30 | - | 460984 |
|  |  | & | Mid | 2350 | 470000 | 2262.88 | 452576 | 102 |  | - | 467984 |
|  |  | Uplink | High | 2385 | 477000 | 2008.44 | 401688 | 504 |  | - | 474984 |
| 40 | 51 | Downlink | Low | 2320 | 464000 | 2301.64 | 460328 | 0 | 30 | - | 461048 |
|  |  | & | Mid | 2350 | 470000 | 2258.2 | 451640 | 102 |  | - | 467048 |
|  |  | Uplink | High | 2380 | 476000 | 1998.76 | 399752 | 504 |  | - | 473048 |
| 50 | 65 | Downlink | Low | 2325 | 465000 | 2301.6 | 460320 | 0 | 30 | - | 461040 |
|  |  | & | Mid | 2350 | 470000 | 2253.16 | 450632 | 102 |  | - | 466040 |
|  |  | Uplink | High | 2375 | 475000 | 1988.72 | 397744 | 504 |  | - | 471040 |
| 60 | 79 | Downlink | Low | 2330 | 466000 | 2301.56 | 460312 | 0 | 30 | - | 461032 |
|  |  | & | Mid | 2350 | 470000 | 2248.12 | 449624 | 102 |  | - | 465032 |
|  |  | Uplink | High | 2370 | 474000 | 1978.68 | 395736 | 504 |  | - | 469032 |
| 70 | 93 | Downlink | Low | 2335 | 467000 | 2301.52 | 460304 | 0 | 30 | - | 461024 |
|  |  | & | Mid | 2350 | 470000 | 2243.08 | 448616 | 102 |  | - | 464024 |
|  |  | Uplink | High | 2365 | 473000 | 1968.64 | 393728 | 504 |  | - | 467024 |
| 80 | 107 | Downlink | Low | 2340 | 468000 | 2301.48 | 460296 | 0 | 30 | - | 461016 |
|  |  | & | Mid | 2350 | 470000 | 2238.04 | 447608 | 102 |  | - | 463016 |
|  |  | Uplink | High | 2360 | 472000 | 1958.6 | 391720 | 504 |  | - | 465016 |
| 90 | 121 | Downlink | Low | 2345 | 469000 | 2301.44 | 460288 | 0 | 30 | - | 461008 |
|  |  | & | Mid | 2350 | 470000 | 2233 | 446600 | 102 |  | - | 462008 |
|  |  | Uplink | High | 2355 | 471000 | 1948.56 | 389712 | 504 |  | - | 463008 |
| 100 | 135 | Downlink | Low | 2350 | 470000 | 2301.4 | 460280 | 0 | 30 | - | 461000 |
|  |  | & | Mid | 2350 | 470000 | 2227.96 | 445592 | 102 |  | - | 461000 |
|  |  | Uplink | High | 2350 | 470000 | 1938.52 | 387704 | 504 |  | - | 461000 |
| Note: FR1 carrier without CORSESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.41	Reference test frequencies for NR operating band n41

Table 4.3.1.1.1.41-1: Test frequencies for NR operating band n41, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 2501.01 | 500202 | 2496.33 | 499266 | 0 | 15 | 6246 | 499710 | 4 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2569.965 | 513993 | 102 |  | 6477 | 518190 | 7 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
| 15 | 79 | Downlink | Low | 2503.5 | 500700 | 2496.39 | 499278 | 0 | 15 | 6246 | 499710 | 0 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2567.535 | 513507 | 102 |  | 6471 | 517710 | 9 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2682.495 | 536499 | 2584.665 | 516933 | 504 |  | 6693 | 535470 | 11 | 0 | 0 (0) | 504 |
| 20 | 106 | Downlink | Low | 2506.005 | 501201 | 2496.465 | 499293 | 0 | 15 | 6246 | 499710 | 7 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2565.105 | 513021 | 102 |  | 6465 | 517230 | 11 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2679.99 | 535998 | 2579.73 | 515946 | 504 |  | 6681 | 534510 | 8 | 1 | 0 (0) | 505 |
| 30 | 160 | Downlink | Low | 2511 | 502200 | 2496.6 | 499320 | 0 | 15 | 6246 | 499710 | 10 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2560.245 | 512049 | 102 |  | 6453 | 516270 | 3 | 1 | 2 (4) | 107 |
|  |  | Uplink | High | 2674.995 | 534999 | 2569.875 | 513975 | 504 |  | 6657 | 532590 | 1 | 1 | 1 (2) | 507 |
| 40 | 216 | Downlink | Low | 2516.01 | 503202 | 2496.57 | 499314 | 0 | 15 | 6246 | 499710 | 0 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2555.205 | 511041 | 102 |  | 6441 | 515310 | 7 | 2 | 2 (4) | 108 |
|  |  | Uplink | High | 2670 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
| 50 | 270 | Downlink | Low | 2521.005 | 504201 | 2496.705 | 499341 | 0 | 15 | 6246 | 499710 | 3 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2550.345 | 510069 | 102 |  | 6426 | 514110 | 3 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2664.99 | 532998 | 2549.97 | 509994 | 504 |  | 6606 | 528510 | 4 | 0 | 0 (0) | 504 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-2: Test frequencies for NR operating band n41, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2501.01 | 500202 | 2496.69 | 499338 | 0 | 30 | 6252 | 500190 | 20 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2551.95 | 510390 | 102 |  | 6483 | 518670 | 0 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (0) | 1008 |
| 15 | 38 | Downlink | Low | 2503.5 | 500700 | 2496.66 | 499332 | 0 | 30 | 6252 | 500190 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2549.43 | 509886 | 102 |  | 6474 | 517950 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2682.48 | 536496 | 2494.2 | 498840 | 504 |  | 6699 | 535950 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 2506.02 | 501204 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2547.09 | 509418 | 102 |  | 6471 | 517710 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2679.99 | 535998 | 2489.37 | 497874 | 504 |  | 6687 | 534990 | 12 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2511 | 502200 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2542.23 | 508446 | 102 |  | 6456 | 516510 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2674.98 | 534996 | 2479.5 | 495900 | 504 |  | 6663 | 533070 | 6 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 | 6252 | 500190 | 4 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2537.19 | 507438 | 102 |  | 6444 | 515550 | 16 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (0) | 1008 |
| 50 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 | 6252 | 500190 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2592.99 | 518598 | 2532.33 | 506466 | 102 |  | 6432 | 514590 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  | 6612 | 528990 | 20 | 0 | 0 (0) | 1008 |
| 60 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2527.11 | 505422 | 102 |  | 6420 | 513630 | 0 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  | 6588 | 527070 | 14 | 0 | 2 (2) | 1012 |
| 70 | 189 | Downlink | Low | 2531.01 | 506202 | 2496.99 | 499398 | 0 | 30 | 6252 | 500190 | 0 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2522.25 | 504450 | 102 |  | 6408 | 512670 | 4 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2655 | 531000 | 2439.54 | 487908 | 504 |  | 6564 | 525150 | 6 | 0 | 3 (3) | 1014 |
| 80 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2517.21 | 503442 | 102 |  | 6396 | 511710 | 20 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  | 6537 | 522990 | 4 | 0 | 1 (1) | 1010 |
| 90 | 245 | Downlink | Low | 2541 | 508200 | 2496.9 | 499380 | 0 | 30 | 6252 | 500190 | 6 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2512.17 | 502434 | 102 |  | 6381 | 510510 | 4 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2644.98 | 528996 | 2419.44 | 483888 | 504 |  | 6513 | 521070 | 10 | 0 | 2 (2) | 1012 |
| 100 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 | 6252 | 500190 | 8 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2507.13 | 501426 | 102 |  | 6369 | 509550 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  | 6486 | 518910 | 6 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-3: Test frequencies for NR operating band n41, SCS 60 kHz and ΔFRaster 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2501.01 | 500202 | 2497.05 | 499410 | 0 | 15 | - | 499770 |
|  |  | & | Mid | 2593.005 | 518601 | 2515.605 | 503121 | 102 |  | - | 518169 |
|  |  | Uplink | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
| 15 | 18 | Downlink | Low | 2503.5 | 500700 | 2497.02 | 499404 | 0 | 15 | - | 499764 |
|  |  | & | Mid | 2593.005 | 518601 | 2513.085 | 502617 | 102 |  | - | 517665 |
|  |  | Uplink | High | 2682.495 | 536499 | 2313.135 | 462627 | 504 |  | - | 535563 |
| 20 | 24 | Downlink | Low | 2506.005 | 501201 | 2497.365 | 499473 | 0 | 15 | - | 499833 |
|  |  | & | Mid | 2593.005 | 518601 | 2510.925 | 502185 | 102 |  | - | 517233 |
|  |  | Uplink | High | 2679.99 | 535998 | 2308.47 | 461694 | 504 |  | - | 534630 |
| 30 | 38 | Downlink | Low | 2511 | 502200 | 2497.32 | 499464 | 0 | 15 | - | 499824 |
|  |  | & | Mid | 2593.005 | 518601 | 2505.885 | 501177 | 102 |  | - | 516225 |
|  |  | Uplink | High | 2674.995 | 534999 | 2298.435 | 459687 | 504 |  | - | 532623 |
| 40 | 51 | Downlink | Low | 2516.01 | 503202 | 2497.65 | 499530 | 0 | 15 | - | 499890 |
|  |  | & | Mid | 2593.005 | 518601 | 2501.205 | 500241 | 102 |  | - | 515289 |
|  |  | Uplink | High | 2670 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
| 50 | 65 | Downlink | Low | 2521.005 | 504201 | 2497.605 | 499521 | 0 | 15 | - | 499881 |
|  |  | & | Mid | 2593.005 | 518601 | 2496.165 | 499233 | 102 |  | - | 514281 |
|  |  | Uplink | High | 2664.99 | 532998 | 2278.71 | 455742 | 504 |  | - | 528678 |
| 60 | 79 | Downlink | Low | 2526 | 505200 | 2497.56 | 499512 | 0 | 15 | - | 499872 |
|  |  | & | Mid | 2593.005 | 518601 | 2491.125 | 498225 | 102 |  | - | 513273 |
|  |  | Uplink | High | 2659.995 | 531999 | 2268.675 | 453735 | 504 |  | - | 526671 |
| 70 | 93 | Downlink | Low | 2531.01 | 506202 | 2497.53 | 499506 | 0 | 15 | - | 499866 |
|  |  | & | Mid | 2593.005 | 518601 | 2486.085 | 497217 | 102 |  | - | 512265 |
|  |  | Uplink | High | 2655 | 531000 | 2258.64 | 451728 | 504 |  | - | 524664 |
| 80 | 107 | Downlink | Low | 2536.005 | 507201 | 2497.485 | 499497 | 0 | 15 | - | 499857 |
|  |  | & | Mid | 2593.005 | 518601 | 2481.045 | 496209 | 102 |  | - | 511257 |
|  |  | Uplink | High | 2649.99 | 529998 | 2248.59 | 449718 | 504 |  | - | 522654 |
| 90 | 121 | Downlink | Low | 2541 | 508200 | 2497.44 | 499488 | 0 | 15 | - | 499848 |
|  |  | & | Mid | 2593.005 | 518601 | 2476.005 | 495201 | 102 |  | - | 510249 |
|  |  | Uplink | High | 2644.995 | 528999 | 2238.555 | 447711 | 504 |  | - | 520647 |
| 100 | 135 | Downlink | Low | 2546.01 | 509202 | 2497.41 | 499482 | 0 | 15 | - | 499842 |
|  |  | & | Mid | 2593.005 | 518601 | 2470.965 | 494193 | 102 |  | - | 509241 |
|  |  | Uplink | High | 2640 | 528000 | 2228.52 | 445704 | 504 |  | - | 518640 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.42 to 4.3.1.1.1.45	FFS

###### 4.3.1.1.1.46	Reference test frequencies for NR operating band n46

Table 4.3.1.1.1.46-1: Test frequencies for NR operating band n46 and SCS 15 kHz (Note 3)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 5730 | 782000 | 5725.32 | 781688 | 0 | 30 | - | 781928 | 31 | - | - | - |
|  |  | & | Mid |  |  |  |  |  |  | - |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5734.62 | 782308 | 504 |  | - | 788596 | 31 | - | - | - |
| 20 | 106 | Downlink | Low | 5160 | 744000 | 5150.46 | 743364 | 0 | 30 | - | 743604 | 31 | - | - | - |
|  |  | & | Mid | 5479.98 | 765332 | 5452.08 | 763472 | 102 |  | - | 764936 | 31 | - | - | - |
|  |  | Uplink | High | 5905.02 | 793668 | 5804.76 | 786984 | 504 |  | - | 793272 | 31 | - | - | - |
| 40 | 216 | Downlink | Low | 5170.02 | 744668 | 5150.58 | 743372 | 0 | 30 | - | 743612 | 31 | - | - | - |
|  |  | & | Mid | 5509.98 | 767332 | 5472.18 | 764812 | 102 |  | - | 766276 | 31 | - | - | - |
|  |  | Uplink | High | 5895 | 793000 | 5784.84 | 785656 | 504 |  | - | 791944 | 31 | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: NR band n46 with SCS =15 kHz is restricted for operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.46-2: Test frequencies for NR operating band n46 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 5730 | 782000 | 5725.68 | 781712 | 0 | 30 | - | 781952 | 31 | - | - | - |
| Note 3 |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5644.26 | 776284 | 504 |  | - | 788620 | 31 | - | - | - |
| 20 | 51 | Downlink | Low | 5160 | 744000 | 5150.82 | 743388 | 0 | 30 | 8996 | 743712 | 12 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5479.98 | 765332 | 5434.08 | 762272 | 102 |  | 9218 | 765024 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5905.02 | 793668 | 5714.4 | 780960 | 504 |  | 9513 | 793344 | 0 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 5170.02 | 744668 | 5150.94 | 743396 | 0 | 30 | 8996 | 743712 | 4 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5509.98 | 767332 | 5454.18 | 763612 | 102 |  | 9232 | 766368 | 20 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5895 | 793000 | 5694.48 | 779632 | 504 |  | 9499 | 792000 | 8 | 0 | 1 (1) | 1010 |
| 60 | 162 | Downlink | Low | 5200.02 | 746668 Note 5 | 5170.86 | 744724 | 0 | 30 | 9010 | 745056 | 20 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5500.02 | 766668 | 5434.14 | 762276 | 102 |  | 9218 | 765024 | 12 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5884.98 | 792332 | 5674.38 | 778292 | 504 |  | 9485 | 790656 | 4 | 0 | 1 (1) | 1010 |
| 80 | 217 | Downlink | Low | 5190 | 746000 | 5150.94 | 743396 | 0 | 30 | 8996 | 743712 | 4 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 5530,02 | 768668 | 5454,24 | 763616 | 102 |  | 9232 | 766368 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 5875.02 | 791668 | 5654.52 | 776968 | 504 |  | 9472 | 789408 | 8 | 1 | 3 (3) | 1016 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. Note 5: The next lowest allowed ARFCN values has been selected as the lowest ARFCN value in TS 38.101 [7], Table 5.4.2.3-2 for CBW=60 MHz will cause the required guardband to exceed the bands lower edge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.46-3: Test frequencies for NR operating band n46 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 2 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 5730 | 782000 | 5726.04 | 781736 | 0 | 30 | - | 781976 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |
|  |  | Uplink | High | 5830.02 | 788668 | 5463.18 | 764212 | 504 |  | - | 788644 |
| 20 | 24 | Downlink | Low | 5160 | 744000 | 5151.36 | 743424 | 0 | 30 | - | 743664 |
|  |  | & | Mid | 5479.98 | 765332 | 5397.9 | 759860 | 102 |  | - | 764996 |
|  |  | Uplink | High | 5905.02 | 793668 | 5533.5 | 768900 | 504 |  | - | 793332 |
| 40 | 51 | Downlink | Low | 5170.02 | 744668 | 5151.66 | 743444 | 0 | 30 | - | 743684 |
|  |  | & | Mid | 5509.98 | 767332 | 5418.18 | 761212 | 102 |  | - | 766348 |
|  |  | Uplink | High | 5895 | 793000 | 5513.76 | 767584 | 504 |  | - | 792016 |
| 60 | 79 | Downlink | Low | 5179.98 | 745332 | 5151.54 | 743436 | 0 | 30 | - | 743676 |
|  |  | & | Mid | 5500.02 | 766668 | 5398.14 | 759876 | 102 |  | - | 765012 |
|  |  | Uplink | High | 5884.98 | 792332 | 5493.66 | 766244 | 504 |  | - | 790676 |
| 80 | 107 | Downlink | Low | 5190 | 746000 | 5151.48 | 743432 | 0 | 30 | - | 743672 |
|  |  | & | Mid | 5530,02 | 768668 | 5418,06 | 761204 | 102 |  | - | 766340 |
|  |  | Uplink | High | 5875.02 | 791668 | 5473.62 | 764908 | 504 |  | - | 789340 |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-2. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.47	FFS

###### 4.3.1.1.1.48	Reference test frequencies for NR operating band n48

Table 4.3.1.1.1.48-1: Test frequencies for NR operating band n48 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | (Note 4) | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 3552.51 | 636834 | 3550.26 | 636684 | 0 | (Note 3) | - | - | - | - | - | - |
|  |  | & | Mid | 3625.005 | 641667 | 3604.395 | 640293 | 102 |  | - | - | - | - | - | - |
|  |  | Uplink | High | 3697.5 | 646500 | 3604.53 | 640302 | 504 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3601.95 | 640130 | 102 |  | 7933 | 641664 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3599.52 | 639968 | 102 |  | 7931 | 641472 | 4 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 20 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3597.09 | 639806 | 102 |  | 7930 | 641376 | 10 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 3565.02 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3592.23 | 639482 | 102 |  | 7926 | 640992 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 40 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3587.19 | 639146 | 102 |  | 7923 | 640704 | 10 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 50 | 270 | Downlink | Low | 3575.01 | 638334 | 3550.71 | 636714 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 4) | Mid | 3624.99 | 641666 | 3582.33 | 638822 | 102 |  |  | 640320 | 31 |  |  |  |
|  |  | Uplink | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  |  | 643680 | 31 |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell. Note 4: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-2: Test frequencies for NR operating band n48 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3583.95 | 638930 | 102 |  | 7933 | 641664 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3581.43 | 638762 | 102 |  | 7931 | 641472 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3579.09 | 638606 | 102 |  | 7930 | 641376 | 10 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3574.23 | 638282 | 102 |  | 7926 | 640992 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3569.19 | 637946 | 102 |  | 7923 | 640704 | 22 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 3575.01 | 638334 | 3551.07 | 636738 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3564.33 | 637622 | 102 |  |  | 640320 | 31 |  |  |  |
|  |  | (Note 3) | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  |  | 643680 | 31 |  |  |  |
| 60 | 162 | Downlink | Low | 3580.02 | 638668 | 3550.86 | 636724 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3559.11 | 637274 | 102 |  |  | 640032 | 31 |  |  |  |
|  |  | (Note 3) | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  |  | 643008 | 31 |  |  |  |
| 70 | 189 | Downlink (Note 3) | Low | 3585 | 639000 | 3550.98 | 636732 | 0 | 30 | - | 637056 | 31 | - | - | - |
|  |  |  | Mid | 3624.99 | 641666 | 3554.25 | 636950 | 102 |  | - | 639648 | 31 | - | - | - |
|  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | - | 642336 | 31 | - | - | - |
| 80 | 217 | Downlink | Low | 3590.01 | 639334 | 3550.95 | 636730 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3549.21 | 636614 | 102 |  |  | 639360 | 31 |  |  |  |
|  |  | (Note 3) | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  |  | 641664 | 31 |  |  |  |
| 90 | 245 | Downlink | Low | 3595.02 | 639668 | 3550.92 | 636728 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3544.17 | 636278 | 102 |  |  | 638976 | 31 |  |  |  |
|  |  | (Note 3) | High | 3654.99 | 643666 | 3429.45 | 628630 | 504 |  |  | 640992 | 31 |  |  |  |
| 100 | 273 | Downlink | Low | 3600 | 640000 | 3550.86 | 636724 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3539.13 | 635942 | 102 |  |  | 638688 | 31 |  |  |  |
|  |  | (Note 3) | High | 3649.98 | 643332 | 3419.4 | 627960 | 504 |  |  | 640320 | 31 |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-3: Test frequencies for NR operating band n48 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3555 | 637000 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3547.59 | 636506 | 102 |  | - | 641642 |
|  |  | Uplink | High | 3694.98 | 646332 | 3328.14 | 621876 | 504 |  | - | 646308 |
| 15 | 18 | Downlink | Low | 3557.52 | 637168 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3545.07 | 636338 | 102 |  | - | 641474 |
|  |  | Uplink | High | 3692.49 | 646166 | 3323.13 | 621542 | 504 |  | - | 645974 |
| 20 | 24 | Downlink | Low | 3560.01 | 637334 | 3551.37 | 636758 | 0 | 30 | - | 636998 |
|  |  | & | Mid | 3624.99 | 641666 | 3542.91 | 636194 | 102 |  | - | 641330 |
|  |  | Uplink | High | 3690 | 646000 | 3318.48 | 621232 | 504 |  | - | 645664 |
| 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 |
|  |  | & | Mid | 3624.99 | 641666 | 3574.23 | 638282 | 102 |  | 7926 | 640992 |
|  |  | Uplink | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 |
| 40 | 51 | Downlink | Low | 3570 | 638000 | 3551.64 | 636776 | 0 | 30 | - | 637016 |
|  |  | & | Mid | 3624.99 | 641666 | 3533.19 | 635546 | 102 |  | - | 640682 |
|  |  | Uplink | High | 3679.98 | 645332 | 3298.74 | 619916 | 504 |  | - | 644348 |
| 50 | 65 | Downlink(Note 2) | Low | 3575.01 | 638334 | 3551.61 | 636774 | 0 | 30 | - | 637014 |
|  |  |  | Mid | 3624.99 | 641666 | 3528.15 | 635210 | 102 |  | - | 640346 |
|  |  |  | High | 3675 | 645000 | 3288.72 | 619248 | 504 |  | - | 643680 |
| 60 | 79 | Downlink(Note 2) | Low | 3580.02 | 638668 | 3551.58 | 636772 | 0 | 30 | - | 637012 |
|  |  |  | Mid | 3624.99 | 641666 | 3523.11 | 634874 | 102 |  | - | 640010 |
|  |  |  | High | 3669.99 | 644666 | 3278.67 | 618578 | 504 |  | - | 643010 |
| 70 | 93 | Downlink (Note 2) | Low | 3585 | 639000 | 3551.52 | 636768 | 0 | 30 | - | 637008 |
|  |  |  | Mid | 3624.99 | 641666 | 3518.07 | 634538 | 102 |  | - | 639674 |
|  |  |  | High | 3664.98 | 644332 | 3268.62 | 617908 | 504 |  | - | 642340 |
| 80 | 107 | Downlink(Note 2) | Low | 3590.01 | 639334 | 3551.49 | 636766 | 0 | 30 | - | 637006 |
|  |  |  | Mid | 3624.99 | 641666 | 3513.03 | 634202 | 102 |  | - | 639338 |
|  |  |  | High | 3660 | 644000 | 3258.6 | 617240 | 504 |  | - | 641672 |
| 90 | 121 | Downlink(Note 2) | Low | 3595.02 | 639668 | 3551.46 | 636764 | 0 | 30 | - | 637004 |
|  |  |  | Mid | 3624.99 | 641666 | 3507.99 | 633866 | 102 |  | - | 639002 |
|  |  |  | High | 3654.99 | 643666 | 3248.55 | 616570 | 504 |  | - | 641002 |
| 100 | 135 | Downlink(Note 2) | Low | 3600 | 640000 | 3551.4 | 636760 | 0 | 30 | - | 637000 |
|  |  |  | Mid | 3624.99 | 641666 | 3502.95 | 633530 | 102 |  | - | 638666 |
|  |  |  | High | 3649.98 | 643332 | 3238.5 | 615900 | 504 |  | - | 640332 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.49	FFS

###### 4.3.1.1.1.50	Reference test frequencies for NR operating band n50

Table 4.3.1.1.1.50-1: Test frequencies for NR operating band n50 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset[RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | (Note 3) | - | 287170 | - | - | - | - |
|  |  | & | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | - | 295170 | - | - | - | - |
|  |  | Uplink | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | - | 303170 | - | - | - | - |
| 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 30 | 3591 | 287310 | 6 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3687 | 294990 | 6 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3780 | 302430 | 10 | 0 | 1 (6) | 510 |
| 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 30 | 3592 | 287330 | 8 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3678 | 294270 | 0 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3767 | 301450 | 0 | 1 | 1 (6) | 511 |
| 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (2) | 506 |
| 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3648 | 291870 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3705 | 296430 | 6 | 3 | 0 (2) | 509 |
| 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 30 | 3593 | 287530 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3636 | 290910 | 2 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3682 | 294530 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-2: Test frequencies for NR operating band n50 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 30 | 3591 | 287310 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (2) | 1012 |
| 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 30 | 3592 | 287330 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (2) | 1012 |
| 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (1) | 1010 |
| 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 30 | 3593 | 287530 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 1462 | 292400 | 1432.84 | 286568 | 0 | 30 | 3592 | 287330 | 14 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1408.62 | 281724 | 102 |  | 3624 | 289950 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1487 | 297400 | 1276.4 | 255280 | 504 |  | 3653 | 292330 | 14 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 1472 | 294400 | 1432.94 | 286588 | 0 | 30 | 3593 | 287530 | 2 | 0 | 3 (3) | 6 |
| (Note 3) |  |  | Mid | 1474.5 | 294900 | 1398.72 | 279744 | 102 |  | 3600 | 288030 | 2 | 0 | 3 (3) | 210 |
|  |  |  | High | 1477 | 295400 | 1256.5 | 251300 | 504 |  | 3607 | 288530 | 2 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-3: Test frequencies for NR operating band n50 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1437 | 287400 | 1433.04 | 286608 | 0 | 30 | - | 287328 |
|  |  | & | Mid | 1474.5 | 294900 | 1397.1 | 279420 | 102 |  | - | 294828 |
|  |  | Uplink | High | 1512 | 302400 | 1145.16 | 229032 | 504 |  | - | 302328 |
| 15 | 18 | Downlink | Low | 1439.5 | 287900 | 1433.02 | 286604 | 0 | 30 | - | 287324 |
|  |  | & | Mid | 1474.5 | 294900 | 1394.58 | 278916 | 102 |  | - | 294324 |
|  |  | Uplink | High | 1509.5 | 301900 | 1140.14 | 228028 | 504 |  | - | 301324 |
| 20 | 24 | Downlink | Low | 1442 | 288400 | 1433.36 | 286672 | 0 | 30 | - | 287392 |
|  |  | & | Mid | 1474.5 | 294900 | 1392.42 | 278484 | 102 |  | - | 293892 |
|  |  | Uplink | High | 1507 | 301400 | 1135.48 | 227096 | 504 |  | - | 300392 |
| 40 | 51 | Downlink | Low | 1452 | 290400 | 1433.64 | 286728 | 0 | 30 | - | 287448 |
|  |  | & | Mid | 1474.5 | 294900 | 1382.7 | 276540 | 102 |  | - | 291948 |
|  |  | Uplink | High | 1497 | 299400 | 1115.76 | 223152 | 504 |  | - | 296448 |
| 50 | 65 | Downlink | Low | 1457 | 291400 | 1433.6 | 286720 | 0 | 30 | - | 287440 |
|  |  | & | Mid | 1474.5 | 294900 | 1377.66 | 275532 | 102 |  | - | 290940 |
|  |  | Uplink | High | 1492 | 298400 | 1105.72 | 221144 | 504 |  | - | 294440 |
| 60 | 79 | Downlink | Low | 1462 | 292400 | 1433.56 | 286712 | 0 | 30 | - | 287432 |
|  |  | & | Mid | 1474.5 | 294900 | 1372.62 | 274524 | 102 |  | - | 289932 |
|  |  | Uplink | High | 1487 | 297400 | 1095.68 | 219136 | 504 |  | - | 292432 |
| 80 | 107 | Downlink | Low | 1472 | 294400 | 1433.48 | 286696 | 0 | 30 | - | 287416 |
| (Note 2) |  |  | Mid | 1474.5 | 294900 | 1362.54 | 272508 | 102 |  | - | 287916 |
|  |  |  | High | 1477 | 295400 | 1075.6 | 215120 | 504 |  | - | 288416 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.51	Reference test frequencies for NR operating band n51

Table 4.3.1.1.1.51-1: Test frequencies for NR operating band n51 and SCS 15 kHz

| [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.52	FFS

###### 4.3.1.1.1.53	Reference test frequencies for NR operating band n53

Table 4.3.1.1.1.53-1: Test frequencies for NR operating band n53 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2486 | 497200 | 2483.75 | 496750 | 0 | 15 | 6215 | 497290 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2468.69 | 493738 | 102 |  | 6221 | 497770 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2492.5 | 498500 | 2399.53 | 479906 | 504 |  | 6232 | 498530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6216 | 497310 | 2 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2466.6 | 493252 | 102 |  | 6215 | 497290 | 2 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2490 | 498000 | 2394.6 | 478920 | 504 |  | 6219 | 497550 | 6 | 1 | 1 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-2: Test frequencies for NR operating band n53 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2488.5 | 497700 | 2484.18 | 496836 | 0 | 30 | 6222 | 497790 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2489.3 | 497860 | 2448.26 | 489652 | 102 |  | 6221 | 497770 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2490 | 498000 | 2304.24 | 460848 | 504 |  | 6225 | 498030 | 10 | 0 | 2 (2) | 1012 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-3: Test frequencies for NR operating band n53 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2488.5 | 497700 | 2484.54 | 496908 | 0 | 15 | - | 497268 |
|  |  | & | Mid | 2489.3 | 497860 | 2411.9 | 482380 | 102 |  | - | 497428 |
|  |  | Uplink | High | 2490 | 498000 | 2123.16 | 424632 | 504 |  | - | 497568 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.54	Reference test frequencies for NR operating band n54

Table 4.3.1.1.1.54-1: Test frequencies for NR operating band n54 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1672.5 | 334500 | 1670.25 | 334050 | 0 | 15 | 4182 | 334590 | 0 | 1 | 2(4) | 5 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.55 to 4.3.1.1.1.64	FFS

###### 4.3.1.1.1.65	Reference test frequencies for NR operating band n65

Table 4.3.1.1.1.65-1: Test frequencies for NR operating band n65 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2134.39 | 426878 | 102 |  | 5389 | 431090 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 2197.5 | 439500 | 2104.53 | 420906 | 504 |  | 5493 | 439470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1872.03 | 374406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2007.5 | 401500 | 2004.17 | 400834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2131.96 | 426392 | 102 |  | 5383 | 430610 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 2195 | 439000 | 2099.6 | 419920 | 504 |  | 5480 | 438490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1869.6 | 373920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1999.24 | 399848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2129.53 | 425906 | 102 |  | 5377 | 430130 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1867.17 | 373434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1994.31 | 398862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2155 | 431000 | 2127.1 | 425420 | 102 |  | 5371 | 429650 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 2190 | 438000 | 2089.74 | 417948 | 504 |  | 5457 | 436590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1864.74 | 372948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1989.38 | 397876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.65-2: Test frequencies for NR operating band n65 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2155 | 431000 | 2113.96 | 422792 | 102 |  | 5389 | 431090 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 2195 | 439000 | 2009.24 | 401848 | 504 |  | 5486 | 438970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1779.24 | 355848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1998.52 | 399704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2155 | 431000 | 2111.44 | 422288 | 102 |  | 5380 | 430370 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 2192.5 | 438500 | 2004.22 | 400844 | 504 |  | 5476 | 438050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1776.72 | 355344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1993.5 | 398700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2155 | 431000 | 2109.1 | 421820 | 102 |  | 5374 | 429890 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 2190 | 438000 | 1999.38 | 399876 | 504 |  | 5463 | 437070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1774.38 | 354876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1988.66 | 397732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.65-3: Test frequencies for NR operating band n65 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  | Mid | 2155 | 431000 | 2077.6 | 415520 | 102 |  | - | 430568 |
|  |  |  | High | 2195 | 439000 | 1828.16 | 365632 | 504 |  | - | 438568 |
|  |  | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1598.16 | 319632 | 504 |  | - | - |
|  |  |  | High | 2005 | 401000 | 1996.72 | 399344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  | Mid | 2155 | 431000 | 2075.08 | 415016 | 102 |  | - | 430064 |
|  |  |  | High | 2192.5 | 438500 | 1823.14 | 364628 | 504 |  | - | 437564 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1595.64 | 319128 | 504 |  | - | - |
|  |  |  | High | 2002.5 | 400500 | 1991.7 | 398340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  | Mid | 2155 | 431000 | 2072.92 | 414584 | 102 |  | - | 429632 |
|  |  |  | High | 2190 | 438000 | 1818.48 | 363696 | 504 |  | - | 436632 |
|  |  | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1593.48 | 318696 | 504 |  | - | - |
|  |  |  | High | 2000 | 400000 | 1987.04 | 397408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.66	Reference test frequencies for NR operating band n66

Table 4.3.1.1.1.66-1: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 15 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2124.39 | 424878 | 102 |  | 5361 | 428910 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  | 5443 | 435410 | 0 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2152.5 | 430500 | 2124.6 | 424920 | 102 |  | 5364 | 429150 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2084.74 | 416948 | 504 |  | 5446 | 435650 | 6 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2124.67 | 424934 | 102 |  | 5365 | 429170 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2187.5 | 437500 | 2084.81 | 416962 | 504 |  | 5444 | 435610 | 0 | 0 | 2 (4) | 508 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1742.5 | 348500 | 1649.53 | 329906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1769.17 | 353834 | 6 |  | - | - | - | - | - | - |
| 5/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1737.5 | 347500 | 1644.53 | 328906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1762.5 | 352500 | 1759.17 | 351834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2122.1 | 424420 | 102 |  | 5357 | 428650 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2180 | 436000 | 2079.74 | 415948 | 504 |  | 5432 | 434650 | 6 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2122.17 | 424434 | 102 |  | 5358 | 428670 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2182.5 | 436500 | 2079.81 | 415962 | 504 |  | 5433 | 434670 | 8 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1644.6 | 328920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1759.24 | 351848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  | 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2114.67 | 422934 | 102 |  | 5337 | 426990 | 8 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2167.5 | 433500 | 2064.81 | 412962 | 504 |  | 5394 | 431550 | 4 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2114.7 | 422940 | 102 |  | 5337 | 426990 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2064.84 | 412968 | 504 |  | 5394 | 431550 | 2 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2112.24 | 422448 | 102 |  | 5331 | 426510 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2165 | 433000 | 2059.88 | 411976 | 504 |  | 5381 | 430570 | 6 | 0 | 1 (2) | 506 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2112.2 | 422440 | 102 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2059.84 | 411968 | 504 |  | 5383 | 430610 | 10 | 1 | 1 (2) | 507 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  | 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
| N.A./40 (Note4) | 40 | 216 | Downlink | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
| 45/45 | 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2104.86 | 420972 | 102 |  | 5313 | 425070 | 10 | 1 | 0 (0) | 103 |
|  |  |  |  | High | 2157.5 | 431500 | 2045 | 409000 | 504 |  | 5345 | 427690 | 2 | 1 | 2 (4) | 509 |
|  | 45 | 242 | Uplink | Low | 1732.5 | 346500 | 1710.72 | 342144 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1632.5 | 326500 | 504 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1734.64 | 346928 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. Note 4: Used only for non-contiguous CA x+40 combination and CA x+x+40 combination with no UL configuration on the SB in Table 4.3.1.1.5.66.1-1 and Table 4.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-1A: Void

Table 4.3.1.1.1.66-2: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 30 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  | 5361 | 428910 | 18 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  | 5436 | 434910 | 18 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2104.1 | 420820 | 102 |  | 5360 | 428890 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1989.38 | 397876 | 504 |  | 5435 | 434890 | 2 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2104.08 | 420816 | 102 |  | 5361 | 428910 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2182.5 | 436500 | 1989.36 | 397872 | 504 |  | 5436 | 434910 | 10 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2104.24 | 420848 | 102 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2185 | 437000 | 1989.52 | 397904 | 504 |  | 5437 | 434930 | 6 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1554.24 | 310848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1758.52 | 351704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  | 5355 | 428430 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  | 5423 | 433930 | 2 | 0 | 1 (6) | 1020 |
|  | 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  | 5349 | 427950 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  | 5413 | 433010 | 18 | 0 | 1 (6) | 1020 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2096.58 | 419316 | 102 |  | 5343 | 427470 | 6 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2167.5 | 433500 | 1974.36 | 394872 | 504 |  | 5400 | 432030 | 2 | 0 | 2 (7) | 1022 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2096.7 | 419340 | 102 |  | 5343 | 427470 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1974.48 | 394896 | 504 |  | 5400 | 432030 | 18 | 0 | 1 (6) | 1020 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2094.24 | 418848 | 102 |  | 5337 | 426990 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2165 | 433000 | 1969.52 | 393904 | 504 |  | 5387 | 431050 | 22 | 0 | 1 (6) | 1020 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2094.2 | 418840 | 102 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1969.48 | 393896 | 504 |  | 5389 | 431090 | 14 | 0 | 2 (7) | 1022 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2089.2 | 417840 | 102 |  | 5325 | 426030 | 18 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  | 5361 | 428910 | 2 | 0 | 0 (5) | 1018 |
|  | 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  | - | - | - | - | - | - |
| N.A./40 (Note4) | 40 | 106 | Downlink | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
| 45/45 | 45 | 119 | Downlink | Low | 2132.5 | 426500 | 2111.08 | 422216 | 0 | 15 | 5290 | 423170 | 6 | 0 | 3 (8) | 16 |
|  |  |  |  | Mid | 2145 | 429000 | 2086.86 | 417372 | 102 |  | 5319 | 425550 | 14 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2157.5 | 431500 | 1954.64 | 390928 | 504 |  | 5351 | 428170 | 6 | 0 | 3 (8) | 1024 |
|  | 45 | 119 | Uplink | Low | 1732.5 | 346500 | 1711.08 | 342216 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1542.14 | 308428 | 504 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1733.92 | 346784 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. Note 4: Used only for non-contiguous CA x+40 combination and CA x+x+40 combination with no UL configuration on the SB in Table 4.3.1.1.5.66.1-2 and Table 4.3.1.1.5.66.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-2A: Void

Table 4.3.1.1.1.66-3: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 60 kHz without CORESET#0

| UL/DLCBW (Asymmetric BCS) Note 2 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  |  | Mid | 2145 | 429000 | 2067.6 | 413520 | 102 |  | - | 428568 |
|  |  |  |  | High | 2175 | 435000 | 1808.16 | 361632 | 504 |  | - | 434568 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2150 | 430000 | 2067.92 | 413584 | 102 |  | - | 428632 |
|  |  |  |  | High | 2180 | 436000 | 1808.48 | 361696 | 504 |  | - | 434632 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2067.9 | 413580 | 102 |  | - | 428628 |
|  |  |  |  | High | 2182.5 | 436500 | 1808.46 | 361692 | 504 |  | - | 434628 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2067.88 | 413576 | 102 |  | - | 428624 |
|  |  |  |  | High | 2185 | 437000 | 1808.44 | 361688 | 504 |  | - | 434624 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1373.16 | 274632 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1756.72 | 351344 | 6 |  | - | - |
| 15/15 | 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  |  | Mid | 2145 | 429000 | 2065.08 | 413016 | 102 |  | - | 428064 |
|  |  |  |  | High | 2172.5 | 434500 | 1803.14 | 360628 | 504 |  | - | 433564 |
|  | 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |  | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |  | - | - |
| 20/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2145 | 429000 | 2062.92 | 412584 | 102 |  | - | 427632 |
|  |  |  |  | High | 2170 | 434000 | 1798.48 | 359696 | 504 |  | - | 432632 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 20/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 25/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  |  | Mid | 2145 | 429000 | 2060.4 | 412080 | 102 |  | - | 427128 |
|  |  |  |  | High | 2167.5 | 433500 | 1793.46 | 358692 | 504 |  | - | 431628 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 25/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2060.7 | 412140 | 102 |  | - | 427188 |
|  |  |  |  | High | 2175 | 435000 | 1793.76 | 358752 | 504 |  | - | 431688 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 30/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  |  | Mid | 2145 | 429000 | 2057.88 | 411576 | 102 |  | - | 426624 |
|  |  |  |  | High | 2165 | 433000 | 1788.44 | 357688 | 504 |  | - | 430624 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| 30/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2058.2 | 411640 | 102 |  | - | 426688 |
|  |  |  |  | High | 2170 | 434000 | 1788.76 | 357752 | 504 |  | - | 430688 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| 40/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2145 | 429000 | 2053.2 | 410640 | 102 |  | - | 425688 |
|  |  |  |  | High | 2160 | 432000 | 1778.76 | 355752 | 504 |  | - | 428688 |
|  | 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |  | - | - |
|  |  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 6 |  | - | - |
| 45/45 | 45 | 58 | Downlink | Low | 2132.5 | 426500 | 2111.62 | 422324 | 0 | 15 | - | 422684 |
|  |  |  |  | Mid | 2145 | 429000 | 2050.68 | 410136 | 102 | 15 | - | 425184 |
|  |  |  |  | High | 2157.5 | 431500 | 1773.74 | 354748 | 504 | 15 | - | 427684 |
|  | 45 | 58 | Uplink | Low | 1732.5 | 346500 | 1711.62 | 342324 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1361.24 | 272248 | 504 | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1732.3 | 346460 | 6 | - | - | - |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2 Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-3A: Void

###### 4.3.1.1.1.67 – 4.3.1.1.1.69	FFS

###### 4.3.1.1.1.70	Reference test frequencies for NR operating band n70

Table 4.3.1.1.1.70-1: Test frequencies for NR operating band n70 and SCS 15 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 1997.5 | 399500 | 1995.25 | 399050 | 0 | 15 | 4993 | 399410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1981.89 | 396378 | 102 |  | 5007 | 400590 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2007.5 | 401500 | 1914.53 | 382906 | 504 |  | 5018 | 401530 | 4 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1609.53 | 321906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1707.5 | 341500 | 1704.17 | 340834 | 6 |  | - | - | - | - | - | - |
| 5/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1982.03 | 396406 | 102 |  | 5006 | 400570 | 8 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2012.5 | 402500 | 1914.67 | 382934 | 504 |  | 5020 | 401570 | 8 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1609.53 | 321906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1707.5 | 341500 | 1704.17 | 340834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1979.6 | 395920 | 102 |  | 5000 | 400090 | 10 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2010 | 402000 | 1909.74 | 381948 | 504 |  | 5007 | 400590 | 10 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1700 | 340000 | 1607.03 | 321406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1702.5 | 340500 | 1699.17 | 339834 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/10 | 10 | 52 | Downlink | Low | 2000 | 400000 | 1995.32 | 399064 | 0 | 15 | 4994 | 399610 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1979.46 | 395892 | 102 |  | 5001 | 400110 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2005 | 401000 | 1909.6 | 381920 | 504 |  | 5008 | 400610 | 2 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2005 | 401000 | 1979.53 | 395906 | 102 |  | 5002 | 400130 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2007.5 | 401500 | 1909.67 | 381934 | 504 |  | 5006 | 400570 | 8 | 1 | 1 (2) | 507 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1979.6 | 395920 | 102 |  | 5000 | 400090 | 10 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2010 | 402000 | 1909.74 | 381948 | 504 |  | 5007 | 400590 | 10 | 1 | 1 (2) | 507 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 5/25 MHz, 10/25 MHz, 15/15 MHz, 15/20 MHz and 15/25 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.70-2: Test frequencies for NR operating band n70 and SCS 30 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2000 | 400000 | 1995.68 | 399136 | 0 | 15 | 5000 | 400090 | 6 | 0 | 3 (8) | 16 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1961.46 | 392292 | 102 |  | 5007 | 400590 | 6 | 0 | 3 (8) | 220 |
|  |  |  |  | High | 2005 | 401000 | 1819.24 | 363848 | 504 |  | 5014 | 401090 | 6 | 0 | 3 (8) | 1024 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/15 | 15 | 38 | Downlink | Low | 2002.5 | 400500 | 1995.66 | 399132 | 0 | 15 | 4998 | 399870 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2005 | 401000 | 1961.44 | 392288 | 102 |  | 5005 | 400370 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2007.5 | 401500 | 1819.22 | 363844 | 504 |  | 5012 | 401050 | 18 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2005 | 401000 | 1995.82 | 399164 | 0 | 15 | 4999 | 399890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1961.6 | 392320 | 102 |  | 5006 | 400570 | 14 | 0 | 2 (7) | 218 |
|  |  |  |  | High | 2010 | 402000 | 1819.38 | 363876 | 504 |  | 5013 | 401070 | 14 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 2007.5 | 401500 | 1995.8 | 399160 | 0 | 15 | 5000 | 400090 | 22 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 38 | Downlink | Low | 2002.5 | 400500 | 1995.66 | 399132 | 0 | 15 | 4998 | 399870 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 51 | Downlink | Low | 2005 | 401000 | 1995.82 | 399164 | 0 | 15 | 4999 | 399890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 65 | Downlink | Low | 2007.5 | 401500 | 1995.8 | 399160 | 0 | 15 | 5000 | 400090 | 22 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 10/25 MHz, 15/15 MHz, 15/20 MHz and 15/25 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.70-3: Test frequencies for NR operating band n70 and SCS 60 kHz without CORESET#0

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 11 | Downlink | Low | 2000 | 400000 | 1996.04 | 399208 | 0 | 15 | - | 399568 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1925.1 | 385020 | 102 |  | - | 400068 |
|  |  |  |  | High | 2005 | 401000 | 1638.16 | 327632 | 504 |  | - | 400568 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/15 | 15 | 18 | Downlink | Low | 2002.5 | 400500 | 1996.02 | 399204 | 0 | 15 | - | 399564 |
|  |  |  |  | Mid | 2005 | 401000 | 1925.08 | 385016 | 102 |  | - | 400064 |
|  |  |  |  | High | 2007.5 | 401500 | 1638.14 | 327628 | 504 |  | - | 400564 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/20 | 20 | 24 | Downlink | Low | 2005 | 401000 | 1996.36 | 399272 | 0 | 15 | - | 399632 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1925.42 | 385084 | 102 |  | - | 400132 |
|  |  |  |  | High | 2010 | 402000 | 1638.48 | 327696 | 504 |  | - | 400632 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/25 | 25 | 31 | Downlink | Low | 2007.5 | 401500 | 1996.34 | 399268 | 0 | 15 | - | 399628 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 18 | Downlink | Low | 2002.5 | 400500 | 1996.02 | 399204 | 0 | 15 | - | 399564 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 24 | Downlink | Low | 2005 | 401000 | 1996.36 | 399272 | 0 | 15 | - | 399632 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 31 | Downlink | Low | 2007.5 | 401500 | 1996.34 | 399268 | 0 | 15 | - | 399628 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: 10/25 MHz, 15/15 MHz, 15/20 MHz and 15/25 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.71	Reference test frequencies for NR operating band n71

Table 4.3.1.1.1.71-1: Test frequencies for NR operating band n71 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 619.5 | 123900 | 617.25 | 123450 | 0 | 15 | 1548 | 123870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 613.89 | 122778 | 102 |  | 1587 | 126990 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 649.5 | 129900 | 556.53 | 111306 | 504 |  | 1623 | 129870 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 587.53 | 117506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 695.5 | 139100 | 692.17 | 138434 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 622 | 124400 | 617.32 | 123464 | 0 | 15 | 1549 | 123890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 611.46 | 122292 | 102 |  | 1581 | 126510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  | 25 | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 585.03 | 117006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 687.17 | 137434 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 622 | 124400 | 617.32 | 123464 | 0 | 15 | 1549 | 123890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 611.46 | 122292 | 102 |  | 1581 | 126510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 585.1 | 117020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 687.24 | 137448 | 6 |  | - | - | - | - | - | - |
| 10/15 | 79 | Downlink | Low | 624.5 | 124900 | 617.39 | 123478 | 0 | 15 | 1547 | 123850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 609.03 | 121806 | 102 |  | 1575 | 126030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 582.6 | 116520 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 682.24 | 136448 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 624.5 | 124900 | 617.39 | 123478 | 0 | 15 | 1547 | 123850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 609.03 | 121806 | 102 |  | 1575 | 126030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 582.67 | 116534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 682.31 | 136462 | 6 |  | - | - | - | - | - | - |
| 15/20 | 106 | Downlink | Low | 627 | 125400 | 617.46 | 123492 | 0 | 15 | 1548 | 123870 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 606.6 | 121320 | 102 |  | 1569 | 125550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  | 79 | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 580.17 | 116034 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 677.31 | 135462 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 627 | 125400 | 617.46 | 123492 | 0 | 15 | 1548 | 123870 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 606.6 | 121320 | 102 |  | 1569 | 125550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 673 | 134600 | 663.46 | 132692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 677.38 | 135476 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 629.5 | 125900 | 617.53 | 123506 | 0 | 15 | 1549 | 123890 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 604.17 | 120834 | 102 |  | 1563 | 125070 | 8 | 1 | 2 (4) | 107 |
|  |  |  | High | 639.5 | 127900 | 536.81 | 107362 | 504 |  | 1574 | 126010 | 0 | 0 | 2 (4) | 508 |
|  | 106 | Uplink | Low | 675.5 | 135100 | 665.96 | 133192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.88 | 134976 | 6 |  | - | - | - | - | - | - |
| 20/30 | 160 | Downlink | Low | 632 | 126400 | 617.6 | 123520 | 0 | 15 | 1550 | 124090 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 634.5 | 126900 | 601.74 | 120348 | 102 |  | 1557 | 124590 | 10 | 1 | 2 (4) | 107 |
|  |  |  | High | 637 | 127400 | 531.88 | 106376 | 504 |  | 1564 | 125090 | 10 | 1 | 2 (4) | 509 |
|  | 106 | Uplink | Low | 678 | 135600 | 668.46 | 133692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 683 | 136600 | 672.38 | 134476 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 634.5 | 126900 | 617.58 | 123516 | 0 | 15 | 1551 | 124110 | 6 | 2 | 2 (4) | 6 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 | Uplink | Low | 680.5 | 136100 | 670.96 | 134192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.71-2: Test frequencies for NR operating band n71 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 622 | 124400 | 617.68 | 123536 | 0 | 15 | 1555 | 124370 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 593.46 | 118692 | 102 |  | 1587 | 126990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 647 | 129400 | 461.24 | 92248 | 504 |  | 1616 | 129370 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 494.74 | 98948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 686.52 | 137304 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 624.5 | 124900 | 617.66 | 123532 | 0 | 15 | 1553 | 124330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 590.94 | 118188 | 102 |  | 1578 | 126270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 492.24 | 98448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 681.52 | 136304 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 624.5 | 124900 | 617.66 | 123532 | 0 | 15 | 1553 | 124330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 590.94 | 118188 | 102 |  | 1578 | 126270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 492.22 | 98444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 681.5 | 136300 | 6 |  | - | - | - | - | - | - |
| 15/20 | 51 | Downlink | Low | 627 | 125400 | 617.82 | 123564 | 0 | 15 | 1554 | 124350 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 588.6 | 117720 | 102 |  | 1572 | 125790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  | 38 | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 678 | 135600 | 489.72 | 97944 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 676.5 | 135300 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 627 | 125400 | 617.82 | 123564 | 0 | 15 | 1554 | 124350 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 588.6 | 117720 | 102 |  | 1572 | 125790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 673 | 134600 | 663.82 | 132764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 676.66 | 135332 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 629.5 | 125900 | 617.8 | 123560 | 0 | 15 | 1555 | 124370 | 6 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 586.08 | 117216 | 102 |  | 1566 | 125310 | 10 | 0 | 0 (5) | 214 |
|  |  |  | High | 639.5 | 127900 | 446.36 | 89272 | 504 |  | 1580 | 126490 | 22 | 0 | 2 (7) | 1022 |
|  | 51 | Uplink | Low | 675.5 | 135100 | 666.32 | 133264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.16 | 134832 | 6 |  | - | - | - | - | - | - |
| 20/30 | 78 | Downlink | Low | 632 | 126400 | 617.96 | 123592 | 0 | 15 | 1553 | 124330 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 583.74 | 116748 | 102 |  | 1560 | 124830 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 637 | 127400 | 441.52 | 88304 | 504 |  | 1567 | 125330 | 6 | 0 | 0 (5) | 1018 |
|  | 51 | Uplink | Low | 678 | 135600 | 668.82 | 133764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 683 | 136600 | 671.66 | 134332 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 634.5 | 126900 | 617.94 | 123588 | 0 | 15 | 1554 | 124350 | 14 | 0 | 0 (5) | 10 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | Uplink | Low | 680.5 | 136100 | 671.32 | 134264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.72	Reference test frequencies for NR operating band n72

Table 4.3.1.1.1.72-1: Test frequencies for NR operating band n72 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 462.5 | 92500 | 461.15 | 92230 | 0 | 15 | 28947 | 92470 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 463.5 | 92700 | 443.79 | 88758 | 102 |  | 28954 | 92730 | 4 | 0 | 3 (2) | 104 |
|  |  |  | High | 464.5 | 92900 | 372.43 | 74486 | 504 |  | 28958 | 92870 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 452.5 | 90500 | 451.15 | 90230 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 453.5 | 90700 | 361.43 | 72286 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 454.5 | 90900 | 452.07 | 90414 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 463.5 | 92700 | 461.25 | 92250 | 102 | 15 | 1158 | 92670 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 453.5 | 90700 | 451.25 | 90250 | 504 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.73	FFS

###### 4.3.1.1.1.74	Reference test frequencies for NR operating band n74

Table 4.3.1.1.1.74-1: Test frequencies for NR operating band n74 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1477.5 | 295500 | 1475.25 | 295050 | 0 | 15 | 3693 | 295470 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1496.5 | 299300 | 1475.89 | 295178 | 102 |  | 3742 | 299330 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 1515.5 | 303100 | 1422.53 | 284506 | 504 |  | 3788 | 303130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1355.53 | 271106 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1467.5 | 293500 | 1464.17 | 292834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1480 | 296000 | 1475.32 | 295064 | 0 | 15 | 3694 | 295490 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1496.5 | 299300 | 1473.46 | 294692 | 102 |  | 3736 | 298850 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 1513 | 302600 | 1417.6 | 283520 | 504 |  | 3778 | 302210 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1432 | 286400 | 1427.32 | 285464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1353.1 | 270620 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1465 | 293000 | 1459.24 | 291848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1482.5 | 296500 | 1475.39 | 295078 | 0 | 15 | 3692 | 295450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1496.5 | 299300 | 1471.03 | 294206 | 102 |  | 3730 | 298370 | 8 | 1 | 1 (2) | 105 |
|  |  |  | High | 1510.5 | 302100 | 1412.67 | 282534 | 504 |  | 3765 | 301230 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1427.39 | 285478 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1350.67 | 270134 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1462.5 | 292500 | 1454.31 | 290862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1485 | 297000 | 1475.46 | 295092 | 0 | 15 | 3693 | 295470 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1496.5 | 299300 | 1468.6 | 293720 | 102 |  | 3724 | 297890 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 1508 | 301600 | 1407.74 | 281548 | 504 |  | 3752 | 300250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1437 | 287400 | 1427.46 | 285492 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1348.24 | 269648 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1460 | 292000 | 1449.38 | 289876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.74-2: Test frequencies for NR operating band n74 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1480 | 296000 | 1475.68 | 295136 | 0 | 15 | 3700 | 295970 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1496.5 | 299300 | 1455.46 | 291092 | 102 |  | 3742 | 299330 | 10 | 0 | 2 (7) | 218 |
|  |  |  | High | 1513 | 302600 | 1327.24 | 265448 | 504 |  | 3784 | 302690 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1432 | 286400 | 1427.68 | 285536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1262.74 | 252548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1465 | 293000 | 1458.52 | 291704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1482.5 | 296500 | 1475.66 | 295132 | 0 | 15 | 3698 | 295930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1496.5 | 299300 | 1452.94 | 290588 | 102 |  | 3736 | 298850 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 1510.5 | 302100 | 1322.22 | 264444 | 504 |  | 3768 | 301470 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1427.66 | 285532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1260.22 | 252044 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1462.5 | 292500 | 1453.5 | 290700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1485 | 297000 | 1475.82 | 295164 | 0 | 15 | 3699 | 295950 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1496.5 | 299300 | 1450.6 | 290120 | 102 |  | 3730 | 298370 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 1508 | 301600 | 1317.38 | 263476 | 504 |  | 3755 | 300490 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1437 | 287400 | 1427.82 | 285564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1257.88 | 251576 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1460 | 292000 | 1448.66 | 289732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.74-3: Test frequencies for NR operating band n74 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1480 | 296000 | 1476.04 | 295208 | 0 | 15 | - | 295568 |
|  |  |  | Mid | 1496.5 | 299300 | 1419.1 | 283820 | 102 |  | - | 298868 |
|  |  |  | High | 1513 | 302600 | 1146.16 | 229232 | 504 |  | - | 302168 |
|  |  | Uplink | Low | 1432 | 286400 | 1428.04 | 285608 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1081.66 | 216332 | 504 |  | - | - |
|  |  |  | High | 1465 | 293000 | 1456.72 | 291344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1482.5 | 296500 | 1476.02 | 295204 | 0 | 15 | - | 295564 |
|  |  |  | Mid | 1496.5 | 299300 | 1416.58 | 283316 | 102 |  | - | 298364 |
|  |  |  | High | 1510.5 | 302100 | 1141.14 | 228228 | 504 |  | - | 301164 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1428.02 | 285604 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1079.14 | 215828 | 504 |  | - | - |
|  |  |  | High | 1462.5 | 292500 | 1451.7 | 290340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1485 | 297000 | 1476.36 | 295272 | 0 | 15 | - | 295632 |
|  |  |  | Mid | 1496.5 | 299300 | 1414.42 | 282884 | 102 |  | - | 297932 |
|  |  |  | High | 1508 | 301600 | 1136.48 | 227296 | 504 |  | - | 300232 |
|  |  | Uplink | Low | 1437 | 287400 | 1428.36 | 285672 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1076.98 | 215396 | 504 |  | - | - |
|  |  |  | High | 1460 | 292000 | 1447.04 | 289408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.75	Reference test frequencies for NR operating band n75 (SDL)

Table 4.3.1.1.1.75-1: Test frequencies for NR operating band n75 and SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | - | 286810 |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | - | 294810 |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | - | 302810 |
| 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | - | 286824 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | - | 294324 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | - | 301824 |
| 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | - | 286838 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | - | 293838 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | - | 300838 |
| 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | - | 286852 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | - | 293352 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | - | 299852 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.75-2: Test frequencies for NR operating band n75 and SCS 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | - | 286896 |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | - | 294396 |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | - | 301896 |
| 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | - | 286892 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | - | 293892 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | - | 300892 |
| 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | - | 286924 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | - | 293424 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | - | 299924 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.75-3: Test frequencies for NR operating band n75 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1437 | 287400 | 1433.04 | 286608 | 0 | 15 | - | 286968 |
|  |  |  | Mid | 1474.5 | 294900 | 1397.1 | 279420 | 102 |  | - | 294468 |
|  |  |  | High | 1512 | 302400 | 1145.16 | 229032 | 504 |  | - | 301968 |
| 15 | 18 | Downlink | Low | 1439.5 | 287900 | 1433.02 | 286604 | 0 | 15 | - | 286964 |
|  |  |  | Mid | 1474.5 | 294900 | 1394.58 | 278916 | 102 |  | - | 293964 |
|  |  |  | High | 1509.5 | 301900 | 1140.14 | 228028 | 504 |  | - | 300964 |
| 20 | 24 | Downlink | Low | 1442 | 288400 | 1433.36 | 286672 | 0 | 15 | - | 287032 |
|  |  |  | Mid | 1474.5 | 294900 | 1392.42 | 278484 | 102 |  | - | 293532 |
|  |  |  | High | 1507 | 301400 | 1135.48 | 227096 | 504 |  | - | 300032 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.76	Reference test frequencies for NR operating band n76 (SDL)

Table 4.3.1.1.1.76-1: Test frequencies for NR operating band n76 and SCS 15 kHz without CORESET#0

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low, Mid, High | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | - | 285810 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.77	Reference test frequencies for NR operating band n77

Table 4.3.1.1.1.77-1: Test frequencies for NR operating band n77, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3750 | 650000 | 3726.96 | 648464 | 102 |  | 8020 | 650016 | 4 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 4194.99 | 679666 | 4099.59 | 673306 | 504 |  | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 15 | 79 | Downlink | Low | 3307.5 | 620500 | 3300.39 | 620026 | 0 | 30 | 7711 | 620352 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3750 | 650000 | 3724.53 | 648302 | 102 |  | 8018 | 649824 | 10 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 4192.5 | 679500 | 4094.67 | 672978 | 504 |  | 8325 | 679296 | 6 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 3310.02 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3722.1 | 648140 | 102 |  | 8016 | 649632 | 4 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 4189.98 | 679332 | 4089.72 | 672648 | 504 |  | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 25 | 133 | Downlink | Low | 3312.51 | 620834 | 3300.54 | 620036 | 0 | 30 | 7711 | 620352 | 4 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3719.67 | 647978 | 102 |  | 8015 | 649536 | 10 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 4187.49 | 679166 | 4084.8 | 672320 | 504 |  | 8319 | 678720 | 4 | 3 | 1 (6) | 513 |
| 30 | 160 | Downlink | Low | 3315 | 621000 | 3300.6 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3717.24 | 647816 | 102 |  | 8013 | 649344 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 4185 | 679000 | 4079.88 | 671992 | 504 |  | 8315 | 678336 | 8 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3712.2 | 647480 | 102 |  | 8010 | 649056 | 4 | 3 | 1 (6) | 111 |
|  |  | Uplink | High | 4179.99 | 678666 | 4069.83 | 671322 | 504 |  | 8308 | 677664 | 6 | 2 | 0 (2) | 508 |
| 50 | 270 | Downlink | Low | 3325.02 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 3750 | 650000 | 3707.34 | 647156 | 102 |  | 8006 | 648672 | 4 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 4174.98 | 678332 | 4059.96 | 670664 | 504 |  | 8301 | 676992 | 4 | 1 | 0 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.77-2: Test frequencies for NR operating band n77, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3708.96 | 647264 | 102 |  | 8020 | 650016 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 4194.99 | 679666 | 4009.23 | 667282 | 504 |  | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 15 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 | 7711 | 620352 | 20 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3706.44 | 647096 | 102 |  | 8018 | 649824 | 16 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4192.5 | 679500 | 4004.22 | 666948 | 504 |  | 8325 | 679296 | 12 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3704.1 | 646940 | 102 |  | 8016 | 649632 | 4 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 4189.98 | 679332 | 3999.36 | 666624 | 504 |  | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 25 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 | 7711 | 620352 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3701.58 | 646772 | 102 |  | 8015 | 649536 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4187.49 | 679166 | 3994.35 | 666290 | 504 |  | 8319 | 678720 | 22 | 0 | 3 (3) | 1014 |
| 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3699.24 | 646616 | 102 |  | 8013 | 649344 | 16 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4185 | 679000 | 3989.52 | 665968 | 504 |  | 8315 | 678336 | 8 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3694.2 | 646280 | 102 |  | 8010 | 649056 | 16 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4179.99 | 678666 | 3979.47 | 665298 | 504 |  | 8308 | 677664 | 6 | 0 | 1 (1) | 1010 |
| 50 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3750 | 650000 | 3689.34 | 645956 | 102 |  | 8006 | 648672 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4174.98 | 678332 | 3969.6 | 664640 | 504 |  | 8301 | 676992 | 16 | 0 | 0 (0) | 1008 |
| 60 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3684.12 | 645608 | 102 |  | 8003 | 648384 | 16 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4170 | 678000 | 3959.4 | 663960 | 504 |  | 8294 | 676320 | 0 | 0 | 1 (1) | 1010 |
| 70 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3750 | 650000 | 3679.26 | 645284 | 102 |  | 7999 | 648000 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4164.99 | 677666 | 3949.53 | 663302 | 504 |  | 8287 | 675648 | 10 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3674.22 | 644948 | 102 |  | 7996 | 647712 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4159.98 | 677332 | 3939.48 | 662632 | 504 |  | 8280 | 674976 | 8 | 0 | 0 (0) | 1008 |
| 90 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3669.18 | 644612 | 102 |  | 7992 | 647328 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4155 | 677000 | 3929.46 | 661964 | 504 |  | 8273 | 674304 | 4 | 0 | 0 (0) | 1008 |
| 100 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3664.14 | 644276 | 102 |  | 7989 | 647040 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4149.99 | 676666 | 3919.41 | 661294 | 504 |  | 8266 | 673632 | 2 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.77-3: Test frequencies for NR operating band n77, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3305.01 | 620334 | 3301.05 | 620070 | 0 | 30 | - | 620310 |
|  |  | & | Mid | 3750 | 650000 | 3672.6 | 644840 | 102 |  | - | 649976 |
|  |  | Uplink | High | 4194.99 | 679666 | 3828.15 | 655210 | 504 |  | - | 679642 |
| 15 | 18 | Downlink | Low | 3307.5 | 620500 | 3301.02 | 620068 | 0 | 30 | - | 620308 |
|  |  | & | Mid | 3750 | 650000 | 3670.08 | 644672 | 102 |  | - | 649808 |
|  |  | Uplink | High | 4192.5 | 679500 | 3823.14 | 654876 | 504 |  | - | 679308 |
| 20 | 24 | Downlink | Low | 3310.02 | 620668 | 3301.38 | 620092 | 0 | 30 | - | 620332 |
|  |  | & | Mid | 3750 | 650000 | 3667.92 | 644528 | 102 |  | - | 649664 |
|  |  | Uplink | High | 4189.98 | 679332 | 3818.46 | 654564 | 504 |  | - | 678996 |
| 25 | 31 | Downlink | Low | 3312.51 | 620834 | 3301.35 | 620090 | 0 | 30 | - | 620330 |
|  |  | & | Mid | 3750 | 650000 | 3665.4 | 644360 | 102 |  | - | 649496 |
|  |  | Uplink | High | 4187.49 | 679166 | 3813.45 | 654230 | 504 |  | - | 678662 |
| 30 | 38 | Downlink | Low | 3315 | 621000 | 3301.32 | 620088 | 0 | 30 | - | 620328 |
|  |  | & | Mid | 3750 | 650000 | 3662.88 | 644192 | 102 |  | - | 649328 |
|  |  | Uplink | High | 4185 | 679000 | 3808.44 | 653896 | 504 |  | - | 678328 |
| 40 | 51 | Downlink | Low | 3320.01 | 621334 | 3301.65 | 620110 | 0 | 30 | - | 620350 |
|  |  | & | Mid | 3750 | 650000 | 3658.2 | 643880 | 102 |  | - | 649016 |
|  |  | Uplink | High | 4179.99 | 678666 | 3798.75 | 653250 | 504 |  | - | 677682 |
| 50 | 65 | Downlink | Low | 3325.02 | 621668 | 3301.62 | 620108 | 0 | 30 | - | 620348 |
|  |  | & | Mid | 3750 | 650000 | 3653.16 | 643544 | 102 |  | - | 648680 |
|  |  | Uplink | High | 4174.98 | 678332 | 3788.7 | 652580 | 504 |  | - | 677012 |
| 60 | 79 | Downlink | Low | 3330 | 622000 | 3301.56 | 620104 | 0 | 30 | - | 620344 |
|  |  | & | Mid | 3750 | 650000 | 3648.12 | 643208 | 102 |  | - | 648344 |
|  |  | Uplink | High | 4170 | 678000 | 3778.68 | 651912 | 504 |  | - | 676344 |
| 70 | 93 | Downlink | Low | 3335.01 | 622334 | 3301.53 | 620102 | 0 | 30 | - | 620342 |
|  |  | & | Mid | 3750 | 650000 | 3643.08 | 642872 | 102 |  | - | 648008 |
|  |  | Uplink | High | 4164.99 | 677666 | 3768.63 | 651242 | 504 |  | - | 675674 |
| 80 | 107 | Downlink | Low | 3340.02 | 622668 | 3301.5 | 620100 | 0 | 30 | - | 620340 |
|  |  | & | Mid | 3750 | 650000 | 3638.04 | 642536 | 102 |  | - | 647672 |
|  |  | Uplink | High | 4159.98 | 677332 | 3758.58 | 650572 | 504 |  | - | 675004 |
| 90 | 121 | Downlink | Low | 3345 | 623000 | 3301.44 | 620096 | 0 | 30 | - | 620336 |
|  |  | & | Mid | 3750 | 650000 | 3633 | 642200 | 102 |  | - | 647336 |
|  |  | Uplink | High | 4155 | 677000 | 3748.56 | 649904 | 504 |  | - | 674336 |
| 100 | 135 | Downlink | Low | 3350.01 | 623334 | 3301.41 | 620094 | 0 | 30 | - | 620334 |
|  |  | & | Mid | 3750 | 650000 | 3627.96 | 641864 | 102 |  | - | 647000 |
|  |  | Uplink | High | 4149.99 | 676666 | 3738.51 | 649234 | 504 |  | - | 673666 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.78	Reference test frequencies for NR operating band n78

Table 4.3.1.1.1.78-1: Test frequencies for NR operating band n78, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3549.99 | 636666 | 3526.95 | 635130 | 102 |  | 7881 | 636672 | 6 | 0 | 1 (6) | 108 |
|  |  | Uplink | High | 3795 | 653000 | 3699.6 | 646640 | 504 |  | 8051 | 652992 | 4 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 3307.5 | 620500 | 3300.39 | 620026 | 0 | 30 | 7711 | 620352 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3549.99 | 636666 | 3524.52 | 634968 | 102 |  | 7879 | 636480 | 0 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 3792.48 | 652832 | 3694.65 | 646310 | 504 |  | 8048 | 652704 | 10 | 2 | 1 (6) | 512 |
| 20 | 106 | Downlink | Low | 3310.02 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3522.09 | 634806 | 102 |  | 7878 | 636384 | 6 | 3 | 1 (6) | 111 |
|  |  | Uplink | High | 3789.99 | 652666 | 3689.73 | 645982 | 504 |  | 8044 | 652320 | 2 | 2 | 0 (2) | 508 |
| 25 | 133 | Downlink | Low | 3312.51 | 620834 | 3300.54 | 620036 | 0 | 30 | 7711 | 620352 | 4 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3519.66 | 634644 | 102 |  | 7876 | 636192 | 0 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 3787.5 | 652500 | 3684.81 | 645654 | 504 |  | 8041 | 652032 | 6 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 3315 | 621000 | 3300.6 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3517.23 | 634482 | 102 |  | 7874 | 636000 | 6 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 3784.98 | 652332 | 3679.86 | 645324 | 504 |  | 8037 | 651648 | 0 | 1 | 0 (2) | 507 |
| 40 | 216 | Downlink | Low | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3512.19 | 634146 | 102 |  | 7871 | 635712 | 6 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 3780 | 652000 | 3669.84 | 644656 | 504 |  | 8030 | 650976 | 8 | 0 | 0 (2) | 506 |
| 50 | 270 | Downlink | Low | 3325.02 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 3549.99 | 636666 | 3507.33 | 633822 | 102 |  | 7867 | 635328 | 6 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3774.99 | 651666 | 3659.97 | 643998 | 504 |  | 8024 | 650400 | 6 | 3 | 1 (6) | 513 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.78-2: Test frequencies for NR operating band n78, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3508.95 | 633930 | 102 |  | 7881 | 636672 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3795 | 653000 | 3609.24 | 640616 | 504 |  | 8051 | 652992 | 16 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 | 7711 | 620352 | 20 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3506.43 | 633762 | 102 |  | 7879 | 636480 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3792.48 | 652832 | 3604.2 | 640280 | 504 |  | 8048 | 652704 | 16 | 0 | 3 (3) | 1014 |
| 20 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3504.09 | 633606 | 102 |  | 7878 | 636384 | 18 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3789.99 | 652666 | 3599.37 | 639958 | 504 |  | 8044 | 652320 | 2 | 0 | 1 (1) | 1010 |
| 25 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 | 7711 | 620352 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3501.57 | 633438 | 102 |  | 7876 | 636192 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3787.5 | 652500 | 3594.36 | 639624 | 504 |  | 8041 | 652032 | 0 | 0 | 3 (3) | 1014 |
| 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3499.23 | 633282 | 102 |  | 7874 | 636000 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3784.98 | 652332 | 3589.5 | 639300 | 504 |  | 8037 | 651648 | 12 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3494.19 | 632946 | 102 |  | 7871 | 635712 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3780 | 652000 | 3579.48 | 638632 | 504 |  | 8030 | 650976 | 8 | 0 | 0 (0) | 1008 |
| 50 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3549.99 | 636666 | 3489.33 | 632622 | 102 |  | 7867 | 635328 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  | 8024 | 650400 | 18 | 0 | 3 (3) | 1014 |
| 60 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3484.11 | 632274 | 102 |  | 7864 | 635040 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  | 8016 | 649632 | 4 | 0 | 0 (0) | 1008 |
| 70 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3549.99 | 636666 | 3479.25 | 631950 | 102 |  | 7860 | 634656 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 |  | 8010 | 649056 | 12 | 0 | 3 (3) | 1014 |
| 80 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3474.21 | 631614 | 102 |  | 7857 | 634368 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  | 8003 | 648384 | 10 | 0 | 3 (3) | 1014 |
| 90 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3469.17 | 631278 | 102 |  | 7853 | 633984 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 |  | 7996 | 647712 | 8 | 0 | 3 (3) | 1014 |
| 100 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3464.13 | 630942 | 102 |  | 7850 | 633696 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  | 7989 | 647040 | 4 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.78-3: Test frequencies for NR operating band n78, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3305.01 | 620334 | 3301.05 | 620070 | 0 | 30 | - | 620310 |
|  |  | & | Mid | 3549.99 | 636666 | 3472.59 | 631506 | 102 |  | - | 636642 |
|  |  | Uplink | High | 3795 | 653000 | 3428.16 | 628544 | 504 |  | - | 652976 |
| 15 | 18 | Downlink | Low | 3307.5 | 620500 | 3301.02 | 620068 | 0 | 30 | - | 620308 |
|  |  | & | Mid | 3549.99 | 636666 | 3470.07 | 631338 | 102 |  | - | 636474 |
|  |  | Uplink | High | 3792.48 | 652832 | 3423.12 | 628208 | 504 |  | - | 652640 |
| 20 | 24 | Downlink | Low | 3310.02 | 620668 | 3301.38 | 620092 | 0 | 30 | - | 620332 |
|  |  | & | Mid | 3549.99 | 636666 | 3467.91 | 631194 | 102 |  | - | 636330 |
|  |  | Uplink | High | 3789.99 | 652666 | 3418.47 | 627898 | 504 |  | - | 652330 |
| 25 | 31 | Downlink | Low | 3312.51 | 620834 | 3301.35 | 620090 | 0 | 30 | - | 620330 |
|  |  | & | Mid | 3549.99 | 636666 | 3465.39 | 631026 | 102 |  | - | 636162 |
|  |  | Uplink | High | 3787.5 | 652500 | 3413.46 | 627564 | 504 |  | - | 651996 |
| 30 | 38 | Downlink | Low | 3315 | 621000 | 3301.32 | 620088 | 0 | 30 | - | 620328 |
|  |  | & | Mid | 3549.99 | 636666 | 3462.87 | 630858 | 102 |  | - | 635994 |
|  |  | Uplink | High | 3784.98 | 652332 | 3408.42 | 627228 | 504 |  | - | 651660 |
| 40 | 51 | Downlink | Low | 3320.01 | 621334 | 3301.65 | 620110 | 0 | 30 | - | 620350 |
|  |  | & | Mid | 3549.99 | 636666 | 3458.19 | 630546 | 102 |  | - | 635682 |
|  |  | Uplink | High | 3780 | 652000 | 3398.76 | 626584 | 504 |  | - | 651016 |
| 50 | 65 | Downlink | Low | 3325.02 | 621668 | 3301.62 | 620108 | 0 | 30 | - | 620348 |
|  |  | & | Mid | 3549.99 | 636666 | 3453.15 | 630210 | 102 |  | - | 635346 |
|  |  | Uplink | High | 3774.99 | 651666 | 3388.71 | 625914 | 504 |  | - | 650346 |
| 60 | 79 | Downlink | Low | 3330 | 622000 | 3301.56 | 620104 | 0 | 30 | - | 620344 |
|  |  | & | Mid | 3549.99 | 636666 | 3448.11 | 629874 | 102 |  | - | 635010 |
|  |  | Uplink | High | 3769.98 | 651332 | 3378.66 | 625244 | 504 |  | - | 649676 |
| 70 | 93 | Downlink& Uplink | Low | 3335.01 | 622334 | 3301.53 | 620102 | 0 | 30 |  | 620342 |
|  |  |  | Mid | 3549.99 | 636666 | 3443.07 | 629538 | 102 |  |  | 634674 |
|  |  |  | High | 3765 | 651000 | 3368.64 | 624576 | 504 |  |  | 649008 |
| 80 | 107 | Downlink | Low | 3340.02 | 622668 | 3301.5 | 620100 | 0 | 30 | - | 620340 |
|  |  | & | Mid | 3549.99 | 636666 | 3438.03 | 629202 | 102 |  | - | 634338 |
|  |  | Uplink | High | 3759.99 | 650666 | 3358.59 | 623906 | 504 |  | - | 648338 |
| 90 | 121 | Downlink | Low | 3345 | 623000 | 3301.44 | 620096 | 0 | 30 | - | 620336 |
|  |  | & | Mid | 3549.99 | 636666 | 3432.99 | 628866 | 102 |  | - | 634002 |
|  |  | Uplink | High | 3754.98 | 650332 | 3348.54 | 623236 | 504 |  | - | 647668 |
| 100 | 135 | Downlink | Low | 3350.01 | 623334 | 3301.41 | 620094 | 0 | 30 | - | 620334 |
|  |  | & | Mid | 3549.99 | 636666 | 3427.95 | 628530 | 102 |  | - | 633666 |
|  |  | Uplink | High | 3750 | 650000 | 3338.52 | 622568 | 504 |  | - | 647000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.79	Reference test frequencies for NR operating band n79

Table 4.3.1.1.1.79-1: Test frequencies for NR operating band n79, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 103 | 52 | Downlink | Low | 4405.02 | 693668 | 4400.34 | 693356 | 0 | 30 | 8475 | 693696 | 4 | 4 | 0 (4) | 8 |
|  |  | & | Mid (Note 4) | 4700.13 | 713342 | 4677.09 | 711806 | 102 |  | 8680 | 713376 | 10 | 4 | 0 (4) | 110 |
|  |  | Uplink | High (Note 4) | 4994.76 | 732984 | 4899.36 | 726624 | 504 |  | 8884 | 732960 | 0 | 0 | 0 (4) | 508 |
| 20 | 106 | Downlink | Low | 4410 | 694000 | 4400.46 | 693364 | 0 | 30 | 8475 | 693696 | 8 | 3 | 0 (4) | 7 |
|  |  | & | Mid | 4699.98 | 713332 | 4672.08 | 711472 | 102 |  | 8676 | 712992 | 8 | 0 | 0 (4) | 106 |
|  |  | Uplink | High | 4989.99 | 732666 | 4889.73 | 725982 | 504 |  | 8878 | 732384 | 6 | 5 | 0 (4) | 513 |
| 40 | 216 | Downlink | Low | 4420.02 | 694668 | 4400.58 | 693372 | 0 | 30 | 8480 | 694176 | 0 | 43 | 0 (4) | 47 |
|  |  | & | Mid | 4699.98 | 713332 | 4662.18 | 710812 | 102 |  | 8672 | 712608 | 8 | 23 | 0 (4) | 129 |
|  |  | Uplink | High | 4980 | 732000 | 4869.84 | 724656 | 504 |  | 8864 | 731040 | 0 | 4 | 0 (4) | 512 |
| 50 | 270 | Downlink | Low | 4425 | 695000 | 4400.7 | 693380 | 0 | 30 | 8480 | 694176 | 4 | 42 | 0 (4) | 46 |
|  |  | & | Mid | 4699.98 | 713332 | 4657.32 | 710488 | 102 |  | 8672 | 712608 | 8 | 50 | 0 (4) | 156 |
|  |  | Uplink | High | 4974.99 | 731666 | 4859.97 | 723998 | 504 |  | 8864 | 731040 | 10 | 58 | 0 (4) | 566 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The High range frequency can’t cover the two RBs at the right edge of band due to the limit of GSCN range in TS 38.101-1 table 5.4.3.3-1. Note 4: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-2: Test frequencies for NR operating band n79, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low (Note 3) | 4405.83 | 693722 | 4401.51 | 693434 | 0 | 30 | 8475 | 693696 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 4699.92 | 713328 | 4658.88 | 710592 | 102 |  | 8679 | 713280 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 4995 | 733000 | 4809.24 | 720616 | 504 |  | 8884 | 732960 | 8 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 4410 | 694000 | 4400.82 | 693388 | 0 | 30 | 8475 | 693696 | 20 | 2 | 0 (0) | 4 |
|  |  | & | Mid | 4700.01 | 713334 | 4654.11 | 710274 | 102 |  | 8676 | 712992 | 6 | 1 | 0 (0) | 206 |
|  |  | Uplink | High | 4989.99 | 732666 | 4799.37 | 719958 | 504 |  | 8878 | 732384 | 18 | 3 | 0 (0) | 1014 |
| 40 | 106 | Downlink | Low | 4420.02 | 694668 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4644.21 | 709614 | 102 |  | 8672 | 712608 | 18 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 4980 | 732000 | 4779.48 | 718632 | 504 |  | 8864 | 731040 | 0 | 3 | 0 (0) | 1014 |
| 50 | 133 | Downlink | Low | 4425 | 695000 | 4401.06 | 693404 | 0 | 30 | 8480 | 694176 | 4 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4639.35 | 709290 | 102 |  | 8672 | 712608 | 6 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 4974.99 | 731666 | 4769.61 | 717974 | 504 |  | 8864 | 731040 | 10 | 26 | 1 (4) | 1068 |
| 60 | 162 | Downlink | Low | 4430.01 | 695334 | 4400.85 | 693390 | 0 | 30 | 8480 | 694176 | 18 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4634.13 | 708942 | 102 |  | 8672 | 712608 | 18 | 36 | 1 (4) | 284 |
|  |  | Uplink | High | 4969.98 | 731332 | 4759.38 | 717292 | 504 |  | 8864 | 731040 | 20 | 54 | 1 (4) | 1124 |
| 80 | 217 | Downlink | Low | 4440 | 696000 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4624.23 | 708282 | 102 |  | 8656 | 711072 | 6 | 0 | 1 (4) | 212 |
|  |  | Uplink | High | 4959.99 | 730666 | 4739.49 | 715966 | 504 |  | 8848 | 729504 | 2 | 46 | 1 (4) | 1108 |
| 100 | 273 | Downlink | Low | 4450.02 | 696668 | 4400.88 | 693392 | 0 | 30 | 8480 | 694176 | 16 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4614.15 | 707610 | 102 |  | 8656 | 711072 | 6 | 28 | 1 (4) | 268 |
|  |  | Uplink | High | 4950 | 730000 | 4719.42 | 714628 | 504 |  | 8832 | 727968 | 20 | 37 | 1 (4) | 1090 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-6 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-3: Test frequencies for NR operating band n79, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 4405.02 | 693668 | 4401.06 | 693404 | 0 | 30 | - | 693644 |
|  |  | & | Mid | 4700.01 | 713334 | 4622.61 | 708174 | 102 |  | - | 713310 |
|  |  | Uplink | High | 4995 | 733000 | 4628.16 | 708544 | 504 |  | - | 732976 |
| 20 | 24 | Downlink | Low | 4410 | 694000 | 4401.36 | 693424 | 0 | 30 | - | 693664 |
|  |  | & | Mid | 4700.01 | 713334 | 4617.93 | 707862 | 102 |  | - | 712998 |
|  |  | Uplink | High | 4989.99 | 732666 | 4618.47 | 707898 | 504 |  | - | 732330 |
| 40 | 51 | Downlink | Low | 4420.02 | 694668 | 4401.66 | 693444 | 0 | 30 | - | 693684 |
|  |  | & | Mid | 4700.01 | 713334 | 4608.21 | 707214 | 102 |  | - | 712350 |
|  |  | Uplink | High | 4980 | 732000 | 4598.76 | 706584 | 504 |  | - | 731016 |
| 50 | 65 | Downlink | Low | 4425 | 695000 | 4401.6 | 693440 | 0 | 30 | - | 693680 |
|  |  | & | Mid | 4700.01 | 713334 | 4603.17 | 706878 | 102 |  | - | 712014 |
|  |  | Uplink | High | 4974.99 | 731666 | 4588.71 | 705914 | 504 |  | - | 730346 |
| 60 | 79 | Downlink | Low | 4430.01 | 695334 | 4401.57 | 693438 | 0 | 30 | - | 693678 |
|  |  | & | Mid | 4700.01 | 713334 | 4598.13 | 706542 | 102 |  | - | 711678 |
|  |  | Uplink | High | 4969.98 | 731332 | 4578.66 | 705244 | 504 |  | - | 729676 |
| 80 | 107 | Downlink | Low | 4440 | 696000 | 4401.48 | 693432 | 0 | 30 | - | 693672 |
|  |  | & | Mid | 4700.01 | 713334 | 4588.05 | 705870 | 102 |  | - | 711006 |
|  |  | Uplink | High | 4959.99 | 730666 | 4558.59 | 703906 | 504 |  | - | 728338 |
| 100 | 135 | Downlink | Low | 4450.02 | 696668 | 4401.42 | 693428 | 0 | 30 | - | 693668 |
|  |  | & | Mid | 4700.01 | 713334 | 4577.97 | 705198 | 102 |  | - | 710334 |
|  |  | Uplink | High | 4950 | 730000 | 4538.52 | 702568 | 504 |  | - | 727000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.80	Reference test frequencies for NR operating band n80 (SUL)

Table 4.3.1.1.1.80-1: Test frequencies for NR operating band n80 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1654.53 | 330906 | 504 |
|  |  |  | High | 1782.5 | 356500 | 1779.17 | 355834 | 6 |
| 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1652.1 | 330420 | 504 |
|  |  |  | High | 1780 | 356000 | 1774.24 | 354848 | 6 |
| 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1649.67 | 329934 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1769.31 | 353862 | 6 |
| 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1647.24 | 329448 | 504 |
|  |  |  | High | 1775 | 355000 | 1764.38 | 352876 | 6 |
| 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1644.81 | 328962 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1759.45 | 351890 | 6 |
| 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1642.38 | 328476 | 504 |
|  |  |  | High | 1770 | 354000 | 1754.52 | 350904 | 6 |

Table 4.3.1.1.1.80-2: Test frequencies for NR operating band n80 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1561.74 | 312348 | 504 |
|  |  |  | High | 1780 | 356000 | 1773.52 | 354704 | 6 |
| 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1559.22 | 311844 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1768.5 | 353700 | 6 |
| 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1556.88 | 311376 | 504 |
|  |  |  | High | 1775 | 355000 | 1763.66 | 352732 | 6 |
| 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1554.36 | 310872 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1758.64 | 351728 | 6 |
| 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1552.02 | 310404 | 504 |
|  |  |  | High | 1770 | 354000 | 1753.8 | 350760 | 6 |

Table 4.3.1.1.1.80-3: Test frequencies for NR operating band n80 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1380.66 | 276132 | 504 |
|  |  |  | High | 1780 | 356000 | 1771.72 | 354344 | 6 |
| 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1378.14 | 275628 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1766.7 | 353340 | 6 |
| 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1375.98 | 275196 | 504 |
|  |  |  | High | 1775 | 355000 | 1762.04 | 352408 | 6 |
| 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1373.46 | 274692 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1757.02 | 351404 | 6 |
| 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1370.94 | 274188 | 504 |
|  |  |  | High | 1770 | 354000 | 1752 | 350400 | 6 |

###### 4.3.1.1.1.81	Reference test frequencies for NR operating band n81 (SUL)

Table 4.3.1.1.1.81-1: Test frequencies for NR operating band n81 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |
| 10 | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |
| 15 | 79 | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |
| 20 | 106 | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |

Table 4.3.1.1.1.81-2: Test frequencies for NR operating band n81 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |
| 15 | 38 | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |
| 20 | 51 | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |

###### 4.3.1.1.1.82	Reference test frequencies for NR operating band n82 (SUL)

Table 4.3.1.1.1.82-1: Test frequencies for NR operating band n82 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |
| 10 | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |
| 15 | 79 | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |
| 20 | 106 | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |

Table 4.3.1.1.1.82-2: Test frequencies for NR operating band n82 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |
| 15 | 38 | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |
| 20 | 51 | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |

###### 4.3.1.1.1.83	Reference test frequencies for NR operating band n83 (SUL)

Table 4.3.1.1.1.83-1: Test frequencies for NR operating band n83 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |
| 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |
| 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |
| 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 |
| (Note 1) |  |  | Mid | 728 | 145600 | 627.74 | 125548 | 504 |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |
| 30 | 160 | Uplink | Low Mid | 718 | 143600 | 703.6 | 140720 | 0 |
| (Note 2) |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |
| Note 1: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 Note 2: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.83-2: Test frequencies for NR operating band n83 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 539.7 | 107948 | 504 |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |
| 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |
| 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 |
| (Note 1) |  |  | Mid | 728 | 145600 | 537.38 | 107476 | 504 |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |
| 30 | 78 | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 |
| (Note 2) |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |
| Note 1: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 Note 2: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.84	Reference test frequencies for NR operating band n84 (SUL)

Table 4.3.1.1.1.84-1: Test frequencies for NR operating band n84 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |
| 10 | 52 | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |
| 15 | 79 | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |
| 20 | 106 | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |
| 25 | 133 | Uplink | Low | 1932.5 | 386500 | 1920.53 | 384106 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1847.31 | 369462 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1954.45 | 390890 | 6 |
| 30 | 160 | Uplink | Low | 1935 | 387000 | 1920.6 | 384120 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1844.88 | 368976 | 504 |
|  |  |  | High | 1965 | 393000 | 1949.52 | 389904 | 6 |
| 40 | 216 | Uplink | Low | 1940 | 388000 | 1920.56 | 384112 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1839.84 | 367968 | 504 |
|  |  |  | High | 1960 | 392000 | 1939.48 | 387896 | 6 |
| 50 | 270 | Uplink | Low | 1945 | 389000 | 1920.7 | 384140 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1834.98 | 366996 | 504 |
|  |  |  | High | 1955 | 391000 | 1929.62 | 385924 | 6 |

Table 4.3.1.1.1.84-2: Test frequencies for NR operating band n84 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |
| 15 | 38 | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |
| 20 | 51 | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |
| 25 | 65 | Uplink | Low | 1932.5 | 386500 | 1920.8 | 384160 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1756.86 | 351372 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1953.64 | 390728 | 6 |
| 30 | 78 | Uplink | Low | 1935 | 387000 | 1920.96 | 384192 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1754.52 | 350904 | 504 |
|  |  |  | High | 1965 | 393000 | 1948.8 | 389760 | 6 |
| 40 | 106 | Uplink | Low | 1940 | 388000 | 1920.92 | 384184 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1749.48 | 349896 | 504 |
|  |  |  | High | 1960 | 392000 | 1938.76 | 387752 | 6 |
| 50 | 133 | Uplink | Low | 1945 | 389000 | 1921.06 | 384212 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1744.62 | 348924 | 504 |
|  |  |  | High | 1955 | 391000 | 1928.9 | 385780 | 6 |

Table 4.3.1.1.1.84-3: Test frequencies for NR operating band n84 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |
| 15 | 18 | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |
| 20 | 24 | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |
| 25 | 31 | Uplink | Low | 1932.5 | 386500 | 1921.34 | 384268 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1575.96 | 315192 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1952.02 | 390404 | 6 |
| 30 | 38 | Uplink | Low | 1935 | 387000 | 1921.32 | 384264 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1573.44 | 314688 | 504 |
|  |  |  | High | 1965 | 393000 | 1947 | 389400 | 6 |
| 40 | 51 | Uplink | Low | 1940 | 388000 | 1921.64 | 384328 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1568.76 | 313752 | 504 |
|  |  |  | High | 1960 | 392000 | 1937.32 | 387464 | 6 |
| 50 | 65 | Uplink | Low | 1945 | 389000 | 1921.6 | 384320 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1563.72 | 312744 | 504 |
|  |  |  | High | 1955 | 391000 | 1927.28 | 385456 | 6 |

###### 4.3.1.1.1.85	Reference test frequencies for NR operating band n85

Table 4.3.1.1.1.85-1: Test frequencies for NR operating band n85 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 729.5 | 145900 | 728.15 | 145630 | 0 | 15 | 30282 | 145870 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 737 | 147400 | 717.29 | 143458 | 102 |  | 30321 | 147430 | 4 | 0 | 3 (2) | 104 |
|  |  |  | High | 744.5 | 148900 | 652.43 | 130486 | 504 |  | 30357 | 148870 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 699.5 | 139900 | 698.15 | 139630 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 614.93 | 122986 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 714.5 | 142900 | 712.07 | 142414 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 730.5 | 146100 | 728.25 | 145650 | 0 | 15 | 1826 | 146170 | 0 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 737 | 147400 | 716.39 | 143278 | 102 |  | 1843 | 147410 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 743.5 | 148700 | 650.53 | 130106 | 504 |  | 1858 | 148610 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 700.5 | 140100 | 698.25 | 139650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 614.03 | 122806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 713.5 | 142700 | 710.17 | 142034 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 733 | 146600 | 728.32 | 145664 | 0 | 15 | 1826 | 146170 | 1 | 2 | 2 (4) | 6 |
|  |  |  | Mid | 737 | 147400 | 713.96 | 142792 | 102 |  | 1835 | 146890 | 2 | 2 | 1 (2) | 106 |
|  |  |  | High | 741 | 148200 | 645.6 | 129120 | 504 |  | 1846 | 147650 | 2 | 2 | 0 (0) | 506 |
|  |  | Uplink | Low | 703 | 140600 | 698.32 | 139664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 611.6 | 122320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 705.24 | 141048 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 735.5 | 147100 | 728.39 | 145678 | 0 | 15 | 1827 | 146190 | 2 | 4 | 2 (4) | 8 |
|  |  |  | Mid | 737 | 147400 | 711.53 | 142306 | 102 |  | 1829 | 146410 | 2 | 4 | 1 (2) | 108 |
|  |  |  | High | 738.5 | 147700 | 640.67 | 128134 | 504 |  | 1832 | 146650 | 2 | 4 | 0 (0) | 508 |
|  |  | Uplink | Low | 705.5 | 141100 | 698.39 | 139678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 609.17 | 121834 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 700.31 | 140062 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.85-2: Test frequencies for NR operating band n85 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 733 | 146600 | 728.68 | 145736 | 0 | 15 | 1832 | 146650 | 12 | 0 | 1 (2) | 4 |
|  |  |  | Mid | 737 | 147400 | 695.96 | 139192 | 102 |  | 1847 | 147850 | 16 | 0 | 3 (8) | 220 |
|  |  |  | High | 741 | 148200 | 555.24 | 111048 | 504 |  | 1852 | 148130 | 16 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 703 | 140600 | 698.68 | 139736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 521.24 | 104248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 704.52 | 140904 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 735.5 | 147100 | 728.66 | 145732 | 0 | 15 | 1837 | 146930 | 22 | 1 | 2 (4) | 8 |
|  |  |  | Mid | 737 | 147400 | 693.44 | 138688 | 102 |  | 1838 | 147130 | 23 | 1 | 0 (0) | 206 |
|  |  |  | High | 738.5 | 147700 | 550.22 | 110044 | 504 |  | 1843 | 147410 | 23 | 1 | 0 (0) | 1010 |
|  |  | Uplink | Low | 705.5 | 141100 | 698.66 | 139732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707 | 141400 | 518.72 | 103744 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 699.5 | 139900 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.86	Reference test frequencies for NR operating band n86 (SUL)

Table 4.3.1.1.1.86-1: Test frequencies for NR operating band n86 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |
| 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |
|  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |
| 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |
| 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |
|  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |
| 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |
|  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |

Table 4.3.1.1.1.86-2: Test frequencies for NR operating band n86 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |
|  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |
| 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |
| 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |
|  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |
| 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |
|  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |

Table 4.3.1.1.1.86-3: Test frequencies for NR operating band n86 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |
|  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |
| 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |
| 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |
|  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |
| 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |
|  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 0 |

###### 4.3.1.1.1.87 to 4.3.1.1.1.90	FFS

###### 4.3.1.1.1.91	Reference test frequencies for NR operating band n91

Table 4.3.1.1.1.91-1: Test frequencies for NR operating band n91 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - | - |
| 10/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.92	Reference test frequencies for NR operating band n92

Table 4.3.1.1.1.92-1: Test frequencies for NR operating band n92 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | 3584 | 286810 | 0 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | 3687 | 294990 | 0 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | 3787 | 302930 | 4 | 1 | 1 (2) |  | 507 |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 5/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) |  | 507 |
|  | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 5/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) |  | 507 |
|  | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 5/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) |  | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) |  | 509 |
|  | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - |  | - |
| 10/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) |  | 507 |
|  |  | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - |  | - |
| 10/15 | 70 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) |  | 507 |
|  | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - |  | - |
| 10/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) |  | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) |  | 509 |
|  | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - |  | - |
| 15/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) |  | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) |  | 507 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |  | - | - | - | - | - |  | - |
| 20/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) |  | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) |  | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) |  | 509 |
|  |  | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 | - | - | - | - | - | - |  | - |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |  | - | - | - | - | - |  | - |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |  | - | - | - | - | - |  | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.92-2: Test frequencies for NR operating band n92 and SCS 30 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | 3591 | 287310 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 10/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 24 | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.93	Reference test frequencies for NR operating band n93

Table 4.3.1.1.1.93-1: Test frequencies for NR operating band n93 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.94	Reference test frequencies for NR operating band n94

Table 4.3.1.1.1.94-1: Test frequencies for NR operating band n94 and SCS 15 kHz

| UL/DL Bandwidth combination[MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | 3584 | 286810 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | 3687 | 294990 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | 3787 | 302930 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/15 | 70 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.94-2: Test frequencies for NR operating band n94 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | 3591 | 287310 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.95	Reference test frequencies for NR operating band n95 (SUL)

Table 4.3.1.1.1.95-1: Test frequencies for NR operating band n95 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 2012.5 | 402500 | 2010.25 | 402050 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1924.53 | 384906 | 504 |
|  |  |  | High | 2022.5 | 404500 | 2019.17 | 403834 | 6 |
| 10 | 52 | Uplink | Low | 2015 | 403000 | 2010.32 | 402064 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1922.1 | 384420 | 504 |
|  |  |  | High | 2020 | 404000 | 2014.24 | 402848 | 6 |
| 15 | 79 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2010.39 | 402078 | 0 |

Table 4.3.1.1.1.95-2: Test frequencies for NR operating band n95 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 2015 | 403000 | 2010.68 | 402136 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1831.74 | 366348 | 504 |
|  |  |  | High | 2020 | 404000 | 2013.52 | 402704 | 6 |
| 15 | 38 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2010.66 | 402132 | 0 |

Table 4.3.1.1.1.95-3: Test frequencies for NR operating band n95 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 2015 | 403000 | 2011.04 | 402208 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1650.66 | 330132 | 504 |
|  |  |  | High | 2020 | 404000 | 2011.72 | 402344 | 6 |
| 15 | 18 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2011.02 | 402204 | 0 |

###### 4.3.1.1.1.96	 Reference test frequencies for NR operating band n96

Table 4.3.1.1.1.96-1: Test frequencies for NR operating band n96 and SCS 15 kHz (Note 3)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 4 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 106 | Downlink | Low | 5955 | 797000 | 5945.46 | 796364 | 0 | 30 | - | 796604 | 31 | - | - | - |
|  |  | & | Mid | 6535.02 | 835668 | 6507.12 | 833808 | 102 |  | - | 835272 | 31 | - | - | - |
|  |  | Uplink | High | 7114.98 | 874332 | 7014.72 | 867648 | 504 |  | - | 873936 | 31 | - | - | - |
| 40 | 216 | Downlink | Low | 5965.02 | 797668 | 5945.58 | 796372 | 0 | 30 | - | 796612 | 31 | - | - | - |
|  |  | & | Mid | 6525 | 835000 | 6487.2 | 832480 | 102 |  | - | 833944 | 31 | - | - | - |
|  |  | Uplink | High | 7084.98 | 872332 | 6974.82 | 864988 | 504 |  | - | 871276 | 31 | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: NR band n46 with SCS =15 kHz is restricted for operation when carrier is configured as an SCell part of a DC or CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 4: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.96-2: Test frequencies for NR operating band n96 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 3 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 51 | Downlink | Low | 5955 | 797000 | 5945.82 | 796388 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6535.02 | 835668 | 6489.12 | 832608 | 102 |  | 9951 | 835392 | 22 | 0 | 1 (0) | 204 |
|  |  | Uplink | High | 7114.98 | 874332 | 6924.36 | 861624 | 504 |  | 10353 | 873984 | 22 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 5965.02 | 797668 | 5945.94 | 796396 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6525 | 835000 | 6469.2 | 831280 | 102 |  | 9937 | 834048 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 7084.98 | 872332 | 6884.46 | 858964 | 504 |  | 10339 | 872640 | 22 | 40 | 3 (3) | 1112 |
| 60 | 162 | Downlink | Low | 5974.98 | 798332 | 5945.82 | 796388 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6985.02 | 865668 | 6919.14 | 861276 | 102 |  | 10256 | 864672 | 22 | 16 | 3 (3) | 260 |
|  |  | Uplink | High | 7095 | 873000 | 6884.4 | 858960 | 504 |  | 10325 | 871296 | 0 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 5985 | 799000 | 5945.94 | 796396 | 0 | 30 | 9548 | 796704 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 6544.98 | 836332 | 6469.2 | 831280 | 102 |  | 9937 | 834048 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 7024.98 | 868332 | 6804.48 | 853632 | 504 |  | 10270 | 866016 | 22 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.96-3: Test frequencies for NR operating band n96 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] Note 2 | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 24 | Downlink | Low | 5955 | 797000 | 5946.36 | 796424 | 0 | 30 | - | 796664 |
|  |  | & | Mid | 6535.02 | 835668 | 6452.94 | 830196 | 102 |  | - | 835332 |
|  |  | Uplink | High | 7114.98 | 874332 | 6743.46 | 849564 | 504 |  | - | 873996 |
| 40 | 51 | Downlink | Low | 5965.02 | 797668 | 5946.66 | 796444 | 0 | 30 | - | 796684 |
|  |  | & | Mid | 6525 | 835000 | 6433.2 | 828880 | 102 |  | - | 834016 |
|  |  | Uplink | High | 7084.98 | 872332 | 6703.74 | 846916 | 504 |  | - | 871348 |
| 60 | 79 | Downlink | Low | 5974.98 | 798332 | 5946.54 | 796436 | 0 | 30 | - | 796676 |
|  |  | & | Mid | 6985.02 | 865668 | 6883.14 | 858876 | 102 |  | - | 864012 |
|  |  | Uplink | High | 7095 | 873000 | 6703.68 | 846912 | 504 |  | - | 871344 |
| 80 | 107 | Downlink | Low | 5985 | 799000 | 5946.48 | 796432 | 0 | 30 | - | 796672 |
|  |  | & | Mid | 6544.98 | 836332 | 6433.02 | 828868 | 102 |  | - | 834004 |
|  |  | Uplink | High | 7024.98 | 868332 | 6623.58 | 841572 | 504 |  | - | 866004 |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: The selection of Low, Mid and High test frequencies are restricted to the limited allowed ARFCN values as specified in TS 38.101 [7], Table 5.4.2.3-3. |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.97	Reference test frequencies for NR operating band n97 (SUL)

Table 4.3.1.1.1.97-1: Test frequencies for NR operating band n97 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 2302.5 | 460500 | 2300.25 | 460050 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2329.39 | 465878 | 102 |
|  |  |  | High | 2397.5 | 479500 | 2304.53 | 460906 | 504 |
| 10 | 52 | Uplink | Low | 2305 | 461000 | 2300.32 | 460064 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2326.96 | 465392 | 102 |
|  |  |  | High | 2395 | 479000 | 2299.6 | 459920 | 504 |
| 15 | 79 | Uplink | Low | 2307.5 | 461500 | 2300.39 | 460078 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2324.53 | 464906 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2294.67 | 458934 | 504 |
| 20 | 106 | Uplink | Low | 2310 | 462000 | 2300.46 | 460092 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2322.1 | 464420 | 102 |
|  |  |  | High | 2390 | 478000 | 2289.74 | 457948 | 504 |
| 25 | 133 | Uplink | Low | 2312.5 | 462500 | 2300.53 | 460106 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2319.67 | 463934 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2284.81 | 456962 | 504 |
| 30 | 160 | Uplink | Low | 2315 | 463000 | 2300.6 | 460120 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2317.24 | 463448 | 102 |
|  |  |  | High | 2385 | 477000 | 2279.88 | 455976 | 504 |
| 40 | 216 | Uplink | Low | 2320 | 464000 | 2300.56 | 460112 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2312.2 | 462440 | 102 |
|  |  |  | High | 2380 | 476000 | 2269.84 | 453968 | 504 |
| 50 | 270 | Uplink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2307.34 | 461468 | 102 |
|  |  |  | High | 2375 | 475000 | 2259.98 | 451996 | 504 |

Table 4.3.1.1.1.97-2: Test frequencies for NR operating band n97 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 2305 | 461000 | 2300.68 | 460136 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2308.96 | 461792 | 102 |
|  |  |  | High | 2395 | 479000 | 2209.24 | 441848 | 504 |
| 15 | 38 | Uplink | Low | 2307.5 | 461500 | 2300.66 | 460132 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2306.44 | 461288 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2204.22 | 440844 | 504 |
| 20 | 51 | Uplink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2304.1 | 460820 | 102 |
|  |  |  | High | 2390 | 478000 | 2199.38 | 439876 | 504 |
| 25 | 65 | Uplink | Low | 2312.5 | 462500 | 2300.8 | 460160 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2301.58 | 460316 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2194.36 | 438872 | 504 |
| 30 | 78 | Uplink | Low | 2315 | 463000 | 2300.96 | 460192 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2299.24 | 459848 | 102 |
|  |  |  | High | 2385 | 477000 | 2189.52 | 437904 | 504 |
| 40 | 106 | Uplink | Low | 2320 | 464000 | 2300.92 | 460184 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2294.2 | 458840 | 102 |
|  |  |  | High | 2380 | 476000 | 2179.48 | 435896 | 504 |
| 50 | 133 | Uplink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2289.34 | 457868 | 102 |
|  |  |  | High | 2375 | 475000 | 2169.62 | 433924 | 504 |
| 60 | 162 | Uplink | Low | 2330 | 466000 | 2300.84 | 460168 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2284.12 | 456824 | 102 |
|  |  |  | High | 2370 | 474000 | 2159.4 | 431880 | 504 |
|  |  |  | Low | 2335 | 467000 | 2300.98 | 460196 | 0 |
| 70 | 189 | Uplink | Mid | 2350 | 470000 | 2279.26 | 455852 | 102 |
|  |  |  | High | 2365 | 473000 | 2149.54 | 429908 | 504 |
| 80 | 217 | Uplink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2274.22 | 454844 | 102 |
|  |  |  | High | 2360 | 472000 | 2139.5 | 427900 | 504 |
|  |  |  | Low | 2345 | 469000 | 2300.9 | 460180 | 0 |
| 90 | 245 | Uplink | Mid | 2350 | 470000 | 2269.18 | 453836 | 102 |
|  |  |  | High | 2355 | 471000 | 2129.46 | 425892 | 504 |
|  |  |  | Low | 2350 | 470000 | 2300.86 | 460172 | 0 |
| 100 | 273 | Uplink | Mid | 2350 | 470000 | 2264.14 | 452828 | 102 |
|  |  |  | High | 2350 | 470000 | 2119.42 | 423884 | 504 |

Table 4.3.1.1.1.97-3: Test frequencies for NR operating band n97 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 2305 | 461000 | 2301.04 | 460208 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2272.6 | 454520 | 102 |
|  |  |  | High | 2395 | 479000 | 2028.16 | 405632 | 504 |
| 15 | 18 | Uplink | Low | 2307.5 | 461500 | 2301.02 | 460204 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2270.08 | 454016 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2023.14 | 404628 | 504 |
| 20 | 24 | Uplink | Low | 2310 | 462000 | 2301.36 | 460272 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2267.92 | 453584 | 102 |
|  |  |  | High | 2390 | 478000 | 2018.48 | 403696 | 504 |
| 25 | 31 | Uplink | Low | 2312.5 | 462500 | 2301.34 | 460268 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2265.4 | 453080 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2013.46 | 402692 | 504 |
| 30 | 38 | Uplink | Low | 2315 | 463000 | 2301.32 | 460264 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2262.88 | 452576 | 102 |
|  |  |  | High | 2385 | 477000 | 2008.44 | 401688 | 504 |
| 40 | 51 | Uplink | Low | 2320 | 464000 | 2301.64 | 460328 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2258.2 | 451640 | 102 |
|  |  |  | High | 2380 | 476000 | 1998.76 | 399752 | 504 |
| 50 | 65 | Uplink | Low | 2325 | 465000 | 2301.6 | 460320 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2253.16 | 450632 | 102 |
|  |  |  | High | 2375 | 475000 | 1988.72 | 397744 | 504 |
| 60 | 79 | Uplink | Low | 2330 | 466000 | 2301.56 | 460312 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2248.12 | 449624 | 102 |
|  |  |  | High | 2370 | 474000 | 1978.68 | 395736 | 504 |
|  |  |  | Low | 2335 | 467000 | 2301.52 | 460304 | 0 |
| 70 | 93 | Uplink | Mid | 2350 | 470000 | 2243.08 | 448616 | 102 |
|  |  |  | High | 2365 | 473000 | 1968.64 | 393728 | 504 |
| 80 | 107 | Uplink | Low | 2340 | 468000 | 2301.48 | 460296 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2238.04 | 447608 | 102 |
|  |  |  | High | 2360 | 472000 | 1958.6 | 391720 | 504 |
|  |  |  | Low | 2345 | 469000 | 2301.44 | 460288 | 0 |
| 90 | 121 | Uplink | Mid | 2350 | 470000 | 2233 | 446600 | 102 |
|  |  |  | High | 2355 | 471000 | 1948.56 | 389712 | 504 |
|  |  |  | Low | 2350 | 470000 | 2301.4 | 460280 | 0 |
| 100 | 135 | Uplink | Mid | 2350 | 470000 | 2227.96 | 445592 | 102 |
|  |  |  | High | 2350 | 470000 | 1938.52 | 387704 | 504 |

###### 4.3.1.1.1.98	Reference test frequencies for NR operating band n98 (SUL)

Table 4.3.1.1.1.98-1: Test frequencies for NR operating band n98 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1882.5 | 376500 | 1880.25 | 376050 | 0 |
|  |  |  | Mid | 1899.1 | 379820 | 1878.49 | 375698 | 102 |
|  |  |  | High | 1917.5 | 383500 | 1824.53 | 364906 | 504 |
| 10 | 52 | Uplink | Low | 1885 | 377000 | 1880.32 | 376064 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1876.96 | 375392 | 102 |
|  |  |  | High | 1915 | 383000 | 1819.6 | 363920 | 504 |
| 15 | 79 | Uplink | Low | 1887.5 | 377500 | 1880.39 | 376078 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1874.53 | 374906 | 102 |
|  |  |  | High | 1912.5 | 382500 | 1814.67 | 362934 | 504 |
| 20 | 106 | Uplink | Low | 1890 | 378000 | 1880.46 | 376092 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1872.1 | 374420 | 102 |
|  |  |  | High | 1910 | 382000 | 1809.74 | 361948 | 504 |
| 25 | 133 | Uplink | Low | 1892.5 | 378500 | 1880.53 | 376106 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1869.67 | 373934 | 102 |
|  |  |  | High | 1907.5 | 381500 | 1804.81 | 360962 | 504 |
| 30 | 160 | Uplink | Low | 1895 | 379000 | 1880.6 | 376120 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1867.24 | 373448 | 102 |
|  |  |  | High | 1905 | 381000 | 1799.88 | 359976 | 504 |
| 35 | 188 | Uplink | Low | 1897.5 | 379500 | 1880.58 | 376116 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1864.72 | 372944 | 102 |
|  |  |  | High | 1902.5 | 380500 | 1794.86 | 358972 | 504 |
| 40 | 216 | Uplink | Low | 1900 | 380000 | 1880.56 | 376112 | 0 |
|  |  |  | Mid |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |

Table 4.3.1.1.1.98-2: Test frequencies for NR operating band n98 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1885 | 377000 | 1880.68 | 376136 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1858.96 | 371792 | 102 |
|  |  |  | High | 1915 | 383000 | 1729.24 | 345848 | 504 |
| 15 | 38 | Uplink | Low | 1887.5 | 377500 | 1880.66 | 376132 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1856.44 | 371288 | 102 |
|  |  |  | High | 1912.5 | 382500 | 1724.22 | 344844 | 504 |
| 20 | 51 | Uplink | Low | 1890 | 378000 | 1880.82 | 376164 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1854.1 | 370820 | 102 |
|  |  |  | High | 1910 | 382000 | 1719.38 | 343876 | 504 |
| 25 | 65 | Uplink | Low | 1892.5 | 378500 | 1880.8 | 376160 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1851.58 | 370316 | 102 |
|  |  |  | High | 1907.5 | 381500 | 1714.36 | 342872 | 504 |
| 30 | 78 | Uplink | Low | 1895 | 379000 | 1880.96 | 376192 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1849.24 | 369848 | 102 |
|  |  |  | High | 1905 | 381000 | 1709.52 | 341904 | 504 |
| 35 | 92 | Uplink | Low | 1897.5 | 379500 | 1880.94 | 376188 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1846.72 | 369344 | 102 |
|  |  |  | High | 1902.5 | 380500 | 1704.5 | 340900 | 504 |
| 40 | 106 | Uplink | Low | 1900 | 380000 | 1880.92 | 376184 | 0 |
|  |  |  | Mid |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |

Table 4.3.1.1.1.98-3: Test frequencies for NR operating band n98 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1885 | 377000 | 1881.04 | 376208 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1822.6 | 364520 | 102 |
|  |  |  | High | 1915 | 383000 | 1548.16 | 309632 | 504 |
| 15 | 18 | Uplink | Low | 1887.5 | 377500 | 1881.02 | 376204 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1820.08 | 364016 | 102 |
|  |  |  | High | 1912.5 | 382500 | 1543.14 | 308628 | 504 |
| 20 | 24 | Uplink | Low | 1890 | 378000 | 1881.36 | 376272 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1817.92 | 363584 | 102 |
|  |  |  | High | 1910 | 382000 | 1538.48 | 307696 | 504 |
| 25 | 31 | Uplink | Low | 1892.5 | 378500 | 1881.34 | 376268 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1815.4 | 363080 | 102 |
|  |  |  | High | 1907.5 | 381500 | 1533.46 | 306692 | 504 |
| 30 | 38 | Uplink | Low | 1895 | 379000 | 1881.32 | 376264 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1812.88 | 362576 | 102 |
|  |  |  | High | 1905 | 381000 | 1528.44 | 305688 | 504 |
| 35 | 44 | Uplink | Low | 1897.5 | 379500 | 1881.66 | 376332 | 0 |
|  |  |  | Mid | 1900 | 380000 | 1810.72 | 362144 | 102 |
|  |  |  | High | 1902.5 | 380500 | 1523.78 | 304756 | 504 |
| 40 | 51 | Uplink | Low | 1900 | 380000 | 1881.64 | 376328 | 0 |
|  |  |  | Mid |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |

###### 4.3.1.1.1.99	Reference test frequencies for NR operating band n99 (SUL)

Table 4.3.1.1.1.99-1: Test frequencies for NR operating band n99 and SCS 15 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 |
|  |  |  | Mid | 1635.0 | 327000 | 1542.03 | 308406 | 504 |
|  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |
| 10 | 52 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |

Table 4.3.1.1.1.99-2: Test frequencies for NR operating band n99 and SCS 30 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.02 | 329004 | 6 |

Table 4.3.1.1.1.99-3: Test frequencies for NR operating band n99 and SCS 60 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low Mid | 1632.5 | 326500 | 1628.54 | 325708 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |

###### 4.3.1.1.1.100	Reference test frequencies for NR operating band n100

Table 4.3.1.1.1.100-1: Test frequencies for NR operating band n100 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 920.9 | 184180 | 919.55 | 183910 | 0 | 15 | 31239 | 184150 | 8 | 0 | 2 (0) | 0 |
|  |  |  | Mid | 922.2 | 184440 | 902.49 | 180498 | 102 |  | 31246 | 184410 | 8 | 0 | 2 (0) | 102 |
|  |  |  | High | 923.5 | 184700 | 831.43 | 166286 | 504 |  | 31253 | 184670 | 8 | 0 | 2 (0) | 504 |
|  |  | Uplink | Low | 875.9 | 175180 | 874.55 | 174910 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 877.2 | 175440 | 785.13 | 157026 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 878.5 | 175700 | 876.07 | 175214 | 6 |  | - | - | - | - | - | - |
| 5 | 25 | Downlink | Low | 921.9 | 184380 | 919.65 | 183930 | 0 | 15 | 2304 | 184350 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 922.2 | 184440 | 901.59 | 180318 | 102 |  | 2304 | 184350 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 922.5 | 184500 | 829.53 | 165906 | 504 |  | 2307 | 184590 | 0 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 876.9 | 175380 | 874.65 | 174930 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 877.2 | 175440 | 784.23 | 156846 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 877.5 | 175500 | 874.17 | 174834 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 with minimum channel bandwidth 3 MHz and channel bandwidth 3 MHz or 5 MHz in TS 38.213 [22]. The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.101	Reference test frequencies for NR operating band n101

Table 4.3.1.1.1.101-1: Test frequencies for NR operating band n101 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink & Uplink | Low | 1902.5 | 380500 | 1900.25 | 380050 | 0 | 15 | 4754 | 380410 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1905 | 381000 | 1884.39 | 376878 | 102 |  | 4761 | 380910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1907.5 | 381500 | 1814.53 | 362906 | 504 |  | 4768 | 381410 | 0 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink & Uplink | Low | 1905 | 381000 | 1900.32 | 380064 | 0 | 15 | 4755 | 380430 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.101-2: Test frequencies for NR operating band n101 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink & Uplink | Low | 1905 | 381000 | 1900,68 | 380136 | 0 | 15 | 4762 | 380930 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.102 to 4.3.1.1.1.105	FFS

###### 4.3.1.1.1.106	Reference test frequencies for NR operating band n106

Table 4.3.1.1.1.106-1: Test frequencies for NR operating band n106 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | Downlink | Low | 938.0 | 187600 | 918.29 | 183658 | 102 |  | 31326 | 187630 | 4 | 0 | 3 (2) | 104 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 899.0 | 179800 | 806.93 | 161386 | 504 |  | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-0 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.107 to 4.3.1.1.1.108	FFS

###### 4.3.1.1.1.109	Reference test frequencies for NR operating band n109

Table 4.3.1.1.1.109-1: Test frequencies for NR operating band n109 and SCS 15 kHz

| UL/DLCBW Combination [MHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | 3584 | 286810 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | 3687 | 294990 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | 3787 | 302930 | 4 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 5/10 | 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 |  | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 5/15 | 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 1444.5 | 288900 | 1432.53 | 286506 | 0 | 15 | 3588 | 287070 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1444.17 | 288834 | 102 |  | 3663 | 293070 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1504.5 | 300900 | 1401.81 | 280362 | 504 |  | 3738 | 299070 | 8 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 5/30 | 30 | 160 | Downlink | Low | 1447 | 289400 | 1432.6 | 286520 | 0 | 15 | 3589 | 287090 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1441.74 | 288348 | 102 |  | 3657 | 292590 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1502 | 300400 | 1396.88 | 279376 | 504 |  | 3725 | 298090 | 10 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 5/40 | 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 15 | 3588 | 287070 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3642 | 291390 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3699 | 295950 | 2 | 0 | 1 (2) | 506 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 5/50 | 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 15 | 3587 | 287050 | 2 | 2 | 1 (2) | 4 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3630 | 290430 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3676 | 294050 | 2 | 2 | 1 (2) | 508 |
|  | 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 625.03 | 125006 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 730.5 | 146100 | 727.17 | 145434 | 6 |  | - | - | - | - | - | - |
| 10/10 | 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | 3585 | 286830 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3681 | 294510 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3774 | 301950 | 6 | 1 | 1 (2) | 507 |
|  | 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 622.6 | 124520 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 722.24 | 144448 | 6 |  | - | - | - | - | - | - |
| 10/15 | 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 622.6 | 124520 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 722.24 | 144448 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 622.6 | 124520 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 722.24 | 144448 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 1444.5 | 288900 | 1432.53 | 286506 | 0 | 15 | 3588 | 287070 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1444.17 | 288834 | 102 |  | 3663 | 293070 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1504.5 | 300900 | 1401.81 | 280362 | 504 |  | 3738 | 299070 | 8 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 622.6 | 124520 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 722.24 | 144448 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 160 | Downlink | Low | 1447 | 289400 | 1432.6 | 286520 | 0 | 15 | 3589 | 287090 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1441.74 | 288348 | 102 |  | 3657 | 292590 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1502 | 300400 | 1396.88 | 279376 | 504 |  | 3725 | 298090 | 10 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 622.6 | 124520 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 722.24 | 144448 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 15 | 3588 | 287070 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3642 | 291390 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3699 | 295950 | 2 | 0 | 1 (2) | 506 |
|  | 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 622.6 | 124520 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 722.24 | 144448 | 6 |  | - | - | - | - | - | - |
| 10/50 | 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 15 | 3587 | 287050 | 2 | 2 | 1 (2) | 4 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3630 | 290430 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3676 | 294050 | 2 | 2 | 1 (2) | 508 |
|  | 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 622.6 | 124520 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 722.24 | 144448 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | 3586 | 286850 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3675 | 294030 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3761 | 300970 | 8 | 1 | 1 (2) | 507 |
|  | 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 620.17 | 124034 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 717.31 | 143462 | 6 |  | - | - | - | - | - | - |
| 15/20 | 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 620.17 | 124034 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 717.31 | 143462 | 6 |  | - | - | - | - | - | - |
| 15/25 | 25 | 133 | Downlink | Low | 1444.5 | 288900 | 1432.53 | 286506 | 0 | 15 | 3588 | 287070 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1444.17 | 288834 | 102 |  | 3663 | 293070 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1504.5 | 300900 | 1401.81 | 280362 | 504 |  | 3738 | 299070 | 8 | 1 | 2 (4) | 509 |
|  | 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 620.17 | 124034 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 717.31 | 143462 | 6 |  | - | - | - | - | - | - |
| 15/30 | 30 | 160 | Downlink | Low | 1447 | 289400 | 1432.6 | 286520 | 0 | 15 | 3589 | 287090 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1441.74 | 288348 | 102 |  | 3657 | 292590 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1502 | 300400 | 1396.88 | 279376 | 504 |  | 3725 | 298090 | 10 | 1 | 2 (4) | 509 |
|  | 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 620.17 | 124034 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 717.31 | 143462 | 6 |  | - | - | - | - | - | - |
| 15/40 | 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 15 | 3588 | 287070 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3642 | 291390 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3699 | 295950 | 2 | 0 | 1 (2) | 506 |
|  | 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 620.17 | 124034 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 717.31 | 143462 | 6 |  | - | - | - | - | - | - |
| 15/50 | 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 15 | 3587 | 287050 | 2 | 2 | 1 (2) | 4 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3630 | 290430 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3676 | 294050 | 2 | 2 | 1 (2) | 508 |
|  | 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 620.17 | 124034 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 717.31 | 143462 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | 3587 | 287050 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3669 | 293550 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3751 | 300050 | 6 | 1 | 2 (4) | 509 |
|  | 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 617.74 | 123548 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 712.38 | 142476 | 6 |  | - | - | - | - | - | - |
| 20/25 | 25 | 133 | Downlink | Low | 1444.5 | 288900 | 1432.53 | 286506 | 0 | 15 | 3588 | 287070 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1444.17 | 288834 | 102 |  | 3663 | 293070 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1504.5 | 300900 | 1401.81 | 280362 | 504 |  | 3738 | 299070 | 8 | 1 | 2 (4) | 509 |
|  | 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 617.74 | 123548 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 712.38 | 142476 | 6 |  | - | - | - | - | - | - |
| 20/30 | 30 | 160 | Downlink | Low | 1447 | 289400 | 1432.6 | 286520 | 0 | 15 | 3589 | 287090 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1441.74 | 288348 | 102 |  | 3657 | 292590 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1502 | 300400 | 1396.88 | 279376 | 504 |  | 3725 | 298090 | 10 | 1 | 2 (4) | 509 |
|  | 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 617.74 | 123548 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 712.38 | 142476 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 15 | 3588 | 287070 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3642 | 291390 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3699 | 295950 | 2 | 0 | 1 (2) | 506 |
|  | 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 617.74 | 123548 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 712.38 | 142476 | 6 |  | - | - | - | - | - | - |
| 20/50 | 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 15 | 3587 | 287050 | 2 | 2 | 1 (2) | 4 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3630 | 290430 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3676 | 294050 | 2 | 2 | 1 (2) | 508 |
|  | 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 617.74 | 123548 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 712.38 | 142476 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 133 | Downlink | Low | 1444.5 | 288900 | 1432.53 | 286506 | 0 | 15 | 3588 | 287070 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1444.17 | 288834 | 102 |  | 3663 | 293070 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1504.5 | 300900 | 1401.81 | 280362 | 504 |  | 3738 | 299070 | 8 | 1 | 2 (4) | 509 |
|  | 25 | 133 | Uplink | Low | 715.5 | 143100 | 703.53 | 140706 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 615.31 | 123062 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 707.45 | 141490 | 6 |  | - | - | - | - | - | - |
| 25/30 | 30 | 160 | Downlink | Low | 1447 | 289400 | 1432.6 | 286520 | 0 | 15 | 3589 | 287090 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1441.74 | 288348 | 102 |  | 3657 | 292590 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 1502 | 300400 | 1396.88 | 279376 | 504 |  | 3725 | 298090 | 10 | 1 | 2 (4) | 509 |
|  | 25 | 133 | Uplink | Low | 715.5 | 143100 | 703.53 | 140706 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 615.31 | 123062 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 707.45 | 141490 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 15 | 3588 | 287070 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3642 | 291390 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3699 | 295950 | 2 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 715.5 | 143100 | 703.53 | 140706 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 615.31 | 123062 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 707.45 | 141490 | 6 |  | - | - | - | - | - | - |
| 25/50 | 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 15 | 3587 | 287050 | 2 | 2 | 1 (2) | 4 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3630 | 290430 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3676 | 294050 | 2 | 2 | 1 (2) | 508 |
|  | 25 | 133 | Uplink | Low | 715.5 | 143100 | 703.53 | 140706 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 615.31 | 123062 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 707.45 | 141490 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 160 | Downlink | Low | 1474.5 | 294900 | 1460.1 | 292020 | 0 | 15 | 3657 | 292590 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 160 | Uplink | Low | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 30/40 | 40 | 216 | Downlink | Low | 1474.5 | 294900 | 1455.06 | 291012 | 0 | 15 | 3642 | 291390 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 160 | Uplink | Low | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 30/50 | 50 | 270 | Downlink | Low | 1474.5 | 294900 | 1450.2 | 290040 | 0 | 15 | 3630 | 290430 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 160 | Uplink | Low | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.109-2: Test frequencies for NR operating band n109 and SCS 30 kHz

| UL/DLCBW Combination [MHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | 3591 | 287310 | 18 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (8) | 220 |
|  |  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 532.24 | 106448 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 721.52 | 144304 | 6 |  | - | - | - | - | - | - |
| 10/15 | 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 532.24 | 106448 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 721.52 | 144304 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 532.24 | 106448 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 721.52 | 144304 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 1444.5 | 288900 | 1432.8 | 286560 | 0 | 15 | 3591 | 287310 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1426.08 | 285216 | 102 |  | 3666 | 293310 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1504.5 | 300900 | 1311.36 | 262272 | 504 |  | 3741 | 299310 | 10 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 532.24 | 106448 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 721.52 | 144304 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 78 | Downlink | Low | 1447 | 289400 | 1432.96 | 286592 | 0 | 15 | 3592 | 287330 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1423.74 | 284748 | 102 |  | 3660 | 292830 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1502 | 300400 | 1306.52 | 261304 | 504 |  | 3731 | 298570 | 14 | 0 | 3 (8) | 1024 |
|  | 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 532.24 | 106448 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 721.52 | 144304 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 15 | 3591 | 287310 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (6) | 1020 |
|  | 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 532.24 | 106448 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 721.52 | 144304 | 6 |  | - | - | - | - | - | - |
| 10/50 | 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 15 | 3593 | 287530 | 18 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 532.24 | 106448 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 728 | 145600 | 721.52 | 144304 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | 3592 | 287330 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (7) | 1022 |
|  | 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 529.72 | 105944 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 716.5 | 143300 | 6 |  | - | - | - | - | - | - |
| 15/20 | 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 529.72 | 105944 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 716.5 | 143300 | 6 |  | - | - | - | - | - | - |
| 15/25 | 25 | 65 | Downlink | Low | 1444.5 | 288900 | 1432.8 | 286560 | 0 | 15 | 3591 | 287310 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1426.08 | 285216 | 102 |  | 3666 | 293310 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1504.5 | 300900 | 1311.36 | 262272 | 504 |  | 3741 | 299310 | 10 | 0 | 0 (5) | 1018 |
|  | 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 529.72 | 105944 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 716.5 | 143300 | 6 |  | - | - | - | - | - | - |
| 15/30 | 30 | 78 | Downlink | Low | 1447 | 289400 | 1432.96 | 286592 | 0 | 15 | 3592 | 287330 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1423.74 | 284748 | 102 |  | 3660 | 292830 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1502 | 300400 | 1306.52 | 261304 | 504 |  | 3731 | 298570 | 14 | 0 | 3 (8) | 1024 |
|  | 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 529.72 | 105944 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 716.5 | 143300 | 6 |  | - | - | - | - | - | - |
| 15/40 | 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 15 | 3591 | 287310 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (6) | 1020 |
|  | 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 529.72 | 105944 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 716.5 | 143300 | 6 |  | - | - | - | - | - | - |
| 15/50 | 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 15 | 3593 | 287530 | 18 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (7) | 1022 |
|  | 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 529.72 | 105944 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 725.5 | 145100 | 716.5 | 143300 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | 3590 | 287290 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (5) | 1018 |
|  | 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 527.38 | 105476 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 711.66 | 142332 | 6 |  | - | - | - | - | - | - |
| 20/25 | 25 | 65 | Downlink | Low | 1444.5 | 288900 | 1432.8 | 286560 | 0 | 15 | 3591 | 287310 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1426.08 | 285216 | 102 |  | 3666 | 293310 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1504.5 | 300900 | 1311.36 | 262272 | 504 |  | 3741 | 299310 | 10 | 0 | 0 (5) | 1018 |
|  | 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 527.38 | 105476 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 711.66 | 142332 | 6 |  | - | - | - | - | - | - |
| 20/30 | 30 | 78 | Downlink | Low | 1447 | 289400 | 1432.96 | 286592 | 0 | 15 | 3592 | 287330 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1423.74 | 284748 | 102 |  | 3660 | 292830 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1502 | 300400 | 1306.52 | 261304 | 504 |  | 3731 | 298570 | 14 | 0 | 3 (8) | 1024 |
|  | 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 527.38 | 105476 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 711.66 | 142332 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 15 | 3591 | 287310 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (6) | 1020 |
|  | 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 527.38 | 105476 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 711.66 | 142332 | 6 |  | - | - | - | - | - | - |
| 20/50 | 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 15 | 3593 | 287530 | 18 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (7) | 1022 |
|  | 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 527.38 | 105476 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 723 | 144600 | 711.66 | 142332 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 65 | Downlink | Low | 1444.5 | 288900 | 1432.8 | 286560 | 0 | 15 | 3591 | 287310 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1426.08 | 285216 | 102 |  | 3666 | 293310 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1504.5 | 300900 | 1311.36 | 262272 | 504 |  | 3741 | 299310 | 10 | 0 | 0 (5) | 1018 |
|  | 25 | 65 | Uplink | Low | 715.5 | 143100 | 703.8 | 140760 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 524.86 | 104972 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 706.64 | 141328 | 6 |  | - | - | - | - | - | - |
| 25/30 | 30 | 78 | Downlink | Low | 1447 | 289400 | 1432.96 | 286592 | 0 | 15 | 3592 | 287330 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1423.74 | 284748 | 102 |  | 3660 | 292830 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1502 | 300400 | 1306.52 | 261304 | 504 |  | 3731 | 298570 | 14 | 0 | 3 (8) | 1024 |
|  | 25 | 65 | Uplink | Low | 715.5 | 143100 | 703.8 | 140760 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 524.86 | 104972 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 706.64 | 141328 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 15 | 3591 | 287310 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (6) | 1020 |
|  | 25 | 65 | Uplink | Low | 715.5 | 143100 | 703.8 | 140760 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 524.86 | 104972 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 706.64 | 141328 | 6 |  | - | - | - | - | - | - |
| 25/50 | 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 15 | 3593 | 287530 | 18 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (7) | 1022 |
|  | 25 | 65 | Uplink | Low | 715.5 | 143100 | 703.8 | 140760 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 718 | 143600 | 524.86 | 104972 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 720.5 | 144100 | 706.64 | 141328 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 78 | Downlink | Low | 1474.5 | 294900 | 1460.46 | 292092 | 0 | 15 | 3660 | 292830 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 78 | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 30/40 | 40 | 106 | Downlink | Low | 1474.5 | 294900 | 1455.42 | 291084 | 0 | 15 | 3648 | 291870 | 22 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 78 | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 30/50 | 50 | 133 | Downlink | Low | 1474.5 | 294900 | 1450.56 | 290112 | 0 | 15 | 3636 | 290910 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 78 | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |