3GPP TSG-RAN5 Meeting #105	R5-247329

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0236 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to BDS SV IDs for Signaling scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN5#104, the BDS SVIDs were corrected to introduce BDS-2 satellites, and rename the existing BDS SVIDs as BDS-3, only affecting performance scenarios. Ref R5-245981. Similar change needs to be added for signaling scenarios. - As per the BDS-SIS-ICD-B1I-3.0 specification, there 2 flavours of BDS: BDS-2 (BDS-SIS-ICD-2.1) with support with SV IDs 1-37 BDS-3 (BDS-SIS-ICD-B1I-3.0) with support with SV IDs >37 The support of each of the flavours is a UE capability. Since the current SV IDs defined for BDS testing are > 37, they don’t covers the UEs supporting BDS-2 only. - For closing this coverage gap, without changing the current tests for BDS-3, we propose to add additional dedicated SV-IDs for BDS-2 UEs. - Also in order to keep the perfomance comparable between BDS-2 and BDS-3, we propose to keep the orbits the same. This result in some artificial scenarios, however which fulfill the purpose of testing. - The resulting mapping between BDS-2 and BDS-3 SV IDs would then be: BDS-3; BDS-2. 38; 6. 40; 7. 42; 11. 43; 12. 44; 13. 45; 14. 59; 2. 60; 1. |
| Summary of change: |  | - Added SV-IDs set for BDS testing for UE supporting BDS-2 - Added respective RINEX files and references in the spec Sig GNSS BDS-2 2020_9_17 Rinex.txt - Marked the current SV-IDs set for BDS testing as for BDS-3 - In the .zip “GNSS_Orbital_Data_Sig_V3.zip”, below changes compared to ““GNSS_Orbital_Data_Sig_V2.zip” - “Sig GNSS BDS-2 2020_9_17 Rinex.txt” file added - “Sig GNSS BDS 2020_9_17 Rinex.txt” file renamed to “Sig GNSS BDS-3 2020_9_17 Rinex.txt” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UEs supporting only BDS-2, will not be able to be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2, B.1 .zip file “GNSS_Orbital_Data_Sig_V2.zip” |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 37-571-3 CR 0173 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note to Editor: Please replace “GNSS_Orbital_Data_Sig_V2.zip” with “GNSS_Orbital_Data_Sig_V3.zip” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 6.1.2	GNSS Scenario

Table 6.1.2-0: Void

The following GNSS scenario shall be used. The assistance data specified in the following subclauses is consistent with this GNSS scenario:

-	Rinex navigation data files: the required file(s) in the GNSS orbital data sig zip file specified in Annex B are given below.

Table 6.1.2-1: Rinex navigation data files for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 2 | Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 3 | Sig GNSS GPS 2020_9_17 Rinex.txt |
| 4 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Sig GNSS BDS-2 2020_9_17 Rinex.txt or Sig GNSS BDS-3 2020_9_17 Rinex.txt (1) |
| 10 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS BDS-2 2020_9_17 Rinex.txt or Sig GNSS BDS-3 2020_9_17 Rinex.txt (1) |
| Note 1: If pc_BDS2_only = TRUE, the rinex files for BDS-2 shall be used. If pc_BDS2_only = FALSE, the rinex files for BDS-3 shall be used. |  |

Table 6.1.2-2: Rinex navigation data files for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | Rinex navigation datafile(s) (1) |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15, 25 | GPS | Sig GNSS GPS 2020_9_17 Rinex.txt |
|  | GLONASS | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
|  | Galileo | Sig GNSS Galileo 2020_9_17 Rinex.txt |
|  | BDS | Sig GNSS BDS-2 2020_9_17 Rinex.txt or Sig GNSS BDS-3 2020_9_17 Rinex.txt (2) |
| Note 1: Where the UE supports more than one GNSS then all the relevant Rinex navigation data files are used Note 2: If pc_BDS2_only = TRUE, the rinex files for BDS-2 shall be used. If pc_BDS2_only = FALSE, the rinex files for BDS-3 shall be used. |  |  |

-	UE location and Reference location: Static at latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan), height: = 300m.

-	Nominal start time: 17th September 2020 23:40:00 (GPS time).

-	The visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given in Table 6.1.2-2A.

Table 6.1.2-2A: SV IDs of Visible satellites

| GNSS | SV IDs of Visible satellites |
| --- | --- |
| GPS | 3, 4, 6, 17, 19, 28 |
| GLONASS | 3, 4, 5, 10, 18, 19 |
| Galileo | 3, 5, 13, 15, 21, 27 |
| BDS | 1, 2, 6, 7, 11, 12 (BDS-2) or 38, 40, 42, 43, 59, 60 (BDS-3) (1) |
| Note 1: If pc_BDS2_only = TRUE, the SV IDs for BDS-2 shall be used. If pc_BDS2_only = FALSE, the SV IDs for BDS-3 shall be used. |  |

-	For BDS, the satellite types are given in Table 6.1.2-2B

Table 6.1.2-2B: BDS satellite types

| BDS Satellite type | SV IDs of Satellites |
| --- | --- |
| GEO | 59, 60, 1, 2 |
| IGSO | 38, 40, 6, 7 |
| MEO | 42, 43, 11, 12 |

-	The visible satellites to be simulated in each sub-test case are given in Table 6.1.2-3 and Table 6.1.2-4

Table 6.1.2-3: Satellites to be simulated for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 5, 10, 18, 19 (GLONASS) |
| 2 | 3, 5, 13, 15, 21, 27 (Galileo) |
| 3 | 3, 4, 6, 17, 19, 28 (GPS) (1) |
| 4 | GPS: 3, 4, 6, 28. GLONASS: 5, 16, 18, 19. |
| 8 | GPS: 3, 4, 6, 28. Galileo: 3, 5, 13, 21. |
| 9 | 1, 2, 6, 7, 11, 12 (BDS-2) or 38, 40, 42, 43, 59, 60 (BDS-3) (2) |
| 10 | GPS: 3, 4, 6, 28. BDS: 1, 2, 6, 7 (BDS-2) or 38, 40, 59, 60 (BDS-3). (2) |
| Note 1: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. Note 2: If pc_BDS2_only = TRUE, the SV IDs for BDS-2 shall be used. If pc_BDS2_only = FALSE, the SV IDs for BDS-3 shall be used. |  |

Table 6.1.2-4: Satellites to be simulated for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | SV IDs of Satellites to be simulated |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15, 25 (1) | GPS | 3, 4, 6, 17, 19, 28 |
|  | GLONASS | 3, 4, 5, 10, 18, 19 |
|  | Galileo | 3, 5, 13, 15, 21, 27 |
|  | BDS | 1, 2, 6, 7, 11, 12 (BDS-2) or 38, 40, 42, 43, 59, 60 (BDS-3) (2) |
| Note 1: For this sub-test the satellite simulator shall generate all the GNSS signals supported by the UE for all the simulated satellites. Note 2: If pc_BDS2_only = TRUE, the SV IDs for BDS-2 shall be used. If pc_BDS2_only = FALSE, the SV IDs for BDS-3 shall be used. |  |  |

# < Unchanged sections omitted >

## B.1	GNSS data files for signalling tests

The GNSS orbital data files for use in GNSS signalling tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9 are contained in archive GNSS_Orbital_Data_Sig_V3.zip which accompanies the present document.

< End of changes >