3GPP TSG-RAN WG5 Meeting #105	R5-247783

# Orlando, USA,  18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3400 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test channel bandwidths and test frequencies for band n254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test CBWs and test frequencies need to be specified for NR band n254 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses updated to add test frequencies for band n254 - Mid, Low, High test channel bandwidths in clause 4.3.1. - Add Reference test frequencies for NR operating band n109 in Clause 4.3.1.9.1.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test CBWs and test frequencies for band n254 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 4.3.1.9.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-247251 |  |  |  |  |  |  |  |  |

### 4.3.1	Test frequencies

Editor's Note: n65 (High: 50MHz), n66 (High: 45MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

#### 4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8], TS 38.101-3 [9] and TS 38.101-5 [56].

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1, Table 4.3.1.0A-2 and Table 4.3.1.0A-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap and eRedCap UE Mid Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 25 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 15 | 15 |
| n12 | 10 | 10 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 2511 | 15 |
| n26 | 15 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n31 | 5 | 5 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 20 | 15 |
| n40 | 50 | 15 |
| n41 | 50 | 15 |
| n46 | 604, 405 | N/A |
| n48 | 2011 | 15 |
| n50 | 404, 205,11 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n54 | 5 | 5 |
| n65 | 15 | 15 |
| n66 | 20 | 15 |
| n70 | 152, 103,12 | 152, 103 |
| n71 | 153,12, 202 | 15 |
| n72 | 5 | 5 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 50 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n85 | 10 | 10 |
| n86 | 203 | [15] |
| n91 | 52,12, 103 | [52,103] |
| n92 | 15 | [15] |
| n93 | 52,12, 103 | [52,103] |
| n94 | 15 | [15] |
| n95 | 103 | [10] |
| n96 | 60 | N/A |
| n97 | 503 | [15] |
| n99 | 103 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n106 | 3 | N/A |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 (most updated version of latest release available) [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: This Mid test channel bandwidth is chosen since it is more commonly used. Note 11: This is an exception to Note 1 in this table to avoid not allowed asymmetric channel bandwidths. Note 12: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 200 | 100 |
| n258 | 200 | 100 |
| n259 | 200 | N/A |
| n260 | 200 | N/A |
| n261 | 200 | 100 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-2 [15] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. |  |  |

Table 4.3.1.0A-3: Mid Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 15 |
| n255 | 15 |
| n254 | 10 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-5 [57] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in TS 38.306 clause 4.2.7.2 [55] for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

#### 4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1, Table 4.3.1.0B-2 and Table 4.3.1.0B-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band | UE Low Test Channel bandwidth [MHz]1, 1a, 1b | RedCap and eRedCap UE Low Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 5 | 5 |
| n2 | 5 | 5 |
| n3 | 5 | 5 |
| n5 | 5 | 5 |
| n7 | 5 | 5 |
| n8 | 5 | 5 |
| n12 | 5 | 5 |
| n13 | 5 | 5 |
| n14 | 5 | 5 |
| n20 | 5 | 5 |
| n24 | 5 | 5 |
| n25 | 5 | 5 |
| n26 | 36,5 | 5 |
| n28 | 36,5 | 5 |
| n29 | 52 | N/A |
| n30 | 5 | 5 |
| n31 | 3 | 5 |
| n34 | 5 | 5 |
| n38 | 5 | 5 |
| n39 | 5 | 5 |
| n40 | 54,105 | 10 |
| n41 | 10 | 10 |
| n46 | 104,205 | N/A |
| n48 | 54, 105 | 10 |
| n50 | 54,105 | 10 |
| n51 | 5 | 5 |
| n53 | 5 | 5 |
| n54 | 5 | 5 |
| n65 | 5 | 5 |
| n66 | 5 | 5 |
| n70 | 5 | 5 |
| n71 | 5 | 5 |
| n72 | 3 | 5 |
| n74 | 5 | 5 |
| n75 | 52 | N/A |
| n76 | 52 | N/A |
| n77 | 10 | 10 |
| n78 | 10 | 10 |
| n79 | 10 | 10 |
| n80 | 53 | [5] |
| n81 | 53 | [5] |
| n82 | 53 | [5] |
| n83 | 53 | [5] |
| n84 | 53 | [5] |
| n85 | 36,5 | 5 |
| n86 | 53 | [5] |
| n91 | 5 | [5] |
| n92 | 5 | [5] |
| n93 | 5 | [5] |
| n94 | 5 | [5] |
| n95 | 53 | [5] |
| n96 | 20 | N/A |
| n97 | 53 | [5] |
| n99 | 53 | [5] |
| n100 | 5 | 5 |
| n101 | 36,5 | 5 |
| n106 | 3 | N/A |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as SCell in CA or SCell in DC configuration. Note 5: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 6: This UE channel bandwidth is optional in Release 18. |  |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR Band | UE Low Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 50 | 50 |
| n258 | 50 | 50 |
| n259 | 50 | N/A |
| n260 | 50 | N/A |
| n261 | 50 | 50 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. |  |  |

Table 4.3.1.0B-3: Low Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 5 |
| n255 | 5 |
| n254 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

#### 4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1, Table 4.3.1.0C-2 and Table 4.3.1.0C-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz]10, 11, 12 | RedCap and eRedCap UE High Test Channel bandwidth [MHz] 10, 11, 12 |
| --- | --- | --- |
| n1 | 50 | 20 |
| n2 | 40 | 20 |
| n3 | 50 | 20 |
| n5 | 201,14, 252 | 20 |
| n7 | 50 | 20 |
| n8 | 201,14, 352 | 20 |
| n12 | 15 | 15 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 20 | 20 |
| n24 | 10 | 10 |
| n25 | 401,14, 452 | 20 |
| n26 | 20 | 20 |
| n28 | 30 | 20 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n31 | 5 | 5 |
| n34 | 15 | 15 |
| n38 | 25, 4013 | 15 |
| n39 | 40 | 20 |
| n40 | 100 | 20 |
| n41 | 100 | 20 |
| n46 | 80 | N/A |
| n48 | 403, 1004 | 20 |
| n50 | 601,14, 802 | 20 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n54 | 5 | 5 |
| n65 | 20 | 20 |
| n66 | 40 | 20 |
| n70 | 151,14, 252 | 151, 202 |
| n71 | 201,14, 352 | 20 |
| n72 | 5 | 5 |
| n74 | 20 | 20 |
| n75 | 202 | N/A |
| n76 | 52 | N/A |
| n77 | 100 | 20 |
| n78 | 100 | 20 |
| n79 | 100 | 20 |
| n80 | 301 | [20] |
| n81 | 201 | [20] |
| n82 | 201 | [20] |
| n83 | 301 | [20] |
| n84 | 501 | [20] |
| n85 | 15 | 15 |
| n86 | 401 | [20] |
| n91 | 101, 52,14 | [10] |
| n92 | 20 | [20] |
| n93 | 101, 52,14 | [10] |
| n94 | 20 | [20] |
| n95 | 151 | [15] |
| n96 | 80 | N/A |
| n97 | 1001 | [20] |
| n99 | 101 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n106 | 3 | N/A |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-1 [7] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 11: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-1 [7] is supported. However, these channel bandwidths (except for 100MHz of bands n41, n48, n77, n78, n79) are mandatory with capability signalling as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL and channelBWs-DL-v1590/channelBWs-UL-v1590 parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Channel bandwidth 100MHz of bands n41, n48, n77, n78, and n79 is mandatory without capability signalling (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Note 12: For CA, DC, SDL and SUL, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA, DC, SDL or SUL, testing could be performed with one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth instead of the values described in the Table. Note 13: These UE channel bandwidths are applicable to sidelink operation. Note 14: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

NOTE 1: Void.

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 400 | 100 |
| n258 | 400 | 100 |
| n259 | 400 | N/A |
| n260 | 400 | N/A |
| n261 | 400 | 100 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-2 [8] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-2 [8] is mandatory with capability bit mandated to be set to 1 (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Note 3: For CA and DC, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA and DC, testing could be performed with one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth instead of the values described in the Table. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination), testing for Rel-15 and Rel-16 could be performed using the value signalled by the UE in single carrier operation instead of the values described in Table 4.3.1.0C-2.

Table 4.3.1.0C-3: High Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 20 |
| n255 | 20 |
| n254 | 5 |
| Note 1: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-5 [56] is mandatory without capability parameter (i.e., purely mandatory) as defined in TS 38.306 clauses 4.2.7.6/4.2.7.8 [55] for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). |  |

#### 4.3.1.0D	Bandwidth part

The value of locationAndBandwidth in BWP for FR1 is given in Table 4.3.1.0D-1. The value of locationAndBandwidth in BWP for FR2 is given in Table 4.3.1.0D-2.

Table 4.3.1.0D-1: locationAndBandwidth in BWP for FR1

| BW (MHz) | SCS (kHz) | LRBs (Note 2) | locationAndBandwidth (Note 1) |
| --- | --- | --- | --- |
| 3 | 15 | 15 | 3850 |
| 3 | 30 | N/A | N/A |
| 3 | 60 | N/A | N/A |
| 5 | 15 | 25 | 6600 |
| 5 | 30 | 11 | 2750 |
| 5 | 60 | N/A | N/A |
| 10 | 15 | 52 | 14025 |
| 10 | 30 | 24 | 6325 |
| 10 | 60 | 11 | 2750 |
| 15 | 15 | 79 | 21450 |
| 15 | 30 | 38 | 10175 |
| 15 | 60 | 18 | 4675 |
| 20 | 15 | 106 | 28875 |
| 20 | 30 | 51 | 13750 |
| 20 | 60 | 24 | 6325 |
| 25 | 15 | 133 | 36300 |
| 25 | 30 | 65 | 17600 |
| 25 | 60 | 31 | 8250 |
| 30 | 15 | 160 | 32174 |
| 30 | 30 | 78 | 21175 |
| 30 | 60 | 38 | 10175 |
| 35 | 15 | 188 | 24474 |
| 35 | 30 | 92 | 25025 |
| 35 | 60 | 44 | 11825 |
| 40 | 15 | 216 | 16774 |
| 40 | 30 | 106 | 28875 |
| 40 | 60 | 51 | 13750 |
| 45 | 15 | 242 | 9624 |
| 45 | 30 | 119 | 32450 |
| 45 | 60 | 58 | 15675 |
| 50 | 15 | 270 | 1924 |
| 50 | 30 | 133 | 36300 |
| 50 | 60 | 65 | 17600 |
| 60 | 15 | N/A | N/A |
| 60 | 30 | 162 | 31624 |
| 60 | 60 | 79 | 21450 |
| 70 | 15 | N/A | N/A |
| 70 | 30 | 189 | 24199 |
| 70 | 60 | 93 | 25300 |
| 80 | 15 | N/A | N/A |
| 80 | 30 | 217 | 16499 |
| 80 | 60 | 107 | 29150 |
| 90 | 15 | N/A | N/A |
| 90 | 30 | 245 | 8799 |
| 90 | 60 | 121 | 33000 |
| 100 | 15 | N/A | N/A |
| 100 | 30 | 273 | 1099 |
| 100 | 60 | 135 | 36850 |
| Note 1: The value for locationAndBandwidth parameter is calculated as the RIV value in accordance to TS 38.214 [21] with = 275, =0 and for each bandwidth and subcarrier spacing. Note 2: = Max NRB is the default configuration, unless explicitly specified. |  |  |  |

Table 4.3.1.0D-2: locationAndBandwidth in BWP for FR2

| BW (MHz) | SCS (kHz) | LRBs (Note 2) | locationAndBandwidth (Note 1) |
| --- | --- | --- | --- |
| 50 | 60 | 66 | 17875 |
| 50 | 120 | 32 | 8525 |
| 100 | 60 | 132 | 36025 |
| 100 | 120 | 24(Note 3) | 6325 |
| 100 | 120 | 48(Note 3) | 12925 |
| 100 | 120 | 66 | 17875 |
| 200 | 60 | 264 | 3574 |
| 200 | 120 | 132 | 36025 |
| 400 | 60 | N/A | N/A |
| 400 | 120 | 264 | 3574 |
| Note 1: The value for locationAndBandwidth parameter is calculated as the RIV value in accordance to TS 38.214 [21] with = 275 and =0 for each bandwidth and subcarrier spacing. Note 2: = Max NRB is the default configuration, unless explicitly specified. Note 3: For configurations where the is explicitly specified to be < Max NRB. |  |  |  |

#### 4.3.1.0E	Void

#### 4.3.1.9	Test frequencies for NR NTN operating bands

##### 4.3.1.9.1	NR NTN operating bands

4.3.1.9.1.1	Reference test frequencies for NR NTN operating band n256

Table 4.3.1.9.1.1-1: Test frequencies for NR operating band n256 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2172.5 | 434500 | 2170.25 | 434050 | 0 | 15 | 5429 | 434410 | 0 | 0 | 0(0) | 0 |
|  |  |  | Mid | 2185 | 437000 | 2164.39 | 432878 | 102 |  | 5464 | 437090 | 0 | 1 | 2(4) | 107 |
|  |  |  | High | 2197.5 | 439500 | 2104.53 | 420906 | 504 |  | 5493 | 439470 | 8 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1982.5 | 396500 | 1980.25 | 396050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1902.03 | 380406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2007.5 | 401500 | 2004.17 | 400834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2175 | 435000 | 2170.32 | 434064 | 0 | 15 | 5430 | 434430 | 2 | 0 | 0(0) | 0 |
|  |  |  | Mid | 2185 | 437000 | 2161.96 | 432392 | 102 |  | 5458 | 436610 | 2 | 1 | 2(4) | 107 |
|  |  |  | High | 2195 | 439000 | 2099.6 | 419920 | 504 |  | 5480 | 438490 | 10 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1985 | 397000 | 1980.32 | 396064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1899.6 | 379920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1999.24 | 399848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2177.5 | 435500 | 2170.39 | 434078 | 0 | 15 | 5431 | 434450 | 4 | 0 | 0(0) | 0 |
|  |  |  | Mid | 2185.0 | 437000 | 2159.53 | 431906 | 102 |  | 5452 | 436130 | 4 | 1 | 2(4) | 107 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1987.5 | 397500 | 1980.39 | 396078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1897.17 | 379434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1994.31 | 398862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2180 | 436000 | 2170.46 | 434092 | 0 | 15 | 5432 | 434650 | 6 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2185 | 437000 | 2157.1 | 431420 | 102 |  | 5446 | 435650 | 6 | 1 | 2(4) | 107 |
|  |  |  | High | 2190 | 438000 | 2089.74 | 417948 | 504 |  | 5457 | 436590 | 10 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1990 | 398000 | 1980.46 | 396092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1894.74 | 378948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1989.38 | 397876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.1-2: Test frequencies for NR operating band n256 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 2175 | 435000 | 2165.64 | 433128 | 0 | 15 | 5424 | 433950 | 10 | 0 | 1(6) | 12 |
|  |  |  | Mid | 2185 | 437000 | 2138.92 | 427784 | 102 |  | 5449 | 435890 | 14 | 0 | 0(5) | 214 |
|  |  |  | High | 2195 | 439000 | 2004.2 | 400840 | 504 |  | 5474 | 438010 | 6 | 0 | 2(7) | 1022 |
|  |  | Uplink | Low | 1985 | 397000 | 1975.64 | 395128 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1804.2 | 360840 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1993.48 | 398696 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2177.5 | 435500 | 2163.28 | 432656 | 0 | 15 | 5419 | 433490 | 14 | 0 | 1(6) | 12 |
|  |  |  | Mid | 2185 | 437000 | 2134.06 | 426812 | 102 |  | 5437 | 434930 | 18 | 0 | 0(5) | 2146 |
|  |  |  | High | 2192.5 | 438500 | 1996.84 | 399368 | 504 |  | 5458 | 436610 | 6 | 0 | 3(8) | 1024 |
|  |  | Uplink | Low | 1987.5 | 397500 | 1973.28 | 394656 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1799.34 | 359868 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1986.12 | 397224 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2180 | 436000 | 2160.92 | 432184 | 0 | 15 | 5411 | 432970 | 22 | 0 | 0(5) | 10 |
|  |  |  | Mid | 2185 | 437000 | 2129.2 | 425840 | 102 |  | 5425 | 433970 | 22 | 0 | 0(5) | 214 |
|  |  |  | High | 2190 | 438000 | 1989.48 | 397896 | 504 |  | 5436 | 434910 | 2 | 0 | 0(5) | 1018 |
|  |  | Uplink | Low | 1990 | 398000 | 1970.92 | 394184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1794.48 | 358896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1978.76 | 395752 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.1-3: Test frequencies for NR operating band n256 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2175 | 435000 | 2171.04 | 434208 | 0 | 15 | - | 434568 |
|  |  |  | Mid | 2185 | 437000 | 2107.6 | 421520 | 102 |  | - | 436568 |
|  |  |  | High | 2195 | 439000 | 1828.16 | 365632 | 504 |  | - | 438568 |
|  |  | Uplink | Low | 1985 | 397000 | 1981.04 | 396208 | 0 | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1628.16 | 325632 | 504 |  | - | - |
|  |  |  | High | 2005 | 401000 | 1996.72 | 399344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2177.5 | 435500 | 2171.02 | 434204 | 0 | 15 | - | 434564 |
|  |  |  | Mid | 2185 | 437000 | 2105.08 | 421016 | 102 |  | - | 436064 |
|  |  |  | High | 2192.5 | 438500 | 1823.14 | 364628 | 504 |  | - | 437564 |
|  |  | Uplink | Low | 1987.5 | 397500 | 1981.02 | 396204 | 0 | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1625.64 | 325128 | 504 |  | - | - |
|  |  |  | High | 2002.5 | 400500 | 1991.7 | 398340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2180 | 436000 | 2171.36 | 434272 | 0 | 15 | - | 434632 |
|  |  |  | Mid | 2185 | 437000 | 2102.92 | 420584 | 102 |  | - | 435632 |
|  |  |  | High | 2190 | 438000 | 1818.48 | 363696 | 504 |  | - | 436632 |
|  |  | Uplink | Low | 1990 | 398000 | 1981.36 | 396272 | 0 | - | - | - |
|  |  |  | Mid | 1995 | 399000 | 1623.48 | 324696 | 504 |  | - | - |
|  |  |  | High | 2000 | 400000 | 1987.04 | 397408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.9.1.2	Reference test frequencies for NR NTN operating band n255

Table 4.3.1.9.1.2-1: Test frequencies for NR operating band n255 with SCS 15 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1527.5 | 305500 | 1525.25 | 305050 | 0 | 15 | 3818 | 305530 | 4 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1521.39 | 304278 | 102 |  | 3855 | 308430 | 4 | 1 | 1(2) | 105 |
|  |  |  | High | 1556.5 | 311300 | 1463.53 | 292706 | 504 |  | 3892 | 311330 | 4 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1629 | 325800 | 1626.75 | 325350 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1550.53 | 310106 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1658 | 331600 | 1654.67 | 330934 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1530 | 306000 | 1525.32 | 305064 | 0 | 15 | 3819 | 305550 | 6 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1518.96 | 303792 | 102 |  | 3849 | 307950 | 6 | 1 | 1(2) | 105 |
|  |  |  | High | 1554 | 310800 | 1458.6 | 291720 | 504 |  | 3879 | 310350 | 6 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1626.82 | 325364 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1548.1 | 309620 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1655.5 | 331100 | 1649.74 | 329948 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1532.5 | 306500 | 1525.39 | 305078 | 0 | 15 | 3820 | 305570 | 8 | 1 | 1(2) | 3 |
|  |  |  | Mid | 1542 | 308400 | 1516.53 | 303306 | 102 |  | 3843 | 307470 | 8 | 1 | 1(2) | 105 |
|  |  |  | High | 1551.5 | 310300 | 1453.67 | 290734 | 504 |  | 3866 | 309370 | 8 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1634 | 326800 | 1626.89 | 325378 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1545.67 | 309134 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1653 | 330600 | 1644.81 | 328962 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1535 | 307000 | 1525.46 | 305092 | 0 | 15 | 3818 | 305530 | 2 | 0 | 1(2) | 2 |
|  |  |  | Mid | 1542 | 308400 | 1514.1 | 302820 | 102 |  | 3837 | 306990 | 10 | 1 | 1(2) | 105 |
|  |  |  | High | 1549 | 309800 | 1448.74 | 289748 | 504 |  | 3856 | 308450 | 6 | 1 | 2(4) | 509 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1626.96 | 325392 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1543.24 | 308648 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1639.88 | 327976 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2-2: Test frequencies for NR operating band n255 with SCS 30 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 1530 | 306000 | 1520.64 | 304128 | 0 | 15 | 3813 | 305070 | 2 | 0 | 3(8) | 16 |
|  |  |  | Mid | 1542 | 308400 | 1495.92 | 299184 | 102 |  | 3843 | 307470 | 2 | 0 | 3(8) | 220 |
|  |  |  | High | 1554 | 310800 | 1363.2 | 272640 | 504 |  | 3873 | 309870 | 2 | 0 | 3(8) | 1024 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1622.14 | 324428 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1452.7 | 290540 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1655.5 | 331100 | 1643.98 | 328796 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1532.5 | 306500 | 1518.28 | 303656 | 0 | 15 | 3808 | 304610 | 6 | 0 | 3(8) | 16 |
|  |  |  | Mid | 1542 | 308400 | 1491.06 | 298212 | 102 |  | 3831 | 306510 | 6 | 0 | 3(8) | 220 |
|  |  |  | High | 1551.5 | 310300 | 1355.84 | 271168 | 504 |  | 3854 | 308410 | 6 | 0 | 3(8) | 1024 |
|  |  | Uplink | Low | 1634 | 326800 | 1619.78 | 323956 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1447.84 | 289568 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1653 | 330600 | 1636.62 | 327324 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1535 | 307000 | 1515.92 | 303184 | 0 | 15 | 3800 | 304090 | 14 | 0 | 2(7) | 14 |
|  |  |  | Mid | 1542 | 308400 | 1486.2 | 297240 | 102 |  | 3816 | 305310 | 2 | 0 | 0(5) | 214 |
|  |  |  | High | 1549 | 309800 | 1348.48 | 269696 | 504 |  | 3835 | 306770 | 22 | 0 | 0(5) | 1018 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1617.42 | 323484 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1442.98 | 288596 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1650.5 | 330100 | 1629.26 | 325852 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.2-3: Test frequencies for NR operating band n255 with SCS 60 kHz and SSB SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1530 | 306000 | 1526.04 | 305208 | 0 | 15 | - | 305568 |
|  |  |  | Mid | 1542 | 308400 | 1464.6 | 292920 | 102 |  | - | 307968 |
|  |  |  | High | 1554 | 310800 | 1187.16 | 237432 | 504 |  | - | 310368 |
|  |  | Uplink | Low | 1631.5 | 326300 | 1627.54 | 325508 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1276.66 | 255332 | 504 |  | - | - |
|  |  |  | High | 1655.5 | 331100 | 1647.22 | 329444 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1532.5 | 306500 | 1526.02 | 305204 | 0 | 15 | - | 305564 |
|  |  |  | Mid | 1542 | 308400 | 1462.08 | 292416 | 102 |  | - | 307464 |
|  |  |  | High | 1551.5 | 310300 | 1182.14 | 236428 | 504 |  | - | 309364 |
|  |  | Uplink | Low | 1634 | 326800 | 1627.52 | 325504 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1274.14 | 254828 | 504 |  | - | - |
|  |  |  | High | 1653 | 330600 | 1642.2 | 328440 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1535 | 307000 | 1526.36 | 305272 | 0 | 15 | - | 305632 |
|  |  |  | Mid | 1542 | 308400 | 1459.92 | 291984 | 102 |  | - | 307032 |
|  |  |  | High | 1549 | 309800 | 1177.48 | 235496 | 504 |  | - | 308432 |
|  |  | Uplink | Low | 1636.5 | 327300 | 1627.86 | 325572 | 0 | - | - | - |
|  |  |  | Mid | 1643.5 | 328700 | 1271.98 | 254396 | 504 |  | - | - |
|  |  |  | High | 1650.5 | 330100 | 1637.54 | 327508 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.9.1.3	Reference test frequencies for NR NTN operating band n254

Table 4.3.1.9.1.3-1: Test frequencies for NR operating band n254 with SCS 15 kHz and SSB SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 2486 | 497200 | 2483.75 | 496750 | 0 | 15 | 6215 | 497290 | 12 | 0 | 2(4) | 4 |
|  |  |  | Mid | 2491.75 | 498350 | 2471.14 | 494228 | 102 |  | 6229 | 498290 | 10 | 0 | 0(0) | 102 |
|  |  |  | High | 2497.5 | 499500 | 2404.53 | 480906 | 504 |  | 6243 | 499470 | 8 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1612.5 | 322500 | 1610.25 | 322050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1525.28 | 305056 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1624 | 324800 | 1620.67 | 324134 | 6 |  | - | - | - | - | - | - |
| 5/10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6219 | 497550 | 10 | 7 | 2(4) | 11 |
|  |  |  | Mid | 2491.75 | 498350 | 2468.71 | 493742 | 102 |  | 6229 | 498290 | 4 | 10 | 2(4) | 116 |
|  |  |  | High | 2495 | 499000 | 2399.6 | 479920 | 504 |  | 6233 | 498730 | 6 | 4 | 2(4) | 512 |
|  | 25 | Uplink | Low | 1612.5 | 322500 | 1610.25 | 322050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1525.28 | 305056 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1624 | 324800 | 1620.67 | 324134 | 6 |  | - | - | - | - | - | - |
| 5/15 | 79 | Downlink | Low | 2491 | 498200 | 2483.89 | 496778 | 0 | 15 | 6217 | 497330 | 4 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.75 | 498350 | 2466.28 | 493256 | 102 |  | 6217 | 497330 | 2 | 1 | 0(0) | 103 |
|  |  |  | High | 2492.5 | 498500 | 2394.67 | 478934 | 504 |  | 6220 | 497570 | 8 | 1 | 1(2) | 507 |
|  | 25 | Uplink | Low | 1612.5 | 322500 | 1610.25 | 322050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1525.28 | 305056 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1624 | 324800 | 1620.67 | 324134 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6216 | 497310 | 2 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.75 | 498350 | 2468.71 | 493742 | 102 |  | 6223 | 497810 | 0 | 1 | 0(0) | 103 |
|  |  |  | High | 2495 | 499000 | 2399.6 | 479920 | 504 |  | 6233 | 498730 | 6 | 4 | 2(4) | 512 |
|  |  | Uplink | Low | 1615 | 323000 | 1610.32 | 322064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1522.85 | 304570 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.74 | 323148 | 6 |  | - | - | - | - | - | - |
| 10/15 | 79 | Downlink | Low | 2491 | 498200 | 2483.89 | 496778 | 0 | 15 | 6217 | 497330 | 4 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.75 | 498350 | 2466.28 | 493256 | 102 |  | 6217 | 497330 | 2 | 1 | 0(0) | 103 |
|  |  |  | High | 2492.5 | 498500 | 2394.67 | 478934 | 504 |  | 6220 | 497570 | 8 | 1 | 1(2) | 507 |
|  | 52 | Uplink | Low | 1615 | 323000 | 1610.32 | 322064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1522.85 | 304570 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.74 | 323148 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 2491 | 498200 | 2483.89 | 496778 | 0 | 15 | 6217 | 497330 | 4 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2491.75 | 498350 | 2466.28 | 493256 | 102 |  | 6217 | 497330 | 2 | 1 | 0(0) | 103 |
|  |  |  | High | 2492.5 | 498500 | 2394.67 | 478934 | 504 |  | 6220 | 497570 | 8 | 1 | 1(2) | 507 |
|  |  | Uplink | Low | 1617.5 | 323500 | 1610.39 | 322078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1520.42 | 304084 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1619 | 323800 | 1610.81 | 322162 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.3-2: Test frequencies for NR operating band n254 with SCS 30 kHz and SSB SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 2488.5 | 497700 | 2484.18 | 496836 | 0 | 15 | 6222 | 497790 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 2491.75 | 498350 | 2450.71 | 490142 | 102 |  | 6229 | 498290 | 4 | 0 | 1 (6) | 216 |
|  |  |  | High | 2495 | 499000 | 2309.24 | 461848 | 504 |  | 6236 | 498970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1615 | 323000 | 1610.68 | 322136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1432.49 | 286498 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.02 | 323004 | 6 |  | - | - | - | - | - | - |
| 10/15 | 38 | Downlink | Low | 2491 | 498200 | 2484.16 | 496832 | 0 | 15 | 6220 | 497570 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2491.75 | 498350 | 2448.19 | 489638 | 102 |  | 6223 | 497810 | 12 | 0 | 1 (6) | 216 |
|  |  |  | High | 2492.5 | 498500 | 2304.22 | 460844 | 504 |  | 6223 | 497810 | 10 | 2 | 0 (5) | 1022 |
|  | 24 | Uplink | Low | 1615 | 323000 | 1610.68 | 322136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1432.49 | 286498 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1621.5 | 324300 | 1615.02 | 323004 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 2491 | 498200 | 2484.16 | 496832 | 0 | 15 | 6223 | 497810 | 14 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 2491.75 | 498350 | 2448.19 | 489638 | 102 |  | 6223 | 497810 | 12 | 0 | 1 (6) | 216 |
|  |  |  | High | 2492.5 | 498500 | 2304.22 | 460844 | 504 |  | 6226 | 498050 | 18 | 2 | 0 (5) | 1022 |
|  |  | Uplink | Low | 1617.5 | 323500 | 1610.66 | 322132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1429.97 | 285994 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1619 | 323800 | 1610 | 322000 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: 20/35 MHz UL/DL Bandwidth combination is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.9.1.3-3: Test frequencies for NR operating band n254 with SCS 60 kHz and SSB SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2488.5 | 497700 | 2484.54 | 496908 | 0 | 15 | - | 496908 |
|  |  |  | Mid | 2491.75 | 498350 | 2414.35 | 482870 | 102 |  | - | 497558 |
|  |  |  | High | 2495 | 499000 | 2128.16 | 425632 | 504 |  | - | 498208 |
|  |  | Uplink | Low | 1615 | 323000 | 1611.04 | 322208 | 0 | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1251.41 | 250282 | 504 |  | - | - |
|  |  |  | High | 1621.5 | 324300 | 1613.22 | 322644 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2491 | 498200 | 2484.52 | 496904 | 0 | 15 | - | 496904 |
|  |  |  | Mid | 2491.75 | 498350 | 2411.83 | 482366 | 102 |  | - | 497054 |
|  |  |  | High | 2492.5 | 498500 | 2123.14 | 424628 | 504 |  | - | 497204 |
|  |  | Uplink | Low | 1617.5 | 323500 | 1611.02 | 322204 | 0 | - | - | - |
|  |  |  | Mid | 1618.25 | 323650 | 1248.89 | 249778 | 504 |  | - | - |
|  |  |  | High | 1619 | 323800 | 1608.2 | 321640 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |