Title:	Adding test point selection for NT band n254 for NS_04N and NS_05N A-MPR and A-SE in FR1

Document for:	Discussion and approval

Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_04N and NS_05N. The intention with NS_04N and NS_05N is to protect 1559-1610 MHz and 1628.5-2200 MHz as indicated in Table 1 below.

Table 1: Additional requirements for “NS_04N” and “NS_05N” (Table 6.5.3.3.4-1 from [4])

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f < 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f < 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f < 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f < 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f < 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Discussion

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] for FR1. For NS_04N and NS_05N the operating band n254 (1610-1626.5 MHz) has specific A-MPR requirements, see Table 2.

Table 2: A-MPR regions for “NS_04N” (Table 6.2.3.3-1 from [4])

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |

Table 3: A-MPR regions for “NS_05N” (Table 6.2.3.4-1 from [4])

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 1622.4 < fc <= 1624 | >= 3.6 | > 0 | A3 |
|  |  |  | >= 2.88 | A1 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc <= 1621.5 |  | <= 7.2 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  |  | > 7.2 | > 0 | A6 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

The RB allocation for testing of these A-MPR regions depends on both channel bandwidth and SCS.

Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_04N and NS_05N.

Test Frequencies

Low and High range are selected for operating band n254, since band n254 is located between the bands to be protected.

PROPOSAL 2: Select Low and high Range as test frequency for NS_04N and NS_05N A-MPR tests

Test Channel Bandwidth

The minimum requirements for NS_04N apply for channel bandwidths 5MHz.

The minimum requirements for NS_05N apply for channel bandwidths 5MHz, 10MHz and 15MHz.

PROPOSAL 3: If supported, select 5MHz for NS_04N, select 5MHz, 10MHz, 15MHz channel bandwidth for NS_05N.

Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. Lowest and Highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing.

UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms). But in order to reduce the testing workload, down selection of UL modulation can be considered. The highest modulation type needs to be tested considering the high PAPR feature. But with the highest modulation, MPR might be higher than A-MPR, and in that case A-MPR is not verified properly, therefore the lowest modulation also needs to be tested. For DFT-s-OFDM QPSK is selected instead of BPSK since BPSK is optionally supported.

PROPOSAL 5: Test QPSK and 256QAM for DFT-s-OFDM and CP-OFDM.

Uplink RB allocation

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full. These common uplink configurations are listed in [5].

A-MPR for NS_04N annd NS_05N has some Region specific RB allocations in Table 2 and Table 3, where the ranges for the RB allocation is specified in terms of RBend × 12 × SCS and LCRB × 12 × SCS. Table 4 and Table 5 shows the corresponding RB ranges for 15/30/60 kHz SCS, and proposed allocation expressed as LCRB@RBstart.

Table 4: UL RB allocation for NS_04N

| Ch BW (MHz) | SCS (kHz) | RBstart | LCRB | Prop. Alloc (DFT-s-OFDM) | Prop. Alloc (CP-OFDM) | A-MPR |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 15 | <=2 | <=2 | Edge_1RB_left | Edge_1RB_left | A1 |
|  | 15 |  | >=16 | Outer_Full | Outer_Full | A2 |
|  | 15 |  | >=18 | Outer_Full | Outer_Full | A3 |

Table 5: UL RB allocation for NS_05N

| Ch BW (MHz) | SCS (kHz) | RBstart | LCRB | Prop. Alloc (DFT-s-OFDM) | Prop. Alloc (CP-OFDM) | A-MPR |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 15 | >=20 | >0 | 5@20 | 5@20 | A3 |
|  | 15 |  | >=16 | Outer_Full | Outer_Full | A1 |
| 10 | 15 | <=10 | <=28 | Edge_1RB_left | Edge_1RB_left | A4 |
|  | 30 | <=5 | <=14 | Edge_1RB_left | Edge_1RB_left | A4 |
|  | 60 | <=2 | <=7 | Edge_1RB_left | Edge_1RB_left | A4 |
|  | 15 | <=10 | >28 | Outer_Full | Outer_Full | A5 |
|  | 30 | <=5 | >14 | Outer_Full | Outer_Full | A5 |
|  | 60 | <=2 | >7 | Outer_Full | Outer_Full | A5 |
|  | 15 | >40 | >0 | 10@41 | 11@41 | A6 |
|  | 30 | >20 | >0 | 3@21 | 3@21 | A6 |
|  | 60 | >10 | >0 | 1@10 | 1@10 | A6 |
|  | 15 | >10 | >=16 | 40@11 | 41@11 | A2 |
|  | 30 | >5 | >=8 | 18@6 | 18@6 | A2 |
|  | 60 | >2 | >=4 | 8@3 | 8@3 | A2 |
|  | 15 |  | <=40 | 40@0 | 40@0 | A6 |
|  | 30 |  | <=20 | 20@0 | 20@0 | A6 |
|  | 60 |  | <=10 | 10@0 | 10@0 | A6 |
|  | 15 | <=2 | <=2 | Edge_1RB_left | Edge_1RB_left | A1 |
|  | 30 | <=1 | <=1 | Edge_1RB_left | Edge_1RB_left | A1 |
|  | 60 | <=0 | <=1 | Edge_1RB_left | Edge_1RB_left | A1 |
|  | 15 | >40 | >0 | Edge_1RB_right | Edge_1RB_right | A6 |
|  | 30 | >20 | >0 | Edge_1RB_right | Edge_1RB_right | A6 |
|  | 60 | >10 | >0 | Edge_1RB_right | Edge_1RB_right | A6 |
| 15 | 15 | <=20 | <=28 | Edge_1RB_left | Edge_1RB_left | A4 |
|  | 30 | <=10 | <=14 | Edge_1RB_left | Edge_1RB_left | A4 |
|  | 60 | <=5 | <=7 | Edge_1RB_left | Edge_1RB_left | A4 |
|  | 15 | <=20 | >28 | Outer_Full | Outer_Full | A5 |
|  | 30 | <=10 | >14 | Outer_Full | Outer_Full | A5 |
|  | 60 | <=5 | >7 | Outer_Full | Outer_Full | A5 |
|  | 15 | >58 |  | 20@59 | 20@59 | A6 |
|  | 30 | >29 |  | 8@30 | 8@30 | A6 |
|  | 60 | >14 |  | 3@15 | 3@15 | A6 |
|  | 15 | >20 | >=24 | 24@55 | 24@55 | A2 |
|  | 30 | >10 | >=12 | 12@26 | 12@26 | A2 |
|  | 60 | >5 | >=6 | 6@12 | 6@12 | A2 |

The proposals for LCRB@RBstart in Table 4 and Table 5 are all based on allocating as many RBs as possible located to either lower part or higher part of the channel bandwidth, i.e. select lowest or highest RBstart.

PROPOSAL 6: Select as many RBs as possible close to the band edge.

2.7.	Number of Test Points

Based on the proposals the maximum number of test points for A-MPR tests for NS_04N AND NS_05N is:

Table 6: Numbers of Test Points for A-MPR NS_04N

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_04N | 1 | 2 (L/H) | 1 (5) | 4 | 2*3 | 48 |

Table 7: Numbers of Test Points for A-MPR NS_05N

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_05N | 1 | 2 (L/H) | 3 (5, 10, 15) | 4 | 2*7 | 336 |

Handling of Additional spurious emission

Given the test point analysis of A-MPR for NS_04N and NS_05N has already taken Additional spurious emission requirement into consideration, the test point of A-SE could reuse that of A-MPR as much as possible.

PROPOSAL 7: A-SE for NS_04N and NS_05N reuses the test points selected for A-MPR for NS_04N and NS_05N.

The maximum number of test points for A-SE tests for NS_04N and NS_05N:

Table 8: Numbers of Test Points for A-SE NS_04N

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_04N | 1 | 2 (L/H) | 1 (5) | 4 | 2*3 | 48 |

Table 9: Numbers of Test Points for A-SE NS_05N

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_05N | 1 | 2 (L/H) | 3 (5, 10, 15) | 4 | 2*7 | 336 |

Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_04N and NS_05N A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_04N and NS_05N.

PROPOSAL 2: Select Low and high Range as test frequency for NS_04N and NS_05N A-MPR tests

PROPOSAL 3: If supported, select 5MHz for NS_04N, select 5MHz, 10MHz, 15MHz channel bandwidth for NS_05N.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing.

PROPOSAL 5: Test QPSK and 256QAM for DFT-s-OFDM and CP-OFDM.

PROPOSAL 6: Select as many RBs as possible close to the band edge.

PROPOSAL 7: A-SE for NS_04N and NS_05N reuses the test points selected for A-MPR for NS_04N and NS_05N.

References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-5, V18.7.0 (2024-09)

- 3GPP TS 38.521-1, V18.4.0 (2024-09), Table 6.1-1