3GPP TSG-RAN WG5 Meeting #105	R5-246685r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3055 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update ASEM TC for n41 CBW 5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-9-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ASEM TC needs to be updated for adding n41 CBW 5MHz according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ASEM TC has been updated for adding n41 CBW 5MHz . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.101-1 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Align with TS 38.101-1 i70 version |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct the incorrect meeting number in the coversheet |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.5.2.3	Additional spectrum emission mask

6.5.2.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5.2.3.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.5.2.3.3	Minimum conformance requirements

6.5.2.3.3.1	Minimum requirement for "NS_35"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When " NS_35" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3.3.1-1.

Table 6.5.2.2.3.3.1-1: Additional requirements for “NS_35”

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.3.1.

6.5.2.3.3.2	Requirements for network signalling value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier centre frequency and one above the carrier c frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power. Since the 26-dB emission bandwidth is implementation dependent, the maximum transmission bandwidths in MHz (NRB * SCS * 12 / 1,000) is used for the SEM.

Table 6.5.2.3.3.2-1: n41 maximum transmission bandwidths (MHz) for CP-OFDM

| SCS (kHz) | Channel bandwidths (MHz) / Maximum transmission bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 |  | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 | 4.50 | 9.36 |  | 14.22 | 19.08 | 28.80 | 38.88 | 48.6 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 8.64 |  | 13.68 | 18.36 | 28.08 | 38.16 | 47.88 | 58.32 | 68.04 | 78.12 | 88.02 | 98.28 |
| 60 | N/A | 7.92 |  | 12.96 | 17.28 | 27.36 | 36.72 | 46.8 | 56.88 | 66.96 | 77.04 | 87.12 | 97.20 |

Table 6.5.2.3.3.2-2: n41 maximum transmission bandwidths (MHz) for DFT-S-OFDM

| SCS (kHz) | Channel bandwidths (MHz) / Maximum transmission bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 |  | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 | 4.50 | 9.00 |  | 13.50 | 18.00 | 28.80 | 38.88 | 48.60 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 8.64 |  | 12.96 | 18.00 | 27.00 | 36.00 | 46.08 | 58.32 | 64.80 | 77.76 | 87.48 | 97.20 |
| 60 | N/A | 7.20 |  | 12.96 | 17.28 | 25.92 | 36.00 | 46.08 | 54.00 | 64.80 | 72.00 | 86.40 | 97.20 |

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.2-3.

Table 6.5.2.3.3.2-3: n41 SEM with “NS_04”

| ΔfOOB MHz |  | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| ± 0 - 1 |  | -10 | -10 | -10 | -10 | -10 | -10 |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  |  |  |  | -10 |  |  |  |  | 1 MHz |
| ± 1 - 5 |  |  |  | -10 |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X |  |  |  | -13 |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
|  | NOTE: X is defined in Table 6.5.2.3.3.2-1 for CP-OFDM and 6.5.2.3.3.2-2 for DFT-S-OFDM |  |  |  |  |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.3.2.

<< UNCHANGED PARTS SKIPPED >>

6.5.2.3.5.2	Test requirements (network signalling value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-2 for UE power class 2 or Table 6.2.3.5-3 UE power class 3.

and

-	the power of any UE emission shall fulfil requirements in table 6.5.2.3.5.2-1.

Table 6.5.2.3.5.2-1: Additional test requirements for "NS_04"

| ΔfOOB MHz |  |  |  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 | 40 | 50 |  | 60 | 70 | 80 | 90 | 100 |  |
| ± 0 - 1 |  | -10+TT | -10+TT | -10+TT | -10+TT | -10+TT | -10+TT |  |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  |  |  |  | -10 |  |  |  |  |  | 1 MHz |
| ± 1 - 5 |  |  |  | -10 + TT |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X |  |  |  | -13 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: X is defined in Table 6.5.2.3.3.2-1 for CP-OFDM and 6.5.2.3.3.2-2 for DFT-S-OFDM. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5.2.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< END OF CHANGES >>