3GPP TSG-RAN5 Meeting #105	R5-247015r2

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3084 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test requirement in REFSENS for 3DL CA exceptions testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The REFSENS test requirement for 8 Rx antenna port need to be updated in test case 7.3A.2 and 7.3A.2_1. For reducing the burden of maintenance in the future, the format of table 7.3A.2_1.5-1 and 7.3A.2_1.5-1a could be updated. And NOTE for 8Rx testing is updated in test configuration tables |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating notes in test configuration tables for UE supporting 8R - Updating test requirement in 7.3A.2.5 for 8Rx antenna ports. - Updating the format of table 7.3A.2_1.5-1 and Table 7.3A.2_1.5-1a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirement of REFSENS for 8Rx is absent for 3DL CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2, 7.3A.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8Rx are introduced in R5-247012 (CR 3081) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Merge changes to table 7.3A.2_1.5-1 in R5-246253 from KDDI Merge changes to table 7.3A.2_1.5-1 in R5-247391 from ZTE - As per comments, using “symbol ‘REFSENS2Rx’ + MSD” value as the test requirements for victim bands for easier maintenance in the future. And keep the original numbers of notes in test requirement tables, meanwhile VOID unnecessary notes and introducing new notes below them. R2：remove typos in NOTEs in table 7.3A.2_1.5-1a |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 7.3A.2	Reference sensitivity power level for 3DL CA without exceptions

### <Unchanged Skipped>

7.3A.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.2A.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2.4.1-1: Test Configuration Table for 3DL CA without exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For Inter-band CA: CA_nXA-nYA-nZA: Mid range for PCC and SCC with exceptions (Note 11): CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA :Low range, High Range for nXC and nXB, mid range for nYA for PCC and SCC with exceptions : CA configurations containing the following band combinations: CA_n1-n77: Mid in band n1 and Low in band n77 CA_n2-n77: UL and DL Mid in band n2 and band n77 at 3850MHz CA_n3-n77: TBD in band 3 and TBD in band 77. CA_n3-n78: Mid in band 3 and High in band 78. CA_n8-nX: Low range for PCC in Band 8 CA_n70-n71: High range for PCC in band 71 |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “NRB_PCC”, “NRB_SCC1” and “NRB_SCC2” columns |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0 to NOTE 5) |  |  |
|  | CA configuration |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | Low CC1 | N/A | nX | Low CC2 | N/A | nX | Low CC3 | Highest NRB_agg | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nX | High CC1 | N/A | nX | High CC2 | N/A | nX | High CC3 | Highest NRB_agg | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0 to NOTE 5) |  |  |
|  | CA configuration |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nY | default | N/A | nZ | default | Highest (NOTE 12) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nZ | default | N/A | nX | default | Highest (NOTE 12) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nY | default | N/A | nX | default | Highest (NOTE 12) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nX | default | N/A | nY | default | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX | default | N/A | nX | default | Highest (NOTE 12) | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0 to NOTE 5) |  |  |
|  | CA configuration |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | nY | Mid | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | Mid | NA | nX | CC1 | Max (NOTE 7) | nX | CC2 | Highest (NOTE 12) | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nX(3A) Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | Max (NOTE 7) | nX | CC3 | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nX(A-C) and CA_nX(A-B) Configuration (Intra-band contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | nX | CC3 | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2.0: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: Inter-band: nX,nY,nZ correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, nX=n1, nY=n3, nZ=n8. Note 4: Intra-band contiguous + Inter-band: nX, nY correspond to the different bands in the CA Configuration, e.g. for CA_n1C-n3A, nX=n1, nY=n3 Note 5: Intra-band non-contiguous + Inter-band: nX and nY correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, nX=n1, nY =n8. Note 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 7: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration for Intra-band non-contiguous Note 8: For band combinations including operating bands without uplink band (as noted in Table 5.2-1), only the CA configurations where PCC band has uplink band shall be tested Note 9: The fallback configuration CA_nXA-nYA for 3CA configurations CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA does not need to be tested even if the test frequency differs Note 10: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 11: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. Note 12: Each of Highest UL and Highest DL shall be selected according to clause 5.5A.3.2. DL channel bandwidth shall be selected first. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <Unchanged Skipped>

7.3A.2.5	Test requirement

For 3DL carrier aggregation, test parameters are specified in table 7.3A.2.4.1-1. For the CA configurations listed in table 7.3A.2.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1a and table 7.3.2.5-1b for each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2a and 7.3.2.5-2b for each non-SDL carrier for 4 Rx antenna port, in table 7.3.2.5-2e and table 7.3.2.5-2f for non-SDL carrier for 8 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

### <Unchanged Skipped>

### 7.3A.2_1	Reference sensitivity power level for 3DL CA exceptions

### <Unchanged Skipped>

7.3A.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n77 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Unchanged rows skipped |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n71A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 693/ DL 647 | n78 | 3546 | n66 | DL 2160 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <Unchanged Skipped>

7.3A.2_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 ans A.2.3 with parameters specified in Table 7.3A.2_1.5-1 for PC3 and Table 7.3A.2_1.5-1a for PC2.

Table 7.3A.2_1.5-1: Reference sensitivity requirement for PC3 interband 3DL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 7, Note 8) |
| --- | --- | --- | --- | --- |
| DL_n1A-n3A-n77A_UL_n1A-n3A | 1 | n1 | 5 | REFSENS |
|  |  | n3 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +28.4 |
| DL_n1A-n3A-n77A_UL_n3A-n77A | 1 | n1 | 5 | REFSENS2Rx +31.0 |
|  |  | n3 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n1A-n3A-n78A_UL_n1A-n3A | 1 | n1 | 5 | REFSENS |
|  |  | n3 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +28.4 |
|  | 2 | n1 | 5 | REFSENS |
|  |  | n3 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +11.2 |
| DL_n1A-n3A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REFSENS |
|  |  | n3 | 5 | REFSENS2Rx +27.9 |
|  |  | n78 | 10 | REFSENS |
| DL_n1A-n5A-n78A_UL_n5A-n78A | 1 | n1 | 5 | REFSENS2Rx +18.1 |
|  |  | n5 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS |
| DL_n1A-n5A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS2Rx +3.1 |
|  |  | n78 | 10 | REFSENS |
| DL_n1A-n5A-n78A_UL_n1A-n5A | 1 | n1 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +15.7 |
| DL_n1A-n8A-n78A_UL_n1A-n8A | 1 | n1 | 5 | REFSENS |
|  |  | n8 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +14.9 |
| DL_n1A-n8A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REFSENS |
|  |  | n8 | 5 | REFSENS2Rx +3.3 |
|  |  | n78 | 10 | REFSENS |
| DL_n1A-n28A-n78A_UL_n1A-n28A | 1 | n1 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +15.7 |
| DL_n1A-n28A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS2Rx +4.2 |
|  |  | n78 | 10 | REFSENS |
| DL_n1A-n28A-n78A_UL_n28A-n78A | 1 | n1 | 5 | REFSENS2Rx +15.7 |
|  |  | n28 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS |
| DL_n1A-n28A-n77A_UL_n28A-n77A | 1 | n1 | 5 | REFSENS2Rx +15.7 |
|  |  | n28 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n1A-n41A-n77A_UL_n1A-n41A | 1 | n1 | 5 | REFSENS |
|  |  | n41 | 10 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +19.6 |
| DL_n1A-n41A-n77A_UL_n41A-n77A | 1 | n1 | 5 | REFSENS2Rx +9.3 |
|  |  | n41 | 10 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n2A-n5A-n48A_UL_n5A-n48A | 1 | n2 | 5 | REFSENS2Rx +15.6 |
|  |  | n5 | 5 | REFSENS |
|  |  | n48 | 5 | REFSENS |
| DL_n2A-n5A-n48A_UL_n2A-n5A | 1 | n2 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS |
|  |  | n48 | 5 | REFSENS2Rx +16.6 |
| DL_n2A-n5A-n66A_UL_n2A-n5A | 1 | n2 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +7.2 |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS2Rx +3.8 |
|  |  | n77 | 10 | REFSENS |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 5 | REFSENS2Rx +16.5 |
|  |  | n5 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n2A-n5A-n77A_UL_n2A-n5A | 1 | n2 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +16.0 |
| DL_n2A-n48A-n66A_UL_n2A-n66A | 1 | n2 | 5 | REFSENS |
|  |  | n48 | 5 | REFSENS2Rx +32 |
|  |  | n66 | 5 | REFSENS |
| DL_n2A-n48A-n66A_UL_n2A-n48A | 1 | n2 | 5 | REFSENS |
|  |  | n48 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +12.1 |
| DL_n2A-n48A-n66A_UL_n48A-n66A | 1 | n2 | 5 | REFSENS2Rx +28.3 |
|  |  | n48 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS |
| DL_n2A-n66A-n77A_UL_n2A-n66A | 1 | n2 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +29.4 |
|  | 2 | n2 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +8.9 |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +29.2 |
|  |  | n77 | 10 | REFSENS |
|  | 2 | n2 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +10.4 |
|  |  | n77 | 10 | REFSENS |
|  | 3 | n2 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +4.0 |
|  |  | n77 | 10 | REFSENS |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 5 | REFSENS2Rx +32.1 |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
|  | 2 | n2 | 5 | REFSENS2Rx +9.1 |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
|  | 3 | n2 | 5 | REFSENS2Rx +2.1 |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n3A-n5A-n78A_UL_n3A-n5A | 1 | n3 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +16.1 |
|  | 2 | n3 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +4.5 |
| DL_n3A-n5A-n78A_UL_n5A-n78A | 1 | n3 | 5 | REFSENS2Rx + +15.7 |
|  |  | n5 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS |
| DL_n3A-n8A-n78A_UL_n3A-n8A | 1 | n3 | 5 | REFSENS |
|  |  | n8 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +16.1 |
|  | 2 | n3 | 5 | REFSENS |
|  |  | n8 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +4.5 |
| DL_n3A-n8A-n78A_UL_n8A-n78A | 1 | n3 | 5 | REFSENS2Rx +15.7 |
|  |  | n8 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS |
| DL_n3A-n28A-n78A_UL_n3A-n28A | 1 | n3 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +17.3 |
|  | 2 | n3 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +4.5 |
| DL_n3A-n28A-n41A_UL_n3A-n28A | 1 | n3 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS |
|  |  | n41 | 5 | REFSENS2Rx +27.4 |
|  | 2 | n3 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS |
|  |  | n41 | 5 | REFSENS2Rx +15.9 |
| DL_n3A-n28A-n41A_UL_n3A-n41A | 1 | n3 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS2Rx +26.0 |
|  |  | n41 | 5 | REFSENS |
| DL_n3A-n28A-n41A_UL_n28A-n41A | 1 | n3 | 5 | REFSENS2Rx +26.0 |
|  |  | n28 | 5 | REFSENS |
|  |  | n41 | 10 | REFSENS |
| DL_n3A-n28A-n77A_UL_n3A-n28A | 1 | n3 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx 15.9 |
| DL_n3A-n28A-n77A_UL_n28A-n77A | 1 | n3 | 5 | REFSENS2Rx +17.0 |
|  |  | n28 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n3A-n28A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REFSENS |
|  |  | n28 | 5 | REFSENS2Rx +15.3 |
|  |  | n77 | 10 | REFSENS |
| DL_n3A-n40A-n77A_UL_n3A-n40A | 1 | n3 | 5 | REFSENS |
|  |  | n40 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx + 19.0 |
| DL_n3A-n40A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REFSENS |
|  |  | n40 | 5 | REFSENS2Rx + 29.4 |
|  |  | n77 | 10 | REFSENS |
| DL_n3A-n40A-n77A_UL_n40A-n77A | 1 | n3 | 5 | REFSENS2Rx + 29.9 |
|  |  | n40 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n3A-n41A-n77A_UL_n3A-n41A | 1 | n3 | 5 | REFSENS |
|  |  | n41 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +16.8 |
| DL_n3A-n41A-n77A_UL_n41A-n77A | 1 | n3 | 5 | REFSENS2Rx +16.4 |
|  |  | n41 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n3A-n41A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REFSENS |
|  |  | n41 | 5 | REFSENS2Rx +5.3 |
|  |  | n77 | 10 | REFSENS |
| DL_n5A-n48A-n66A_UL_n5A-n66A | 1 | n5 | 5 | REFSENS |
|  |  | n48 | 5 | REFSENS2Rx +3.6 |
|  |  | n66 | 5 | REFSENS |
| DL_n5A-n66A-n77A_UL_n5A-n66A | 1 | n5 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +16.1 |
|  | 2 | n5 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +8.2 |
|  | 3 | n5 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +3.3 |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +13.2 |
|  |  | n77 | 10 | REFSENS |
| DL_n28A-n41A-n77A_UL_n28A-n41A | 1 | n28 | 5 | REFSENS |
|  |  | n41 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +28.2 |
| DL_n28A-n41A-n77A_UL_n28A-n77A | 1 | n28 | 5 | REFSENS |
|  |  | n41 | 5 | REFSENS2Rx +29.5 |
|  |  | n77 | 10 | REFSENS |
| DL_n28A-n41A-n77A_UL_n41A-n77A | 1 | n28 | 5 | REFSENS2Rx +30.8 |
|  |  | n41 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
|  | 2 | n28 | 5 | REFSENS2Rx +3.0 |
|  |  | n41 | 10 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n28A-n41A-n79A_UL_n41A-n79A | 1 | n28 | 5 | REFSENS2Rx +13 |
|  |  | n41 | 10 | REFSENS |
|  |  | n79 | 40 | REFSENS |
| DL_n28A-n41A-n79A_UL_n28A-n41A | 1 | n28 | 5 | REFSENS |
|  |  | n41 | 10 | REFSENS |
|  |  | n79 | 40 | REFSENS2Rx +10.1 |
| DL_n28A-n41A-n79A_UL_n28A-n79A | 1 | n28 | 5 | REFSENS |
|  |  | n41 | 10 | REFSENS2Rx +10.4 |
|  |  | n79 | 40 | REFSENS |
| DL_n66A-n71A-n77A_UL_n66A-n71A | 1 | n66 | 5 | REFSENS |
|  |  | n71 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS2Rx +15.9 |
| DL_n66A-n71A-n77A_UL_n66A-n77A | 1 | n66 | 5 | REFSENS |
|  |  | n71 | 5 | REFSENS2Rx +15.3 |
|  |  | n77 | 10 | REFSENS |
| DL_n66A-n71A-n78A_UL_n66A-n71A | 1 | n66 | 5 | REFSENS |
|  |  | n71 | 5 | REFSENS |
|  |  | n78 | 10 | REFSENS2Rx +9 |
| DL_n66A-n71A-n78A_UL_n71A-n78A | 1 | n66 | 5 | REFSENS2Rx +15.5 |
|  |  | n71 | 5 | REFSENS |
|  |  |  |  | n78 |  | 10 |  | REFSENS |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Void Note 5: Void Note 6: Void Note 7: The symbol “REFSENS2Rx” in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b. The requirements in this column are applicable for 2Rx, 4Rx or 8Rx antenna ports NR band in the CA configuration. Note 8: The symbol "REFSENS" in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b for 4 antenna port NR band, Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8 antenna port NR band. |  |  |  |  |

Table 7.3A.2_1.5-1a: Reference sensitivity requirement for PC2 interband 3DL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 5, Note 6) |
| --- | --- | --- | --- | --- |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REFSENS |
|  |  | n5 | 5 | REFSENS2Rx +13.6 |
|  |  | n77 | 10 | REFSENS |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 5 | REFSENS2Rx +24.8 |
|  |  | n5 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +34.7 |
|  |  | n77 | 10 | REFSENS |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 5 | REFSENS2Rx +37.6 |
|  |  | n66 | 5 | REFSENS |
|  |  | n77 | 10 | REFSENS |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 5 | REFSENS |
|  |  | n66 | 5 | REFSENS2Rx +22.2 |
|  |  | n77 | 10 | REFSENS |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Void Note 5: The symbol “REFSENS2Rx” in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b. The requirements in this column are applicable for 2Rx, 4Rx or 8Rx antenna ports NR band in the CA configuration. Note 6: The symbol "REFSENS" in this table refers to the reference sensitivity values (including TT) for single carrier specified in Table 7.3.2.5-1a and Table 7.3.2.5-1b for 2 antenna port NR band, Table 7.3.2.5-2a and Table 7.3.2.5-2b for 4 antenna port NR band, Table 7.3.2.5-2e and Table 7.3.2.5-2f for 8 antenna port NR band. |  |  |  |  |

### <End of Changes>